3GPP TSG RAN WG1 Meeting #101-e                                                                                   R1-2003296

E-meeting, May 25th – June 5th, 2020

Agenda Item:	8.2.2

Source:	Huawei, HiSilicon

Title:	Performance evaluation for Rel-17 positioning

Document for:	Discussion and Decision

Introduction

In RAN#86, Rel-17 NR positioning SI was approved in [1], with the exemplary performance targets of

- Sub-meter level accuracy and 100ms latency for general commercial use cases

- Accuracy <0.2m and latency in the order of 10ms for IIoT use cases

The objective of the SI for RAN1 includes the following:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)): - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

This contribution presents the simulation results with Rel-16 positioning techniques for the IIoT scenario specified in [2] so as to identify the performance gaps.

Scenarios for evaluations

Five sub-scenarios are defined in TR 38.901 [3], while only four of them are calibrated for large scale parameters. The calibrated scenarios and simulation assumption are as follows:

Table 1 calibrated scenarios assumption in 38.901

| Parameter | Values |
| --- | --- |
| Scenario | InF-SL(Sparse-clutter, Low BS), InF-DL(Dense-clutter, Low BS), InF-SH(Sparse-clutter, High BS), InF-DH(Dense-clutter, High BS) |
| Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| Sectorization | None |
| BS antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| UT antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| Handover margin (for calibration) | 0dB |
| BS deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH |
| UT distribution | uniform dropping for indoor with minimum 2D distance of 1 m UT height = 1.5 m |
| UT attachment | Based on pathloss |
| UT noise figure | 9 dB |
| Carrier frequency | 3.5 GHz, 28 GHz |
| Bandwidth | 100 MHz |
| Clutter density: r | Low clutter density: 20% High clutter density: 60% |
| Clutter height: hc | Low clutter density: 2 m High clutter density: 6 m |
| Clutter size: dclutter | Low clutter density: 10 m High clutter density: 2 m |

Firstly, the LOS probability of all the above sub-scenarios are evaluated based on the calibration parameters. As shown in Figure 1 below:

Figure 1 The distribution of the number of LOS BS

As discussed in our companion contribution [2], scenario InF-SH is prioritized for position accuracy evaluation as shown in section 3.

Evaluated cases

The following cases in Table 2 are evaluated for FR1. We will evaluate the impact of the bandwidth, precoding algorithm and super resolution measurement algorithm. Other parameters can be found in the Appendix A.

We used SVD as the upper bound to show the impact from precoding, and identify the performance gap between the upper bound and the DFT codebook based or single port codebook based precoding.

We also used 200MHz bandwidth to show potential benefit of further extending PRS bandwidth to improve the positioning accuracy, and identify gap from using the existing maximum bandwidth of FR1, i.e. 100MHz.

Table 2 Evaluated parameter combinations

| Case | Fc/BW/SCS | Precoding assumptions | Measurement Algorithm | BS number used for positioning |
| --- | --- | --- | --- | --- |
| Case1 | {4GHz, 100MHz, 30kHz} | Single port | Regular | 7 |
| Case2 | {4GHz, 100MHz, 30kHz} | Single port | MUSIC | 7 |
| Case3 | {4GHz, 200MHz, 60kHz} | Single port | Regular | 7 |
| Case4 | {4GHz, 200MHz, 60kHz} | Single port | MUSIC | 7 |
| Case5 | {4GHz, 100MHz, 30kHz} | SVD | Regular | 7 |
| Case6 | {4GHz, 100MHz, 30kHz} | SVD | MUSIC | 7 |
| Case7 | {4GHz, 200MHz, 60kHz} | SVD | Regular | 7 |
| Case8 | {4GHz, 200MHz, 60kHz} | SVD | MUSIC | 7 |
| Case9 | {4GHz, 100MHz, 30kHz} | SimpleCodebook16 | Regular | 7 |
| Case10 | {4GHz, 100MHz, 30kHz} | SimpleCodebook16 | MUSIC | 7 |
| Case11 | {4GHz, 200MHz, 60kHz} | SimpleCodebook16 | Regular | 7 |
| Case12 | {4GHz, 200MHz, 60kHz} | SimpleCodebook16 | MUSIC | 7 |

Simulation result

The positioning error of 50%, 67%, 80%, 90% and 95% for all cases are summarized in Table 3. The detailed simulation results can be found in Figure 2, Figure 3 and Figure 4 in the Appendix B.

Table 3 CDF of DL-TDOA positioning accuracy (in unit of meter)

|  | Case | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | Case1 {100MHz, SP, Regular} | 1.050 | 1.631 | 2.692 | 5.364 | 10.049 |
|  | Case2 {100MHz, SP, Music} | 0.123 | 0.304 | 1.200 | 5.441 | 12.855 |
|  | Case3 {200MHz, SP, Regular} | 0.542 | 0.802 | 1.468 | 3.259 | 7.095 |
|  | Case4 {200MHz, SP, Music} | 0.035 | 0.091 | 0.450 | 2.925 | 7.628 |
|  | Case5 {100MHz, SVD, Regular} | 0.962 | 1.307 | 1.881 | 3.234 | 5.013 |
|  | Case6 {100MHz, SVD, Music} | 0.050 | 0.078 | 0.136 | 0.295 | 0.598 |
|  | Case7 {200MHz, SVD, Regular} | 0.500 | 0.668 | 1.014 | 1.835 | 3.002 |
|  | Case8 {200MHz, SVD, Music} | 0.020 | 0.031 | 0.051 | 0.112 | 0.248 |
|  | Case9 {100MHz, CB16, Regular} | 0.968 | 1.364 | 2.024 | 3.380 | 5.321 |
|  | Case10 {100MHz, CB16, Music} | 0.059 | 0.104 | 0.220 | 0.584 | 1.727 |
|  | Case11 {200MHz, CB16, Regular} | 0.502 | 0.692 | 1.062 | 1.906 | 3.230 |
|  | Case12 {200MHz, CB16, Music} | 0.062 | 0.103 | 0.200 | 0.487 | 1.349 |

It is observed that the upper bound performance using SVD and 200MHz bandwidth among cases is 0.11m@90% (case8).

Comparing with bandwidths (100MHz, and 200MHz), we have the following observation.

Observation 1: Bandwidth is still an important factor affecting positioning accuracy.

- When the bandwidth is doubled, the corresponding positioning error is reduced by almost half.

Comparing with precoding algorithms (single port precoder, DFT precoder, and SVD precoder), we have the following observation.

Observation 2: The beamforming of the transmitted signal can further improve the positioning accuracy, but the gain is limited for non-MUSIC algorithm (case 1, case 5, and case 9).

Comparing with TOA estimate algorithms (regular, and MUSIC), we have the following observation.

Observation 3: Super resolution algorithms (e.g., MUSIC) can greatly improve positioning accuracy at least for beamformed procoder (case 5 and case 6, case 9 and case 10).

Overall, using precoding of single port or DFT precoder with 100MHz bandwidth, we have the following observation for achievable positioning accuracy using Rel-16 DL-TDOA positioning method.

Observation 4: Rel-16 DL-TDOA with 100MHz@FR1 can achieve 0.58m@90% in case10. The performance gap to the target performance [0.2-0.5m]@90% is derived accordingly.

Conclusion

Based on the simulation results and the discussion, we have the following observations:

Observation 1: Bandwidth is still an important factor affecting positioning accuracy.

- When the bandwidth is doubled, the corresponding positioning error is reduced by almost half.

Observation 2: The beamforming of the transmitted signal can further improve the positioning accuracy, but the gain is limited for non-MUSIC algorithm (case 1, case 5, and case 9).

Observation 3: Super resolution algorithms (e.g., MUSIC) can greatly improve positioning accuracy at least for beamformed procoder (case 5 and case 6, case 9 and case 10).

Observation 4: Rel-16 DL-TDOA with 100MHz@FR1 can achieve 0.58m@90% in case10. The performance gap to the target performance [0.2-0.5m]@90% is derived accordingly.

- References

- 3GPP RP-193237, New SID on NR Positioning Enhancements, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019

- 3GPP R1-2003295, Discussion on scenarios and evaluation methodology for Rel-17 positioning Huawei, HiSilicon, RAN1#101-e, 25th May – 5th June, 2020.

- 3GPP TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- Appendix A

The common parameters can be found in Table 4. The other unlisted deployment parameters can refer to Table 1.

Table 4

Table 4 Common parameter

| Parameter | value |
| --- | --- |
| Channel model (baseline, otherwise state any modifications) | 38.901 |
| gNB antenna configuration | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ– Note 1 |
| gNB antenna radiation pattern | Single sector– Note 1 |
| Total gNB TX power, dBm | 24dBm |
| UE drop procedure | 100% indoor, uniformly distributed over the horizontal area |
| Min gNB-UE distance (2D), m | 1m |
| UE antenna configuration | Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) – Note 1 |
| UE antenna radiation pattern | Omni, 0dBi |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) (reference to figure in contribution) | Comb-6 |
| Reference signal | PRS: Gold, 1-port |
| Number of symbols used per occasion | 14 |
| number of occasions used per positioning estimate | 4 |
| Power-boosting level | 7.8dB |
| Uplink power control (applied/not applied) | Not applied |
| interference modelling (ideal muting, or other) | Ideal muting |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, taylor series, etc) | OTDOA, 2WLS |
| Network synchronization assumptions | No sync error |
| Note 1: According to 3GPP TR 38.802. |  |

- Appendix B

The detailed simulation results are as follows:

Figure 2 The CDF for positioning error when precoding assumption is Single port

Figure 3 The CDF for positioning error when precoding assumption is SVD

Figure 4 The CDF for positioning error when precoding assumption is Codebook