3GPP TSG RAN WG1 #101	R1-2003314

e-Meeting, May 25th – June 5th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining details of QoS management for sidelink

Agenda item:	7.2.4.6

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss remaining RAN1 aspects of QoS management and sidelink congestion control.

# Discussion

## Sidelink Congestion Control

### CBR-based power control in resource allocation mode 1

For the LTE V2X sidelink, CBR-based power control was only applied in resource allocation mode 3. The current specification in TS 38.213 on the other hand applies CBR-based power control regardless of resource allocation mode. So far there has not been any explicit discussion on this deviation from LTE behaviour. Supporting CBR-based power control in resource allocation mode 1 provides an additional tool for congestion control; moreover, if its use is not desired, it can be disabled by setting maximumtransmitPower-SL to its maximum value for all values of PPPP.

Proposal 1: CBR-based power control is applied in both resource allocation modes.

No text proposal is needed since this is anyway the behaviour currently specified in TS 38.213.

### CR – how to deal with unused resources

One issue that has been raised in some contributions is how resources which have been reserved but then released due to HARQ feedback or have been pre-empted should be counted in channel occupancy ratio (CR).

This aspect is not directly covered by the existing agreements that LTE CR is the baseline, since the LTE sidelink did not support HARQ feedback. A second case of unused resources is when resources have been reserved but then pre-empted.

The current CR definition in TS 38.215 counts resources “used for … transmissions in slots [n-a, n-1] and granted in slots [n, n+b]”. Hence, for the past segment of the evaluation window, resources which have been reserved but released or pre-empted are not counted, since they have not been used for transmission.

Proposal 2: For CR evaluation, past resources which had been reserved, but then released due to HARQ feedback or pre-empted, are not counted.

No text proposal is needed since this is anyway the behaviour currently specified in TS 38.215.

For the future segment of the evaluation window, the current definition counts all the resources reserved for retransmissions, even if the UE already knows at the time of CR evaluation that, due to received HARQ feedback or pre-emption, it will not use the reserved resources.

There are two points of view on this issue:

On the one hand, one can argue that, for the future segment of the evaluation window, CR is supposed to capture the expected resource occupancy as closely as possible. From that point of view, resources which the UE knows will not be occupied, should not be counted as occupied.

On the other hand, it can be argued that an incentive against excessive resource reservation is beneficial; e.g. in the case of HARQ feedback, if for a unicast link the initial TX of a TB is almost always successful, but the UE always reserves 2 retransmissions then one can argue that the UE is greedy and should be penalized by counting the reserved, but almost always unused, resources in CR.

Moreover, modifying the behaviour requires a change to the specification. So one question to address is if the expected benefit is worth the effort. Given that the CR window is at least of length 1000 slots, while the reserved but unused resource could be at most 32 slots away (ignoring distinction between physical slots and logical slots),  it would seem that the impact of the reserved but unused resources in the future window on the value of CR will, in most cases, not be significant. Moreover, it is up to the UE if it even includes future slots in its evaluation window.

Proposal 3: For CR evaluation, do not amend the specification to support that future resources which had been reserved, but then released due to HARQ feedback or pre-empted, are not counted.

# Conclusions

In this contribution we discussed the remaining RAN1 aspects of QoS management and sidelink congestion control and make the following observations and proposals:

Proposal 1: CBR-based power control is applied in both resource allocation modes.

Proposal 2: For CR evaluation, past resources which had been reserved, but then released due to HARQ feedback or pre-empted, are not counted.

Proposal 3: For CR evaluation, do not amend the specification to support that future resources which had been reserved, but then released due to HARQ feedback or pre-empted, are not counted.

# References

- RP-200129, “Revised WID: 5G V2X with NR sidelink”

- TR 37.885, Study on evaluation methodology of new Vehicle-to-Everything V2X use cases for LTE and NR

- TR 38.885, Study on NR Vehicle-to-Everything (V2X)

# Background

## WI Objectives

At RAN#83, a new work item “5G V2X with NR sidelink” (5G_V2X_NRSL) was approved ‎[1]. Two of the objectives are relevant for the present agenda item:

1. NR sidelink: Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage. - … - Congestion control [RAN1, RAN2] 4. Specify support for QoS management [RAN2, RAN3, RAN1]

## Earlier Agreements

The following relevant agreements have been reached in previous meetings:

### QoS

Agreements: From RAN1 perspective, at least the following QoS-related parameters relevant to physical layer studies are considered: - Priority - latency - reliability

Agreements: RAN1 studies further how to use - priority, - latency, - reliability, - minimum required communication range (as defined by higher layers) if agreed to use in the physical layer aspects of at least - resource allocation and - congestion control and - resolution of in-device coexistence issues and - power control

In the Sidelink resource allocation mode 2 agenda item, the following working assumption was reached:

Working assumption: - An indication of a priority of a sidelink transmission is carried by SCI payload - This indication is used for sensing and resource (re)selection procedures - This priority is not necessarily the higher layer priority

Agreements: - For the priority indication in 1st stage SCI: - Up to RAN2 on how to define the mapping between the priority indication and the corresponding QoS - Size is 3 bits (as a working assumption)

### Sidelink Congestion Control

Agreements: - Introduce at least one congestion metric for NR sidelink - FFS details – to be done in WI phase (if included)

Agreements: - Congestion control is supported at least for sidelink mode 2 - Note: details of congestion control can be covered in the work item phase, not in this SI.

Conclusion: - It is deemed beneficial to report Sidelink Congestion Metrics(s) to a gNB - Consequently, it is recommended to specify the corresponding details in the WI phase

Agreements: Support at least NR CBR as congestion metric for NR sidelink congestion control. - LTE CBR is the baseline for defining NR CBR.

Agreements: - LTE V2X sidelink congestion control is the starting point for defining NR sidelink congestion control.

Agreements: - Higher-layer reporting of CBR to the gNB is supported for RRC_CONNECTED UEs.

Agreements: - For PSCCH/PSSCH multiplexing option 3, one CBR measurement over a resource pool is defined. - PSFCH resources, if (pre)configured, are excluded from this CBR measurement.

Agreements: Define NR sidelink Channel Occupancy Ratio (CR) measurement. - LTE CR is the baselines

Agreements: - Congestion control can restrict the values of at least the following PSSCH/PSCCH TX parameters per resource pool: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions – already agreed in mode 2 AI - Upper bound of TX power (including zero TX power) - Congestion control can set an upper bound on channel occupancy ratio (CR), CRlimit. - Ranges/bounds of the transmission parameters and CRlimit are functions of QoS and CBR. - In addition to congestion control (in use or not in use), the above parameters can be restricted by reusing the same mechanism as in LTE - For speed, further discussion on absolute vs. relative speed - FFS other parameter(s) that can be restricted - FFS whether or not to tie the speed with a UE capability

Agreements: Lookup table links CBR range with values of the transmission parameters and CRlimit for each value of the indication of a priority of a sidelink transmission carried by SCI payload (as per WA from RAN1#98), Lookup table is (pre)configured. Details up to RAN2. - Up to 16 (as a working assumption) CBR ranges are supported - The working assumption will be automatically confirmed in RAN1#99 if no further input

Agreements: - Sidelink RSSI (SL-RSSI) measurement is used for CBR estimation

Agreements: A sidelink resource is busy for the purpose of CBR measurement if Sidelink RSSI measured by the UE in that resource exceeds a (pre-)configured threshold.

Agreements: The CBR measurement time window size is 100 ms and 100 slots by (pre-)configuration. CR window size is { 1000 ms, 1000 slots } by (pre)-configuration

Agreement: - The future segment of the CR evaluation window reuses the same behaviour as in the LTE V2X sidelink. - FFS whether additional constraints on UE’s choice of values for a and b are needed

Agreement: For the constraints on past/future window in CR evaluation: - n+b shall not exceed the last transmission opportunity of the grant for the current transmission - b >= 0 - b < (a+b+1)/2 Notes: - in the first bullet point above, LTE’s “should” has been replaced by “shall”

Agreement: - UE evaluates CR and applies CR_limit for every (re)transmission.

Agreement: - The CBR processing time is given by UE capability according to the following table µ; Congestion process time 1 (slots); Congestion processing time 2 (slots). 0; 2; 2. 1; 2; 4. 2; 4; 8. 3; 8; 16. - A UE shall only apply a single CBR/CR processing time capability in SL. - CR processing time is the same as CBR processing time.

Agreement: - The slot index in the definition of CBR is the physical slot index. Agreement: - The slot index in the definition of CR is the physical slot index.

### TX Parameter Restrictions

Agreements: - Only TX parameter restriction based on absolute speed can be (pre)configured in Rel-16.

Agreement: - In addition to congestion control (in use or not in use), the following PSSCH/PSCCH TX parameters per resource pool can be restricted by reusing the same mechanism as in LTE: - Range of MCS for a given MCS table supported within the resource pool - Range of number of sub-channels - Upper bound of number of (re)transmissions Note: This reverts the agreement made in RAN1#98b, which included “Upper bound of TX power” in the set of TX parameters that can be restricted using this mechanism.