3GPP TSG RAN WG1 #101	R1-2003315

e-Meeting, May 25th – June 5th, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining details of NR Uu controlling LTE sidelink

Agenda item:	7.2.4.7

Document for:	Discussion and Decision

# Introduction

In this contribution we discuss remaining RAN1 aspects of NR Uu controlling LTE sidelink.

# Discussion

## SL index field in DCI format 3-1

According to TS 38.212, one of the fields in NR DCI format 3-1 is “SL index”:

-	SL index – 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

TS 36.212 in turn specifies the following for  “SL index”:

-	SL index – 2 bits as defined in clause 14.2.1 of [3] (this field is present only for cases with TDD operation with uplink-downlink configuration 0-6).

From the wording in 38.212 it is ambiguous if SL index is unconditionally present in DCI format 3-1, or present only “for cases with TDD operation with uplink-downlink configuration 0-6”. This ambiguity arises because there are two plausible readings of the text in 38.212:

- The field “SL index” of length 2 bits is unconditionally present, the reference to 36.212 is just about the meaning of the field.

- The reference to 36.212 is for both the presence condition and meaning of the field.

We have an earlier agreement which implies that “SL index” may not be present; so the intention that “SL index” may be absent or, alternatively, may be of size 0 bits should be made clear.

Looking at the description of other DCI fields in TS 38.212 it seems that in general the range of the number of bits for the field is stated explicitly; e.g. for DCI format 0_1:

-	Carrier indicator – 0 or 3 bits, as defined in Clause 10.1 of [5, TS38.213].

To avoid any ambiguity on presence of SL index we have the following proposal:

Proposal 1: Adopt the following text proposal on field SL index in NR DCI format 3-1:

----------------------------------------------------begin text proposal for 38.212-----------------------------------------------

-	SL index – 0 or 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

----------------------------------------------------end text proposal for 38.212-----------------------------------------------

## Open loop power control of LTE SL based on DL pathloss

From the RAN4 point of view, LTE sidelink is only supported in band 47. Some recent RAN4 LS responses to RAN2 illustrate this:

- R4-1912874: RAN4 respond to RAN2 that the answer to “whether NR Uu can share the same frequency with LTE SL” is “this case is not possible to share same frequency”.

- R4-1915985: RAN4 inform RAN2 that “LTE-SL/NR-UL is not valid under licensed band since LTE-SL is only allowed in ITS band in LTE V2X”.

It follows that in practice LTE SL TX power does not need to be controlled to protect NR UL or LTE UL.

On the other hand, there are some arguments in favour of introducing LTE SL open loop power control based on DL pathloss:

- In Rel-14, RAN1 specified V2X sidelink to support both shared carrier (LTE SL shares LTE UL carrier) and dedicated carrier scenario scenarios. Open loop power control for LTE SL based on DL pathloss from an LTE serving cell is supported in TS 36.213. The specification could be considered incomplete if the case of an NR serving cell is not included.

- If in the future it was decided to fully support LTE V2X sidelink in shared carrier scenarios then, provided RAN1 have completed the specification for this scenario in Rel-16, RAN1 may not need to get involved in the future work.

Regarding the second argument, a counterargument is that supporting NR UL + LTE SL in a shared carrier scenario would anyway require additional work not confined to just RAN4. Aspects such as cross-RAT UL/SL prioritization will have to be considered. RAN1 have considered this in the past and, in LS R1-1911720, provided the following response to RAN2:

RAN1 has so far not considered such operating scenario where UL and SL transmissions from different RAT are in the shared/same carrier, and RAN1 has no plan to discuss this in Rel-16.

Hence it appears that, even if we agree OLPC based on DL pathloss, specification support for the scenario of NR UL + LTE SL in shared carrier would be incomplete. For this reason we prefer to ensure that, if the cross-RAT shared carrier scenario ever is supported, the related work include all aspects, not just RAN4. Hence we don’t find that argument of restricting any future work to RAN4 compelling.

Overall, we are not in favour of specifying open loop power control of LTE SL based on NR DL pathloss. If, however, this feature is specified then we don’t see a reason to treat mode 3 and mode 4 differently.

Proposal 2: In Rel-16, do not specify open loop power control of LTE SL based on NR DL pathloss.

Proposal 3: If open loop power control of LTE SL based on NR DL pathloss is specified in Rel-16 then it shall include both mode 3 and mode 4.

# Conclusions

In this contribution we discussed remaining aspects of NR Uu control of the LTE V2X sidelink and make the following observations and proposals:

Proposal 1: Adopt the following text proposal on field SL index in NR DCI format 3-1:

----------------------------------------------------begin text proposal for 38.212-----------------------------------------------

-	SL index – 0 or 2 bits as defined in 5.3.3.1.9A of [11, TS 36.212]

----------------------------------------------------end text proposal for 38.212-----------------------------------------------

Proposal 2: In Rel-16, do not specify open loop power control of LTE SL based on NR DL pathloss.

Proposal 3: If open loop power control of LTE SL based on NR DL pathloss is specified in Rel-16 then it shall include both mode 3 and mode 4.

# References

- RP-200129, “Revised WID: 5G V2X with NR sidelink”

# Background

## WID Objectives

At RAN#83, a new work item “5G V2X with NR sidelink” (5G_V2X_NRSL) was approved. The WID has recently been revised ‎[1]. One of the objectives deals with the cross-RAT scenario of control of the LTE sidelink by NR Uu:

2. Specify support for NR Uu to provide control for LTE sidelink - Sidelink mode 4 as per the study outcome [RAN2, RAN1]; and - Sidelink mode 3-like RRC-configured SPS scheduling with DCI-based activation/deactivation as per the agreement in RAN1#97 [RAN1, RAN2].

## Previous agreements

The following agreements have been reached in previous meetings:

Agreements: - RRC-based activation/deactivation is not supported - DCI-based activation/deactivation is supported - Support of LTE PC5 scheduling by NR Uu (mode 3-like ) is based on UE capability - NR DCI provides the fields of DCI 5A in LTE-V that are related to SPS scheduling - The size of DCI for activation/deactivation is one of the DCI size(s) that will be defined for NR Uu scheduling NR V2V - FFS whether the DCI format is the same as one of the DCI formats that will be defined for NR Uu scheduling NR V2V - Activation/deactivation applies to the first LTE subframe after Z+X ms after receiving the DCI - Z is the same timing offset in current LTE V2X specs - X>0. FFS value(s) of X, and if one or multiple values of X are possible Draft LS to RAN to be prepared in R1-1907826 (Phillippe, Huawei), which is approved with the following update: for LTE sidelink for mode 3 with final LS in R1-1907909.

Agreements: - A new RNTI is introduced to scramble the NR DCI used for scheduling LTE PC5.

Agreements: - X is dynamically indicated using a field in the DCI - FFS whether the DCI field provides an index to a table or the value of X - The minimum value of X is subject to UE capability - UE reports a single value subject to UE capability

Agreements: - The NR DCI field to indicate X provides an index to a table of values - The table of values is configurable, and has 8 values - The size of the DCI field is fixed at 3 bits

Agreements: - Use a separate PDCCH monitoring configuration (as configured in Rel-15) for NR DCI scheduling LTE SL - The per-CC and across-CC blind decoding budget and the maximum number of non-overlapped CCEs for channel estimation are not increased. - The per-CC and across-CC maximum number of search spaces is not increased. - The per-CC and across-CC maximum number of CORESETs is not increased - When in the same slot, there is both PDCCH monitoring for Uu SL and PDCCH monitoring for SL for the same CC, the search space(s) for LTE SL is configured to be the same or a subset of those for Uu for the same CC or vice versa

Agreements: - The minimum value of X signalled in the UE capability is one of the values (excluding spare values) that can be signalled in DCI 3_1

Agreements: The supported values of X signaled in the DCI are: - 0.75ms, 1ms, [1.25ms], [1.5ms], 2ms, 4ms, 5ms, 8ms, 10ms, 20 ms - Additional value(s) can be discussed during the Feb. meeting - Spare values are reserved for future deployments

Agreement: - For NR Uu scheduling LTE sidelink, the subframe of the first sidelink transmission is the first SL subframe of the corresponding resource pool that starts not earlier than TDCI-NTA2×TC+X+4+m ms, where TDCI is the start timing of the slot carrying DCI format 3_1, NTA and Tc are defined in TS 38.211, X is the value indicated by Timing offset field in DCI format 3_1, m is the value indicated by SL index in DCI format 3_1 if SL index is present, otherwise m=0.

Agreements:  If UE is configured to monitor DCI 3_0, the sizes of DCI 3_1 and DCI 3_0 are is aligned by zero padding  If UE is not configured to monitor DCI 3_0, the mechanism for size alignment between DCI 3_0 and Uu DCI is reused for size alignment between DCI 3_1 and Uu DCI

Agreement: the set of possible values for X is: 0ms, 0.25ms, 0.5ms, 0.625ms, 0.75ms, 1ms, 1.25ms, 1.5ms,1.75ms, 2ms, 2.5ms, 3ms, 4ms, 5ms, 6ms, 8ms, 10ms, 20 ms

Conclusion: - Regarding the proposal of “For NR Uu scheduling LTE sidelink with NR Uu and LTE SL on the same band, open loop power control based on the LTE PSSCH power control formula is reused with DL pathloss from the gNB” - No consensus, re-discuss in May as needed - Regarding the proposal of “For NR Uu scheduling LTE sidelink with LTE Uu and LTE SL on the same band, open loop power control based on the LTE PSSCH power control formula is reused with DL pathloss from the eNB - Not agreed - No consensus to address open loop power control for LTE V2X mode-4 resource allocation