3GPP TSG RAN WG1 #101                                                                                     R1-2003338

e-Meeting, May 25th – June 5th, 2020

Source:	ZTE

Title:                 Discussion on baseline coverage performance for FR1

Agenda item:	8.8.1.1

Document for: Discussion and Decision

Introduction

In the RAN#86 meeting, a new SID on NR coverage enhancement was approved [1]. The objective of this study item is to identify the performance target for coverage enhancement for specific scenarios for both FR1 and FR2 and study the potential solutions for both DL and UL. The scenarios include urban scenario and rural scenario for FR1 as well as indoor scenario and urban/suburban scenario for FR2.

In this contribution, the baseline coverage performance for PUSCH, PUCCH, PDCCH, PBCH and PRACH in FR1 are provided and discussed.

Baseline performance for PUSCH

In this section, we provide the baseline performance (i.e. SNR from LLS) for PUSCH carrying eMBB services and VoIP in urban scenario and rural scenario for FR1.

2.1 PUSCH for eMBB services

2.1.1 Simulation scenarios

For eMBB service in FR1, rural scenario, rural with long distance and urban scenario are considered in the SID. Considering the typical carrier frequency and the path loss model for each scenario, the following 7 simulation scenarios as given in Table 2-1 are evaluated in this contribution. Note that, the main difference between O-to-I and O-to-O is the UE speed, which is 3km/h for O-to-I case and 120 km/h for O-to-O case.

Table 2-1 Simulation scenarios for PUSCH for eMBB services

| Scenario 1-1 | Rural scenario_700MHz_O-to-O |
| --- | --- |
| Scenario 1-2 | Rural scenario_2GHz_O-to-O |
| Scenario 1-3 | Rural scenario_2GHz_O-to-I |
| Scenario 1-4 | Rural scenario_4GHz_O-to-O |
| Scenario 1-5 | Rural scenario_4GHz_O-to-I |
| Scenario 1-6 | Long distance rural scenario_700MHz_O-to-O |
| Scenario 1-7 | Urban scenario_4GHz_O-to-I |

2.1.2 Simulation assumptions

For PUSCH with eMBB services, the transmission parameter, e.g., TBS, MCS, and number of RBs etc., can be derived from target data rate, which is 1 Mbps for urban scenario and 100 Kbps for rural scenario respectively in FR1 as suggested in the SID. More specifically, the TBS is equal to {Target data rate * PUSCH time duration} for FDD. For TDD, a scaling factor T is applied to account for the impact of UL duty cycle. Take configuration 'DDDSUDDSUU' and ‘S’ with 10:2:2 for instance, only T=30% or 32.857% time is available for PUSCH transmission w/ or w/o considering the uplink symbols in 'S' slot. Then, the TBS of TDD should be {Target data rata * PUSCH time duration /T} for TDD. In case of repetitions, the time duration of PUSCH should include all PUSCH repetitions. Based on the TBS, the MCS can be easily determined based on the number of RBs and DMRS overhead.

The simulation cases for rural scenario with FDD, rural scenario with TDD and urban scenario with TDD are provided in Table 2-2, 2-3 and 2-4 respectively. The other simulation assumptions are provided in Table A-1 in the appendix.

Table 2-2 Simulation cases for rural scenario with (FDD, 700MHz/2GHz, 14-symbol PUSCH, one DMRS per hop, SCS = 15 kHz, Rank =1, O-to-I or O-to-O)

| Target data rate = 100kbps |  |  |  |
| --- | --- | --- | --- |
| Simulation cases | # of RB | MCS (Target SE) | Actual data rate/ kbps |
| Case 1 | 1 | MCS 5 (0.7402) | 106.5888 |
| Case 2 | 2 | MCS 2(0.3770) | 108.5760 |
| Case 3 | 3 | MCS 0(0.2344) | 101.2608 |

Table 2-3 Simulation cases for rural scenario with (TDD 'DDDSUDDSUU' and ‘S’:10:2:2, 4 GHz, 14-symbol PUSCH, one DMRS per hop, SCS = 30 kHz, Rank =1, O-to-I or O-to-O, T=30%)

| Target data rate = 100kbps |  |  |  |
| --- | --- | --- | --- |
| Simulation cases | # of RB | MCS (Target SE) | Actual data rate/ kbps |
| Case 1 | 1 | MCS 8(1.1758) | 101.5891 |
| Case 2 | 2 | MCS 4(0.6016) | 103.9565 |
| Case 3 | 3 | MCS 3(0.4902) | 127.0598 |
| Case 4 | 4 | MCS 1(0.3066) | 105.9610 |
| Case 5 | 5 | MCS 0(0.2344) | 101.2608 |

Table 2-4 Simulation cases for urban scenario with (TDD 'DDDSUDDSUU' and ‘S’:10:2:2, 14-symbol PUSCH, one DMRS without hopping, SCS = 30 kHz, O-to-I, T=30%)

| Target data rate = 1 Mbps |  |  |  |
| --- | --- | --- | --- |
| Simulation cases | # of RB | MCS (Target SE) | Actual data rate/ Mbps |
| Case 0 | 1 | NA | NA |
| Case 1 | 2 | MCS 27 (5.5547) | 1.0398 |
| Case 2 | 3 | MCS 20 (3.6094) | 1.0135 |
| Case 3 | 4 | MCS 17 (2.7305) | 1.0223 |
| Case 4 | 5 | MCS 14 (2.1602) | 1.011 |
| Case 5 | 6 | MCS 13 (1.9141) | 1.075 |
| Case 7 | 8 | MCS 11 (1.4766) | 1.1057 |
| Case 8 | 9 | MCS 9 (1.3262) | 1.1172 |
| Case 9 | 10 | MCS 8 (1.1758) | 1.1005 |
| Case 10 | 12 | MCS 7 (1.0273) | 1.1539 |
| Case 11 | 15 | MCS 5 (0.7402) | 1.0392 |
| Case 12 | 18 | MCS 4 (0.6016) | 1.0136 |
| Case 13 | 24 | MCS 3 (0.4902) | 1.1012 |
| Case 14 | 30 | MCS 2 (0.3770) | 1.0586 |
| Case 15 | 36 | MCS 1 (0.3066) | 1.0331 |
| Case 16 | 48 | MCS 0 (0.2344) | 1.0531 |

2.1.3 Simulation results

- Scenario 1-1, Rural scenario_700MHz_O-to-O

For scenario 1-1, the baseline performance for different simulation cases with 2 Rx, 4 Rx and 8 Rx are provided in Table 2-5. To get a fair comparison among different cases, the results with power normalization to 1 RB case are calculated. As it can be observed, the performance of the case with (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, (2 RBs, MCS#2) has the best performance for 8Rx.

Table 2-5 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-1, Rural scenario_700MHz_O-to-O}.

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 RB, MCS#5 | 2.53 dB | 2.53 dB | -1.61 dB | -1.61 dB | -5.36 dB | -5.36 dB |
| 2 RBs, MCS#2 | -0.94 dB | 2.07 dB | -4.94 dB | -1.93 dB | -8.58 dB | -5.57 dB |
| 3 RBs, MCS#0 | -3.33 dB | 1.44 dB | -6.94 dB | -2.17 dB | -10.03 dB | -5.25 dB |

Observation 1: For PUSCH with 100kbps eMBB service in scenario 1-1 (rural scenario, 700 MHz, O-to-O), the case (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, and (2RBs, MCS#2) has the best performance for 8Rx. The corresponding baseline performance is given in Table 2-5.

- Scenario 1-2, Rural scenario_2GHz_O-to-O

For scenario 1-2, the baseline performance for different simulation cases with 2 Rx, 4 Rx and 8 Rx are provided in Table 2-6. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, (2RBs, MCS#2) has the best performance for 8Rx.

Table 2-6 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-2, Rural scenario_2GHz_O-to-O}.

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 RB, MCS#5 | 2.70 dB | 2.70 dB | -1.76 dB | -1.76 dB | -5.04 dB | -5.04dB |
| 2 RBs, MCS#2 | -0.9 dB | 2.11dB | -4.93 dB | -1.92 dB | -8.36 dB | -5.35 dB |
| 3 RBs, MCS#0 | -3.31 dB | 1.46 dB | -6.89 dB | -2.12 dB | -9.91 dB | -5.13 dB |

Observation 2: For PUSCH with 100kbps eMBB service in scenario 1-2 (rural scenario, 2GHz, O-to-O), the case (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, and (2RBs, MCS#2) has the best performance for 8Rx. The corresponding baseline performance is given in Table 2-6.

- Scenario 1-3, Rural scenario_2GHz_O-to-I

For scenario 1-3, the baseline performance for different simulation cases with 2 Rx, 4 Rx and 8 Rx are provided in Table 2-7. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, (2RBs, MCS#2) has the best performance for 8Rx.

Table 2-7 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-3, Rural scenario_2GHz_O-to-I} .

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 RB, MCS#5 | 2.77 dB | 2.77 dB | -1.55 dB | -1.55 dB | -6.09 dB | -6.09 dB |
| 2 RBs, MCS#2 | -0.67 dB | 2.34 dB | -4.83 dB | -1.82 dB | -9.15 dB | -6.14 dB |
| 3 RBs, MCS#0 | -3.16 dB | 1.61 dB | -6.9 dB | -2.13 dB | -10.49 dB | -5.72 dB |

Observation 3: For PUSCH with 100kbps eMBB service in scenario 1-3 (rural scenario, 2GHz, O-to-I), the case (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, and (2RBs, MCS#2) has the best performance for 8Rx. The corresponding baseline performance is given in Table 2-7.

- Scenario 1-4, Rural scenario_4GHz_O-to-O

For scenario 1-4, the baseline performance for different simulation cases with 2 Rx, 4 Rx and 8 Rx are provided in Table 2-8. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (5 RBs, MCS#0) has the best performance for 2Rx, (4 RBs, MCS#1) has the best performance for 4Rx, (2RBs, MCS#4) has the best performance for 8Rx.

Table 2-8 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-4, Rural scenario_4GHz_O-to-O}.

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 RB, MCS#8 | 8.15 dB | 8.15 dB | 1.91 dB | 1.91 dB | -2.29 dB | -2.29 dB |
| 2 RBs, MCS#4 | 2.12 dB | 5.13 dB | -3.01 dB | 0 dB | -6.27 dB | -3.26 dB |
| 3 RBs, MCS#3 | 1.17 dB | 5.94 dB | -3.43 dB | 1.34 dB | -6.68 dB | -1.91 dB |
| 4 RBs, MCS#1 | -1.37 dB | 4.65 dB | -6.06 dB | -0.04 dB | -9.26 dB | -3.24 dB |
| 5 RBs, MCS#0 | -3.16 dB | 3.83 dB | -6.79 dB | 0.2 dB | -9.89 dB | -2.9 dB |

Observation 4: For PUSCH with 100kbps eMBB service in scenario 1-4 (rural scenario, 4GHz, O-to-O), the case (5 RBs, MCS#0) or (4RBs, MCS#1) or (2RBs, MCS#4) has the best performance for 2Rx or 4Rx or 8 Rx respectively. The corresponding baseline performance for 2Rx/4Rx/8Rx is given in Table 2-8.

- Scenario 1-5, Rural scenario_4GHz_O-to-I

For scenario 1-5, the baseline performance for different simulation cases with 2 Rx, 4 Rx and 8 Rx are provided in Table 2-9. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (5 RBs, MCS#0) has the best performance for 2Rx, (4 RBs, MCS#1) has the best performance for 4Rx, (2RBs, MCS#4) has the best performance for 8Rx.

Table 2-9 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-5, Rural scenario_4GHz_O-to-I}.

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 RB, MCS#8 | 6.02 dB | 6.02 dB | 1.16 dB | 1.16 dB | -3.29 dB | -3.29 dB |
| 2 RBs, MCS#4 | 1.78 dB | 4.79 dB | -3.15 dB | 0.14 dB | -6.62 dB | -3.61 dB |
| 3 RBs, MCS#3 | 0.82 dB | 5.59 dB | -3.72 dB | 1.05 dB | -7.44 dB | -2.67 dB |
| 4 RBs, MCS#1 | -1.31 dB | 4.71 dB | -6.12 dB | -0.1 dB | -9.61 dB | -3.59 dB |
| 5 RBs, MCS#0 | -3.13 dB | 3.86 dB | -7.02 dB | -0.03 dB | -10.97 dB | -3.58 dB |

Observation 5: For PUSCH with 100kbps eMBB service in scenario 1-5 (rural scenario, 4GHz, O-to-I), the case (5 RBs, MCS#0) or (4RBs, MCS#1) or (2RBs, MCS#4) has the the best performance for 2Rx or 4Rx or 8 Rx respectively. The corresponding baseline performance for 2Rx/4Rx/8Rx is given in Table 2-9.

- Scenario 1-6, Long distance rural scenario_700MHz_O-to-O

For scenario 1-6, the baseline performance for different simulation cases with 2 Rx, 4 Rx and 8 Rx are provided in Table 2-10. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (3 RBs, MCS#0) has the best performance.

Table 2-10 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-6, Long distance rural scenario_700MHz_O-to-O}.

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 RB, MCS#5 | -0.32 dB | -0.32 dB | -3.37 dB | -3.37 dB | -6.28 dB | -6.28 dB |
| 2 RBs, MCS#2 | -3.44 dB | -0.43 dB | -6.41 dB | -3.40 dB | -9.31 dB | -6.30 dB |
| 3 RBs, MCS#0 | -6.02 dB | -1.25 dB | -9.08 dB | -4.31 dB | -11.34 dB | -6.57 dB |

Observation 6: For PUSCH with 100kbps eMBB service in scenario 1-6 (long distance rural scenario, 700MHz, O-to-O), the case (3 RBs, MCS#0) has the best performance. The corresponding baseline performance for 2Rx/4Rx/8Rx is given in Table 2-10.

- Scenario 1-7, Urban scenario_4GHz_O-to-I

For scenario 1-7, the baseline performance for different simulation cases with 4 Rx, 8 Rx and 64 Rx are provided in Table 2-11. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with 24 RBs/MCS#3 has the best performance for 4Rx and 8Rx, 15 RBs/MCS#5 has the best performance for 64Rx.

Table 2-11 Baseline performance for PUSCH at 10% iBLER for {Scenario 1-7, Urban scenario_4GHz_O-to-I}.

| Simulation case | 4RX (w/o power normalization) | 4RX (w/ power normalization) | 8RX (w/o power normalization) | 8RX (w/ power normalization) | 64RX (w/o power normalization) | 64RX (w/ power normalization) |
| --- | --- | --- | --- | --- | --- | --- |
| 12 RBs, MCS#7 | -1.88 dB | 8.91 dB | -6.08 dB | 4.71 dB | -13.6 dB | -2.8 dB |
| 15 RBs, MCS#5 | -4.33 dB | 7.43 dB | -6.91 dB | 4.85 dB | -15.01 dB | -3.25 dB |
| 18 RBs, MCS#4 | -4.97 dB | 7.59 dB | -7.84 dB | 4.71 dB | -15.35 dB | -2.8 dB |
| 24 RBs, MCS#3 | -6.53 dB | 7.27 dB | -9.2 dB | 4.6 dB | -16.53 dB | -2.73 dB |
| 30 RBs, MCS#2 | -7.39 dB | 7.38 dB | -9.55 dB | 5.22 dB | -16.74 dB | -1.97 dB |

Observation 7: For PUSCH with 1Mbps eMBB service in scenario 1-7 (urban scenario, 4GHz, O-to-I), the case (24 RBs,MCS#3) has the best performance for 4Rx and 8Rx, and (15RBs, MCS#5) has the best performance for 64Rx. The corresponding baseline performance for 4Rx/8Rx/64Rx is given in Table 2-11.

2.2 PUSCH for VoIP services

2.2.1 Simulation scenarios

For VoIP service in FR1, rural scenario and rural with long distance are considered in the SID. Considering the typical carrier frequency and the path loss model for each scenario, the following 4 simulation scenarios as given in Table 2-12 are evaluated in this contribution.

Table 2-12 Simulation scenarios for PUSCH for VoIP services

| Scenario 2-1 | Rural scenario_700MHz_O-to-O |
| --- | --- |
| Scenario 2-2 | Rural scenario_2GHz_O-to-O |
| Scenario 2-3 | Long distance Rural scenario_700MHz_O-to-O |
| Scenario 2-4 | Urban scenario_4GHz_O-to-I |

2.2.2 Simulation assumptions

According to TR 25.912 [2], the encoder frame length of VoIP is 20ms with a total of voice payload on air interface of 320 bits (without RLC segmentation). The performance of VoIP can be improved by repetitions/re-transmissions as long as the repetitions/re-transmissions do not exceed the encoder frame length of 20 ms.

In the following simulation, 2% rBLER for VoIP is assumed, maximum number of re-transmissions is 3 for repetition number 1 and 4, and maximum number of re-transmission is 1 for repetition number 8. The time interval between two (re)-transmissions is one slot. The other simulation assumptions are provided in appendix A-1.

2.2.3 Simulation results

- Scenario 2-1, Rural scenario_700MHz_O-to-O

For scenario 2-1, the baseline performance for different simulation cases with 2 Rx are provided in Table 2-13. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (10 RBs, MCS#0) has the best performance. Therefore, the results of 4 or 8 repetitions based on (10 RBs, MCS#0) are further provided.

Table 2-13 Baseline performance for PUSCH at 2% rBLER for {Scenario 2-1, Rural scenario_700MHz_O-to-O}.

| Simulation case | # of repetitions | Max. # of re-transmissions | 2RX (w/o power normalization) | 2RX (w/ power normalization) |
| --- | --- | --- | --- | --- |
| 1 RB, MCS#15 | 1 | 3 | 8.48 dB | 8.48 dB |
| 2 RBs, MCS#8 | 1 | 3 | 2.39 dB | 5.4 dB |
| 3 RBs, MCS#6 | 1 | 3 | 0.44 dB | 5.21 dB |
| 4 RBs, MCS#4 | 1 | 3 | -1.7 dB | 4.32 dB |
| 5 RBs, MCS#3 | 1 | 3 | -2.73 dB | 4.26 dB |
| 6 RBs, MCS#2 | 1 | 3 | -3.5 dB | 4.28 dB |
| 8 RBs, MCS#1 | 1 | 3 | -4.73 dB | 4.3 dB |
| 10 RBs, MCS#0 | 1 | 3 | -6.17 dB | 3.83 dB |
| 10 RBs, MCS#0 | 4 | 3 | -11.46 dB | -1.46 dB |
| 10 RBs, MCS#0 | 8 | 1 | -12.54 dB | -2.54 dB |

Observation 8: For PUSCH with VoIP service in scenario 2-1 (rural scenario, 700MHz, O-to-O), the baseline performance for 2 Rx is given in Table 2-13.

  - For cases with one repetition, (10 RBs, MCS#0) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 5.29 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 6.37 dB gain over one repetition with max 3 re-transmissions.

- Scenario 2-2, Rural scenario_2GHz_O-to-O

For scenario 2-2, the baseline performance for different simulation cases with 2 Rx are provided in Table 2-14. To get a fair comparison among different cases with one repetition, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (10 RBs, MCS#0) has the best performance. Therefore, the results of 4 or 8 repetitions based on (10 RBs, MCS#0) are further provided.

Table 2-14 Baseline performance for PUSCH at 2% rBLER for {Scenario 2-2, Rural scenario_2GHz_O-to-O}.

| Simulation case | # of repetitions | Max. # of re-transmissions | 2RX (w/o power normalization) | 2RX (w/ power normalization) |
| --- | --- | --- | --- | --- |
| 1 RB, MCS#15 | 1 | 3 | 8.81 dB | 8.81 dB |
| 2 RBs, MCS#8 | 1 | 3 | 2.48 dB | 5.49 dB |
| 3 RBs, MCS#6 | 1 | 3 | 0.42 dB | 5.19 dB |
| 4 RBs, MCS#4 | 1 | 3 | -1.74 dB | 4.28 dB |
| 5 RBs, MCS#3 | 1 | 3 | -2.95 dB | 4.04 dB |
| 6 RBs, MCS#2 | 1 | 3 | -3.59 dB | 4.19 dB |
| 8 RBs, MCS#1 | 1 | 3 | -4.82 dB | 4.21 dB |
| 10 RBs, MCS#0 | 1 | 3 | -6.24 dB | 3.76 dB |
| 10 RBs, MCS#0 | 4 | 3 | -12.12 dB | -2.12 dB |
| 10 RBs, MCS#0 | 8 | 1 | -13.64 dB | -3.64 dB |

Observation 9: For PUSCH with VoIP service in scenario 2-2 (rural scenario, 2GHz, O-to-O), the baseline performance for 2 Rx is given in Table 2-14.

  - For cases with one repetition, (10 RBs, MCS#0) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 5.88 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 7.4 dB gain than over repetition with max 3 re-transmissions.

- Scenario 2-3, Long distance Rural scenario_700MHz_O-to-O

For scenario 2-3, the baseline performance for different simulation cases with 2 Rx are provided in Table 2-15. To get a fair comparison among different cases with one repetition, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (6 RBs, MCS#2) has the best performance. Therefore, the results of 4 or 8 repetitions based on (6 RBs, MCS#2) are further provided.

Table 2-15 Baseline performance for PUSCH at 2% rBLER for {Long distance rural scenario_700MHz_O-to-O}.

| Simulation case | # of repetitions | Max. # of re-transmissions | 2RX (w/o power normalization) | 2RX (w/ power normalization) |
| --- | --- | --- | --- | --- |
| 1RBs, MCS#15 | 1 | 3 | 5.97 dB | 5.97 dB |
| 2RBs, MCS#8 | 1 | 3 | -0.03 dB | 2.98 dB |
| 3RBs, MCS#6 | 1 | 3 | -1.59 dB | 3.19 dB |
| 4RBs, MCS#4 | 1 | 3 | -3.24 dB | 2.78 dB |
| 5RBs, MCS#3 | 1 | 3 | -4.57 dB | 2.42 dB |
| 6RBs, MCS#2 | 1 | 3 | -5.47 dB | 2.31 dB |
| 8RBs, MCS#1 | 1 | 3 | -6.23 dB | 2.8 dB |
| 10RBs, MCS#0 | 1 | 3 | -7.62 dB | 2.38 dB |
| 6RBs, MCS#2 | 4 | 3 | -10.82 dB | -3.04 dB |
| 6RBs, MCS#2 | 8 | 1 | -12.81 dB | -5.03 dB |

Observation 10: For PUSCH with VoIP service in scenario 2-3 (long distance rural scenario, 700 MHz, O-to-O), the baseline performance for 2 Rx is given in Table 2-15.

  - For cases with one repetition, (6RBs, MCS#2) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 5.35 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 7.34 dB gain over one repetition with max 3 re-transmissions.

- Scenario 2-4, Urban scenario_4GHz_O-to-I

For scenario 2-4, the baseline performance for different simulation cases with 4 Rx are provided in Table 2-16. To get a fair comparison among different cases with one repetition, the results with power normalization to 1 RB case are also provided. As it can be observed, the performance of the case with (10 RBs, MCS#0) has the best performance. Therefore, the results of 4 or 8 repetitions based on (10 RBs, MCS#0) are further provided.

Table 2-16 Baseline performance for PUSCH at 2% rBLER for {Urban scenario_4GHz_O-to-I}.

| Simulation case | # of repetitions | Max. # of re-transmissions | 4RX (w/o power normalization) | 4RX (w/ power normalization) |
| --- | --- | --- | --- | --- |
| 4RBs, MCS#4 | 1 | 3 | -3.98 dB | 2.04 dB |
| 10RBs, MCS#0 | 1 | 3 | -8.05 dB | 1.95 dB |
| 10RBs, MCS#0 | 4 | 3 | -12.29 dB | -2.29 dB |
| 10RBs, MCS#0 | 8 | 1 | -14.75 dB | -4.75 dB |

Observation 11: For PUSCH with VoIP service in scenario 2-4 (urban scenario, 2GHz, O-to-I), the baseline performance for 4 Rx is given in Table 2-16.

  - For cases with one repetition, (10RBs, MCS#0) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 4.07 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 6.7 dB gain over one repetition with max 3 re-transmissions.

Baseline performance for PUCCH

In this section, we provide baseline PUCCH results for urban scenario and rural scenario for FR1.

  - Simulation scenarios

Similar to PUSCH, the following simulation scenarios for PUCCH listed in Table 3-1 are evaluated in this section.

Table 3-1 Simulation scenarios for PUCCH

| Scenario 3-1 | Rural scenario_700MHz_O-to-O |
| --- | --- |
| Scenario 3-2 | Rural scenario_2GHz_O-to-O |
| Scenario 3-3 | Rural scenario_2GHz_O-to-I |
| Scenario 3-4 | Rural scenario_4GHz_O-to-O |
| Scenario 3-5 | Long distance rural scenario_700MHz_O-to-O |
| Scenario 3-6 | Rural/Urban scenario_4GHz_O-to-I |

  - Simulation assumptions

For PUCCH, 14-symbol long PUCCH format 1 with UCI of 1 or 2 bits and 14-symbol long PUCCH format 3 with UCI of 3, 6, 11 and 22 bits are evaluated in this section. As for the target BLER, it is set to DTX to ACK probability 1% and NACK to ACK probability of  0.1%  and ACK missed detection probability of  1% for 1~2 bits, and block error probability of 1% for more than 2 bits.

More detailed simulation assumptions are attached in Table A-2.

  - Simulation results

- Scenario 3-1, Rural scenario_700MHz_O-to-O

For scenario 3-1, the baseline performance for different cases is provided in Table 3-2.

Table 3-2 Baseline performance for PUCCH for {Scenario 3-1, Rural scenario_700MHz_O-to-O}

| Simulation cases | 2Rx | 4Rx |
| --- | --- | --- |
| PUCCH format 1 1bit | -7.2 dB | - |
| PUCCH format 1 2bits | -6 dB | -10 dB |
| PUCCH format 3 3bits | -6.5 dB | -10.5 dB |
| PUCCH format 3 6bits | -5 dB | -9.3 dB |
| PUCCH format 3 11bits | -3.2 dB | -7.7 dB |
| PUCCH format 3 22bits | -0.8 dB | -5.6 dB |

Observation 12: For PUCCH in scenario 3-1 (Rural scenario, 700MHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx is given in Table 3-2.

- Scenario 3-2, Rural scenario_2GHz_O-to-O

For scenario 3-2, the baseline performance for different cases is provided in Table 3-3.

Table 3-3 Baseline performance for PUCCH for {Scenario 3-2, Rural scenario_2GHz_O-to-O}.

| Simulation cases | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- |
| PUCCH format 1 1bit | -7.6 dB | -11.8 dB | - |
| PUCCH format 1 2bits | -6.0 dB | -10.3 dB | -13.62 dB |
| PUCCH format 3 3bits | -6.6 dB | -10.3 dB | -13.40 dB |
| PUCCH format 3 6bits | -5.2 dB | -9.2 dB | -12.18 dB |
| PUCCH format 3 11bits | -3.2 dB | -7.7 dB | -10.84 dB |
| PUCCH format 3 22bits | -1.0 dB | -5.7 dB | -8.85 dB |

Observation 13: For PUCCH in scenario 3-2 (Rural scenario, 2GHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx is given in Table 3-3.

- Scenario 3-3, Rural scenario_2GHz_O-to-I

For scenario 3-3, the baseline performance for different cases is provided in Table 3-4.

Table 3-4 Baseline performance for PUCCH for {Scenario 3-4, Rural scenario_2GHz_O-to-I}.

| Simulation cases | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- |
| PUCCH format 1 1bit | -7.5 dB | -11.7 dB | - |
| PUCCH format 1 2bits | -6.1 dB | -10.3 dB | -13.6 dB |
| PUCCH format 3 3bits | -6.4 dB | -10.3 dB | -13.43 dB |
| PUCCH format 3 6bits | -5.2 dB | -9.2 dB | -12.19 dB |
| PUCCH format 3 11bits | -3.4 dB | -7.6 dB | -10.83 dB |
| PUCCH format 3 22bits | -1.2 dB | -5.6 dB | -8.92 dB |

Observation 14: For scenario 3-3 (Rural scenario, 2GHz, O-to-I), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx is given in Table 3-4.

- Scenario 3-4, Rural scenario_4GHz_O-to-O

For scenario 3-4, the baseline performance for different cases is provided in Table 3-5.

Table 3-5 Baseline performance for PUCCH for {Scenario 3-4, Rural scenario_4GHz_O-to-O}.

| Simulation cases | 2Rx | 4Rx | 8Rx |
| --- | --- | --- | --- |
| PUCCH format 1 1bit | -7.6 dB | -11.7 dB | - |
| PUCCH format 1 2bits | -6.1 dB | -10.3 dB | -13.3 dB |
| PUCCH format 3 3bits | -6.6 dB | -10.4 dB | -13.2 dB |
| PUCCH format 3 6bits | -5.1 dB | -9.3 dB | -12.05 dB |
| PUCCH format 3 11bits | -3.3 dB | -7.6 dB | -10.54 dB |
| PUCCH format 3 22bits | -0.6 dB | -5.5 dB | -8.65 dB |

Observation 15: For scenario 3-4 (Rural scenario, 4GHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx is given in Table 3-5.

- Scenario 3-5, Long Distance rural scenario_700MHz_O-to-O

For scenario 3-5, the baseline performance for different cases is provided in Table 3-6.

Table 3-6 Baseline performance for PUCCH for {Scenario 3-5, Long Distance rural scenario_700MHz_O-to-O}.

| Simulation cases | 2Rx | 4Rx |
| --- | --- | --- |
| PUCCH format 1 1bit | -10.65 dB | -13.2 dB |
| PUCCH format 1 2bits | -8.45 dB | -11.2 dB |
| PUCCH format 3 3bits | -9.15 dB | -11.75 dB |
| PUCCH format 3 6bits | -7.9 dB | -10.6 dB |
| PUCCH format 3 11bits | -6.35 dB | -9.2 dB |
| PUCCH format 3 22bits | -4.6 dB | -7.5 dB |

Observation 16: For Scenario 3-5 (Long distance rural scenario, 700MHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx is given in Table 3-6.

- Scenario 3-6, Urban/Rural_scenario_4GHz_O-to-I

For scenario 3-5, the baseline performance for different cases is provided in Table 3-7. Note, the simulation assumptions in urban and rural scenario at 4GHz for the simulation below are the same according to Table A-2.

Table 3-7 Baseline performance for PUCCH for {Scenario 3-6, Urban/Rural_scenario_4GHz_O-to-I}.

| Simulation cases | 2Rx | 4Rx | 8Rx | 64Rx |
| --- | --- | --- | --- | --- |
| PUCCH format 1 1bit | -7.6 dB | -11.8 dB | - | - |
| PUCCH format 1 2bits | -6.0 dB | -10.3 dB | -13.6 dB | - |
| PUCCH format 3 3bits | -6.2 dB | -10.2 dB | -13.2 dB | -19.62 dB |
| PUCCH format 3 6bits | -5.3 dB | -9.3 dB | -12 dB | -18.71 dB |
| PUCCH format 3 11bits | -3 dB | -7.2 dB | -10.7 dB | -17.50 dB |
| PUCCH format 3 22bits | -0.9 dB | -5.7 dB | -9 dB | -15.87 dB |

Observation 17: For scenario 3-6 (Urban/Rural_scenario, 4GHz, O-to-I), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx or 64Rx is given in Table 3-7.

Baseline performance for PDCCH

In this section, we provide PDCCH results for urban scenario and rural scenario for FR1.

  - Simulation scenarios

For PDCCH, both rural scenario at 700 MHz and urban scenario at 4GHz are evaluated in this section, as listed in Table 4-1.

Table 4-1 Simulation scenarios for PDCCH

| Scenario 4-1 | Rural scenario_700MHz_O-to-O |
| --- | --- |
| Scenario 4-2 | Urban scenario_4GHz_O-to-I |

  - Simulation assumptions

For PDCCH, both 40 bits for Rel-15 fallback DCI formats and 30 bits for new DCI formats in Rel-16 URLLC are evaluated with AL from 1~16. More detailed simulation assumptions are provided in Table A-3 in the appendix.

  - Simulation results

- Scenario 4-1, Rural scenario_700MHz_O-to-O

For scenario 4-1, the baseline performance for different simulation cases with 2Tx-2 Rx are provided in Table 4-2.

Table 4-2 Baseline performance for PDCCH at 1% BLER for {Scenario 4-1, Rural scenario_700MHz_O-to-O, 2Tx-2 Rx}

| Simulation cases | Required SNR |
| --- | --- |
| 30 bits AL1 | 7.92 dB |
| 30 bits AL2 | 1.59 dB |
| 30 bits AL4 | -1.67 dB |
| 30bits AL8 | -4.80 dB |
| 30bits AL16 | -7.75 dB |
| 40bits AL1 | 9.56 dB |
| 40bits AL2 | 2.94 dB |
| 40bits AL4 | -0.59 dB |
| 40bits AL8 | -4.30 dB |
| 40bits AL16 | -7.43 dB |

Observation 18: For PDCCH in scenario 4-1 (rural scenario, 700 MHz, O-to-O, 2Tx-2Rx), the required SNR for cases of 30 or 40 bits with AL 1~16 is given in Table 4-2.

- The case with information bits of 30bits with AL 16 has the lowest required SNR with -7.75 dB.

- Scenario 4-2, Urban scenario_4GHz_O-to-I

For scenario 4-2, the baseline performance for different simulation cases with 4Tx-4 Rx are provided in Table 4-3.

Table 4-3 Baseline performance for PDCCH at 1% BLER for {Scenario 4-2, Urban scenario_4GHz_O-to-I, 4Tx-4 Rx}

| Simulation cases | Required SNR |
| --- | --- |
| 30bits AL1 | 1.67 dB |
| 30bits AL2 | -2.96 dB |
| 30bits AL4 | -6.41 dB |
| 30bits AL8 | -9.39 dB |
| 30bits AL16 | -11.83 dB |
| 40bits AL1 | 2.72 dB |
| 40bits AL2 | -2.19 dB |
| 40bits AL4 | -5.83 dB |
| 40bits AL8 | -8.78 dB |
| 40bits AL16 | -11.40 dB |

Observation 19: For PDCCH in scenario 4-2 (Urban scenario, 4GHz, O-to-O, 4Tx-4Rx), the required SNR for cases of 30 or 40 bits with AL 1~16 is given in Table 4-3.

- The case with information bits of 30bits with AL 16 has the lowest required SNR with -11.83 dB.

Baseline performance for PBCH

  - Simulation scenarios

For PBCH, both rural scenario at 700 MHz and urban scenario at 4GHz are evaluated in this section, as listed in Table 5-1.

Table 5-1 Simulation scenarios for PBCH

| Scenario 5-1 | Rural scenario_700MHz_O-to-O |
| --- | --- |
| Scenario 5-2 | Urban scenario_4GHz_O-to-I |

  - Simulation assumptions

For PBCH, a combination of 4 SSBs in 80 ms is assumed. More detailed simulation assumptions are attached in Table A-4.

  - Simulation results

- Scenario 5-1, Rural scenario_700MHz_O-to-O

For scenario 5-1, the baseline performance for different simulation cases with 2Tx-2 Rx are provided in Table 5-2.

Table 5-2 Baseline performance for PBCH at 1% BLER for {Scenario 5-1, Rural scenario_700MHz_O-to-O, 2Tx-2 Rx}

| Simulation cases | Required SNR |
| --- | --- |
| PBCH payload 32 bits, 4 SSBs | -9.41 dB |

Observation 20: For PBCH in scenario 5-1 (rural scenario, 700 MHz, O-to-O, 2Tx-2Rx), the required SNR with 32 information bits and a combination of 4 SSBs is -9.41 dB.

- Scenario 5-2, Urban scenario_4GHz_O-to-I

For scenario 5-2, the baseline performance for different simulation cases with 2Tx-2Rx and 4Tx-4Rx are provided in Table 5-3.

Table 5-3 Simulation results for PBCH for {Scenario 5-2, Urban scenario_4GHz_O-to-I}

| Simulation cases | 2Rx | 4Rx |
| --- | --- | --- |
| PBCH payload 32 bits, 4 SSBs | -7.27 dB | -12.70 dB |

Observation 21: For PBCH in scenario 5-2 (rural scenario, 4GHz, O-to-I), the required SNR with 2 or 4 Rx and 32 information bits and a combination of 4 SSBs is -7.27 dB and -12.70 dB.

Baseline performance for PRACH

  - Simulation scenarios

For PRACH, both rural scenario at 700 MHz and rural scenario at 2GHz are evaluated in this section, as listed in Table 6-1.

Table 6-1 Simulation scenarios for PRACH

| Scenario 6-1 | Rural scenario_2GHz_O-to-I |
| --- | --- |
| Scenario 6-2 | Rural scenario_700MHz_O-to-O |

  - Simulation assumptions

NR PRACH preamble format 2 with Ncs = 167 and logical sequence index = 22, v=2 for Scenario 6-1 and NR PRACH preamble format 1 with Ncs = 202 and logical sequence index = 384, v=0 for Scenario 6-2 are considered. Detailed simulation assumptions are attached in Table A-5.

  - Simulation results

The baseline performance for different simulation scenarios are provided in Table 6-2.

  - Table 6-2 Baseline performance for PRACH

| Simulation Scenario | Required SNR |
| --- | --- |
| Scenario 6-1, Rural_scenario_2GHz_O-to-I | -9.01 dB |
| Scenario 6-2, Rural_scenario_700MHz_O-to-O | -6.76 dB |

Observation 22: For PRACH in scenario 6-1 (rural scenario, 2GHz, O-to-I, 2Tx-2Rx), the required SNR is about -9.01dB.

Observation 23: For PRACH in scenario 6-2 (rural scenario, 700MHz, O-to-O, 2Tx-2Rx), the required SNR is about -6.76dB.

Conclusion

According to the analysis given above, we have the following observations:

Observation 1: For PUSCH with 100kbps eMBB service in scenario 1-1 (rural scenario, 700 MHz, O-to-O), the case (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, and (2RBs, MCS#2) has the best performance for 8Rx. The corresponding baseline performance is given in Table 2-5.

Observation 2: For PUSCH with 100kbps eMBB service in scenario 1-2 (rural scenario, 2GHz, O-to-O), the case (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, and (2RBs, MCS#2) has the best performance for 8Rx. The corresponding baseline performance is given in Table 2-6.

Observation 3: For PUSCH with 100kbps eMBB service in scenario 1-3 (rural scenario, 2GHz, O-to-I), the case (3 RBs, MCS#0) has the best performance for 2Rx and 4Rx, and (2RBs, MCS#2) has the best performance for 8Rx. The corresponding baseline performance is given in Table 2-7.

Observation 4: For PUSCH with 100kbps eMBB service in scenario 1-4 (rural scenario, 4GHz, O-to-O), the case (5 RBs, MCS#0) or (4RBs, MCS#1) or (2RBs, MCS#4) has the best performance for 2Rx or 4Rx or 8 Rx respectively. The corresponding baseline performance for 2Rx/4Rx/8Rx is given in Table 2-8.

Observation 5: For PUSCH with 100kbps eMBB service in scenario 1-5 (rural scenario, 4GHz, O-to-I), the case (5 RBs, MCS#0) or (4RBs, MCS#1) or (2RBs, MCS#4) has the the best performance for 2Rx or 4Rx or 8 Rx respectively. The corresponding baseline performance for 2Rx/4Rx/8Rx is given in Table 2-9.

Observation 6: For PUSCH with 100kbps eMBB service in scenario 1-6 (long distance rural scenario, 700MHz, O-to-O), the case (3 RBs, MCS#0) has the best performance. The corresponding baseline performance for 2Rx/4Rx/8Rx is given in Table 2-10.

Observation 7: For PUSCH with 1Mbps eMBB service in scenario 1-7 (urban scenario, 4GHz, O-to-I), the case (24 RBs,MCS#3) has the best performance for 4Rx and 8Rx, and (15RBs, MCS#5) has the best performance for 64Rx. The corresponding baseline performance for 4Rx/8Rx/64Rx is given in Table 2-11.

Observation 8: For PUSCH with VoIP service in scenario 2-1 (rural scenario, 700MHz, O-to-O), the baseline performance for 2 Rx is given in Table 2-13.

  - For cases with one repetition, (10 RBs, MCS#0) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 5.29 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 6.37 dB gain over one repetition with max 3 re-transmissions.

Observation 9: For PUSCH with VoIP service in scenario 2-2 (rural scenario, 2GHz, O-to-O), the baseline performance for 2 Rx is given in Table 2-14.

  - For cases with one repetition, (10 RBs, MCS#0) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 5.88 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 7.4 dB gain than over repetition with max 3 re-transmissions.

Observation 10: For PUSCH with VoIP service in scenario 2-3 (long distance rural scenario, 700 MHz, O-to-O), the baseline performance for 2 Rx is given in Table 2-15.

  - For cases with one repetition, (6RBs, MCS#2) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 5.35 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 7.34 dB gain over one repetition with max 3 re-transmissions.

Observation 11: For PUSCH with VoIP service in scenario 2-4 (urban scenario, 2GHz, O-to-I), the baseline performance for 4 Rx is given in Table 2-16.

  - For cases with one repetition, (10RBs, MCS#0) has the lowest required SNR with power normalization.

  - PUSCH with 4 repetitions and a maximum of 3 re-transmissions can provide additional 4.07 dB gain over one repetition with max 3 re-transmissions.

  - PUSCH with 8 repetitions and a maximum of 1 re-transmission can provide additional 6.7 dB gain over one repetition with max 3 re-transmissions.

Observation 12: For PUCCH in scenario 3-1 (Rural scenario, 700MHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx is given in Table 3-2.

Observation 13: For PUCCH in scenario 3-2 (Rural scenario, 2GHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx is given in Table 3-3.

Observation 14: For scenario 3-3 (Rural scenario, 2GHz, O-to-I), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx is given in Table 3-4.

Observation 15: For scenario 3-4 (Rural scenario, 4GHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx is given in Table 3-5.

Observation 16: For Scenario 3-5 (Long distance rural scenario, 700MHz, O-to-O), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx is given in Table 3-6.

Observation 17: For scenario 3-6 (Urban/Rural_scenario, 4GHz, O-to-I), the required SNR for cases of 1~ 22 bits with 2 or 4 Rx or 8 Rx or 64Rx is given in Table 3-7.

Observation 18: For PDCCH in scenario 4-1 (rural scenario, 700 MHz, O-to-O, 2Tx-2Rx), the required SNR for cases of 30 or 40 bits with AL 1~16 is given in Table 4-2.

- The case with information bits of 30bits with AL 16 has the lowest required SNR with -7.75 dB.

Observation 19: For PDCCH in scenario 4-2 (Urban scenario, 4GHz, O-to-O, 4Tx-4Rx), the required SNR for cases of 30 or 40 bits with AL 1~16 is given in Table 4-3.

- The case with information bits of 30bits with AL 16 has the lowest required SNR with -11.83 dB.

Observation 20: For PBCH in scenario 5-1 (rural scenario, 700 MHz, O-to-O, 2Tx-2Rx), the required SNR with 32 information bits and a combination of 4 SSBs is -9.41 dB.

Observation 21: For PBCH in scenario 5-2 (rural scenario, 4GHz, O-to-I), the required SNR with 2 or 4 Rx and 32 information bits and a combination of 4 SSBs is -7.27 dB and -12.70 dB.

Observation 22: For PRACH in scenario 6-1 (rural scenario, 2GHz, O-to-I, 2Tx-2Rx), the required SNR is about -9.01dB.

Observation 23: For PRACH in scenario 6-2 (rural scenario, 700MHz, O-to-O, 2Tx-2Rx), the required SNR is about -6.76dB.

Reference

- 3GPP, RAN#86, RP-193240 New SID on NR coverage enhancement, China Telecom.

- 3GPP, TR 25.912, Feasibility study for evolved Universal Terrestrial Radio Access (UTRA) and Universal Terrestrial Radio Access Network (UTRAN).

- Appendix-A Simulation assumptions

Table A-1 Simulation assumption for baseline performance of PUSCH for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban , O-to-I | Rural, O-to-O or , O-to-I | Rural with long distance, O-to-O |
| Duplexing scheme and frequency | 4GHz TDD | 4GHz TDD 700MHz /2GHz FDD | 700MHz FDD |
| Transmission bit rate for data channel (bit/s) | 1 Mbps for eMBB 12.2 kbps for VoIP | 100 kbps for eMBB 12.2 kbps for VoIP | 100 kbps for eMBB 12.2 kbps for VoIP |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O-to-O 3 km/h for O-to-I | 120 km/h |
| Number of BS antennas & antenna configuration | 192 (M,N,P,Mg,Ng;Mp,Np) = (12,8,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) | 64 (M,N,P,Mg,Ng;Mp,Np) = (8,4,2,1,1;1,1) |
| Number of TXRU for BS | 4, 8, 64 | 2, 4, 8 | 2, 4, 8 |
| Number of UE antennas | 2 | 1 | 1 |
| Number of TRXU for UE | 2 with PMI adaption | 1 | 1 |
| Occupied channel bandwidth | Derived based on the data rate. |  |  |
| DMRS overhead | - For 3km/h: Type I, one DMRS symbol, no multiplexing with data. - For 120km/h: Type I, 2 DMRS symbol (one front- loaded and one additional), no multiplexing data |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER Target | 10% iBLER for eMBB 2%rBLER for VoIP, maximum # of retransmission is 3 for repetition number 1 and 4; maximum # of retransmission is 1 for repetition number 8. |  |  |

Table A-2 Simulation assumption for baseline performance of PUCCH for FR1

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 4GHz TDD | 4GHz TDD/ 700MHz/2GHz FDD | 700MHz FDD |
| Format type | Format 1 or 3 (long PUCCH with 14 OFDM symbols), 1bit, 2bits, 6bits, 11bits, 22bits. |  |  |
| Subcarrier spacing | 30kHz | 30kHz for TDD and 15kHz for FDD | 15kHz |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-D |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 30ns |
| UE velocity | 3 km/h | 120 km/h for O-to-O 3 km/h for O-to-I | 120 km/h |
| Number of TXRU for BS | 2, 4, 8, 64 | 2, 4 | 2, 4 |
| Number of UE antennas | 1 | 1 | 1 |
| Number of TRXU for UE | 2 | 1 | 1 |
| Occupied channel bandwidth | 1 PRBs |  |  |
| DMRS overhead | No additional DMRS for PUCCH format 3 |  |  |
| Channel estimation | Practical |  |  |
| Receiver type | MMSE |  |  |
| BLER Target | DTX to ACK probability: 1% ; NACK to ACK probability: 0.1%; ACK missed detection probability: 1%; and CSI block error probability: 1% |  |  |

Table A-3 Simulation assumption for baseline performance of PDCCH for FR1

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | Urban | Rural |
| DCI payload (excluding 24bits CRC) | 40 bits for fallback DCI, 30 bits for compact DCI |  |
| Carrier Frequency | 4GHz | 700MHz |
| UE speed | 3 km/h | 120 km/h |
| System bandwidth | 40MHz | 20MHz |
| Number of BS antennas | 4Tx | 2Tx |
| Number of UE antennas | 4Rx | 2Rx |
| Subcarrier spacing | 30KHz | 15kHz |
| Number of symbols for CORESET | 2 |  |
| Transmission type | Interleaved(R=3 for 3OS,others,R=2) |  |
| REG bundling size | 6 |  |
| Modulation | QPSK |  |
| Channel coding | Polar code (DCI) |  |
| Transmission scheme | 1-port precoder cycling |  |
| Channel model | TDL-C (delay spread: 300ns) |  |
| Channel estimation | Practical |  |
| Receiver type | MMSE |  |
| BLER target | 1% |  |

Table A-4 Simulation assumption for baseline performance of PBCH for FR1

| Parameter | Value |  |
| --- | --- | --- |
| Scenario | Urban | Rural |
| Carrier Frequency | 4GHz | 700MHz |
| UE speed | 3 km/h | 120 km/h |
| System bandwidth | 40MHz | 20MHz |
| Number of BS antennas | 2,4Tx | 2Tx |
| Number of UE antennas | 2,4Rx | 2Rx |
| Subcarrier spacing | 30KHz | 15kHz |
| PBCH payload (excluding 24bits CRC) | 32 bits |  |
| Number of symbols for CORESET | 2 |  |
| Modulation | QPSK |  |
| Channel coding | Polar code (DCI) |  |
| Transmission scheme | 1-port precoder cycling |  |
| Channel model | TDL-C (delay spread: 300ns) |  |
| Channel estimation | Practical |  |
| Receiver type | MMSE |  |
| BLER target | 1% |  |
| Number of interfering TRPs | 0 TRP |  |
| Combined number | 4 SSBs |  |
| Frequency Offset | Initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 5, 10, 20 ppm (each company to choose one) Non-initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 0.1 ppm |  |

Table A-5 Simulation assumption for baseline performance of PRACH for FR1

| Parameter | Value |
| --- | --- |
| Scenario | Rural |
| Carrier Frequency | 2GHz /700MHz |
| UE speed | 3 km/h /120 km/h |
| System bandwidth | 5MHz |
| Number of BS antennas | 2Rx |
| Number of UE antennas | 2Tx |
| Subcarrier spacing | 15kHz |
| Channel model | TDL-C (delay spread: 300ns) |
| Miss-detection target | 1% |
| Preamble | NR PRACH preamble format 2 with Ncs = 167 , Logical sequence index = 22, v=2 for O2I NR PRACH preamble format 1 with Ncs = 202, Logical sequence index = 384, v=0 for O2O |