3GPP TSG RAN WG1 #101                                                                                      R1-2003339

e-Meeting, May 25th – June 5th, 2020

Source:	ZTE

Title:                 Discussion on baseline coverage performance for FR2

Agenda item:	8.8.1.2

Document for: Discussion and Decision

Introduction

In the RAN#86 meeting, a new SID on NR coverage enhancement was approved [1]. The objective of this study item is to identify the performance target for coverage enhancement for specific scenarios for both FR1 and FR2 and study the potential solutions for both DL and UL. The scenarios include urban scenario and rural scenario for FR1, as well as indoor scenario, and urban/suburban scenario for FR2.

In this contribution, the baseline coverage performance for PUSCH in FR2 are provided.

Baseline performance for PUSCH

In this section, we provide baseline PUSCH results, i.e., the link-level required SNR, with carrying eMBB services in indoor scenario for FR2.

  - Simulation assumptions

For PUSCH with eMBB services, the transmission parameter, e.g., TBS, MCS, and number of RBs etc., can be derived from target data rate, which is 5 Mbps for indoor scenario in FR2 as suggested in the SID. More specifically, the TBS is equal to {Target data rate * PUSCH time duration} for FDD. For TDD, a scaling factor T is applied to account for the impact of UL duty cycle. Take configuration 'DDDSUDDSUU' and ‘S’ with 10:2:2 for instance, only T=30% or 32.857% time is available for PUSCH transmission w/ or w/o considering the uplink symbols in 'S' slot. Then, the TBS of TDD should be {Target data rata * PUSCH time duration /T} for TDD. In case of repetitions, the time duration of PUSCH should include all PUSCH repetitions. Based on the TBS, the MCS can be easily determined based on the number of RBs and DMRS overhead.

The simulation cases for indoor scenario with TDD is provided in Table 2-2. The other simulation assumptions are provided in Table A-1 in the appendix.

Table 1 Simulation cases for indoor scenario (TDD 'DDDSUDDSUU' and ‘S’:10:2:2, 14-symbol PUSCH, one DMRS with one hop, SCS = 120 kHz, Speed = 3km/h, T=30%)

| Target data rate = 5 Mbps |  |  |  |
| --- | --- | --- | --- |
| Simulation Cases | Number of RBs | MCS (Target SE) | Actual data rate/ Mbps |
| Case 0 | 4 | MCS#27（5.5547） | 5.1192 |
| Case 1 | 5 | MCS#23（4.5234） | 5.2110 |
| Case 2 | 6 | MCS#21（3.9023） | 5.3945 |
| Case 3 | 8 | MCS#17（2.7305） | 5.0329 |
| Case 4 | 9 | MCS#16（2.5703） | 5.3298 |
| Case 5 | 10 | MCS#15（2.4063） | 5.5441 |
| Case 6 | 12 | MCS#13（1.9141） | 5.2921 |
| Case 7 | 15 | MCS#11(1.4766） | 5.1031 |
| Case 8 | 18 | MCS#9(1.3262） | 5.5000 |
| Case 9 | 20 | MCS#8（1.1758） | 5.4181 |
| Case 10 | 24 | MCS#7（1.0273） | 5.6805 |
| Case 11 | 25 | MCS#6（0.877） | 5.0515 |
| Case 12 | 30 | MCS#5（0.7402） | 5.4573 |
| Case 13 | 40 | MCS#4（0.6016） | 5.5443 |
| Case 14 | 45 | MCS#3（0.4902） | 5.0824 |

  - Simulation results

The baseline performance for different simulation cases with 2 Rx is provided in Table 2-3. For the results without power normalization, the power is increased proportionally with the number of RBs. To get a fair comparison among different cases, the results with power normalization to 1 RB case are also calculated. As it can be observed, the performance of the case with (45 RBs, MCS#3) has the best performance.

Table 2 Baseline performance for PUSCH at BLER = 0.1 for {Indoor scenario, 1Tx, 30GHz, TDD with ‘DDDSUDDSUU’, hopping enabled, 3km/h, Target data rate = 5 Mbps}.

| Simulation case | 2RX (w/o power normalization) | 2RX (w/ power normalization) |
| --- | --- | --- |
| 4 RBs, MCS#27 | - | - |
| 5 RBs, MCS#23 | - | - |
| 6 RBs, MCS#21 | - | - |
| 8 RBs, MCS#17 | 15.71 dB | 24.74 dB |
| 9 RBs, MCS#16 | 14.51 dB | 24.05 dB |
| 10 RBs, MCS#15 | 13.89 dB | 23.89 dB |
| 12 RBs, MCS#13 | 11.32 dB | 22.11 dB |
| 15 RBs, MCS#11 | 8.75 dB | 20.51 dB |
| 18 RBs, MCS#9 | 8.44 dB | 21 dB |
| 20 RBs, MCS#8 | 7.21 dB | 20.22 dB |
| 24 RBs, MCS#7 | 5.86 dB | 19.66 dB |
| 25 RBs, MCS#6 | 5.13 dB | 19.11 dB |
| 32 RBs, MCS#5 | 3.9 dB | 18.96 dB |
| 40 RBs, MCS#4 | 2.54 dB | 18.56 dB |
| 45 RBs, MCS#3 | 1.7 dB | 18.24 dB |

Observation 1: For PUSCH with eMBB service in indoor scenario for FR2, the required SNR for cases of 4~45 RBs with 2Rx is given in Table 2.

Conclusion

According to the analysis given above, we have the following observation:

Observation 1: For PUSCH with eMBB service in indoor scenario for FR2, the required SNR for cases of 4~45 RBs with 2Rx is given in Table 2.

Reference

- 3GPP, RAN#86, RP-193240 New SID on NR coverage enhancement, China Telecom.

- Appendix-A Simulation assumptions

Table A-1 Simulation assumption for baseline performance of PUSCH for FR2

| Parameter | Value |
| --- | --- |
| Carrier frequency | 30 GHz |
| Duplex mode | TDD with 'DDDSUDDSUU' and S:10:2:2, |
| Channel model | TDL-B (delay spread: 20ns) as in 38.901 for Indoor scenario |
| UE speed | 3 km/h |
| BS antenna configuration | 2 Rx for 30GHz |
| UE antenna configuration | 1 Tx |
| System bandwidth | 160 MHz |
| Sub-carrier spacing | 120 kHz |
| Waveform | DFT-s-OFDM |
| Number of OS per repetition | 14 (PUSCH mapping type A) |
| DMRS overhead | 1 DMRS symbol per hop |
| Number of repetitions | 1 |
| Frequency hopping | Enabled |
| Channel estimation | Practical |
| Receiver type | MMSE |
| BLER Target | 10% iBLER |