3GPP TSG RAN WG1 #101		R1-2003408

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on remaining issues on UE and gNB measurement for NR positioning

Agenda Item:	7.2.8.3

Document for:	Discussion & Decision

# Introduction

In this contribution, we present our views on some remaining issues on UE and gNB measurement for Rel-16 NR positioning.

# Additional path

Additional path for time-based measurement is agreed in RAN2 and captured in the IE ‘NR-DL-TDOA-MeasElement-r16’ and ‘NR-Multi-RTT-MeasElement-r16’ as below in the latest TS 37.355 [1].

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE { trp-ID-r16 TRP-ID-r16 OPTIONAL, nr-DL-PRS-ResourceId-r16 NR-DL-PRS-ResourceId-r16 OPTIONAL, nr-DL-PRS-ResourceSetId-r16 NR-DL-PRS-ResourceSetId-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-RSTD-r16 INTEGER (0..ffs), -- FFS on the value range nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimingMeasQuality-r16 NR-TimingMeasQuality-r16, nr-PRS-RSRP-Result-r16 INTEGER (FFS) OPTIONAL, -- FFS, value range to be decided in RAN4. nr-DL-TDOA-AdditionalMeasurements-r16 NR-DL-TDOA-AdditionalMeasurements-r16, ... }

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE { trp-ID-r16 TRP-ID-r16 OPTIONAL, nr-DL-PRS-ResourceId-r16 NR-DL-PRS-ResourceId-r16 OPTIONAL, nr-DL-PRS-ResourceSetId-r16 NR-DL-PRS-ResourceSetId-r16 OPTIONAL, nr-UE-RxTxTimeDiff-r16 INTEGER (0..ffs) OPTIONAL, -- FFS on the value range to be decided in RAN4 nr-AdditionalPathList-r16 NR-AdditionalPathList-r16 OPTIONAL, nr-TimeStamp-r16 NR-TimeStamp-r16, nr-TimingMeasQuality-r16 NR-TimingMeasQuality-r16, nr-PRS-RSRP-Result-r16 INTEGER (FFS) OPTIONAL, -- FFS, value range to be decided in RAN4. nr-Multi-RTT-AdditionalMeasurements-r16 NR-Multi-RTT-AdditionalMeasurements-r16, ... }

The corresponding descriptions for ‘NR-AdditionalPathList’ are also listed below for DL-TDOA and Multi-RTT respectively.

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value or the nr-UE-RxTxTimeDiffAdditional value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

More specifically, RAN2 also specifies the number of NR-AdditionalPath and the component IEs for each NR-AdditionalPath included in the list as below.

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16 NR-AdditionalPath-r16 ::= SEQUENCE { nr-relativeTimeDifference-r16 INTEGER (FFS),--FFS to be decided in RAN4 nr-path-Quality-r16 NR-TimingMeasQuality-r16 OPTIONAL, ...}

In RAN2’s understanding, the IE NR-AdditionalPathList is used by the UE to provide information about additional paths in association to the TOA measurements for NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-path-Quality.

However, no agreements related to NR-AdditionalPath have been achieved by RAN1 in Rel-16 WI. To align with RAN2’s conclusion, we think the NR-AdditionalPath related descriptions should also be captured into TS 38.214.

Proposal 1:

- Capture the descriptions related to ‘NR-AdditionalPath’ into TS 38.214.

In addition, it is observed that in both TS 37.355 [1] and TS 36.355 [2], the UE can indicate to the location server whether it can support reporting of timing information for additional detected path for associated ToA measurements as UE capability of additional path report, as shown below.

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE { nr-DL-TDOA-Mode-r16 PositioningModes, nr-DL-TDOA-MeasCapability-r16 NR-DL-PRS-MeasCapability-r16 OPTIONAL, nr-DL-TDOA-MeasSupported-r16 BIT STRING { prsrsrpSup (0)} (SIZE(1..8)), additionalPathsReport-r16 ENUMERATED { supported } OPTIONAL, periodicalReporting-r16 ENUMERATED { supported } OPTIONAL, ...}

NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE { nr-DL-PRS-MeasCapability-r16 NR-DL-PRS-MeasCapability-r16, nr-UL-SRS-MeasCapability-r16 NR-UL-SRS-MeasCapability-r16, nr-Multi-RTT-MeasSupported-r16 BIT STRING { prsrsrpSup (0)} (SIZE(1..8)), additionalPathsReport-r16 ENUMERATED { supported } OPTIONAL, periodicalReporting-r16 ENUMERATED { supported } OPTIONAL, ... }

# OTDOA-ProvideCapabilities ::= SEQUENCE { otdoa-Mode BIT STRING { ue-assisted (0), ue-assisted-NB-r14 (1), ue-assisted-NB-TDD-r15 (2) } (SIZE (1..8)), ..., supportedBandListEUTRA SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA OPTIONAL, supportedBandListEUTRA-v9a0 SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9a0 OPTIONAL, interFreqRSTDmeasurement-r10 ENUMERATED { supported } OPTIONAL, additionalNeighbourCellInfoList-r10 ENUMERATED { supported } OPTIONAL, prs-id-r14 ENUMERATED { supported } OPTIONAL, tp-separation-via-muting-r14 ENUMERATED { supported } OPTIONAL, additional-prs-config-r14 ENUMERATED { supported } OPTIONAL, prs-based-tbs-r14 ENUMERATED { supported } OPTIONAL, additionalPathsReport-r14 ENUMERATED { supported } OPTIONAL, densePrsConfig-r14 ENUMERATED { supported } OPTIONAL, maxSupportedPrsBandwidth-r14 ENUMERATED { n6, n15, n25, n50, n75, n100, ...} OPTIONAL, ... }

Thus, it makes more sense to follow TS 37.355 and TS 36.355 and capture the same UE capability in RAN1 specification. So that for each ToA measurement, the UE may be configured to measure and report, subject to UE capability, up to 2 additional detected path timing values.

Proposal 2:

- Capture UE capability of additional path report for NR DL-TDOA positioning and NR Multi-RTT positioning.

Therefore, we propose that

Proposal 3:

- Adopt the following text proposals into TS 38.214 for the description of additional path report.

TS 38.214 5.1.6.5 PRS reception procedures < Unchanged parts are omitted > The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of cells with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those cells. The up to 4 measurements being performed on the same pair of cells and all DL RSTD measurements in the same report use a single reference timing. The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception. The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers. The UE may be configured to measure and report, subject to UE capability, up to 2 additional detected path timing values relative to the path timing in association to each TOA measurement used to determine each RSTD measurement or RX-TX time difference measurement. The UE may also be configured to report quality metrics corresponding to each additional detected path. < Unchanged parts are omitted >

# Conclusion

In this contribution, we discuss UE and gNB measurement for Rel-16 NR positioning with the following proposals:

Proposal 1:

- Capture the descriptions related to ‘NR-AdditionalPath’ into TS38.214.

Proposal 2:

- Capture UE capability of additional path report for NR DL-TDOA positioning and NR Multi-RTT positioning.

Proposal 3:

- Adopt the following text proposals into TS 38.214 for the description of additional path.

TS 38.214 5.1.6.5 PRS reception procedures < Unchanged parts are omitted > The UE may be configured to measure and report, subject to UE capability, up to 4 DL RSTD measurements per pair of cells with each measurement between a different pair of DL PRS resources or DL PRS resource sets within the DL PRS configured for those cells. The up to 4 measurements being performed on the same pair of cells and all DL RSTD measurements in the same report use a single reference timing. The UE may be configured to measure and report, subject to UE capability, up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate which DL PRS RSRP measurements have been performed using the same spatial domain filter for reception. The UE may be configured to measure and report, subject to UE capability, up to 4 UE Rx-Tx time difference measurements corresponding to a single configured SRS resource or resource set for positioning. Each measurement corresponds to a single received DL PRS resource or resource set which can be in different positioning frequency layers. The UE may be configured to measure and report, subject to UE capability, up to 2 additional detected path timing values relative to the path timing in association to each TOA measurement used to determine each RSTD measurement or RX-TX time difference measurement. The UE may also be configured to report quality metrics corresponding to each additional detected path. < Unchanged parts are omitted >

# References

- R2-2003981, LPP clean-up, RAN2#109b-e, April, 2020

- 3GPP TS 36.355 v15.5.0

- R1-2003142 CR, Corrections of NR positioning support