3GPP TSG RAN WG1 #101		R1-2003415

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on UE features for 2-step RACH

Agenda Item:	7.2.11.1

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the UE features for 2-step RACH, including the simplified basic feature group for 2-step RACH.

- Discussion

According to [1], FGs for 2-step RACH are captured as in the Appendix. In this section, we will discuss the FGs 9-1 to 9-6.

  - FG 9-1

| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection for CBRA according to SSB-based RSRP threshold - msgA PRACH resource configuration including separately configured ROs not applicable to 4-step RO configuration and fully or partially shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration - msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA - Supporting up to two msgA PUSCH configurations in an UL BWP - Validation of MsgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH - msgB monitoring and decoding for 2-step CBRA - (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI - (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE - PUCCH transmission for HARQ-ACK feedback to a msgB - Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For FG 9-1, currently there are still too many detailed components in the basic feature group. If details of functionalities of 2-step RACH in the specification are to be listed in the FG, it will result in great risk that some functionalities would be missed for the basic feature group, which is not the intention and may have inconsistency with the Rel.15 UE features. It is safer and clearer that only the essential functionalities for 2-step RACH are included in the FG, while more details can be found in the specifications.

Therefore, for FG 9-1, a simplified basic feature group with only the essential components is preferred. The remaining issue is what components should be included in the FG 9-1. Starting from the components listed in current FG 9-1, following categorization can be considered based on our analysis.

| 1) | RACH type selection for CBRA according to SSB-based RSRP threshold | Should be mandatory supported behaviour for 2-step RACH Thus, it is no need to be included in 9-1 |
| --- | --- | --- |
| 2) | msgA PRACH resource configuration including separately configured ROs not applicable to 4-step RO configuration and fully or partially shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration | Signaling and behaviour for msgA PRACH configuration and msgA PRACH transmission are already specified in the spec. No additional signalling/behaviour is needed. Thus, it can be included in a component of msgA PRACH and PUSCH transmission |
| 3) | msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA | Signaling and behaviour for msgA PUSCH configuration and msgA PUSCH transmission are already specified in the spec. No additional signalling/behaviour is needed. Thus, it can be included in a component of msgA PRACH and PUSCH transmission |
| 4) | - Supporting up to two msgA PUSCH configurations in an UL BWP | Signaling and behaviour for msgA PUSCH configuration and msgA PUSCH transmission are already specified in the spec. No additional signalling/behaviour is needed. Thus, it can be included in a component of msgA PRACH and PUSCH transmission |
| 5) | Validation of MsgA PRACH and PUSCH | Should be mandatory supported behaviour for 2-step RACH Thus, it is no need to be included in 9-1 |
| 6) | Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH | Should be mandatory supported behaviour for 2-step RACH Thus, it is no need to be included in 9-1 |
| 7) | msgB monitoring and decoding for 2-step CBRA | Signaling and behaviour for msgB monitoring and decoding are already specified in the spec. No additional signalling/behaviour is needed. Besides, it is not necessary to mention msgB for 2-step CBRA. Thus, it can be included in a component of MsgB monitoring, reception, and feedback |
| 8) | (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI | Signaling and behaviour for msgB monitoring and decoding are already specified in the spec. No additional signalling/behaviour is needed. Besides, it is not necessary to mention msgB with msgB-RNTI. Thus, it can be included in a component of MsgB monitoring, reception, and feedback |
| 9) | (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE | Signaling and behaviour for msgB monitoring and decoding are already specified in the spec. No additional signalling/behaviour is needed. Besides, it is not necessary to mention msgB with C-RNTI. Thus, it can be included in a component of MsgB monitoring, reception, and feedback |
| 10) | PUCCH transmission for HARQ-ACK feedback to a msgB | Signaling and behaviour for PUCCH transmission for HARQ-ACK feedback to a msgB are already specified in the spec. No additional signalling/behaviour is needed. Thus, it can be included in a component of MsgB monitoring, reception, and feedback |
| 11) | Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB | Keep it as a component of Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB |

Proposal 1: For FG 9-1, support a more simplified basic feature group with following components included.

- MsgA PRACH and PUSCH transmission

- MsgB monitoring, reception, and feedback

- Power control for MsgA PRACH, MsgA PUSCH, and PUCCH for HARQ-ACK feedback to a MsgB

  - FG 9-3, 9-4 and 9-6

| 9. NR_2step_RACH | [9-3] | [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA] | [Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell] | 9-1 TBD | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | [9-4] | [MsgA operation in a band combination including SUL] | [MsgA operations in a band combination including SUL] | 9-1, 6-16 TBD | Yes | N/A | UE does not support msgA operations in a band combination including SUL | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-6] | [up to X of msgBs per slot/within the msgB window] | [up to X of msgBs per slot/within the msgB window] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |

For FG 9-3, we are fine to keep it with an update to clarify that only MsgA PUSCH is needed to be included in the FG, i.e. parallel MsgA PUSCH and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell. For PRACH transmission for 2-step RACH, there is no difference from 4-step RACH.

For FG 9-4, we don’t see too much necessity to introduce separate FG for 2-step RACH. It can be inherited from the Rel.15 feature of SUL.

For FG 9-6, wait for RAN2 further discussion and conclusion on it before introducing the FG.

Proposal 2: For FG 9-3, we are fine to keep it with an update to clarify that only MsgA PUSCH is needed to be included in the FG, i.e. parallel MsgA PUSCH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell

Proposal 3: For FG 9-4, no need to introduce separate FG for 2-step RACH.

Proposal 4: For FG 9-6, wait for RAN2’s further discussion and conclusion on it before introducing the FG.

- Conclusion

In this contribution, we provide our views on UE features for 2-step RACH. The proposals are summarized below.

Proposal 1: For FG 9-1, support a more simplified basic feature group with following components included.

- MsgA PRACH and PUSCH transmission

- MsgB monitoring, reception, and feedback

- Power control for MsgA PRACH, MsgA PUSCH, and PUCCH for HARQ-ACK feedback to a MsgB

Proposal 2: For FG 9-3, we are fine to keep it with an update to clarify that only MsgA PUSCH is needed to be included in the FG, i.e. parallel MsgA PUSCH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell

Proposal 3: For FG 9-4, no need to introduce separate FG for 2-step RACH.

Proposal 4: For FG 9-6, wait for RAN2’s further discussion and conclusion on it before introducing the FG.

- References

- R1-2003197	Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_2step_RA Moderator (AT&T, NTT DOCOMO, INC.)

- Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection for CBRA according to SSB-based RSRP threshold - msgA PRACH resource configuration including separately configured ROs not applicable to 4-step RO configuration and fully or partially shared ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration - msgA PUSCH resource (DMRS included) and waveform determination for 2-step CBRA - Supporting up to two msgA PUSCH configurations in an UL BWP - Validation of MsgA PRACH and PUSCH - Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH - msgB monitoring and decoding for 2-step CBRA - (for UE in any RRC state) monitoring msgB PDCCH with CRC masked by msgB-RNTI in Type-1 CSS set, and decoding multi-cast msgB PDSCH carrying SuccessRAR, FallbackRAR and BI - (for RRC connected UE only) monitoring msgB PDCCH with CRC masked by C-RNTI in USS set, and decoding the unicast PDSCH carrying absolute TA MAC CE - PUCCH transmission for HARQ-ACK feedback to a msgB - Power control for msgA PRACH, msgA PUSCH and PUCCH carrying HARQ-ACK feedback to msgB |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-3] | [Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA] | [Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell] | 9-1 TBD | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-4] | [MsgA operation in a band combination including SUL] | [MsgA operations in a band combination including SUL] | 9-1, 6-16 TBD | Yes | N/A | UE does not support msgA operations in a band combination including SUL | Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| 9. NR_2step_RACH | [9-6] | [up to X of msgBs per slot/within the msgB window] | [up to X of msgBs per slot/within the msgB window] | TBD | Yes | N/A |  | [Per band] | N/A | N/A | N/A |  | Optional with capability signalling |