3GPP TSG RAN WG1 #101		R1-2003438

e-Meeting, May 25th – June 5th, 2020

Source:	vivo

Title:	Discussion on coverage evaluation assumptions and performance for RedCap UE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss the evaluation assumptions and preliminary results for coverage of different UE type, i.e. reduced capability (RedCap) UEs.

- Evaluation of coverage performance of Redcap UE

In [1][2], the evaluation methodology, assumptions and results for normal UEs in various scenarios have been provided for NR coverage enhancements. And in Rel-17 SI for RedCap UE, coverage enhancement is also one of the potential objectives, since coverage of the RedCap UE would be reduced due to the device complexity reduction, e.g. reduced channel bandwidth, reduced number of receiver antennas, and lower antenna loss. Therefore, coverage evaluation is also needed for SI for RedCap UEs.

To avoid duplicated discussion, basic evaluation methodology and assumptions in coverage enhancement can be considered as baseline for coverage recovery for RedCap UE.

Proposal 1: To avoid duplicated discussion, basic evaluation methodology and assumptions in coverage enhancement can be considered as baseline for coverage recovery for RedCap UE.

As discussed in [3], the RedCap UEs has reduced bandwidth capability and rx antenna numbers. Furthermore, due to the device type constraints and manner of wearing, e.g. wearables, the additional antenna gain loss or body loss should be considered compared with normal UEs. Therefore, these differences should be considered in evaluation assumptions. The detailed evaluation assumptions are given in Table 1.

Table 1  Evaluation assumptions for RedCap UEs

| Parameter | Evaluation assumptions |
| --- | --- |
| Channel model | TDL-C 300ns 3km/h |
| Frame structure | DDDSU DDSUU |
| Tx Power at UE | 23dBm |
| Tx power at gNB | FR1 Urban Macro 44dBm for BW=20MHz |
| Power assignment | Concentrated on occupied BW for UL Uniformed allocated to channel BW for DL |
| Required SNR | 10% BLER for data channels 1% BLER for other channels |
| Target data rate | FR1 Urban DL/UL: 5 Mbps/1Mbps |
| Pathloss model | NLOS for FR1 4GHz |
| # of BS Ants / TxRU | 192 / 2 |
| # of UE Ants | 4Rx for Normal UE (expect for SSB/PBCH) 2Rx for Normal UE SSB/PBCH 1Rx (or 2Rx) for REDCAP UE |
| Antenna gain | -3dB |

Proposal 2: Following differences should be considered for coverage evaluation for RedCap UE

- Reduced bandwidth capability, e.g., up to 20MHz;

- Reduced number of Rx antennas, i.e., 1Rx or 2Rx;

- Reduced antenna gain/higher body loss, e.g. -3dB.

Based on the evaluation assumptions, the evaluation results for different channels are given in Figure 1.

Figure 1: Coverage Evaluation for RedCap UEs in Urban Macro

As illustrated in Figure 1, Both DL and UL coverage for REDCAP UEs are significantly reduced compared with normal UEs. More coverage loss is observed for DL, due to reduction on both Rx ant numbers and UE antenna gain. For UL, the coverage loss is brought about due to reduced antenna again only.

Observation 1: Both DL and UL coverage for REDCAP UEs are significantly reduced compared with normal UEs.

- Conclusion

In this contribution, we discuss the evaluation assumptions for coverage of RedCap, and provide the preliminary evaluation results, and have the following proposals and observations:

Proposal 1: To avoid duplicated discussion, basic evaluation methodology and assumptions in coverage enhancement can be considered as baseline for coverage recovery for RedCap UE.

Proposal 2: Following differences should be considered for coverage evaluation for RedCap UE

- Reduced bandwidth capability, e.g., up to 20MHz;

- Reduced number of Rx antennas, i.e., 1Rx or 2Rx;

- Reduced antenna gain/higher body loss, e.g. -3dB.

Observation 1: Both DL and UL coverage for REDCAP UEs are significantly reduced compared with normal UEs.

- References

- R1-2003919, “Assumptions for NR coverage evaluation”, vivo.

- R1-2003435, “Evaluation on NR coverage performance for FR1”, vivo.

- R1-2003433, “Discussion on functionality for coverage recovery”, vivo.