3GPP TSG RAN WG1 Meeting #101-e                                         R1-2003467

e-Meeting, May 25th – June 5th, 2020

Source: 	ZTE

Title: 	UE features of DL MIMO efficiency enhancements for LTE

Agenda item:	6.2.5.3

Document for:	Discussion/Decision

Introduction

After RAN1#100-e meeting, all the Rel-16 RAN1-led WIs have declared as finished. In RAN1#100bis-e and the post-meeting email discussion, a stable version of LTE UE features has been endorsed and send to RAN2 [1]. In this contribution, we give our views on remaining issues of LTE MIMO UE features after RAN1#100bis-e.

Discussion

3-1: Additional SRS symbols within normal UL subframes without frequency hopping

Regarding FG3-1, it is still FFS on how to capture the limitation that a UE may support additional SRS in cells with PUSCH, but not in PUSCH-less SCells. Since SRS carrier switching on additional SRS symbols in subframe N will affect the transmission of other signal on subframe N-1, N and N+1, but no rules are specified for the priority of different signals so far, we prefer not to support additional SRS in PUSCH-less SCells considering such late stage of Rel-16.

Proposal 1: For FG3-1, add a note saying that additional SRS is not supported in PUSCH-less SCells.

Conclusion

In this contribution, the following proposals on LTE Rel-16 MIMO UE features are given.

Proposal 1: For FG3-1, add a note saying that additional SRS is not supported in PUSCH-less SCells.

Reference

- [1] Chairman notes, RAN1#100bis-e