3GPP TSG RAN WG1 Meeting #101-e                                                                     R1-2003468

e-Meeting, May 25th – June 5th, 2020

Source:              ZTE

Title:                   Maintenance of CSI enhancement for MU-MIMO

Agenda Item:     7.2.6.1

Document for:   Discussion and Decision

# Introduction

In Rel-16, eType-II codebook and eType-II port selection codebook have been introduced to provide better performance-overhead trade-off over Rel-15 Type II codebooks. The specifications for the eType-II CSI introduction has been approved by RAN#86. In this contribution, we discuss the remaining issues on eType-II CSI.

# Remaining issues on CSI UE feature

CBSR

We have the following agreement in RAN1#98bis.

Agreement

For amplitude restriction mechanism:

- Alt 0. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient hard amplitude restriction)

  - Four beam groups are selected via higher-layer configured bitmap B1

  - For each spatial beam in each of the four beam groups, hard restriction (maximum amplitude of 0 or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps

- Alt 3A from RAN1#98 (soft with sum-power-ratio constraint), simplified to 1KNZ(λ,k,l0)×m=0M-1pres,l02λ,m,k≤γl0 for each (λ,k,l0) where KNZ(λ,k,l0) denotes the number of NZCs associated with (λ,k,l0).

  - The value of γl0 is configured from the Rel.15 2-bit amplitude restriction table

  - The number of beam-groups is the same as Rel.15 Type II CBSR

Support Alt0 as mandatory and Alt3A (described above) as optional analogous to Rel.15 Type II codebook.

- Cf. Notes in UE capability agreement in RAN1#98bis regarding “mandatory” and “optional”

Based on this agreement, CBSR without soft amplitude restriction should be basic feature, and CBSR with soft amplitude restriction should be an optional feature.

Proposal 1: CBSR without soft amplitude restriction is a basic feature, and CBSR with soft amplitude restriction is an optional feature.

Concurrent codebook capability

In Rel-15, UE can indicate the list of triplets {Max number of ports per resource, max number of resources, max number of total ports} per codebook type. However, more than one codebook type can be configured simultaneously. Hence UE reporting of the triplets needs to consider the worst case that multiple codebook types are configured simultaneously. It would cause UE to under report its capability if UE cannot report its capability to process multiple codebook types simultaneously. If there is such concurrent capability reporting, UE can be more aggressive to report its per codebook type capability.

The concurrent codebook capability would let UE to indicate the number of {max number of ports per resource, max number of resources, max number of total ports} for processing multiple codebook types simultaneously. Then the first issue is to identify those combinations of concurrent codebook types. As the minimum granularity of the triplets is per band, and different CCs would bring different gain for Type II/eType II codebook over Type I codebook, at least there would be the possibility of configuring Type I reporting in some CCs, and configuring Type II or eType II reporting in other CCs. Hence UE can report its concurrent codebook capability by reporting the following list of codebook combination {codebook 1, codebook 2}, and for each codebook combination, UE can report a list of triplets. In each codebook combination, codebook 1 is restricted to Type I SP or Type I MP codebook, and codebook 2 is restricted to Type II, Type II port selection, eType II and eType II port selection. For the remaining combinations, simultaneous configuration is not supported.

Proposal 2: For concurrent codebook capability, report a list of codebook combinations as {codebook 1, codebook 2}.

- Codebook 1 is one of Type I SP and Type I MP. Codebook 2 is one of Type II, Type II port selection, eType II and eType II port selection.

- For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}.

- For a codebook combination not included in the reported list of codebook combinations, simultaneous configuration of the codebooks are not supported.

# Conclusion

In this contribution, we discuss the remaining issues of eType II and eType II port selection CSI. The following proposals are given.

Proposal 1: CBSR without soft amplitude restriction is a basic feature, and CBSR with soft amplitude restriction is an optional feature.

Proposal 2: For concurrent codebook capability, report a list of codebook combinations as {codebook 1, codebook 2}.

- Codebook 1 is one of Type I SP and Type I MP. Codebook 2 is one of Type II, Type II port selection, eType II and eType II port selection.

- For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}.

- For a codebook combination not included in the reported list of codebook combinations, simultaneous configuration of the codebooks are not supported.

# References

[1] TS 38.214, Physical layer procedures for data, V16.1.0.

[2] TS 38.212, Multiplexing and channel coding, V16.1.0.