3GPP TSG RAN WG1 Meeting #101-e	R1-2003476

e-Meeting, May 25th – June 5th, 2020

Source: 	ZTE

Title: 	NR eMIMO UE features

Agenda item:	7.2.11.6

Document for:	Discussion/Decision

# Introduction

After RAN1#100bis-e meeting, a first consolidated version of RAN1 UE feature list has been sent to RAN2 in LS R1-2003073 [1]. In this contribution, we give our views on the remaining issues on NR eMIMO UE features based on the endorsed R1-2003073.

# Discussion on each feature group

In this section, each of the MIMO feature groups is discussed and the corresponding proposed updates are done in the UE feature table shown in the Appendix.

16-1 Multi-beam operation

The following includes more detailed comments for features of 16-1 multi-beam operation.

16-1a-1: SSB/CSI-RS for L1-SINR measurement

Firstly, the motivation of distinguishing 1-Tx CSI-RS and 2-Tx CSI-RS is unclear, considering that we also have the component “10. Supported density of CSI-RS (CMR)”. Secondly, we only need to consider either the maximum number of CMR/IMR separately (i.e. components 1,2,3), or the maximum number of CMR+IMR together (i.e. components 4 and 5). We don’t need both of them. Consequently, we have the following proposals for revising the components in 16-1a-1.

- The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR (including CMR and IMR) within a slot shall not exceed MB_1-3 - The max number of SSB/CSI-RS (1Tx)/CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR (including CMR and IMR) shall not exceed MC_1-3 - The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_2 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1

16-1b & 16-1b-2: TCI state activation and spatial relation update & spatial relation update across multiple CCs

Firstly, the title of “16-1b: TCI state activation and spatial relation update” should be replaced by “16-1b: TCI state activation and spatial relation update”, considering that it can be only applied to DL transmission.

Then, regarding two FFS parts, we still have concerns about the motivation of reporting which bands can share the same TCI state/spatial relation. Sharing the same TCI state or not is purely up to gNB configuration, but using different TCI state can be seen as a further UE capability enhancement over the Rel-15 UE that only supports single active TCI state for DL and/or UL transmission. Instead, we only need to discuss which bands can be used with different TCI state/spatial relation. For instance, we prefer to update two FFS components as “indicating band pairs that can be indicated with different DL TCI states” or “indicating band pairs that can be indicated with different UL spatial relation infos”, respectively.

16-1c:	Default spatial relation

In our companion contribution [2], we propose that the default spatial relation and path loss approach should be applied for PUSCH scheduled by DCI format 0_1 also. Meanwhile, we also need to consider that this approach can be also used for default path-loss RS determination in FR1. Consequently, we have the following proposal.

Proposal 1:

- The restriction that PUSCH transmission in FG [16-1c] only corresponds to that scheduled by DCI format 0_0 should be removed.

- FG [16-1c] can be applied for both FR1 and FR2, and in FR1 only default pathloss RS can be considered.

16-1d:	MAC CE spatial relation update for AP-SRS

Similar to the agreement for 16-1b, we prefer to update the component as "Support of spatial relation update for AP-SRS via MAC CE" as we did for other components for TCI/spatial relation update.

- For component 16-1d, the maximum number of spatial relation update for AP-SRS via MAC CE is based on the recent Component 2-59: Configured spatial relations.

16-1e: Pathloss reference RS activation via MAC CE

We prefer to have a fixed value for maximum number of activated pathloss reference RS(s) rather than according to UE capability. As a baseline, up to 4 activated pathloss reference RS(s) can be supported mandatorily without capability signaling, which is the same as in Rel-15. As a result, the component of “FFS: The maximum number of activated pathloss reference RS update for PUSCH/SRS/PUCCH [across CCs / within a slot across all CCs / per CC]” should be removed.

16-1f: FFS: Support of PUCCH-BFR

Firstly, the support of PUCCH-BFR shall be mandatory with capability signaling for FR2 and optional for FR1 (as we did for PCell-BFR in Rel-15), in order to guarantee the effectiveness/low-latency of SCell-BFR. Hence, this component 5 in 16-1f should be removed, and the UE supporting the basic feature group FG 16-1f should support both SR based and PUCCH based-BFR.

Then, regarding the component “FFS: Density of CSI-RS for new beam identification for Scell BFR”, we only need to reuse Rel-15 UE capability, and consequently it can be removed.

16-1g	Resources for beam management, [pathloss measurement, BFD, and BFR]

The aspects about pathloss measurement, BFD and BFR should be removed, and we do not need the further components about combining pathloss measurement, BFD or BFR cases, which has been reported through the other UE capability reporting independently. Consequently, we have the following proposals for revising the components in 16-1g.

- The maximum number of SSB/CSI-RS/[CSI-IM] resources [within a slot] across all CCs for any of L1-RSRP measurement, L1-SINR measurement, [pathloss measurement, BFD, and new beam identification] - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for pathloss measurement - FFS: The maximum number of SSB/CSI-RS resources within a slot across all CCs for BFD - FFS: The maximum number of SSB/CSI-RS resources across all CCs for new beam identification

16-2 Multi-TRP

In FG16-2a: Support of fully/partial overlapping in time

Regarding the component 4 about fully/partial overlapping in time in FG 16-2a, there were extensive discussion in RAN1#100bis-e meeting. From our view, it should be a basic component. Once Multi-DCI based MTRP is supported by UE, fully/partial overlapping in time should be supported. Otherwise, only DPS is supported and the basic FG of MDCI based MTRP has no much difference with Rel-15. For progress, we accept only non-overlapping in frequency in the basic component 4, and move partial/fully overlapping in frequency in another FG.

Proposal 2: Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency as a basic component of FG16-2a.

If proposal 2 is adopted, FG 16-2a-0 should be removed.

In NR Rel-15, for single TRP transmission, a UE capability signaling pdsch-ProcessingType1-DifferentTB-PerSlot to report the maximum number of TDMed PDSCHs in a slot as shown in the following table from 38.306, where the candidates include 2, 4 and 7.

pdsch-ProcessingType1-DifferentTB-PerSlot Defines whether the UE capable of processing time capability 1 supports reception of up to two, four or seven unicast PDSCHs for several transport blocks with PDSCH scrambled using C-RNTI, TC-RNTI, or CS-RNTI in one serving cell within the same slot per CC that are multiplexed in time domain only. Note PDSCH(s) for Msg.4 is included.

In case of multi-DCI based MTRP, PDSCHs scheduled by different TRPs in a slot can overlap in the time domain. It is natural to support more PDSCHs in one slot per CC. Straightforwardly, 2, 4, 8 and 14 PDSCHs across two TRPs can be supported corresponding to Rel-15 non-report of pdsch-ProcessingType1-DifferentTB-PerSlot, report values 2, 4 and 7 respectively. Therefore, in FG 16-2a, one new component is needed to report maximum number PDSCHs in the same slot per CC for MDCI based MTRP. Then UE will use the Rel-15 capability signaling to limit the number of PDSCHs in one slot for one TRP, and the new Rel-16 capability signaling to limit the number of PDSCHs for two TRPs.

Without this new component, if only the Rel-15 capability signaling is used to indicate the number of PDSCHs in one slot, the number of PDSCHs in one slot for M-DCI based M-TRP operation will be too restricted. Further, it is unclear for gNB how partial overlapping PDSCHs from different TRPs are taking into accounted to determine the actual scheduling only based on Rel-15 capability report.

Proposal 3: In FG 16-2a, one new component is added to indicate the maximum number of PDSCHs across two TRPs in the same slot per CC for M-DCI based M-TRP. The candidates can be {2, 4, 8, 14}.

In FG16-2a-2 and 16-2a-3: Support of out of order

Regarding FG16-2a-a and 16-2a-3 about out of order operation, we support the current structure that support of out-of-order operation for PDCCH and PDSCH and support of out-of-order operation for PDSCH to HARQ-ACK is in one FG. If UE only supports one of them, e.g. only support out-of-order operation for PDCCH and PDSCH but not support out-of-order operation for PDSCH to HARQ-ACK, separate scheduling between two TRPs are still impossible. Very tight coordination is still needed. Then, this is equivalent to the situation that UE supports neither of them. In RAN1#100bis-e, these two FGs are agreed as working assumption, we suggest confirming them in this meeting.

Proposal 4: Confirm FG16-2a-2 (support out-of-order operation for PDCCH and PDSCH; support out-of-order operation for PDSCH to HARQ-ACK) and 16-2a-3 (support out-of-order operation for PDCCH to PUSCH).

Regarding FG16-2a-8, this FG is only for FR2. The motivation is to report whether UE supports two different receive beams no matter PDSCH or PDCCH. So the FG should be for both PDCCH and PDSCH.

Regarding component 3 of FG16-2a, it overlaps with FG 16-2a-10. Then it should be removed.

Regarding FG 16-2b-1, support of DCI indication of 2 TCI states by a codepoint and DMRS ports within two CDM groups should be a basic component, ‘FFS’ should be removed from the first component. Regarding component 3, it should be a basic component as well, ‘FFS’ can also be removed. Further, two PTRS ports should be optional as what we agreed. Then this FG can be further split into two FGs, the first one is the support of single-DCI based SDM scheme, including two basic components, and the other one is the support of two PTRS ports.

16-3 eType II codebook

16-3a-4 CBSR

We have the following agreement in RAN1#98bis.

Agreement

For amplitude restriction mechanism:

- Alt 0. Analogous to Rel.15 Type II (SD beam group restriction + per coefficient hard amplitude restriction)

  - Four beam groups are selected via higher-layer configured bitmap B1

  - For each spatial beam in each of the four beam groups, hard restriction (maximum amplitude of 0 or 1) is applied to any of the coefficients associated with the beam (the restriction is applied for both polarizations of the beam). This maximum amplitude restriction is higher-layer configured with four bitmaps

- Alt 3A from RAN1#98 (soft with sum-power-ratio constraint), simplified to 1KNZ(λ,k,l0)×m=0M-1pres,l02λ,m,k≤γl0 for each (λ,k,l0) where KNZ(λ,k,l0) denotes the number of NZCs associated with (λ,k,l0).

  - The value of γl0 is configured from the Rel.15 2-bit amplitude restriction table

  - The number of beam-groups is the same as Rel.15 Type II CBSR

Support Alt0 as mandatory and Alt3A (described above) as optional analogous to Rel.15 Type II codebook.

- Cf. Notes in UE capability agreement in RAN1#98bis regarding “mandatory” and “optional”

Based on this agreement, CBSR without soft amplitude restriction should be a basic feature, and CBSR with soft amplitude restriction should be an optional feature.

Proposal 5: CBSR without soft amplitude restriction is a basic feature, and CBSR with soft amplitude restriction is an optional feature.

Basic components in 16-3a and 16-3b

In the current UE feature list, three components are listed in 16-3a and 16-3b. However, only the first one of each of the two FGs is needed. The current formulation may confuse RAN2 on their signaling design. Hence we think it’s better to revise 16-3a and 16-3b as follows.

| 16-3a | Regular eType-II | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 Note: The following are basic components for regular eType II, which do not require extra signaling: - Support of parameter combinations 1-6 - Support of rank 1,2 - CBSR with hard amplitude restriction |
| --- | --- | --- |
| 16-3b | Port selection eType-II | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 Note: The following are basic components for port selection eType II, which do not require extra signaling: - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 |

16-8 Concurrent codebook capability

In Rel-15, UE can indicate the list of triplets {Max number of ports per resource, max number of resources, max number of total ports} per codebook type. However, more than one codebook type can be configured simultaneously. Hence UE reporting of the triplets needs to consider the worst case that multiple codebook types are configured simultaneously. It would cause UE to under report its capability if UE cannot report its capability to process multiple codebook types simultaneously. If there is such concurrent capability reporting, UE can be more aggressive to report its per codebook type capability.

The concurrent codebook capability would let UE to indicate the number of {max number of ports per resource, max number of resources, max number of total ports} for processing multiple codebook types simultaneously. Then the first issue is to identify those combinations of concurrent codebook types. As the minimum granularity of the triplets is per band, and different CCs would bring different gain for Type II/eType II codebook over Type I codebook, at least there would be the possibility of configuring Type I reporting in some CCs, and configuring Type II or eType II reporting in other CCs. Hence UE can report its concurrent codebook capability by reporting the following list of codebook combination {codebook 1, codebook 2}, and for each codebook combination, UE can report a list of triplets. In each codebook combination, codebook 1 is restricted to Type I SP or Type I MP codebook, and codebook 2 is restricted to Type II, Type II port selection, eType II and eType II port selection. For the remaining combinations, simultaneous configuration is not supported.

Proposal 6: For concurrent codebook capability, report a list of codebook combinations as {codebook 1, codebook 2}.

- Codebook 1 is one of Type I SP and Type I MP. Codebook 2 is one of Type II, Type II port selection, eType II and eType II port selection.

- For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}.

- For a codebook combination not included in the reported list of codebook combinations, simultaneous configuration of the codebooks are not supported.

16-5 Power-efficient UL transmission

We have the following views on the detailed components in this feature group.

16-5b/5c:	Number of Tx to support mode 0/1/2: {2Tx, 4Tx, 2Tx_4Tx}

As a basic principle, for full Tx power transmission, the corresponding PUSCH power scaling factor and extension of UL codebook should be determined only according to the full Tx mode and TPMI-group reporting for full Tx power in mode2. The motivation for “16-5b/5c:	Number of Tx to support mode 0/1/2: {2Tx, 4Tx, 2Tx_4Tx}” is unclear with the following analysis, and those three components also restrict the flexibility of gNB configuration significantly.

- For mode 1, the scaling factor is the same as Rel-15, and only extension of the UL codebook is supported herein. It does not make sense that the extension of codebook is only applied to a specific number of Tx ports.

- For mode 2, this component is also redundant, considering that the component “16-5-6: TPMI group which delivers full power” can fully provide the UE capability of which TPMIs can support full power.

Component-4 in 16-5c: FFS: Number of ports per SRS resource

We are not sure why this component is needed. The motivation is not clear to us.

Component-5 in 16-5c: FFS: Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set

We think this component is needed. We can consider the candidate values of "1", "2" or "4" for this component.

Component-6 in 16-5c: TPMI group which delivers full power.

The relationship issue between UL-Tx-Switching and Mode-2 full power transmission was identified and discussed at the last RAN1 meeting, and herein we make further analysis about this issue.

- In our views, for full power transmission, it is assumed that the UE can always support 2-Tx port transmission; but for UL-Tx switching, it is assumed that the UE only can support the 2-Tx port transmission under the case that the Tx from carrier 1 is switched to carrier 2. Considering the latest RAN4 agreement that UE power class is fixed regardless of UL-Tx switching, in UL-Tx switching case, the power scaling factor for PUSCH transmission and enabling multiple SRS resources with different number of ports should be enhanced compared with Rel-15 from spec perspective, which are well aligned with Mode-2 TPMI group reporting and two SRS resources with different ports.

  - In a typical case, for PC3 UE, there are two PAs with 23 dBm for carrier 1 and carrier 2, respectively, and, when a Tx chain is switched from carrier 1 to carrier 2, the corresponding power class is also 23 dBm. The UE can support full power transmission, even in the case of the single non-zero power entry, e.g., [1, 0]. For spec perspective, the UE can report to support full power transmission under [1, 0] and [0, 1] using the bitmap of TPMI group reporting.

- Besides, this capability component 3 in FG16-5c, in addition to the components about TPMI group reporting and two SRS resources with different ports can also be applied to UL-Tx switching, in our views. For instance, in order to reduce the UE complexity of supporting additional SRS resources in mode 2, the UE can report to only support one SRS resource under mode 2 through indicating the value of 1 in the component 3 “The maximum number of SRS resources in set with different number of ports for usage set to‘codebook’” in FG16-5c. If the UE reports 2 SRS resources for this component, one SRS resource is 1-port for 1-port transmission and another SRS resource is 2-port for 2-port transmission when 2Tx is available for carrier 2. In case of 1-port transmission, full power can be supported with the {23dBm 23dBm} PA architecture although no virtualization is needed with 1Tx assumption.

Consequently, regardless of splitting the FG 16-5c or not, we do not see any difference for further applying FG 16-5c for UL-Tx-Switching feature, considering that TPMI reporting is an essential feature for UL-Tx-Switching (For 1-port PUSCH transmission, full power transmission is mandatorily supported in Rel-15).

Observation 1: Further applying FG 16-5c for UL-Tx-switching is not relevant to splitting the FG into 2 FGs.

Proposal 7: The following UE capability reporting in FG 16-5c can be applied for UL-Tx-switching.

- Supported UL full power transmission mode 2, i.e., multiple SRS resources with different number of SRS ports

- The maximum number of SRS resources in set with different number of ports for usage set to‘codebook’.

- TPMI group which delivers full power.

Regarding the case that UE does not support additional TPMI with full power, one entry of “NULL” (i.e., no further TPMI group is supported for full power) can be considered as a candidate value for TPMI group reporting. Consequently, in order to save the FG signaling, the single FG 16-5c seems to be sufficient.

Proposal 8: Do NOT support to further split FG 16-5c into 2 FGs in order to save the FG signaling overhead.

- One entry of “NULL” (i.e., no further TPMI group is supported for full power) can be considered as a candidate value for TPMI group reporting.

16-6 Low PAPR

Regarding FG 16-6b, the wording ‘and/or’ should be clarified. In our view, this FG is for both PUCCH format 3 and 4. So ‘/or’ can be removed.

# Conclusion

In this contribution, the following observation and proposals on NR Rel-16 MIMO UE features are given.

Observation 1: Further applying FG 16-5c for UL-Tx-switching is not relevant to splitting the FG into 2 FGs.

Proposal 1:

- The restriction that PUSCH transmission in FG [16-1c] only corresponds to that scheduled by DCI format 0_0 should be removed.

- FG [16-1c] can be applied for both FR1 and FR2, and in FR1 only default pathloss RS can be considered.

Proposal 2: Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency as a basic component of FG16-2a.

Proposal 3: In FG 16-2a, one new component is added to indicate the maximum number of PDSCHs across two TRPs in the same slot per CC for M-DCI based M-TRP. The candidates can be {2, 4, 8, 14}.

Proposal 4: Confirm FG16-2a-2 (support out-of-order operation for PDCCH and PDSCH; support out-of-order operation for PDSCH to HARQ-ACK) and 16-2a-3 (support out-of-order operation for PDCCH to PUSCH).

Proposal 5: CBSR without soft amplitude restriction is a basic feature, and CBSR with soft amplitude restriction is an optional feature.

Proposal 6: For concurrent codebook capability, report a list of codebook combinations as {codebook 1, codebook 2}.

- Codebook 1 is one of Type I SP and Type I MP. Codebook 2 is one of Type II, Type II port selection, eType II and eType II port selection.

- For each codebook combination, report a list of {max number of ports per resource, max number of resources, max number of total ports}.

- For a codebook combination not included in the reported list of codebook combinations, simultaneous configuration of the codebooks are not supported.

Proposal 7: The following UE capability reporting in FG 16-5c can be applied for UL-Tx-switching.

- Supported UL full power transmission mode 2, i.e., multiple SRS resources with different number of SRS ports

- The maximum number of SRS resources in set with different number of ports for usage set to‘codebook’.

- TPMI group which delivers full power.

Proposal 8: Do NOT support to further split FG 16-5c into 2 FGs in order to save the FG signaling overhead.

- One entry of “NULL” (i.e., no further TPMI group is supported for full power) can be considered as a candidate value for TPMI group reporting.

Based on these proposals, an updated UE feature list on MIMO is provided in the Appendix.

# Reference

R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T, NTT DOCOMO, INC

R1-2003470,	Maintenance of multi-beam operation, ZTE

# Appendix: NR_eMIMO

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16-1a-1 | SSB/CSI-RS for L1-SINR measurement | - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR within a slot shall not exceed MB_1 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-1 - The max number of NZP-IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR within a slot shall not exceed MB_1-2 - The max number of CSI-IM resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-1 - The max number of NZP IMR resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-SINR shall not exceed MC_1-2 - The max number of SSB/CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured as CMR to measure L1-SINR shall not exceed MC_1-3 - Supported density of CSI-RS (CMR) - The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-SINR (including CMR and IMR) shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a |  |  |  | Per band |  |  |  |  | Optional with capability signaling Component-1 to 4, candidate value set for MB_1 and MB_1-x is {0, 8, 16, 32, 64} Component-5 to 8, candidate value set for MC_1 and MC_1-x is {0, 4, 8, 16, 32, 64} Component-9, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-10: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} Component-11, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} |
|  | 16-1a-2 | Non-group based L1-SINR reporting | - Support of non-group based L1-SINR reporting with N_max L1-SINR values reported |  |  |  |  | Per band | [No] | [No] |  |  | Mandatory Mandatory with capability signaling Candidate value set is {1, 2, 4} FFS: default value equals 1 |
|  | 16-1a-3 | Group based L1-SINR reporting | - Support of group based L1-SINR reporting |  |  |  |  | Per band | [No] | [No] |  |  | Optional with capability signalling |
|  | 16-1b | TCI state activation and spatial relation update | - Support of Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - FFS: details on whether/how to indicate band pairs which can use the different DL TCI states | Component 1: 2-1, 2-4 |  | N/A |  | [Per BC or per band] | No | Yes |  |  | FFS |
|  | 16-1b-2 | Spatial relation update across multiple CCs | - Support of Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - FFS: details on whether/how to indicate band pairs which can use the different UL spatial relation infos | Component 1: 2-59, 2-60 |  | N/A |  | [Per BC or per band] | No | Yes |  |  | FFS |
|  | 16-1b-3 | Spatial relation update for PUCCH group | - Support of PUCCH resource groups per BWP for simultaneous spatial relation update | 2-53, 2-59, 4-24 |  | N/A |  | [Per BC or per band] | No | Yes |  |  | FFS |
|  | 16-1c | Default spatial relation | Support of default spatial relation and pathloss reference RS for dedicated-PUCCH/SRS and PUSCH | [Component 1: 2-20,] 2-53, 2-59 |  | N/A |  | [Per band] | [No] | No |  |  | FFS |
|  | 16-1d | MAC CE spatial relation update for AP-SRS | Support of spatial relation update for AP-SRS via MAC CE | 2-53, 2-59 |  | N/A |  | [Per UE] | No | [No] |  |  | FFS |
|  | 16-1e | Pathloss reference RS activation via MAC CE | - The maximum number of configured pathloss reference RSs for PUSCH/SRS by RRC for MAC-CE based pathloss reference RS update - FFS: Number of measurement samples N to apply newly activated pathloss reference RS | 8-2, 8-3 |  | N/A |  | [Per UE] | No | No |  |  | FFS |
|  | 16-1f | SCell beam failure recovery | - The maximum number of SCells [configured] for SCell beam failure recovery [simultaneously] - FFS: The maximum number of CSI-RS and/or SSB resources for new beam identification of SCell BFR [across all CCs / within a slot across all CCs / per CC] | 2-31 |  | N/A |  | FFS | No |  |  |  | FFS |
|  | 16-1g | Resources for beam management, | - The maximum number of SSB/CSI-RS/CSI-IM resources within a slot across all CCs for any of L1-RSRP measurement, L1-SINR measurement, |  |  | N/A |  | FFS | No |  |  |  | FFS |
|  | 16-2a | Multi-DCI based multi-TRP | - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - Support fully/partially overlapping PDSCHs in time and non-overlapping in frequency - The maximum number of PDSCH in a slot. Candidate values include {2, 4, 8, 14} | FFS |  | N/A |  | [per band / per FSPC] | No | FFS |  |  | FFS |
|  | 16-2a-1 | Simultaneous reception of Multiple PDSCHs overlapping in frequency | - Support fully overlapping PDSCHs at frequency and partially/fully overlapping at time - Support partially overlapping PDSCHs in frequency and partially/fully overlapping in time - The maximum number of MIMO layers of each scheduled PDSCH [if PDCCHs scheduling fully/partially/non-overlapped PDSCHs in time and frequency domain] - The maximal number of PDSCH scrambling sequences per serving cell FFS whether default values of component 3/4 to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-2 | Out-of-order operation for DL | 1. Support out-of-order operation for PDCCH to PDSCH 2. Support out-of-order operation for PDSCH to HARQ-ACK |  |  |  |  |  |  |  |  | Note: this FG is a WA in RAN1 |  |
|  | 16-2a-3 | Out-of-order operation for UL | 1. Support out-of-order operation for PDCCH to PUSCH |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP - separate | - Support of separate HARQ-ACK - [Support of two TDMed long PUCCHs in a slot] |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-4a | HARQ-ACK for multi-DCI based multi-TRP - joint | - Support of joint HARQ-ACK |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-5 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-6 | Default QCL enhancement for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-7 | Maximum number of activated TCI states | - Support of maximum per CORESETPoolIndex - Support of total maximum FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-8 | Simultaneous reception with different Type-D | whether UE supports receiving time-overlapping PDSCHs/PDCCHs with different Type-D FFS whether default values to be included in 16-2a |  |  |  |  |  |  |  |  |  |  |
|  | 16-2a-9 | [Simultaneous reception across CCs with Multi-DCI] | [The maximum number of CCs supporting multi-DCI based multi-TRP simultaneously] |  |  |  |  |  |  |  |  | Note: If the type of 16-2a is agreed to be FSPC or FS this FG will be removed |  |
|  | 16-2a-10 | Value of BD factor | R=[(1,2] for BD/CCE |  |  |  |  |  |  |  |  |  |  |
|  | 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | FFS |
|  | 16-2b-1 | Single-DCI based SDM scheme | - Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of DMRS entry {0, 2, 3} | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  |  | FFS |
|  | 16-2b-1a | Two PTRS ports for SDM | Support of two PTRS ports | 16-2b-1 |  |  |  |  |  |  |  |  |  |
|  | 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  |  | FFS |
|  | 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, FFS |  | N/A |  | [per FSPC] | No | FFS |  |  | FFS |
|  | 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  | Component 2 candidate values {10 CBs, FFS} | FFS |
|  | 16-2b-5 | Single-DCI based inter-slot TDM | - Support of MAC CE to activate two TCI states for a TCI codepoint - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, FFS |  | N/A |  | Per band | No | FFS |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, FFS} | FFS |
|  | 16-3a | Regular eType-II | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=1 Note: The following are basic components for regular eType II, which do not require extra signaling: - Support of parameter combinations 1-6 - Support of rank 1,2 - CBSR with hard amplitude restriction | FFS |  | N/A |  | FFS: Per band or Per band per BC | No | No |  |  | Optional |
|  | 16-3a-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support regular eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-2 | Support of parameter combinations 7-8 | Support of parameter combinations 7-8 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-3 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3a-4 | CBSR | CBSR with soft amplitude restriction (capture consequence if not supported hard amplitude restriction is supported) |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b | Port selection eType-II | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=1 Note: The following are basic components for port selection eType II, which do not require extra signaling: - 6 parameter combinations (combos with L=6 don’t apply) - Support of rank 1,2 | FFS |  | N/A |  | FFS: Per band or Per band per BC | No | No |  |  | Optional |
|  | 16-3b-1 | Support of PMI sub-bands with R=2 | {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} to support port selection eType-II for R=2 |  |  |  |  |  |  |  |  |  |  |
|  | 16-3b-2 | Support of rank 3,4 | Support of rank 3,4 |  |  |  |  |  |  |  |  |  |  |
|  | 16-4 | Low PAPR DMRS for DL | Low PAPR DMRS for PDSCH | FFS |  | N/A |  | [Per UE] | No | No |  |  | [Optional] |
|  | 16-5a | UL full power transmission [mode 0] | - Supported UL full power transmission [mode 0] | 2-13, 2-14 | Yes | N/A | Yes | FFS: Per FS or Per band or Per band per BC | N/A | N/A |  |  | FFS |
|  | 16-5b | UL full power transmission mode 1 | - Supported UL full power transmission mode 1 - [New UL codebook set(s) per supported Tx] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | No | No |  |  | FFS |
|  | 16-5c | UL full power transmission mode 2 | - Supported UL full power transmission mode 2 - The maximum number of SRS resources in set with different number of ports [for usage set to ‘codebook’]. FFS on details for supported number of Tx. - Maximum number of different spatial relation info for all SRS resources for usage set to ‘codebook’ in a resource set - TPMI group which delivers full power. FFS on details for supported number of Tx. Note: UE indicating mode 2 shall support full power transmission for 1 antenna port] | 2-13, 2-14 |  | N/A |  | FFS: Per FS or Per band or Per band per BC | No | No |  |  | FFS |
|  | 16-6a | Low PAPR DMRS for PUSCH without transform precoding | - For PUSCH without transform precoding | FFS | Yes | N/A | Yes | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-6b | Low PAPR DMRS for PUCCH | For PUCCH format 3 and PUCCH format 4, if capable with transform precoding and with pi/2 BPSK modulation | [FG 1-7, 4-4, 4-5] |  | N/A |  | FFS: Per band | No | No |  |  | FFS: Optional with capability signalling |
|  | 16-6c | Low PAPR DMRS for PUSCH with transform precoding and with pi/2 BPSK | For PUSCH with transform precoding and with pi/2 BPSK modulation | FFS | Yes | N/A | Yes | FFS: Per band | N/A | N/A |  |  | FFS: Optional with capability signalling |
|  | 16-7 | Extension of the maximum number of configured aperiodic CSI report settings | Extension of the maximum number of configured aperiodic CSI report settings for all codebook types |  |  |  |  | [Per band] |  |  |  |  |  |
|  | 16-8 | Active CSI-RS resources and ports for mixed codebook types in any slot | - A list of supported codebook combinations, which can be configured simultaneously, as {codebook 1, codebook 2}. - For each codebook combination in Component 1, report a list of {max number of ports per resource, max number of resources, max number of total ports}. |  |  |  |  |  |  |  |  |  |  |