3GPP TSG RAN WG1 Meeting #101-e	R1-2003514

E-meeting, 25 May- 5 June, 2020

Agenda Item:	7.2.2.2.3

Source:	Huawei, HiSilicon

Title:	Maintenance on HARQ-ACK enhancement

Document for:	Discussion and Decision

Introduction

In this contribution, we propose to fix the remaining issues of HARQ-ACK feedback for Rel-16 NR-U work item, including the issues for Type-2 HARQ-ACK codebook (Issue A in [2]), Type-3 HARQ-ACK codebook(Issue B in [2]).

Corrections on the Type-2 HARQ-ACK codebook

Single T-DAI for the non-scheduled group (Issue A7)

In R15, when CBG based transmission is configured, two sub-codebooks (one TB-based sub-codebook and one CBG-based sub-codebook) should always be generated as following:

If a UE

-	is provided PDSCH-CodeBlockGroupTransmission for  serving cells; and

-	is not provided PDSCH-CodeBlockGroupTransmission, for  serving cells where

…

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook

If the NFI-TotalDAI-Included-r16 is configured, how to apply the single T-DAI to these two sub-codebooks for the non-scheduled group is still an open issue. Some companies proposed that two separate T-DAI for the two sub-codebooks could be indicated in non-fallback DCI, however, CBG based transmission is configured per cell while the NFI-TotalDAI-Included-r16 is configured per UE, which means that there will be a lot of invalid indications when UE is not provided PDSCH-CodeBlockGroupTransmission for part of cells. A natural way is that gNB indicates the larger of T-DAI for these two sub-codebooks, and UE appends NACK(s) to the smaller one. This is effectively achieved by “setting Vtemp2=VDAIg+1mod2 for both sub-codebooks, if any”, as already specified in TS38.213 v16.1.0. A potential ambiguity remains in the interpretation of “if any”, which could be interpreted either as configuration of two sub-codebooks or detection of DCIs for both sub-codebooks in the set of monitoring occasions corresponding to the PUCCH occasion.

TP#1 for TS 38.213 Clause 9.1.3.3(on top of R1-2003180)

============================= Unchanged part omitted ============================

-	the PUCCH transmission occasion i(g+1mod2) is a last one for multiplexing second HARQ-ACK information and it is not after PUCCH transmission occasion i(g)

-	if VDAIg+1mod2≠∅,

- if the UE is provided PDSCH-CodeBlockGroupTransmission for  NcellsDL,CBG serving cells, and is not provided PDSCH-CodeBlockGroupTransmission for  NcellsDL,TB serving cells , after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2 for both sub-codebooks before appending the second sub-codebook to the first sub-codebook.

- Otherwise, after the completion of the c and m loops for the pseudo-code for the second HARQ-ACK codebook generation in Clause 9.1.3.1, set Vtemp2=VDAIg+1mod2.

============================= Unchanged part omitted ============================

Scell dormancy/SPS release with NNK1 value (Issue A11)

In last meeting, the following agreement was achieved:

Agreement:

- DCI format 1_1 should not simultaneously indicate a NNK1 value and activate a SPS configuration (CRC scrambled with CS-RNTI and NDI=0)

- DCI format 1_1 should not simultaneously indicate a NNK1 value and request feedback of Type-3 HARQ-ACK codebook (one-shot HARQ-ACK request field with value 1)

- FFS: DCI format 1_1 should not simultaneously indicate a NNK1 value and indicate Scell dormancy

- FFS: DCI format 1_1 should not simultaneously indicate a NNK1 value and indicate SPS release

The motivation for introducing the NNK1 value is to postpone the HARQ-ACK feedback for one scheduled PDSCH if there is no time for UE to report it within the current COT. After receiving the first DCI, UE could firstly decode the data, and wait for a second DCI. However, it is unreasonable for gNB to indicate SPS release or Scell dormancy with an NNK1, since for SPS release or Scell dormancy, no data can be scheduled. Sending a DCI with NNK1 to indicate SPS release or Scell dormancy means that UE should wait for a second DCI to indicate SPS release or Scell dormancy again, i.e. the first DCI is meaningless for UE, which should be precluded.

Proposal 1: DCI format 1_1 should not simultaneously indicate a NNK1 value and indicate Scell dormancy.

Proposal 2: DCI format 1_1 should not simultaneously indicate a NNK1 value and indicate SPS release.

TP#2 for TS 38.213 Clause 10.2(on top of R1-2003180)

============================= Unchanged part omitted ============================

A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if

-	the CRC of a corresponding DCI format is scrambled with a CS-RNTI provided by cs-RNTI, and

-	the new data indicator field in the DCI format for the enabled transport block is set to '0', and

-	the DFI flag field, if present, in the DCI format is set to '0', and

-	if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK.

============================= Unchanged part omitted ============================

TP#3 for TS 38.213 Clause 10.3(on top of R1-2003180)

============================= Unchanged part omitted ============================

If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if

-	the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1

-	if the PDSCH-to-HARQ_feedback timing indicator field in the DCI format is present, the PDSCH-to-HARQ_feedback timing indicator field does not provide an inapplicable value from dl-DataToUL-ACK.

the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception or indicating a SPS PDSCH release, and for transport block 1 interprets the sequence of fields of

============================= Unchanged part omitted ============================

UL DAI for a PDSCH group which is not scheduled (Issue A16)

We have agreed that if VDAIUL=4 for one group which is not scheduled for a UE, the UE does not multiplex HARQ-ACK information in the PUSCH transmission for that group. By this assumption, UE could differentiate whether PDCCH(s)/PDSCH(s) has been missed or not, i.e. if UE did not detected any DCI format scheduling PDSCH reception for a group, and UE received VDAIUL=4 for this group, UE knows that gNB does not expect any HARQ-ACK feedback information for this group, otherwise, UE knows that all the PDSCHs for this group are missed, and NACKs for this group should be reported according to the UL DAI indication. It has already been supported by the current specification, and there is no need for the correction. The NFI information for this group should be ignored.

Observation 1: No correction is needed for issue A16.

Power control (Issue A5)

The nHARQ-ACK definition for PUCCH power control is proposed in last meeting. In NR R15, for the case where OACK+OSR+OCSI<11 , if CBG based transmission is not configured, the UE determines a number of HARQ-ACK information bits nHARQ-ACK for obtaining a transmission power for a PUCCH as following:

nHARQ-ACK=nHARQ-ACK,TB=VDAI,mlastDL-c=0NcellsDL-1UDAI,cmodTDNTB,maxDL+c=0NcellsDL-1m=0M-1Nm,creceived+NSPS,c (1)

While if CBG based transmission is configured,  nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,CBG with

nHARQ-ACK,CBG=VDAI,mlastDL-c=0NcellsDL,CBG-1UDAI,cCBGmodTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1m=0M-1Nm,creceived,CBG

(2)

In NR-U, since the DAI are accumulated within each PDSCH group, and the SPS PDSCH does not belong to any group, if HARQ-ACK feedback for both groups are requested, and OACK+OSR+OCSI<11, UE should determine the number of HARQ-ACK information bits nHARQ-ACK for each group and SPS, separately. The TP is provided as following:

TP#4 for TS 38.213 Clause 9.1.3.3

============================= Unchanged part omitted ============================

If q=0, the UE

includes only the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

elseif q=1

if g = 1

appends the first HARQ-ACK information to the second HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

else

append the second HARQ-ACK information to the first HARQ-ACK information for multiplexing in PUCCH transmission occasion i(g)

end if

end if

If a UE is not provided PDSCH-CodeBlockGroupTransmission for each of the  serving cells, or for PDSCH receptions scheduled by a DCI format that does not support CBG-based PDSCH receptions, or for SPS PDSCH reception, or for SPS PDSCH release, and if , the UE determines a number of HARQ-ACK information bits  for obtaining a transmission power for a PUCCH, as

nHARQ-ACK=nHARQ-ACK,TB=g=01VDAI,mlastDL(g)-c=0NcellsDL-1UDAI,c(g)modTDNTB,maxDL+c=0NcellsDL-1g=01m=0M-1Nm,creceived+NSPS,c

where

- NSPS,c is the number of SPS PDSCH receptions by the UE on serving cell  for which the UE transmits corresponding HARQ-ACK information in the PUCCH

- NTB,maxDL is defined in clause 9.1.3.1

- Nm,creceived is defined in clause 9.1.3.1 with m and M counted separately for each PDSCH group

- VDAI,mlastDLg,and UDAI,c(g) are defined in clause 9.1.3.1 except that the numbers are counted separately for each PDSCH group. If  VDAIg+1mod2≠∅, VDAI,mlastDL(g+1mod2)= VDAIg+1mod2.

If a UE

- is provided PDSCH-CodeBlockGroupTransmission for  serving cells; and

- is not provided PDSCH-CodeBlockGroupTransmission, for  serving cells where

If , the UE also determines  for obtaining a PUCCH transmission power, as described in Clause 7.2.1, with

nHARQ-ACK,CBG=g=01VDAI,mlastDL(g)-c=0NcellsDL,CBG-1UDAI,cCBG(g)modTDNHARQ-ACK,maxCBG/TB,max+c=0NcellsDL-1g=01m=0M-1Nm,creceived,CBG

where

- NHARQ-ACK,maxCBG/TB,max is defined in clause 9.1.3.1

- Nm,creceived, CBG is defined in clause 9.1.3.1 with m and M counted separately for each PDSCH group

- VDAI,mlastDLg  and UDAI,cCBG(g)are defined in clause 9.1.3.1 except that the numbers are counted separately for each PDSCH group. If  VDAIg+1mod2≠∅, VDAI,mlastDL(g+1mod2)= VDAIg+1mod2.

============================= Unchanged part omitted ============================

Corrections on the Type-3 HARQ-ACK codebook

One-shot HARQ-ACK request without PDSCH scheduling (Issue B1)

FDRA value for DCI not scheduling PDSCH (Issue B1-1)

In last meeting, it was discussed whether simultaneously triggering the Type-3 HARQ-ACK codebook and SPS release can be supported or not. Besides, the FDRA field values that indicate PDCCH validation for DL SPS with DCI format 1_1 has been changed as below [7]:

Table 10.2-2: Special fields for single DL SPS or single UL grant Type 2 scheduling release PDCCH validation when a UE is provided a single SPS PDSCH or UL grant Type 2 configuration in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| HARQ process number | set to all '0's | set to all '0's |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

Table 10.2-4: Special fields for a single or multiple DL SPS and UL grant Type 2 scheduling release PDCCH validation when a UE is provided multiple DL SPS or UL grant Type 2 configurations in the active DL/UL BWP of the scheduled cell

|  | DCI format 0_0/0_1/0_2 | DCI format 1_0/1_1/1_2 |
| --- | --- | --- |
| Redundancy version | set to all '0's | set to all '0's |
| Modulation and coding scheme | set to all '1's | set to all '1's |
| Frequency domain resource assignment | set to all '0's for FDRA Type 2 with μ=1 set to all '1's, otherwise | set to all '0's for FDRA Type 0 or for dynamicSwitch set to all '1's for FDRA Type 1 |

It can be found that the FDRA field values for SPS release PDCCH validation and Type-3 HARQ-ACK codebook request without scheduling PDSCH are basically the same except when resource allocation scheme is configured as “dynamicSwitch”. To avoid the ambiguity between these two cases, the UE should ignore the value of one-shot HARQ-ACK request field when the DCI signals a validation for DL SPS (Alt-2), and Type-3 HARQ-ACK codebook should not be triggered by DCI with CS-RNTI.

Proposal 3: UE should ignore the value of one-shot HARQ-ACK request field when the DCI signals a validation for DL SPS.

Proposal 4: Type-3 HARQ-ACK codebook request without PDSCH scheduling should not be triggered by DCI with CS-RNTI.

TP#5 for TS 38.212 Clause 10.2

============================= Unchanged part omitted ============================

If validation is achieved, the UE considers the information in the DCI format as a valid activation or valid release of DL SPS or configured UL grant Type 2. If validation is not achieved, the UE discards all the information in the DCI format.

If validation for release of DL SPS is achieved, the UE ignores the value of One-shot HARQ-ACK request field, if any.

============================= Unchanged part omitted ============================

Spatial bundling for Type-3 HARQ-ACK codebook (Issue B2)

Considering the spatial bundling in Type-3 HARQ-ACK codebook, the following alternatives were discussed：

- Alt1: spatial bundling is not applied to type-3 HARQ-ACK codebook even if harq-ACK-SpatialBundlingPUCCH is provided.

- Alt2: spatial bundling is applied to type-3 HARQ-ACK codebook when harq-ACK-SpatialBundlingPUCCH is provided, with AND operation for 2 TBs.

- Alt3: spatial bundling is applied to type-3 HARQ-ACK codebook when harq-ACK-SpatialBundlingPUCCH is provided, with AND operation for 2 TBs and with AND operation for 2 NDIs corresponding to the 2 TBs.

For Alt2 and Alt3, if a UE is provided PDSCH-CodeBlockGroupTransmission for  NcellsDL,CBG serving cells, the spatial bundling could be applied only for the TB based sub-codebook since spatial bundling is not applicable for CBG-based HARQ-ACK reporting in R15, and the reduction of HARQ-ACK feedback overhead is negligible. Furthermore, Type-3 HARQ-ACK codebook is designed to provide a robust fallback mechanism, and the overhead is not a key issue. There is no strong motivation to enable the spatial bundling for Type-3 HARQ-ACK codebook. The following TP is provided:

TP#6 for TS 38.213 Clause 9.1.4

============================= Unchanged part omitted ============================

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c.harq-ACK-SpatialBundlingPUCCH is not applicable.NTB,cDL=1

============================= Unchanged part omitted ============================

Collisions between SPS-release ACK and Type-3 HARQ-ACK codebook feedback (Issue B6)

For SPS PDSCH release, no PDSCH is scheduled and no HARQ process is allocated. If a UE is triggered to report both Type-3 HARQ-ACK codebook and HARQ-ACK information for SPS PDSCH release in the same slot, the HARQ-ACK bit corresponding to the SPS PDSCH release could be appended to the Type-3 HARQ-ACK codebook. To avoid the misalignment of codebook size between gNB and UE (UE may miss the SPS release PDCCH), one bit at the end of Type-3 codebook could be reserved for SPS PDSCH release.

Proposal 5: One bit at the end of Type-3 codebook could be reserved for SPS PDSCH release.

TP#7 for TS 38.213 Clause 9.1.4

============================= Unchanged part omitted ============================

end while

h=0

c=c+1

end while

j=j+1

if the UE receives a PDCCH indicating SPS PDSCH release and indicating a same slot for Type-3 codebook transmission by PDSCH-to-HARQ_feedback timing indicator field

= ACK

else

= NACK

If a UE receives a SPS PDSCH, or a PDSCH that is scheduled by a DCI format 1_0 for a serving cell c and if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c, and pdsch-HARQ-ACK-OneShotFeedbackCBG-r16 is provided, the UE repeats NHARQ-ACK,cCBG/TB,max times the HARQ-ACK information for the transport block in the PDSCH.

============================= Unchanged part omitted ============================

Conclusion

Based on the discussion, we have the following observation:

For the enhanced Type-2 HARQ-ACK codebook,

Proposal 1: DCI format 1_1 should not simultaneously indicate a NNK1 value and indicate Scell dormancy.

Proposal 2: DCI format 1_1 should not simultaneously indicate a NNK1 value and indicate SPS release.

Observation 1: No correction is needed for issue A16.

For the Type-3 HARQ-ACK codebook,

Proposal 3: UE should ignore the value of one-shot HARQ-ACK request field when the DCI signals a validation for DL SPS.

Proposal 4: Type-3 HARQ-ACK codebook request without PDSCH scheduling should not be triggered by DCI with CS-RNTI.

Proposal 5: One bit at the end of Type-3 codebook could be reserved for SPS PDSCH release.

References

- RAN1 Chairman’s Notes, 3GPP TSG RAN1 Meeting#100bis-e, April 20 – 30, 2020

- R1-2001043, Feature lead summary#1 on NR-U HARQ and multi-PUSCH scheduling	, Huawei, February 24 - March 6, 2020

- R1-2001191, Feature lead summary#2 on NR-U HARQ and multi-PUSCH scheduling, Huawei, February 24 - March 6, 2020

- 3GPP TS38.213 v 16.1.0

- 3GPP TS38.212 v 16.1.0

- R1-2002696 Feature lead summary#1 on NR-U HARQ	Moderator (Huawei) April 20 – 30, 2020

- R1-2003176, Corrections on Ultra Reliable Low Latency Communications Enhancements, Samsung, April 20 – 30, 2020