3GPP TSG RAN WG1 Meeting #101-e	R1-2003534

E-meeting, 25 May - 5 June, 2020

Agenda Item:	7.2.6.5

Source:	Huawei, HiSilicon

Title:	Remaining issues on low PAPR sequence in R16

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining issue for the low PAPR RS in TS 38.211 [1]

Correction on Low PAPR RS for PUSCH

In current spec, there is a missing case for determining the nIDRS: “the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH”.

According to the previous agreement, nIDRS=NIDnSCID is only for the case dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is used. So, for the missing case, Rel-15 behaviour is applied. To fix the missing case, we have the following text proposal:

Text proposal for Section 6.2.3.1 of 38.214:

-----------------------------------------------------------------------------------------------------------------------------

6.4.1.1.1.2	Sequence generation when transform precoding is enabled

< Unchanged parts are omitted >

The sequence group u=fgh+nIDRS mod 30, where nIDRS is given by

-	nIDRS=nIDPUSCH if nIDPUSCH is configured by the higher-layer parameter nPUSCH-Identity in the DMRS-UplinkConfig IE, the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is not configured, or the higher-layer parameter dmrsUplinkTransformPrecoding-r16 is configured and π/2-BPSK modulation is not used for PUSCH, and the PUSCH is neither scheduled by RAR UL grant nor scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI according to clause 8.3 in [5, TS 38.213];

-	nIDRS=NIDnSCID if the higher-layer parameter dmrs-UplinkTransformPrecoding-r16 is configured, π/2-BPSK modulation is used for PUSCH, the PUSCH transmission is not a msg3 transmission, and the transmission is not scheduled using DCI format 0_0 in a common search space;

-	nIDRS=NIDcell otherwise

< Unchanged parts are omitted >

------------------------------------------------------------------------------------------------------------------------------

- References

3GPP, TS 38.211 v16.1.0.