3GPP TSG RAN WG1 Meeting #101                                   R1-2003605

e-Meeting, May 25th – June 5th , 2020

Source:	CATT

Title:	Discussion of UE features for NR V2X

Agenda Item:	7.2.11.4

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on UE features for 5G_V2X_NRSL based on the latest version[1].

- Discussion

FG 15-1: Transmitting NR sidelink mode 1 scheduled by NR Uu

FG 15-11: PSFCH format 0

Since the component-4 in FG15-11(Sidelink HARQ-ACK forwarding to gNB) is only related to mode 1 resource allocation, then it is better to move it into 15-2.

Proposal 1: The component-4(Sidelink HARQ-ACK forwarding to gNB) in FG 15-11 shall be moved to FG15-2.

FG 15-5: Sidelink congestion control

Regarding to the mandatory/optional, since sidelink congestion control is a resource pool specific configuration, if congestion control is enabled for a resource pool, and UEs supporting congestion control and UEs not supporting congestion control share this pool, it is unfair to the UEs supporting congestion control. Therefore, this feature shall be a basic FG.

Regarding to the component 1(CBR measurement report to gNB), it is only related to Uu connection, it is unnecessary to bundle with traditional sidelink congestion control. From this point of view, it is better to remove it from 15-5. Component 1 could be either a separate FG or similar restriction as that in 15-1/15-3, that means component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1

Proposal 2: FG 15-5(sidelink congestion control) shall be a basic FG.

Proposal 3: CMR measurement report to gNB(component-1) could be either a separate FG or similar restriction as that in 15-1/15-3, that means component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1

FG 15-6: Short-term time-scale TDM for in-device coexistence

Regarding the FFS part on whether define a set of candidate processing time value for inter-RAT collision resolution. We think it is better for UE to report the processing time, otherwise it would be difficult to test the UE’s behaviour.

Proposal 4: In FG15-6, a new component shall be introduced for reporting UE processing time

- Conclusion

In this contribution, the UE features for 5G_V2X_NRSL are discussed. Particularly, we have following proposals:

Proposal 1: The component-4(Sidelink HARQ-ACK forwarding to gNB) in FG 15-11 shall be moved to FG15-2.

Proposal 2: FG 15-5(sidelink congestion control) shall be a basic FG.

Proposal 3: CMR measurement report to gNB(component-1) could be either a separate FG or similar restriction as that in 15-1/15-3, that means component 1 is not required to be supported in a band indicated with only the PC5 interface in 38.101-1 Table 5.2E-1

Proposal 4: In FG15-6, a new component shall be introduced for reporting UE processing time

- References

- R1-2003073, “RAN1 UE features list for Rel-16 NR after RAN1#100bis-E”, AT&T, NTT DOCOMO