3GPP TSG RAN WG1 Meeting #101                                      R1-2003606

e-Meeting, May 25th – June 5th, 2020

Source:	CATT

Title:	Discussion of UE features for NR URLLC/IIoT

Agenda Item:	7.2.11.5

Document for:	Discussion and Decision

- Introduction

NR Rel-16 UE features were discussed in RAN1#100bis-e meeting and an LS was agreed in [1]. Email discussions were carried out after the meeting and the updated UE features were endorsed as baseline for further discussion. In this contribution, we provide our views on UE features for NR URLLC and IIoT, i.e. FG 11-x and FG 12-x based on [2][3].

- Discussion

FG 11-2: Rel-16 PDCCH monitoring capability

The processing capability is defined on the basis of the same number of PDCCH blind detections, i.e. capability#1 needs more time to processing the control channel, data channel and corresponding feedback while capability#2 can do it faster. There is no reason to define separate UE capability C(X,Y,μ)/m(X,Y,μ) for different processing capability. It should be noted that we don’t differentiate the UE capability when we define the per slot limit on the maximum number of non-overlapped CCE and PDCCH candidate.

The square bracket for component 2 should be removed as it addresses the rule for how to determine the maximum number of M/C if more than one (X,Y, μ) is configured.

At last meeting, the following agreements related to URLLC PDCCH were agreed:

Agreements: For enhanced PDCCH monitoring capability,  For limit C on the maximum number of non-overlapping CCEs for channel estimation per PDCCH monitoring span, - The value of C for combination (4, 3) for 15 kHz and 30 kHz is 36. - The value of C for combination (2, 2) for 15 kHz and 30 kHz is 18 [18 or 16 or 24].  For limit M on the maximum number of monitored PDCCH candidates per monitoring span, - For 15 kHz, - The value of M03 for combination (7, 3) is 44 - The value of M02 for combination (4, 3) is 28 - The value of M01 for combination (2, 2) is 14 [12 or 14] - For 30 kHz, - The value of M13 for combination (7, 3) is 36 - The value of M12 for combination (4, 3) is 24 - The value of M11 for combination (2, 2) is 12 [10 or 12]  For the case with Rel-16 monitoring capability only on all the serving cells (i.e. case 2), - Minimum of pdcch-BlindDetectionCA-R16 = 2 - Candidate values for pdcch-BlindDetectionCA-R16 is 2 to 16 Agreements: For the case with Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells (i.e. case 3), UE will report one or more combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) as UE capability.  If UE reports more than one combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16), - gNB configure which combination for UE to use for scaling PDCCH monitoring capability if the number of CCs configured is larger than the reported capability Agreements: For one reported combination of (pdcch-BlindDetectionCA-R15, pdcch-BlindDetectionCA-R16) for CA:  The minimum value of pdcch-BlindDetectionCA-R15 is 1 and the minimum value of pdcch-BlindDetectionCA-R16 is 1  [3]<=pdcch-BlindDetectionCA-R15 + pdcch-BlindDetectionCA-R16 <=16  Candidate values for pdcch-BlindDetectionCA-R15 is 1 to 15  Candidate values for pdcch-BlindDetectionCA-R16 is 1 to 15

The second last column should be updated accordingly, which is reflected in Table1. It should also be noted that the minimum number of Rel-16 carriers is different for different cases, i.e. Rel-16 only or Rel-15/Rel-16 mixed scenarios. It should also be reflected in the second last column.

FG 11-3: More than one PUCCH for HARQ-ACK transmission within a slot

There is an FFS on whether to keep component 3) which defines the minimum gap between sub-slots containing actual PUCCH transmissions. The motivation seems to be that a UE may support sub-slot configuration of 2-symbol*7 but does not support PUCCH transmission in every sub-slot. The necessity of this component is not clear to us thus we prefer to remove component 3 in FG 11-3.

FG 11-4 & 11-4x: Two HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE

FG 11-4 and 11-4x cover two HARQ-ACK codebooks with different priorities. It is our understanding that the two HARQ-ACK codebooks can overlap in time and the PUCCH carrying HARQ-ACK with higher priority should be prioritized. In addition, for a UE supports FG 11-4 and/or 11-4x but does not support FG 12-1, when the PUCCH carrying HARQ-ACK with high priority overlaps in time with other UL channel/signal including PUSCH, SRS etc., the UE should prioritize the PUCCH carrying HARQ-ACK with high priority. Consequently, a UE supports FG 11-4 and/or 11-4x should also support prioritization between UL channels/signals with different PHY priority levels and the prioritization/cancellation timelines as defined in FG 12-1.

FG 11-7: UL cancelation scheme for self-carrier

The description under component 3) has been captured in the physical specification and our understanding is that it should be followed once UL cancellation is supported. It doesn’t need to be included in the UE capability.

Whether more than one monitoring occasions for DCI format 2_4 per slot is applied depends on the FG 3-5 or FG 3-5a or FG 3-5b. We don’t see the necessity to add new FG with FG 11-7 as prerequisite for the support of more than one monitoring occasion for DCI format 2-4 per slot. The square bracket should be removed accordingly.

FG 11-7a: UL cancelation scheme for cross-carrier

The description under component 3) has been captured in the physical specification and our understanding is that it should be followed once UL cancellation is supported. It doesn’t need to be included in the UE capability.

Whether more than one monitoring occasions for DCI format 2_4 per slot is applied depends on the FG 3-5 or FG 3-5a or FG 3-5b. We don’t see the necessity to add new FG with FG 11-7 as prerequisite for the support of more than one monitoring occasion for DCI format 2-4 per slot. The square bracket should be removed accordingly.

FG 12-1: UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer

It is our understanding that FG 12-1 includes intra-UE multiplexing/prioritization of all the UL overlapping channels/signals instead of PUCCH/PUCCH and PUCCH/PUSCH only. For a UE supports FG 12-1, the UE should support LCH based prioritization in MAC. In addition, a UE may support FG 12-1 but not support FG 11-4 for example a UE supports UL URLLC service only.

FG 12-2: Multiple SPS configurations

It was agreed in RAN2 that up to 32 SPS configurations are supported per MAC entity. It is proposed to update the maximum number of SPS configurations according to RAN2’s agreements.

# Meeting agreements:

- [026] Not to introduce restrictions of how many SPS configurations are supported, e.g. per cell/ per UE (SPS/CG).

- [026] No need to capture limitation of maximum CG/SPS configurations per MAC entity in TS 38.300.

- [026] Support up to 32 SPS configurations per MAC entity.

- [026] SPS-Config and SPS-ConfigList in BWP-DownlinkDedicated cannot be configured simultaneously at a given time.

- [026] ConfiguredGrantConfig and ConfiguredGrantConfigList in BWP-UplinkDedicated cannot be configured simultaneously at a given time.

Based on the above analyses, we provide the suggested modifications on the UE feature lists for NR_L1enh_URLLC and NR_IIOT as shown in Table 1 and Table 2.

Proposal: Adopt the updates corresponding to UE feature lists for NR_L1enh_URLLC and NR_IIOT included in Table 1 and Table 2.

# Table1: NR_L1enh_URLLC

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | - Supports monitoring DCI format 1_2 for DL scheduling - Supports monitoring DCI format 0_2 for UL scheduling |  | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | - Supports monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-2 | Rel-16 PDCCH monitoring capability | - Supported combination(s) of (X, Y, ). For each reported combination, the UE supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span - [If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the combination (X, Y) with the maximum value of C and M from the valid combinations is applied] - Capability on the number of CCs for monitoring a maximum number of BDs and non-overlapped CCEs per span when configured with DL CA with Rel-16 PDCCH monitoring capability on all the serving cells. | 3-5b (TBD) | Yes | N/A |  | [FSPC] FFS: Compoent 3) reported per UE | [N/A] | [N/A] | [N/A] | This capability is signalled for SCS 15 kHz and 30 kHz. For =0 and 1, candidate value set for (X, Y, ): {(7, 3, ), (4, 3, ), (2, 2, )} For component 1, a list of separate UE capabilities (X, Y, )for processing capability #1; For component 1, a list of separate UE capabilities (X, Y, )for processing capability #2; For component 3, if UE supports carrier aggregation with more than 1 or 2 DL carriers with Rel-16 PDCCH monitoring capability on all the carriers, UE should report this capability. Candidate value for component 3: {2, 3, …, 16} if Rel-16 monitoring capability only on all the serving cells; { 1,2, 3, …, 15} if Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells; | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | - Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. - Supported sub-slot configuration - [Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols] |  | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | Candidate value set for component 2: { 7-symbol*2, 2-symbol*7 and 7-symbol*2} for NCP or { 6-symbol*2, 2-symbol*6 and 6-symbol*2} for ECP [Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (7, 7)}] FFS: Whether to keep component 3) and accordingly the above note for component 3) FFS “no more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot” for multi-TRP support” | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook (i.e. slot-based + slot-based, or slot-based + sub-slot based) simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE | - Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. - Supports separate PUCCH configuration for different HARQ-ACK codebooks - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - [Supports a DCI format (from the formats 1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP] - Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. - [Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission | [11-3] (TBD) | Yes | N/A |  | FFS [Per UE or Per FS] | [No] | [No] | [N/A] | [If a UE reports both 11-3 and 11-4, it can support two slot-based HARQ-ACK codebooks, and one slot-based and one-sub-slot-based HARQ-ACK codebooks. If a UE reports 11-4 but not 11-3, it can only support two slot-based HARQ-ACK codebooks.] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-4a | Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting HARQ-ACK codebooks with different priorities at a UE | - Supports two sub-slot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. - Supports separate PUCCH configuration for different HARQ-ACK codebooks - Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. - [Supports a DCI format (from the formats /1_1/1_2) scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP] - Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. - [Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission | [11-3] and [11-4] (TBD) | Yes | N/A |  | FFS [Per UE or Per FS] | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | [11-4b] | [DL priority indication in DCI with mixed DCI formats] | [DL priority indication in DCI with mixed DCI formats] | 11-1a, 11-4 (TBD) | Yes | N/A |  | Per UE | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-6 | PUSCH repetition Type A | - PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be dynamically indicated (as agreed for Rel-16). • When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. | [5-17] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | FFS: Whether to add a component for the supported maximum number of PUSCH repetitions | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7 | UL cancelation scheme for self-carrier | - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication [on the same DL CC as that scheduling PUSCH or SRS] - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols [For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [N/A] | [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-7a | UL cancelation scheme for cross-carrier | - Supports group common DCI (i.e. DCI format 2_4) for cancelation indication [on a different DL CC than that scheduling PUSCH or SRS] - UL cancelation for PUSCH - Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions - UL cancelation for SRS symbols that overlap with the cancelled symbols [For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [N/A] | [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | [11-7b] | [Independent cancellation of the overlapping PUSCHs in an intra-band UL CA] | [For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to.] | 6-23, 11-7 (TBD) | Yes | N/A |  | [Per band] | [N/A] | [N/A] | TBD | FFS: Whether to add this FG and the content for each column if added | Optional with capability signaling |
| 11. NR_L1enh_URLLC | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | - Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations - [Supported maximum number of configured/active configured grant configurations in a BWP of a serving cell] - [Supported maximum number of configured/active configured grant configurations across all serving cells in a cell group] | TBD FFS: 5-19 or 5-20 | Yes | N/A |  | [Per UE] FFS: FSPC | [No] | [No] | [N/A] |  | Optional with capability signalling FFS: Candidate value for component 2: {1, 2, …, 12} FFS: Candidate value for component 3: {2, …, [32]} |
| 11. NR_L1enh_URLLC | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 (TBD) | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | FFS: A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1). A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2). | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-10 | Type 2 configured grant release by DCI format 0_1 | - Support of type 2 configured grant release by DCI format 0_1 | 5-20 | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] FFS: The capability interpretation is from the perspective of a carrier on which the release DCI is received | [A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2).] | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-11 | Type 2 configured grant release by DCI format 0_2 | - Support of type 2 configured grant release by DCI format 0_2 | 5-20, 11-1 | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] FFS: The capability interpretation is from the perspective of a carrier on which the release DCI is received | [A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1).] | Optional with capability signalling |

# Table 2: NR_IIOT

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL overlapping PUCCH/PUCCH and PUCCH/PUSCH channels/signals with two priority levels in physical layer (PHY) - [Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format] - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission | [11-4] | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} [A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]).] The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4 and 11-4xshould be further discussed. | Optional with capability signaling |
| 12. NR_IIOT | [12-1a] | [UL priority indication in DCI with mixed DCI formats] | [UL priority indication in DCI with DCI format 0_1 and 0_2] | 12-1 and 11-1 TBD | Yes | N/A | FFS | Per UE | [No] | [No] | [N/A] |  | Optional with capability signalling |
| 12. NR_IIOT | 12-2 | Multiple SPS configurations | - Support of up to 8 configured SPS configurations in a BWP of a serving cell and up to [16]32 configured SPS configurations in a cell group, including separate RRC parameters and separate activation/release for different SPS configurations - The max number of active SPS configurations in a BWP of a serving cell - The max number of active SPS configurations across all serving cells - The related HARQ-ACK enhancements to support multiple active SPS configurations | 5-18 DL SPS | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] | Component-2, candidate value set is {1, 2, …, 8} Component-3, candidate value set is [{2, …, [16]32}] | Optional with capability signaling |
| 12. NR_IIOT | 12-2a | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication - The related HARQ-ACK enhancements to support joint release | 12-2 | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 12. NR_IIOT | 12-3 | SPS release by DCI format 1_1 | Support of SPS release by DCI format 1_1 | 5-18 DL SPS | Yes | N/A |  | [Per UE] | [No] | [No] | [TBD] | [A UE supporting this FG and 11-1 (DCI format 0_2/1_2) shall also support FG12-3a (SPS release by DCI format 1_2).] | Optional with capability signaling |
| 12. NR_IIOT | 12-3a | SPS release by DCI format 1_2 | Support of SPS release by DCI format 1_2 | 5-18 DL SPS and 11-1 | Yes | N/A |  | [Per UE] | [No] | [No] | [TBD] |  | Optional with capability signaling |
| 12. NR_IIOT | 12-5 | Configuration of aggregation factor per SPS configuration | Support of configurable PDSCH aggregation factor ({1, 2, 4, 8}) per DL SPS configuration | 5-18 DL SPS | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signaling |
| 12. NR_IIOT | 12-6 | Support of SPS periodicity shorter than 10 ms | Support of SPS periodicity shorter than 10 ms | 5-18 DL SPS | Yes | N/A |  | [Per UE] | [No] | [No] | [N/A] |  | Optional with capability signalling |

- Conclusion

In this contribution, we provided our view on UE features for URLLC and IIoT and proposed updates are provided.

Proposal: Adopt the updates corresponding to UE feature list for NR_L1enh_URLLC and NR_IIOT included in Table 1 and Table 2.

- Reference

- R1-2003073, RAN1 UE features list for Rel-16 NR after RAN1#100bis-E, AT&T, NTT DOCOMO, RAN1#100bis-e

- R1-2003199, Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_L1enh_URLLC, Moderator (NTT DOCOMO, INC.), RAN1#100bis-e

- R1-2003200, Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR_IIoT, Moderator (NTT DOCOMO, INC.), RAN1#100bis-e