3GPP TSG RAN WG1 Meeting #101-e						R1-2003639

e-meeting, 25th May – 5th June 2020

Source:	Moderator (CATT)

Title:	Summary of discussion on IIOT Scenarios for NR Positioning Enhancements (prior to the meeting)

Agenda item:	8.2.1

Document for:	Discussion and Decision

# Introduction

At RAN#86 meeting, the study item on NR Positioning Enhancements was approved [1]. From RAN1’s perspective, the SI includes the following objectives:

- Study enhancements and solutions necessary to support the high accuracy (horizontal and vertical), low latency, network efficiency (scalability, RS overhead, etc.), and device efficiency (power consumption, complexity, etc.) requirements for commercial uses cases (incl. general commercial use cases and specifically (I)IoT use cases as exemplified in section 3 above (Justification)):

  - Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT). [RAN1]

  - Evaluate the achievable positioning accuracy and latency with the Rel-16 positioning solutions in (I)IoT scenarios and identify any performance gaps. [RAN1]

  - Identify and evaluate positioning techniques, DL/UL positioning reference signals, signalling and procedures for improved accuracy, reduced latency, network efficiency, and device efficiency.
Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case. [RAN1, RAN2]

NOTE 1:	Sidelink is not part of this objective.

NOTE 2:	Involve RAN4 for validating assumptions for the systems evaluations where appropriate.

NOTE 3:	The commercial use cases and requirements are applicable to a limited geographic area.

As stated above, the SI will define the IIOT use cases with the associated performance requirements for identifying the performance gap and the simulation scenarios for the IIOT use cases with associated parameters. To facilitate the progress of the SI, we would like to collect the options from interested companies on these issues prior to the meeting. This summary will be submitted before the meeting for information purpose only.

# IIOT use cases for positioning enhancements

The following 3 scenarios are defined for NR positioning study in Rel-16 in TR 38.855 [3]

-	Scenario 1. Indoor Office for FR1 and FR2 (Open office)

-	Scenario 2. UMi street canyon for FR1 and FR2 (ISD 200m)

-	Scenario 3. UMa (ISD 500m) for FR1 only (Macro cell only deployment scenario)

The performance targets for RAT dependent solutions for commercial use cases in Rel-16 are also defined in TR 38.855 [3]

-	Horizontal positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Vertical positioning error < 3m for 80% of UEs in indoor deployment scenarios

-	Horizontal positioning error < 10m for 80% of UEs in outdoor deployments scenarios

-	Vertical positioning error < 3m for 80% of UEs in outdoor deployment scenarios

-	End to end latency < 1s

For Rel-17, as indicated in the SI objective, additional scenarios (e.g. IIoT) will be defined for evaluating the achievable positioning accuracy and latency with the Rel-16 positioning solutions and identifying any performance gaps, as well as the enhancements to Rel-16 positioning techniques.

For IIOT positioning requirements, eight IIOT scenarios are defined in TR 22.804 [4] (also see Table 8.1.7  in Appendix), with each of them having different performance requirements in terms of horizontal accuracy, latency, and availability.

The positioning requirements defined in TR 22.804 [4] for most of the IIOT use cases are very challenging and thus it may not be practical to meet all of them in Rel-17. Note also that the requirements defined in TR 22.804 [4] are service requirements which are not meant to be fulfilled with RAT dependent positioning mechanisms alone but rather in combination with RAT independent positioning methods. In addition, it was agreed the in the SI, “Enhancements to Rel-16 positioning techniques, if they meet the requirements, will be prioritized, and new techniques will not be considered in this case.” To evaluate whether the enhancements to Rel-16 positioning techniques meet the requirements, we would need first to define the target performance for Rel-17 in order to identify the performance gap.

The SID also provides the exemplary performance targets in the justification of the SI [1]: “NR Positioning in Rel-17 should evaluate and specify enhancements and solutions to meet the following exemplary performance targets:

(a) For general commercial use cases (e.g., TS 22.261):

- sub-meter level position accuracy (< 1 m)

(b) For IIoT Use Cases (e.g., 22.804):

- position accuracy < 0.2 m

The target latency requirement is < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired.”

Therefore, we may need first to define (or confirm) the target positioning requirements for Rel-17 for selected IIOT use cases, or to define the target positioning requirements for Rel-17 (e.g., using the exemplary performance targets as in the SID) without referring to specific IIOT use cases.

In R16, CDFs of horizontal and vertical positioning errors are used as a performance metrics in NR positioning evaluations with at least the following percentiles of positioning error are analyzed 50%, 67%, 80%, 90%.  In R17, we may consider larger percentiles due to the positioning performance requirements in TR 22.804 and TS 22.261 are defined at {90%, 99%, 99.9%} for different scenarios. In addition, due to the LOS probabilities in different areas in an InF scenarios are different, it may also be helpful in present the simulation results separately in different areas.

Proposal 2-1

- Define target positioning requirements in Rel-17 with one of the following options:

  - Option 1: Select one IIOT scenario (or multiple IIOT scenarios) from Table 8.1.7 in TR 22.804 as the target IIOT scenario(s), and then define the target positioning requirements in Rel-17 based on the positioning requirements defined in TR 22.804 for the selected IIOT scenario(s);

  - Option 2: Define the target positioning requirements in Rel-17 with the consideration of the positioning performance defined in Table 8.1.7 in TR 22.804 (e.g., using the exemplary performance targets in the SID), but the target positioning requirements may not necessarily be associated with particular IIOT scenario(s).

- In  R17, CDFs of horizontal and vertical positioning errors are used as a performance metrics in NR positioning evaluations with at least the following percentiles of positioning errors 67%, 80%, 90%, [95%], [99%].

FFS: whether to consider the confidence level of the accuracy in R17 performance metrics

- In R17, Positioning latency will be considered in terms of:

  - FFS: physical layer only, or RAN (PHY, MAC, RRC, NPP, NPPa etc.) only, or End to End

- FFS: whether to define the target performance for UE heading

Interested companies are welcome to provide their opinions on above proposals and options as well as additional considerations on the target IIOT use cases and the target positioning requirements in Rel-17 in the following table.

Table 2-1 Summary of target IIOT use cases and positioning service performance requirements in Rel-17

| Company | IIOT use cases | Positioning performance requirements |  |  |  | Additional comments |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Accuracy | Availability | Latency | Others |  |
| Huawei |  | Horizontal: [0.2 – 0.5m] @90% Vertical: [1m]@90% |  |  |  | Regarding Proposal 1, if the exemplary target performance requirement in the justification needs to revisited regardless of Option 1 or Option 2, we should consider to set a CDF value, and explicitly state whether the accuracy is horizontal or horizontal+veritical, similar to what we did in Rel-16. So we suggest the following rewording. “Note: For accuracy, the CDF value and the applicability to horizontal and/or vertical should be defined.” |
| CMCC | Inbound logistics for manufacturing (for driving trajectories, and for storage of goods) | < 0.1m (horizontal); < 0.2m (vertical) |  | 10ms |  | Based on our preliminary investigation in vertical industries, one of the most promising applications is the logistics and warehousing, which aligns with the use cases of inbound logistics for manufacturing that defined in 22.804. It should be noted that since the stored goods need to be accurately localized in such a case, it is necessary to define positioning accuracy requirements including both horizontal and vertical dimensions. |
| Fraunhofer | Factories of the Future15.6 Inbound logistics for manufacturing (for storage of goods) Factories of the Future15.5 Inbound logistics for manufacturing | < [20] cm | 99% | 10ms | - PRS required capacity (i.e. assigned RE for X UEs) | In most use cases, devices with very high accuracy and availability requirements are expected to be localized within positioning enhanced areas having good channel conditions (example, factory passage or movement area). Use case examples: - the navigation of AGVs inside a factory building. A positioning accuracy of <30cm is sufficient. - For picking and storage of objects automatically, a higher positioning accuracy of <20cm is required. |
|  | Factories of the Future 13.3 Process automation – plant asset management | < 1 m | 90% | 2s | - PRS required capacity (i.e. assigned RE for X UEs) ; - Power consumption per positioning event (incl. signaling overhead) | Use case example: Position information of IOT devices for asset management and maintenance (process, environmental conditions or inventory of material). Typically, those devices have also to perform with low power consumption and densely distributed all over the campus area (thousands of IOT devices/sensor per km²). |
| OPPO | Processing automation | <1m | 90% | 2s |  | Processing automation is the most important use case. Furthermore, using <1m as first metric for entering sub-meter region is a good start. |
| Futurewei |  |  |  |  |  | Our view is that it is not necessary to pick a specific scenario since many of those scenarios share some common requirements. Instead, the study should proceed based on the parameters you would capture in Section 4 to proceed with the various evaluations. We would then conclude based on these evaluations and with whatever enhancements, which of the requirements are met and which are not or close etc. I believe the SI then is able to conclude effectively similar to what we have done in R16. Between Option 1 and 2, we think Option 2 as an objective is closer to what we have in mind. The exemplary target stated in the SID is also a useful target. |
| Nokia |  |  |  |  |  | In our view Option 2 does not make much sense. Similar to Huawei we see the need to revisit the requirements outlined in the justification section of the SID (namely to decide on CDF value). We don’t see the benefit of selecting a scenario from TR 22.804. As is clear in the remainder of this document the group seems to be moving to selecting one or multiple InF sub-scenarios for evaluation. Whether we can meet the agreed requirements for the agreed InF sub-scenario(s) with Rel-16 solutions seems like the clear first step of this SI in our view. |
| CATT | Factories of the Future 13.3 Process automation – plant asset management Factories of the Future 15.5 Inbound logistics for manufacturing Factories of the Future 15.6 Inbound logistics for manufacturing | < 20 cm (horizontal accuracy); < 20 cm (vertical accuracy) | 90% | 100ms |  | Option 1 is preferred. i.e., select three IIoT use cases (Factories of the Future 13.3, 15.5 and 15.6) from Table 8.1.7 in TR 22.804 as the target IIoT use cases, and the target positioning requirements in Rel-17 should be defined based on the positioning requirements of the selected IIoT use cases. Based on the positioning requirements of the three selected IIoT use cases, one unified target positioning requirements for Rel-17 is defined as shown in the left. It should be noted that only horizontal accuracy is defined in the Table 8.1.7, however, vertical accuracy is included in the SID as the one of objectives of positioning enhancements, moreover, there are also the practical requirements for vertical positioning accuracy in IIoT scenarios. For example, in the scenario of multi-layer placement of goods in the logistics warehouse, vertical positioning is required for the tracking of goods in different layers of goods shelves. Rel-16 Positioning focuses on the study and evaluation of horizontal accuracy, so it will bring great challenges for Rel-17 Pos SI if vertical accuracy also needs to be evaluated, such as the base stations need to be multi-layer placement to carry out vertical positioning solutions. |
| LG |  |  |  |  |  | If we need to redefine the performance targets, we prefer option 1, and we are also fine with the current exemplary performance target which have been described in SID. Similar to Rel-16, the performance results need to be shown as a CDF for horizontal and/or vertical positioning accuracy. Regarding latency, it might be necessary to clarify the definition of it in order to have a common understanding. In consideration of the description of 8.1.7 in TR 22.804, this latency seems including processing time since it is described as “latency for position estimation of UE”, but we think the latency which could be considered in RAN1 would be physical-layer latency. |
| Verizon | Inbound logistics for manufacturing | < 0.1m (horizontal); < 0.2m (vertical) | 99% | 10ms |  | Firstly, for IIOT, we don’t like to overlook vertical accuracy either. Also, based on the feedback from folks serving enterprise/vertical customers, we see strong need for CM level accuracy for IIOT. An item they keep bring up is the need for heading accuracy (delta accuracy?). We are trying to get a more reliable read on the accuracy need for that. As a reference, they want .17 radians accuracy for 2 (not so accurate) commercial use cases in office and sports stadium. |
| Qualcomm |  | < [20] cm |  |  |  | We don’t need to pick a specific IIOT scenario from TR22.804. We can simply consider a set of CDF values for the positioning accuracy (at least including 50%, 80% and 90%) that companies could report for any agreed InF evaluation scenarios discussed in Section 4. Similar approach was also used in Rel-16, and we think it should be used also in this release. Otherwise, we risk spending a lot of time debating on priorities between IIOT scenarios. Then, based on the evaluations, the SI could conclude whether the requirements can be met in any of the InF scenarios in TR38.901 and with what level of accuracy (e.g. CDF point). |
| Ericsson |  |  |  |  |  | Requirements need to be coupled to scenarios and channel models (e.g. an InF model) for which they are relevant. Each requirement need not be fulfilled in all scenarios. Requirements in TR 22.804 are intended to be fulfilled by a combination of RAT dependent and RAT independent methods. It’s not always reasonable to put these requirements on RAT dependent methods only. |

As per the SID [1], besides accuracy, latency, network efficiency and device efficiency aspects should be studied in the study item. The scalability, latency and average power consumption of devices can be related.

.

Proposal 2-2:

Physical layer latency, UE power consumption, scalability/capacity, network efficiency can be considered as important design factors for potential enhancements and solutions.

Interested companies are welcome to provide their opinions on above proposal and additional considerations on target positioning requirements in Rel-17 in Table 2-2.

Table 2-2 Additional Comments

| Company |  | Comments |  |
| --- | --- | --- | --- |
| Huawei |  | #2. Regarding Proposal 2, we understand that those factors are important for positioning; however, we fail to see how to quantize those factors, or even to set a reasonable target for e.g. scalability, UE power consumption. Since those parameters may have impact on positioning accuracy, will those factors influence the evaluation? In the meantime, we do not see a strong connection between UE power consumption and the number of devices to be positioned. Therefore, we suggest to change Proposal 2 similar to what we agreed for Rel-16.  Proposal 2: Physical layer latency, UE power consumption, scalability/capacity, network efficiency can be considered as important design factors for potential enhancements and solutions and can be evaluated analytically. |  |
| Fraunhofer |  | For the UE power consumption a cost function shall be defined. The cost function can, for example, use the following input parameter - number of OFDM symbols to be processed by UE - number of OFDM symbols transmitted by UE. |  |
| Nokia | We are unsure why the latency and power consumption are being tied to the number of devices to be positioned. We agree with Huawei that proposal 2 can be changed to reflect that latency, scalability, UE power consumption, and network efficiency can be considered as important design factors for potential enhancements. |
| CATT | For assessing scalability of positioning solutions, latency of positioning procedure should be studied as a function of number of devices to be positioned. The more number of devices to be positioned, the more Uu signaling and positioning data needed, which challenges the capacity and processing capability of gNBs as well as LMF, and eventually leads to the increase of latency of positioning procedure. The UE-based positioning can reduce signaling overhead and has advantages in terms of positioning data reduction and scalability improvement, compared with UE-assistant positioning. Average power consumption of devices should be studied as a function of configured time and frequency resources for positioning. In order to obtain better positioning accuracy, SRS for positioning occupies up to 12 symbols and up to 272 RBs. The power consumption caused by the configured time and frequency resources for positioning should be considered. |
| LG | We have similar understanding that these should be considered as important design factors in Rel-17, but we also think that it is difficult to make one clear metric, so we suggest that the performance could be evaluated analytically and the performance metric is up to each company. |
| Verizon | We don’t want to get too much into technical details ourselves but in general, we strongly support considering these factors, though we can’t give any more specific info to be useful now. For example, power consumption is important. We want for typical usage (TBS what is typical), <2% battery for positioning purpose every 12 hrs. Also scalability. It is scenario dependent, e.g., for commercial case, we need it to scale to a sports stadium of 65K UEs. Not exactly sure for IIOT yet but it is important to consider. |
| Futurewei | Perhaps the Physical layer Latency aspect should be highlighted, even though evaluating latency requires much more extensive simulation. Scalability and Network Efficiency are not something that can be readily quantified. |
| Qualcomm | We should put realistic targets for this SI (which is going to be only 3 meetings). The proposal shown above seems to be a repetition of the SI scope. In the SI description it is already written that latency, network efficiency, device power are part of potential enhancements, so we do not think that there is a need for this proposal here, nor that it is related to the this subagenda. |
| Ericsson | To evaluate scalability the positioning overhead needs to be studied, depending on number of UEs to position as well as on the latency requirement. We also wanted to note that all these factors (latency, power consumption, scalability, accuracy), may not be possible to achieve all at the same time and some compromise will have to be reached as to what factor is prioritised. We think latency and scalability can be used to distinguish different solution achieving similar accuracy, but should not be the first priority. |

# IIOT InF scenarios

For the evaluation of positioning performance with the target IIOT scenarios, there is a need to define the details of the evaluation parameters, including the channel parameters for the IIOT scenarios based on TR 38.901[2], where the following 5 Indoor Factory (InF) scenarios are defined:

- InF-SL	Indoor Factory with Sparse clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-DL	Indoor Factory with Dense clutter and Low base station height (both Tx and Rx are below the average height of the clutter)

- InF-SH	Indoor Factory with Sparse clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-DH	Indoor Factory with Dense clutter and High base station height (Tx or Rx elevated above the clutter)

- InF-HH	Indoor Factory with High Tx and High Rx (both elevated above the clutter)

Above InF scenarios can be considered for evaluation. Prioritization from above can be done over email discussions.

Proposal 3-1:

Select one InF scenario (or more target InF scenarios) from the InF scenarios defined in TR 38.901 for supporting positioning evolution in Rel-17.

Interested companies are welcome to provide their opinions on above proposal and also the preferred InF scenario(s) for the evaluation of positioning performance in Rel-17 in Table 3-1.

Table 3-1 Summary of company views on InF scenarios in Rel-17

| InF scenarios Company | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (High Tx and High Rx) | Additional comments |
| --- | --- | --- | --- | --- | --- | --- |
| CMCC |  |  |  | High priority |  | Considering the tight schedule and heavy workload of the SI phase, we agree that companies should focus on one or two InF scenarios defined in 38.901. Since the characteristics of the identified logistics for manufacturing use cases are in line with that of the InF-DH scenario, we think that the InF-DH should be evaluated with high priority. |
| Fraunhofer | The high BS scenario is more relevant when going for down selection. Interested companies can bring results with mixed InF-SL and InF-SH scenario (example: 4 or more BSs deployed at high height). Note that the LOS/NLOS probability doesn’t depend on the BS/UE and clutter heights. | Similar to InF-SL, almost all links suffers NLOS conditions. Interested companies can bring results with mixed InF-DL and InF-DH scenario (example: 4 or more BSs deployed at high height). Note that the LOS/NLOS probability doesn’t depend on the BS/UE and clutter heights. | Good LOS conditions (with default clutter parameters Expected LOS Probability ~ 80% ¦ Expected NLOS Probability ~ 20%) | Main simulation scenario; additional remarks: Many links will be under NLOS conditions for the default clutter parameters. Reduce the clutter height (h_c =2m) and clutter density to 40%. (With “relaxed” DH clutter parameters Expected LOS Probability ~ 40% ¦ Expected NLOS Probability ~ 60%) | Scenario not relevant for the positioning use cases targeted by the SI. | We propose the following scenario: - Use InF-DH clutter as the common baseline scenario. - Define a “Passage/ Movement Area” and apply InF-SH or InF-LOS to UE-BS links within this area. For scenarios dominated by NLOS channels no simulation is needed to know that the targeted accuracy (<0.2m) is not achievable especially when taking the NLOS TOA offset from 38.901 into account. The output of the SI shall evaluate if the performance targets are achievable for the general requirements (overall area) as well as the high accuracy requirements (within the Passage/Movement Area). |
| OPPO |  | include | include |  |  | InF-SH is with the highest probability of LOS among these 4 scenarios, whereas InF-DL is with the lowest probability of LOS The simulation parameters in Table 7.8-7 of 38.901 are assumed for the above figure |
| Futurewei |  |  |  |  |  | Our view is that it would be too early to exclude most scenarios without some technical discussions and just settle with one. Ensuring how each model implications on the evaluation and eventually on the positioning accuracy due to its differences e.g. LOS/NLOS properties would be important for the study. Among the 5 listed cases, we support inclusion of at least the first four. The InF-HH is probably the less relevant for our study now. |
| Nokia |  |  |  |  |  | Selecting one or two sub-scenarios makes sense. We agree with the view of Huawei that InF-DH should be evaluated with highest priority. It can be complemented with InF-SH. The remaining scenarios seem to have less relevance for the purpose of positioning of items in a factory hall. Final down selection could be done during RAN1#100 after more progress on simulation assumptions and requirements has been reached. |
| CATT | Lower priority | Higher priority (mandatory) | Higher priority (mandatory) | Higher priority (optional) | Lower priority | For the first four scenarios (i.e., InF-SL, InF-DL, InF-SH and InF-DH), large scale calibration has been completed in Rel-16 stage, including the calibration of coupling loss, geometry, delay spread, ASD, ZSD, ASA and ZSA. A lot of scenarios parameters have been specified for calibration in 38.901, such as BS height, clutter density, clutter height and clutter size. Therefore, it is recommended to select one or more scenarios from the first four scenarios which had been calibrated in Rel-16 for positioning evaluation of Rel-17, so as to reduce the workload of determination of scenario parameters and calibration of simulations results. In general, high BS height and sparse clutter will increase the possibility of LOS, but low BS height and dense clutter will play the opposite role. Therefore, the simulation results for InF-SH scenario can be considered as the upper bound in terms of positioning accuracy among all the four InF scenarios, and InF-DL scenario should be the lower bound. So the InF-SH and InF-DL scenarios should be selected as the mandatory scenarios in the positioning evaluation in Rel-17 to make clear the range of positioning accuracy for different IIoT InF scenarios. Other scenarios already defined in 38.901 (e.g. InF-DH) can also be selected as optional scenarios for evaluation according to the interests of companies. |
| LG |  |  |  |  |  | In consideration of limited time budget, we may need to select one or two scenarios. In our view, InF-SH might be firstly considered to see an upper bound of the performance under the high LOS probability. In addition, InF-DH could be considered as a more severe condition. |
| Verizon |  | High interest | High interest | Highest interest |  |  |
| Qualcomm | Include | Include (with parameter changes) | Include | Include (with parameter changes) | Low Priority | We are ok to give lower priority to the HH scenario. Amongst the remaining 4 scenarios, at least include the SH and SL scenarios. With regards to DH and DL scenarios, discuss further parameter changes to make them more realistic. To decide a new set of clutter parameters, we first need to agree upon a CDF requirement governing the LOS state, which ensures X% of the UEs with a least Y LOS Links. Once (X, Y) is defined, the required value of ksubsce in LOS probability equation (Table 7.4.2-1 of 38.901) can be obtained from a simple UE dropping experiment. The corresponding clutter parameters (e.g. hc, r, dclutter) can then be derived analytically. As an example for InF-DH & 20m ISD, if the requirement is to keep 95% of the UEs with at least 4 LOS links, the following parameter change can be considered: hc = 3, r = 0.4, dclutter = 5. |
| Huawei/HiSilicon [update on 14th May] | Low | Low | High | Middle | Low | We are OK to evaluate InF-SH and InF-DH. If some parameters are changed for InF-DH, we suggest renaming it to InF-DHm to differentiate the original one to avoid potential confusion in the future. |
| Ericsson |  |  | Include two Inf-SH scenarios. 1.Clutter density 0.2, BS height 8m, UE height 1.5m, clutter size 10m, clutter height 2m, resulting in ksubsce=582.6m (parameter setting previously used for InF-SH model calibration). Probability distribution for number of LoS links in ‘large hall’ deployment with 50m between TRPs: 2. Clutter density 0.4, BS height 8m, UE height 1.5m, clutter size 2.6m, clutter height 10m, resulting in ksubsce=115.7m. Probability distribution for number of LoS links in ‘large hall’ deployment with 50m between TRPs: | Main simulation scenario - with clutter density 0.4, BS height 8m, UE height 1.5m, clutter size 2m, clutter height 2m, resulting in ksubsce=50.9m. Probability distribution for number of LoS links in ‘small hall’ deployment with 20m between TRPs: | Include this model to study positioning in e.g. passageways (as discussed by Fraunhofer). There is no need to develop a new combined model for such purposes. Include this model for best case scenario / benchmarking | Each of InF-SH, Inf-DH,… allow for different parameter settings according to Table 7.2-4 in 38.901. We need to select also these parameters. The main difference between all these models and parameter settings is the difference in LoS probability which can be characterized by the parameter k_subsce. Note that the mean NLoS excess delay in the InF models is 48.3ns corresponding to 14.5m, making NLoS links useless/destructive for high accuracy positioning Note that the parameter choices used for the large scale calibration of the models are not necessarily representative for real scenarios. A clutter density of 0.6 (60% of the floor area covered by blockers) used for the large scale calibration of Inf-DH is very extreme. Using that model the probability for a UE to have no LoS link at all is 87% in the ‘small hall’ deployment with 20m between TRPs. |

# Evaluation parameters for IIOT scenarios

Evaluation parameters common to all scenarios

In Rel-16 the scenario parameters common to all scenarios for positioning evaluation are defined in Table 6.1.1-1 in TR 38.855[3] (see Table 6.1.1-1 in Appendix 7.3), which includes the carrier frequency, the PRS/SRS bandwidth, subcarrier spacing, gNB/UE noise figures, UE max. TX power, UE antenna configuration, UE radiation pattern and network synchronization, etc. It is proposed to reuse most of the common parameters defined in Table 6.1.1-1 in TR 38.855 (including Table 6.1.1-2 for UE radiation pattern in FR2) for IIOT scenarios, with possible modifications to minimize simulation overhead.

Proposal 4-1:

- Adopt the common parameters defined in Table 4-1 for the evaluation of InF scenarios

Table 4-1: Common scenario parameters applicable for all InF scenarios

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 in TR 38.855 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Interested companies are welcome to provide their opinions on above proposal and other comments in Table 4-2.

Table 4-2 Summary of company views on evaluation parameters common to all scenarios

| Company | Comments |
| --- | --- |
| Huawei | We have multiple instances of Proposal 3. For the second Proposal 3, when it comes to reuse of Table 6.1.1-1, we do not think 5MHz BW is necessary. |
| Fraunhofer | Include parameters agreed in R1-1901416 for the evaluation parameters, and if agreeable, define common assumption from the list, for example: - Power-boosting level - number of occasions used per positioning estimate - Uplink power control - Interference modelling - Beam-related assumptions - Precoding assumptions - Description of measurement (parameter estimation) algorithm - Description of positioning technique / applied positioning algorithm - … |
| OPPO | Ok with proposal 3 |
| Nokia | 5 MHz BW certainly seems not needed. Further down scoping of BW, carrier frequency choices in Table 6.1.1-1 can be considered to minimize simulation overhead. |
| CATT | Agree proposal 3 to reuse the common parameters defined in 38.901 for the evaluation of IIoT scenarios. |
| LG | We are fine with the common parameters |
| Qualcomm | Consider only 100MHz for SCS of 30kHz (FR1) and 400MHz for SCS of 120kHz (FR2) to minimize the evaluation load. No need to have 5 MHz and 50MHz for FR1 and 100MHz for FR2 as the focus is on the best achievable accuracy. Replace the carrier frequencies with 3.5 GHz (FR1) and 28 GHz (FR2) as defined in Table 7.8-1 of TR38.901. Given that indoor factory is a highly controlled environment, high precision synchronization can be achieved through shared clock and synchronization cable. Even if this is not possible, further enhancements for network calibration using positioning-based solutions can be considered in the potential enhancements sub-agenda. Therefore, at least for UL/DL TDOA, the study shall have as a baseline the cases with perfect synchronization, otherwise it should be clear, without much simulation effort, that even a few nsec of sync errors would result into not meeting the <1 m target (e.g., 3 nsec corresponds to 3 feet error); spending simulation effort to show the evident is not needed. |
| Huawei/HiSilicon [update on 14th May] | Regarding BW/SCS, we prefer only 100MHz with 30kHz for FR1 (high priority), 160MHz with 30kHz for FR1 (middle priority, implemented through CA), and 400MHz with 120kHz for FR2 (middle priority). Others should be low priority. Regarding sync error, we prefer only evaluate the case without sync error. |
| Ericsson | We agree with the fact that 5MHz is not a realistic bandwidth for high accuracy targets. In the Rel. 16 positioning evaluations, the simulation assumptions were based on early assumptions about NR UE design from the time before the specification of the NR standard and before any deployment of NR. We now have knowledge about actual NR UE design and we should modify the simulation assumptions accordingly. The UE should have three to four 1x4x2 antenna panels with 0.5λ antenna element separation and with position and orientation as given below. P1: Θ0=90°; Ω0=270°, x0=0m, y0=0m, z0=0.08m P2: Θ1= Θ0-90°; Ω1=Ω0, x1= x0, y1=y0+0.03m, z1= z0+0.08m P3: Θ2= Θ0; Ω2=Ω0+180°, x2= x0, y2=y0+0.06m, z2= z0 P4: Θ3= Θ0+90°; Ω3=Ω0, x3= x0, y3=y0+0.03m, z1= z0-0.08m The UE RX/TX timing error for antenna panel k is modelled as zero mean stochastic variables kRX/kTX with normal distributions truncated at 3. Evaluations should be performed both without timing errors (RX=TX=0) and for RX and TX of the order of a few nanoseconds, exact values TBD. P1 P2 P3 P4 Px: |

Scenario parameters common to InF scenario(s)

The scenario parameters common to InF scenario(s) may be developed from the consideration of both the simulation assumptions for large scale calibration for InF scenarios in Table 7.8-7 in TR 38.901 (see Appendix 7.4),Table 7.2-4 of 38.901 (see Appendix 7.2) and the evaluation parameters for indoor office scenarios in Table 6.1.1-3 in TR 38.855 (see Appendix 7.3) as the starting point.

Proposal 4-2:

The scenario parameters common to InF scenario(s) can be developed with the consideration of the parameters for InF scenarios defined in Table 7.8-7 in TR 38.901, Table 7.2-4 of 38.901 (see Appendix 7.2)and the parameters for indoor office scenarios in Table 6.1.1-3 in TR 38.855, e.g., as shown in Table 4-3.

Table 4-3 Parameters common to InF scenario(s)

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Channel model |  | [InF-SL, InF-DL, InF-SH, InF-DH and/or InF-HH] | [InF-SL, InF-DL, InF-SH, InF-DHand/or InF-HH] |
| Layout | Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m InF-HH: 300x150 m |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m FFS: asymmetrical location for the BSs FFS: denser BS grid (D=10) |  |
|  | Room height | 10m |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Peneteration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE drop procedure |  | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility |  | 3km/h |  |
| UE antenna height |  | 1.5m FFS: [>3m for InF-HH] FFS: uniformly distributed within a pre-defined range, e.g., [0.5 ~ 9]m, or pre-defined values, e.g., [0.5, 1.5, 4]m for the evaluation of vertical positioning performance |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH FFS:uniform distribution [3-8]m. |  |
| Clutter parameters: {density r, height hc,size dclutter} |  | Low clutter density: {20%, 2m, 10m} High clutter density: {60%, 6m, 2m} FFS: other cases: {40%, 2m, 2m} |  |
| LOS probability |  | LOS probability for InF scenarios is modelled according to Section 7.4.2 in TR 38.901 |  |
| Absolute time of arrival |  | FFS: Absolute time of arrival for InF scenarios is modelled according to Section 7.6.9 in TR 38.901 |  |
| [Blockage modelling] |  | FFS: Blockage model B from Section 7.6.4.2 in TR 38.901 is included in simulation evaluation. (Details of the modelling parameters, e.g., the number of blockers, the blocker extensions, locations, etc.),need to be further discussed if blockage model is included in simulation evaluation. |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |  |

Interested companies are welcome to provide their opinions on above proposal and other comments in the Table 4-4.

Table 4-4 Summary of company views on the evaluation parameters common to InF scenario(s)

| Company | Comments |
| --- | --- |
| Huawei | Regarding Proposal 4 and Table 4-3, we do not think InF-HH maps to a valid positioning use case, but if we really need to list it as a candidate, we should consider a reasonable UT height, e.g., >3m. |
| CMCC | Regarding the positioning of the stored goods on the shelf, in our view, it is more reasonable to drop UE over a three-dimensional space, which is, the UE height should be uniformly distributed within a pre-defined range, e.g., UE antenna height ~U([0.5]m~[9]m). |
| Fraunhofer | - The table shall note that the absolute time of arrival is applied according to TR 38.901 - Channel model: InF-DH (except for positioning area: passage), InF-SH (or InF-LOS) in the positioning area. - gNB antenna height with uniform distribution [3-8]m. Optional: For better comparison of results between companies it may be worthwhile to select the height values. - BS locations: Consider asymmetrical location for the BSs (a simple approach is exclude certain BSs for example every 4th BS [1, 5, 9, 13, 17] from the evaluation or position estimation statistics - UE drop procedure: Random dropping and statistical independent LOS/NLOS probability generate results difficult to compare. The high accuracy is maybe feasible in certain areas (example passage areas with higher probability of LOS reception). Definition of regions with higher LOS probability may help to better demonstrate the impact of a careful selection of gNB positions. Proposal: Compare two areas with same (relative) position to gNBs with different LOS probability, resulting in 3 areas - [20%] of UEs are uniformly dropped in an indoor passage (high accuracy area) with high LOS probability (e.g. LOS probability according to InF-SH, or at least high LOS probability for the direct neighboring gNB ) - [20%] of UEs are uniformly dropped in an area with same relationship to gNB position to allow direct comparison (e.g. LOS probability according to InF-DH) - [60%] of UEs are uniformly distributed over the remaining horizontal area. In this area it may be worthwhile to distinguish the effects resulting from ToA measurement errors and non-ideal geometry (“dilution of precision”). - Many links will be under NLOS conditions for the default clutter parameters. Reduce the clutter height (h_c =2m) and clutter density to 40%: Clutter density: r=40%; Clutter height: hc=2 m ; Clutter size: dclutter=2m |
| Futurewei | We need to include the LOS probability as given in Table 7.2.4.1 as applicable for InF secnarios. Furthermore, as explained in our reply email, we propose to include Blockage model from Section 7.6.4(of 38.901) as additional modelling for further consideration. We are ok with the Table 4-3 above, For our previous comment above, we are proposing to capture to include in the simulation modelling the upper bound of the excess delay in NLOS, , as provided in Table 7.6.9-1 of TR38.901. |
| Nokia | Table 7.2-4 in 38.901 captures evaluation parameters for InF, and Table 7.4.2-1 in 38.901 defines the LOS probability for NLOS and LOS statistics. We think that the scenarios can be taken from both the Table 7.2-4 and Table 7.4.2-1 in 38.901 for the IIoT positioning study. Parameters in Table 5 could be extended as follows: Consider different UE heights: 0.5 m, 1.5m, and 4m (for 6m clutter height). Also, a denser BS grid with D=10m (at same hall size) should be considered. We think it is not required to simulate all possible combinations of parameters but agree on a reasonable set of combinations. |
| CATT | Agree proposal 4 to reuse simulation assumptions for the indoor factory scenario defined in 38.901 for the evaluation of IIoT scenarios. |
| LG | We are generally fine with proposal 4-2. However, regarding UE antenna height, “uniformly distributed within a pre-defined range” needs to be limitedly allowed when evaluating the vertical positioning accuracy. If the UE antenna height is changing when horizontal positioning performance is evaluated, it might be difficult to find critical factors that affect to horizontal accuracy performance. If we need evaluation considering different UE antenna height, one or two different values for UE antenna height could be considered, which are fixed. |
| Verizon | Support CMCC’s view. We need more heights when dropping UE in a 3D space of a warehouse. |
| Qualcomm | - The simulation should drop the UE uniformly in the 3D space so that the result is decoupled from a specific UE height and the vertical accuracy can be fairly assessed. - Place the base stations at different heights to reduce the sensitivity in z-coordinate to TOA error. For example, we can stagger the neighboring base station with different height from 3m to 8m. - The UEs located outside the convex hull of the base stations generally experience higher geometry dilution of geometry (GDOP) and are hence more sensitive to TOA errors in positioning accuracy. In practice, this issue is addressed by installing base station at the perimeter of the target service area. In the simulation, however, this can be simplified to dropping UEs inside the convex hull or reporting CDF only for the UEs inside. - The position tracking aspect can be considered as additional scenarios for use cases like AGV. To enable mobility in the simulation, a simple route or path trajectory can be defined in the layout along with a mobility model defining the velocities and accelerations consistent with the use-case applications. Spatial consistency procedure in TR 38.901 shall also be enabled in the mobility simulation with configurations agreed by the group. |
| Huawei/HiSilicon [update on 14th May] | OK with the randomized UE height distribution. Regarding UE outside the convex hull, we share similar review with QC, as in Rel-16, we also showed the results for central UE in InH scenario, which is much better than the overall UEs. However, if we take height into consideration, should the convex hull be 3-dimensional? Should we only consider that the UE height is between the highest BS and the lowest BS? No need to consider blockage model. |
| Ericsson | Table 7.8-7: Simulation assumptions for large scale calibration for the indoor factory scenario, in 38.901 (reproduced above) gives just example parameters that were used for the calibration of the model. The allowed parameters are instead given by Table 7.2-4 of 38.901 (see Appendix 7.2). Still, we think that most of the parameters in Table 7.8-7 are useful. For the clutter density r and clutter height h_c different values are needed to better reflect realistic and relevant scenarios. For InF-SH we propose clutter density 0.2 and clutter height 2m and for InF-DH we propose clutter density 0.4 and clutter height 2m, as explained in table 3 (companies view on InF models) above. Both the small hall with 20m ISD and the large hall with 50m ISD should be utilized. |

Additional details on evaluation of InF scenario(s)

In addition to the parameters to InF scenario(s) discussed in previous section, interested companies are welcome to provide additional considerations or details on positioning evolution of InF scenario(s), such as NLOS channel modelling etc.

Table 4-5 Additional considerations on the evaluation of InF scenario(s)

| Company | Additional considerations |
| --- | --- |
| Futurewei | During the R16 SI on IIOT channel model, methods to support simulations in which absolute time of arrival is important have been considered. The details are described in Section 7.6.9, where the propagation time delay (or bias) due to the total path length and the impulse response are provided. Furthermore, the upper bound of the excess delay in NLOS, , is provided in Table 7.6.9-1 copied below. Table 7.6.9-1: Parameters for the absolute time of arrival model Scenarios; InF-SL, InF-DL; InF-SH, InF-DH. lgΔτ=log10Δτ1s; μlgΔτ; -7.5; -7.5. σlgΔτ; 0.4; 0.4. Correlation distance in the horizontal plane [m]; 6; 11. To correctly and adequately consider the effects of NLOS on positioning accuracy that are based on time of arrival of received paths, we propose the work as captured in Section 7.6.9 of 38.901 to be considered. The values applicable for us in Positioning SI can be further discussed and decided. |
| Huawei/HiSilicon [update on 14th May] | We would like to discuss the possibility of using explicit ground reflection (7.6.8 of TS 38.901) for improving the vertical accuracy and extending that model to include also wall reflection. |
| Ericsson | In order to simplify the discussions regarding the InF model we would like to make a few observations. The Inf models include a number of sub-models as well as a large range of free parameters. All these parameters and sub-model choices do, however, impact simulations only in three ways that are easily understood: - The parameters for clutter density, clutter height, clutter size, UE height and BS height impact LOS probability but only through the parameter ksubsce as defined in 38.901: - The UE height, the BS height, the size of the hall and the position of the UEs have an obvious impact on the geometry. - The choice between Inf submodels impacts the NLOS pathloss as shown in the figure below. The effect of this on positioning is very limited since NLOS links anyway are useless for submeter positioning since the average NLOS excess delay is 48ns corresponding to 14.5m. The LOS pathloss is the same for all models. Note also that the NLOS pathloss difference between the models relevant for positioning (Inf-SH and Inf-DH) is very small. There are no other impacts on the simulations than the above three. The conclusion from this is that we should select models with representative values of ksubsce and with realistic geometries (UE height, BS height, size of the hall and position of the UEs). All other considerations are illusory since they don’t have any impact on the evaluations we will do. |

DL PRS and UL SRS Configurations

In order to have a practical understanding on the achievable positioning performance with Rel-16 positioning technologies for the IIOT scenarios, we assume the simulation evaluation may use any, or a combination, of the Rel-16 positioning technologies (e.g., OTDOA, UTDOA, multi-RTT). In addition, given that the positioning performance depends heavily on the DL/UL RF resources configured for supporting the positioning, there is also a need to have a common understanding on the reasonable DL PRS and UL SRS configurations for simulation evaluation.

### DL PRS Configuration for simulation evaluation

Table 4-6 presents some suggestions on DL PRS configurations for the purpose of the evaluation of the positioning performance.

Table 4-6 DL PRS Configuration

| Parameters | Values | Additional Comments |
| --- | --- | --- |
| DL-PRS-Periodicity | [10]ms | FFS: [4, 5, 8, 16, 20, 32, 40, 64, 80]ms |
| DL-PRS-ResourceRepetitionFactor | [1] | FFS: [2, 4, 6, 8, 16, 32] |
| DL-PRS-ResourceTimeGap | [1] | FFS: [2, 4, 6, 8, 16, 32] |
| DL-PRS-SFN0-Offset | 0 |  |
| DL-PRS-ResourceSetSlotOffset | Configured for avoiding/minimizing overlapping of the DL PRS from different TRPs |  |
| DL-PRS-ResourceId | Uniquely configured for each DL-PRS-Resource |  |
| DL-PRS-SequenceId | Uniquely configured for each DL-PRS-Sequence |  |
| DL-PRS-CombSizeN | N=[6] | FFS: N = [2, 4, 12] |
| DL-PRS-ReOffset |  | Configured up to companies to support avoiding/minimizing overlapping of the DL PRS from different TRPs |
| DL-PRS-ResourceSlotOffset | Configured for avoiding/minimizing overlapping of the DL PRS from different TRPs |  |
| DL-PRS-ResourceSymbolOffset | [0 to 13] | Configured up to companies |
| DL-PRS-NumSymbols | [6] | FFS: [2, 4, 12] |
| DL-PRS-QCL-Info | Not used in evaluation |  |
| DL-PRS-SubcarrierSpacing | [15/30] kHz for FR1 [120] kHz for FR2 | FFS: [ 60] kHz for FR1 FFS: [60] kHz for FR2 |
| DL-PRS-CyclicPrefix | Normal |  |
| DL-PRS-ResourceBandwidth | [50/100 MHz] with [15/30]kHz SCS for FR1 400 MHz with [120]kHz SCS for FR2 | FFS: [24, 28, 32, 36, 40, …, 268, 272] RBs |
| DL-PRS-StartPRB | Not used in evaluation |  |
| DL-PRS-PointA | Not used in evaluation |  |
| DL-PRS-expectedRSTD | +/- 1 us |  |
| DL-PRS-expectedRSTD-uncertainty | FR1/FR2: +/- 1 us FR2: +/- 1 us |  |
| DL-PRS-MutingPattern | can be used in evaluation | In case overlapping of the DL PRS from different TRPs cannot be avoided |
| DL-PRS-MutingBitRepetitionFactor | can be used in evaluation | In case overlapping of the DL PRS from different TRPs cannot be avoided |
| TRP ID | Uniquely configured for each TRP |  |
| ss-PBCH-BlockPower | Not used in evaluation |  |
| DL-PRS-ResourcePower | Determined based on the maximum TRP Tx power devided by the number of DL PRS REs |  |
| DL–PRS- PositioningFrequencyLayer | 1 |  |
| NumDL-PRS-RSTD-MeasurementsPerTRPPair | [1] | FFS: [,2,3,4] Depending on the positioning algorithm |
| NumPositioningFrequencyLayers | 1 |  |
| NumTrpPerPositioningFrequencyLayer | 18 | All TRPs in simulation layout |
| NumDL-PRS-ResourceSetsPerTRPFrequencyLayer | 1 |  |
| NumDL-PRS-ResourcesPerSet | Configured as nedded for the beamforming in the simulation evaluation |  |

Interested companies are welcome to provide their opinions in the following table for above proposed DL PRS configuration and parameters.

Table 4-7 Summary of company views on the DL PRS configuration parameters

| Company | Comments |
| --- | --- |
| Huawei | For the proposals related to DL PRS and SRS configuration, some parameters are duplicated with Table 6.1.1-1, e.g., BW, numerology, etc. In fact, we do not think it is necessary to explicitly mention the PRS or SRS configuration, since companies still have freedom to select the comb size, number of symbols, etc. |
| Fraunhofer | It may be worthwhile to agree on the beams active at the same time slot (or OFDM symbol) to consider for the interference scenario. |
| OPPO | We do not see the necessity to exhaust the configuration of PRS resource, suggest use the following configuration: DL-PRS-CombSizeN = 6 DL-PRS-ReOffset = {0,3,1,4,2,5} DL-PRS-NumSymbols = 6 DL-PRS-ResourceSymbolOffset = 0 or 7 |
| Nokia | NumTrpPerPositioningFrequencyLayer = {18, 55} for D=20m and D=10m in the small hall. |
| CATT | We are fine for the DL PRS Configuration (table 4-6). Maybe the values of several parameters would be adjusted according to further discussion, but a common understanding is needed on the reasonable DL PRS configurations for Rel-17 positioning simulation evaluation. |
| LG | Fine with the table 4-6. There are many possible cases of PRS configuration. If it is possible, it would be desirable to have a common configuration, and evaluation for additional cases could be up to company decision. As CATT suggested, Comb-6 and 6 symbols could be considered as a starting point of discussion. |
| Qualcomm | Try to minimize the evaluation load by using too many different PRS configurations, unless there a specific reason to do so. For example, for FR1 we are OK to simulate combSizeN = 6 with 6 symbols, and all TRPs orthogonal with single repetition (e.g. 6 TRPs per slot for 3 consecutive slots). For FR2, as analog beam sweeping is not compatible with FDMing too many beams, combSizeN = 2 with 2 symbols is preferred considering the number of beams to be multiplexed in time. If there is no easy consensus on some of the parameters, we are OK to leave freedom to companies to pick different parameters under the assumption that the configuration is reported. Regarding the bandwidth and numerology, suggest to focus on 100 Mhz with 30 kHz for FR1, and 400 MHz with 120 kHz for FR2. |

### UL SRS Configuration for simulation evaluation

Table 4-8 presents some suggestions on UL SRS configurations for the purpose of the evaluation of the positioning performance.

Table 4-8 Configuration of SRS for positioning

| Parameters | Values | Additional Comments |
| --- | --- | --- |
| SRS-ResourceId | Uniquely configured for each SRS-sequence |  |
| transmissionComb: combOffset: cyclicShift | FFS: n4: combOffset (0,..,3); cyclicShift (0,…,11) | FFS: n2: combOffset (0,1); cyclicShift (0,…,7) FFS: n8; comboffset (0,..,7); cyclicShift (0,… 5) |
| resourceMapping nrofSymbols | [n12] | FFS: nrofSymbols : n1,n2,n4,n8, |
| freqDomainShift | Not used in simulation |  |
| freqHopping c-SRS | [23] for FR1(15kHz); [33] for FR2(60kHz) | SRS bandwidths are selected in comparable to PRS bandwidths, e.g., 96 PRBs for FR1 (15kHz SCS); 132 PRBs for FR2 (60kHz SCS) FFS: other values |
| groupOrSequenceHopping | neither | No group or sequence hopping |
| SRS-Periodicity | [10]ms | SRS-Periodicity will match the DL PRS periodicity for supporting multiple-RTT FFS: other SRS periodicities |
| SRS-Offset | Configured for no overlapping of SRS-resources with other UEs |  |
| sequenceID | Uniquely configured for each SRS-sequence |  |
| spatialRelationInfo | Not considered in evaluation |  |
| srsResourceSetId | Uniquely configured for an SRS for positioning of a UE |  |
| srsResourceIdList | Uniquely configured for the SRS-Resources for positioning in an SRS-ResourceSet. |  |
| resourceType | periodic | FFS: periodic, semipersistent |
| alpha | Alpha=[1] | When the field is absent the UE applies the value 1 |
| P0 | [24] dBm | P0 value for SRS power control is configured with maximum value. Assume UE uses maximum power for SRS transmission |
| Parameters related to pathloss reference | Not used in evaluation | Assume UE uses maximum power for SRS transmission |
| SFN0 Offset | 0 | Assume TRPs are SFN aligned |
| NumOfUL-PositioningResourcesPerSet | 16 | FFS: Other values |
| NumOfUL-PositioningResources | 16 | FFS: Other values |
| maxNumOfUL-PositioningResourceSets | 1 | FFS: Other values |

Interested companies are welcome to provide their opinions in the following table for above proposed SRS configuration and parameters.

Table 4-9 Summary of company views on the UL SRS configuration parameters

| Company | Comments |
| --- | --- |
| Huawei | For the proposals related to DL PRS and SRS configuration, some parameters are duplicated with Table 6.1.1-1, e.g., BW, numerology, etc. In fact, we do not think it is necessary to explicitly mention the PRS or SRS configuration, since companies still have freedom to select the comb size, number of symbols, etc. |
| Fraunhofer | UL PRS parameter configuration are relevant in case interference/ positioning RE capacity is taken into account. Proposal: Consider UL-PRS interference between the links using the same REs. (Example: Regular resource reuse scheme: Split scenario in „subareas“where UEs in a given area share the same REs). |
| Nokia | Regarding path loss reference and TPC, we may need further clarification on the assumption. The scenario assumes “UE uses maximum power for SRS transmission,” then interference concerns may be raised between UEs. Is the assumption in Rel-17 similar to Rel-16 work that interference is not considered, and “perfect” muting is assumed both in DL and UL? |
| CATT | We are fine for the UL SRS Configuration (table 4-8). Maybe the values of several parameters would be adjusted according to further discussion, but a common understanding is needed on the reasonable UL SRS configurations for Rel-17 positioning simulation evaluation. On the power control, suggest just keep the fixed power. It will be much simpler to use the fixed power for the simulation and also for the comparison of the results. |
| LG | We are generally fine with the table 4-8. There are many possible cases of SRS configuration, so if it is possible, it would be desirable to have a common configuration, and evaluation for additional cases could be up to company decision. |
| Qualcomm | We have preference to assume perfect muting for both DL and UL (that is, orthogonal resources used for both DL and UL) as the baseline. Also, no need to simulate power control. Regarding the number of resources, consider the following for FR1 baseline: - NumOfUL-PositioningResourcesPerSet = 1 - NumOfUL-PositioningResources = 1 For FR2, these two values can be increased up to 64, and it should be up to each company to report how many resources they used. |
| Ericsson | We think this should be left to companies, provided that the results from each companies come with the RS configurations and method used. |

## Others

Interested companies are welcome to provide their additional comments related to IIOT use cases, the simulation scenarios and associated parameter settings, etc. in the following table.

Table 4-10 Summary of additional comments

| Company | Comments |
| --- | --- |
| ZTE | If we get a good performance in the IIOT scenario with bigger NLOS possibility, the same simulation will get much better results in indoor office with more LOS paths, or UMa and UMi under a reasonable TRP density, maybe O2I is the only exception. In the past, we always pursue a general solution that applies to all the earth, because the E-SMLC or LMF will cover a very wide area, if the LMC was supported in a few months, a global solution would be unnecessary. A solution for a car factory does not need be suitable for a mobile phone factory since they may use different Local-LMF. Maybe the note 3 can remind us that maybe a more specific evaluation assumption besides the 5 general IIOT scenarios should also be considered? |
| Huawei | For the comment from ZTE, we do not think IIOT scenario should be necessarily associated with a higher NLOS probability, since there are multiple sub-scenarios. The five scenarios to be down-selected are already sufficient, we do not see the need to include more scenarios. Aspects related to LCS architecture or evaluations of positioning based on local LMF would require RAN plenary decision before being included in the scope of the study. |
| CATT | Our thinking is that additional IIOT scenarios could be useful for the future work. But, for this SI we would suggest focusing on the scenarios already defined in TS 38.901, since we may not have the time budget to consider additional IIOT scenarios other than those already defined in 38.901 due to the constraint of the SI schedule. |
| Fraunhofer | - Define performance metrics for Rel-17 positioning evaluation: - Evaluate position error statistics for all UEs and UEs in passage area separately - Horizontal accuracy - Vertical accuracy - Availability - To clarify diverse the simulation results as in Rel16 SI, provide additional statistics like - SINR for the 4 (3 for RTT) best links - ToA error statistics of the 4 (3 RTT) best links (method for selecting best links may be implementation specific) - Joint probability of LOS (number of links in LOS) |
| Nokia | The main interest of Rel-17 study is high accuracy performance of IIoT positioning. Therefore, it would be reasonable to prioritize configuration of full resources of PRS, SRS (i.e. comb-2, n12) over other configurations. The goal should be to see what is the best accuracy we can achieve. |
| CATT | Performance evaluation in R17 SI stage should focus on positioning accuracy evaluation. For latency requirements, first of all, a clear definition and calculation method of positioning latency should be clarified, and then try to evaluate the latency of different positioning techniques. In our point of view, RAN1 cannot accurately simulate end-to-end latency, which involves a lot of factors and it should be obtained by analysis instead of simulation. When discussing NR positioning enhancements, we should identify its impact on positioning latency and pay attention to the reduction of physical layer latency. |
| LG | Similar view with CATT. As mentioned earlier, latency is an important design factor, but we need to first clarify meaning of latency. That is, we might need to clarify whether it means the time to calculate and acquire the location of the UE or the time required to send and receive the necessary signals in physical layer. |
| Futurewei | We think eventually for this SI, the enhancements that would be adopted would be those that are addressing issues or challenges unique to IIOT that were not specifically addressed in R16. Aspects such as vertical accuracy, NLOS impacts, and latency performance. |

Conclusions

This contribution provides a summary of the email discussion on the IIOT use cases, the simulation scenarios and associated parameter settings for the Rel-17 SI on NR Positioning Enhancements prior to the meeting. This summary is submitted before the meeting for information purpose.

References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”,V16.0.0 (2019-10)

- 3GPP TR 38.855, “Study on NR positioning support”,  V16.0.0 (2019-03)

- 3GPP TR 22.804, “Study on Communication for Automation in Vertical Domains”, V16.2.0 (2018-12)

- R1-1913674, “Updated consolidated parameter list for Rel-16 NR”

Appendix

## Positioning Service Performance Requirements (TS 22.804)

Table 8.1.7   Positioning Service Performance Requirements (TS 22.804)

| Use case reference | Scenario | Horizontal accuracy | Availability | Heading | Latency for position estimation of UE | UE Mobility |
| --- | --- | --- | --- | --- | --- | --- |
| Factories of the Future 6.5 | Mobile control panels with safety functions in smart factories (within factory danger zones) | < 1 m | 99.9% | < 0,54 rad | < 1 s | N/A |
| Factories of the Future 6.7 | Mobile control panels with safety functions ( non-danger zones) | < 5 m | 90% | N/A | < 5 s- | N/A |
| Factories of the future 10.8 | Augmented reality in smart factories | < 1 m | 99% | < 0,17 rad | < 15 ms | < 10 km/h |
| Factories of the Future 13.3 | Process automation – plant asset management | < 1 m | 90% | N/A | < 2 s | < 30 km/h |
| Factories of the Future15.5 | Inbound logistics for manufacturing (for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems) ) | < 30 cm (if supported by further sensors like camera, GNSS, IMU) | 99.9% | N/A | 10 ms | < 30 km/h |
| Factories of the Future15.6 | Inbound logistics for manufacturing (for storage of goods) | < 20 cm | 99% | N/A | < 1 s | < 30 km/h |
| Factories of the Future18.19 | Flexible, modular assembly area in smart factories (for autonomous vehicles (only for monitoring proposes)) | < 50 cm | 99% | N/A | 1 s | < 30 km/h |
| Factories of the Future18.20 | Flexible, modular assembly area in smart factories (for tracking of tools at the work-place location) | < 1m (relative positioning) | 99% | N/A | 1 s | < 30km/h |

Evaluation parameters for InF (TR 38.901)

Table 7.2-4: Evaluation parameters for InF (TR 38.901)

|  |  | InF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| Layout | Room size | Rectangular: 20-160000 m2 |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height | Clutter-embedded |  |  |  | Above clutter |

## Common scenario parameters applicable for all scenarios (TR 38.855)

Table 6.1.1-1: Common scenario parameters applicable for all scenarios (TR 38.855)

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE model parameters |  |  |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

Table 6.1.1-2: UE antenna radiation pattern model 1 (FR2) (TR 38.855)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

Simulation assumptions for calibration of the indoor factory scenario (TR 38.901)

Table 7.8-7: Simulation assumptions for large scale calibration for the indoor factory scenario

| Parameter | Values |
| --- | --- |
| Scenario | InF-SL, InF-DL, InF-SH, InF-DH |
| Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| Sectorization | None |
| BS antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| UT antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| Handover margin (for calibration) | 0dB |
| BS deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for for InF-SH and InF-DH |
| UT distribution | uniform dropping for indoor with minimum 2D distance of 1 m UT height = 1.5 m |
| UT attachment | Based on pathloss |
| UT noise figure | 9 dB |
| Carrier frequency | 3.5 GHz, 28 GHz |
| Bandwidth | 100 MHz |
| Clutter density: r | Low clutter density: 20% High clutter density: 60% |
| Clutter height: hc | Low clutter density: 2 m High clutter density: 6 m |
| Clutter size: dclutter | Low clutter density: 10 m High clutter density: 2 m |
| Metrics | 1) Coupling loss – serving cell |
|  | 2) Geometry with and without noise |
|  | 3) CDF of delay and angle spread (ASD, ZSD, ASA, ZSA) according to the definition in Annex A.1 |
|  | 4) CDF of first path excess delay for serving cell |

## Performance requirements for Horizontal and Vertical positioning service levels (TS 22.261)

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99.9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0.3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0.3 m | 2 m | 99.9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0.2 m | 0.2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each others (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |