3GPP TSG RAN WG1 #101                                                     R1- 2003663

e-Meeting, May 25th – June 5th, 2020

Source: MediaTek Inc.

Title: maintenance of low PAPR RS

Agenda Item: 7.2.6.5

Document for:  Discussion, Decision

# Introduction

In this contribution, we discuss a remaining issue of low PAPR RS.

# Block-wise spreading for PUCCH format 3 and 4 with pi/2-BPSK DMRS

In RAN1#96 meeting, the following agreement was reached:

Agreement

The PUCCH multiplexing capacity when Rel-16 DMRS is configured for pi/2 BPSK PUCCH is a single port

As the result, PUCCH with pi/2-BPSK DMRS is expected to apply NSFPUCCH,s=1. The same issue was pointed out by company [1]. We have the following text proposal.

Text Proposal: for Section 6.3.2.6.3 of 38.211:

##### 6.3.2.6.3	Block-wise spreading

<Unchanged parts are omitted>

For PUCCH format 3 with interlaced mapping and PUCCH format 4, block-wise spreading shall be applied according to

ylMscPUCCH,s+k=wnkNSFPUCCH,sMscPUCCH,sdlMscPUCCH,sNSFPUCCH,s+k mod MscPUCCH,sNSFPUCCH,s
k=0,1,…, MscPUCCH,s-1
l=0,1,…,NSFPUCCH,sMsymbMscPUCCH,s-1

where

-	for PUCCH format 3 with interlaced mapping, NSFPUCCH,3∈1,2,4 if a single interlace is configured and NSFPUCCH,3=1, wn=1 if two interlaces are configured or pi2BPSK is enabled and dmrsUplinkTransformPrecodingPUCCH-r16 is configured;

-	for PUCCH format 4,MRBPUCCH,4=1,. NSFPUCCH,4=1, wn=1 if pi2BPSK is enabled and dmrsUplinkTransformPrecodingPUCCH-r16 is configured; otherwise NSFPUCCH,4∈2,4;

and  is given by Tables 6.3.2.6.3-1 and 6.3.2.6.3-2 for NSFPUCCH,s>1 where n is the index of the orthogonal sequence to use according to clause 9.2.1 of [5, TS 38.213].

# References

[1] R1-2002297, “Maintenance on Rel-16 Low PAPR RS,” Nokia, Nokia Shanghai Bell, RAN 100bis e-Meeting