3GPP TSG RAN WG1 Meeting #101-e 	R1-2003677

e-Meeting, May 25th – June 05th, 2020

Agenda Item: 7.2.11.10

Source: MediaTek Inc.

Title: 	Views on Rel-16 UE features for MR-DC/CA

Document for: Discussion

- Introduction

This paper provides our views on Rel-16 UE features for MR-DC/CA. Detailed proposals are described in the following sections.

- Views on UE features for MR-DC/CA

For FG 18-4, two Cases of SCell dormancy indication are supported when the indication is sent within DRX Active Time.

- Case 1 SCell dormancy indication:

  - DCI format 0_1 and 1_1 with additional bit field “SCell dormancy indication” are used.

  - Case 1 DCI can still schedule PDSCH/PUSCH, and the timeline for HARQ-ACK information feedback is the same as N1 in Rel-15.

  - 1 bit in “SCell dormancy indication” indicates SCell dormancy/non-dormancy for a group of SCells.

- Case 2 SCell dormancy indication:

  - DCI format 1_1 with some repurposed bit fields is used.

  - Case 2 DCI cannot schedule PDSCH but UE still needs to report HARQ-ACK information. Its HARQ-ACK report timeline is tighter than in Case 1. (working assumption: timeline is the same as in HARQ-ACK information report for SPS PDSCH release).

  - 1 bit of SCell dormancy indication indicates SCell dormancy/non-dormancy for a SCell.

Because of many differences between Case 1 and Case 2 SCell dormancy indication, it is more reasonable to have separated FGs for them. We suggest the following revisions:

Proposal 1:

- FG 18-4: SCell dormancy indication within active time with data scheduling

  - Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 scheduling PUSCH/PDSCH

- Keep FG 18-4b: SCell dormancy indication without data scheduling within active time

  - Support for SCell dormancy indication sent within the active time on PCell via DCI format 1_1 without PDSCH scheduling

For cross-carrier scheduling/ACSI-RS triggering, according to GTW session of DC/CA UE features in RAN1 #100-bis-e, companies agreed that UE capabilities 18-5a/18-6a for default QCL assumption should apply to both same/different numerologies. Hence, it would be better to add add “for same/different numerologies” to FG 18-5a and FG 18-6a for clarification.

Proposal 2: Add “for same/different numerologies” to FG 18-5a and FG 18-6a for clarification (according to previous companies consensus during GTW session of DC/CA UE features in RAN1 #100-bis-e).

For FG 18-5: Cross-carrier scheduling with different SCS, it is sufficient to reuse the Rel. 16 FG 3-5b PDCCH monitoring to have multiple DCIs in one slot of the scheduling cell with lower SCS than the scheduled cell. This has the benefit of avoiding introducing additional impact to the spec (e.g., new design for HARQ-ACK codebook). The value of X should be deleted since RAN1 never achieved consensus to set this X values for basic features. We are open to introduce components 2 of both 18-5/5b as separate featrues, say as 18-5e and 18-5f.

Proposal 3: Delete the descriptions related to value of X (component 2) in FG 18-5 and FG 18-5b.

Proposal 4: FG 18-8 “HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group” should be optional with capability signaling.

Proposal 5: For FG 18-4/4a, Type to be per BC or per UE.

Proposal 6: For FG 18-5/5a/5b/6/6a, Type to be per band and per BC.

For [18-5c] and [18-5d], these are cross-carrier counterpart for the same-carrier features:

- FG 5-5a -- UE PDSCH processing capability #2

- FG 5-5b -- UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS

- FG 5-5c -- UE PUSCH processing capability #2

Since 5-5a/b/c are all applicable to FR1 only, we think [18-5c] and [18-5d] should also be applicable to FR1 only.

Proposal 7: FG [18-5c] and [18-5d] should be applicable to FR1 only.

According to the agreement in RAN1#100e,

Agreements:

- To extend the cross-carrier scheduling with different SCS to be operable with DCI format 1_2 defined for Rel-16 in eURLLC WI.

  - Note: to be discussed under UE feature discussion regarding whether or not to introduce separate UE capability to indicate the support for cross-carrier scheduling with different SCS to be operable with DCI format 1_1 and DCI format 1_2

Proposal 8: Introduce separate UE capability to indicate the support of the DCI format 1_2 for cross-carrier scheduling (FG 18-5e).

# Proposed changes on current RAN1 UE features table

MR-DC/CA enhancement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 18. MR-DC/CA enhancement | 18-1 | Basic UL power sharing for DC | Semi-static power sharing mode1 between MCG and SCG cells of same FR for NR dual connectivity. | [intra-FR DC if such FG is introduced by RAN2] | Yes | N/A |  | Per BC | N/A | N/A | N/A | Absence means intra-FR DC is not supported. | Optional with capability signalling |
|  | 18-1a | Semi-static UL power sharing mode 2 for DC | Semi-static power sharing mode 2 between MCG and SCG cells of same FR for NR dual connectivity. | 18-1 | Yes | N/A |  | Per BC | N/A | N/A | N/A | Semi-static power sharing mode 2 between MCG and SCG cells of same FR is applicable only for synchronous NR dual connectivity | Optional with capability signalling |
|  | 18-1b | Dynamic UL power sharing for DC | Dynamic power sharing between MCG and SCG cells of same FR for NR dual connectivity. - T_offset | 18-1 | Yes | N/A |  | Per BC | N/A | N/A | N/A | 1) {short, long} | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-4 | SCell dormancy indication within active time with data scheduling | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_1/1_1 scheduling PUSCH/PDSCH | 6-5 | Yes | N/A |  | [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-4a | SCell dormancy indication outside active time | Support for SCell dormancy indication sent outside the active time on PCell with DCI format 2_6 | 6-5 and [19-1] | Yes | N/A |  | [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-4b | Support of SCell dormancy indication without data scheduling within active time | Support of SCell dormancy indication without data scheduling within active time on PCell via DCI format 1_1 without PDSCH scheduling | TBD | Yes | N/A |  | [Per UE or Per BC] | No | [Yes or N/A] | N/A |  | Optional with capability signaling |
| 18. MR-DC/CA enhancement | 18-5 | DL cross-carrier scheduling with different SCS | 1. The UE supports DL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} | 6-5 and one of {6-9, 6-9a} | Yes | N/A |  | Per band and per BC | N/A | [Yes or N/A] | N/A | crossCarrierScheduling-OtherSCS Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | 18-5a | Default QCL assumption for cross-carrier scheduling for same/different numerologies | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier scheduling for same/different numerologies. | 6-10 and 18-5 | Yes | N/A |  | Per band and per BC | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
|  | 18-5b | UL cross-carrier scheduling with different SCS | 1. The UE supports UL cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in UL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} | 6-6 and one of {6-9, 6-9a} | Yes | N/A |  | Per band and per BC | N/A | [Yes or N/A] | N/A | crossCarrierScheduling-OtherSCS Note: This applies also to the case where there is a single span in the slot for the scheduling CC. In case UE supports 3-5b, the limits apply for each span for FDD scheduling cell and TDD scheduling cell. | Optional with capability signalling |
|  | [18-5c] | [DL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | [DL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | 18-5a TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | Applicable to FR1 only | N/A |  | [Optional with capability signaling] |
|  | [18-5d] | [UL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | [UL cross-carrier scheduling with different SCS and PDSCH processing capability 2] | 18-5b TBD | Yes | N/A |  | FFS[Per UE or Per band and per BC or Per FS] | N/A | Applicable to FR1 only | N/A |  | [Optional with capability signaling] |
|  | 18-5e | DL cross-carrier scheduling with DCI format 1_2 | DL cross-carrier scheduling with DCI format 1_2 | 6-10 or 18-5 | Yes | N/A |  | Per band and per BC | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-6 | Cross-carrier A-CSI RS triggering with different SCS | Cross-carrier A-CSI RS triggering with different SCS | 2-33 and 6-5 | Yes | N/A |  | Per band and per BC | N/A | [Yes or N/A] | N/A | 1) {PDCCH cell of lower SCS and A-CSI RS cell of higher SCS, PDCCH cell of higher SCS and A-CSI-RS of lower SCS, both} . | Optional with capability signalling |
|  | 18-6a | Default QCL assumption for cross-carrier A-CSI-RS triggering for same/different numerologies | Indicates whether the UE can be configured with enabledDefaultBeamForCCS for default QCL assumption for cross-carrier A-CSI-RS triggering for same/different numerologies. | 6-5 | Yes | N/A |  | Per band and per BC | N/A | [Yes or N/A] | N/A |  | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries for inter-band CA | 6-5 | Yes | N/A |  | Per BC | N/A | N/A | N/A | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the Pcell and the Scell are not aligned, while the slot boundaries are. | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | 6-7 | Yes | N/A |  | Per UE | No | No | N/A | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-2 | Single UL TX operation for TDD PCell in EN-DC | TDM restriction to LTE TDD PCell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) TDD UL/DL configuration#2, #4, #5 configured as DL-reference UL/DL configuration 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (only for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe not limited to the reference TDM pattern (only for type 1 UE) [4) dropping NR transmission when LTE and NR transmissions collide for Type 1 UE] | EN-DC | Yes | N/A |  | Per band combination | Applicable to TDD-TDD EN-DC only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to TDD PCell This FG is for synchronous EN-DC | Optional with capability signalling |
|  | 18-2a | Enhanced single UL TX operation for FDD Pcell EN-DC | TDM restriction to LTE FDD Pcell in EN-DC for single UL-Transmission associated functionality when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (only for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe not limited to the reference TDM pattern (only for type 1 UE) [4) dropping NR transmission when LTE and NR transmissions collide for Type 1 UE] | [6-13] | Yes | N/A |  | Per band combination | Applicable to in FDD-LTE -NR EN-DC | Applicable to FR1 only |  | Enhancement to the R15 capability tdm-Pattern [This FG is for synchronous EN-DC] | Optional with capability signalling |
|  | 18-2b | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | Support of HARQ-offset for SUO case1 in EN-DC with LTE TDD PCell for type 1 UE | 18-2 | Yes | N/A |  | Per band combination | N/A | N/A | N/A | [This FG is for synchronous EN-DC] | Optional with capability signaling |
|  | 18-3 | Dual Tx transmission for EN-DC with FDD PCell(TDM pattern for dual Tx UE) | TDM restriction to LTE FDD PCell in EN-DC for dual UL Tx operation when tdm-patternConfig-r16 is configured 1) DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) PRACH transmission in non- designated UL subframes given by the DL-reference configuration (only for type 1 UE) 3) LTE UL transmissions scheduled/triggered by a DCI in any UL subframe not limited to the reference TDM pattern (only for type 1 UE) | [6-13], EN-DC | Yes | N/A |  | Per band combination | Applicable to EN-DC with LTE FDD PCell only | Applicable to FR1 only |  | Extension of the R15 capability tdm-Pattern to a dual Tx UE [This FG is for synchronous EN-DC] | Optional with capability signalling |
|  | 18-3a | Semi-statically configured LTE UL transmissions in all UL subframes not limited to tdm-pattern | UE configured with tdm-patternConfig-r16 can be semi-statically configured with LTE UL transmissions in all UL subframes not limited to the reference tdm-pattern (only for type 1 UE) | One of {18-2, 18-2a, 18-3} | Yes | N/A |  | [Per band combination] | Applicable to EN-DC only | Applicable to FR1 only |  | [This FG is for synchronous EN-DC] | Optional with capability signaling |