3GPP TSG RAN WG1 #101                                                             R1-2003680

e-Meeting, May 25th – June 5th, 2020

Agenda item:	8.1.2

Source: 	MediaTek Inc.

Title: 	Channel access mechanism for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and decision

# Introduction

In this submission we summarize the regulatory landscape in above 57GHz unlicensed spectrum bands, with particular focus of regulations pertaining to CEPT and the FCC. We observe that the current status of unlicensed mm-wave regulations in these regions imposes a limit on the type and location of devices that can currently be brought to market.

With this is mind we look to the previous studies undertaken by 3GPP and summarized any output of 38.805, 38.913, 38.900 and 38.807 that may be relevant to above 57GHz unlicensed spectrum under these current regulatory constraints.

# A short introduction of regulation, both ETSI and FCC, that are most relevant to defining the simulation assumptions

Currently only one ETSI specification [1] is mature enough to be considered available that covers above 57GHz unlicensed spectrum for wideband data transmission systems. On top of this it only covers low power scenarios associated with the c1 band as defined in annex 3 of [2] or band 75 as defined in [3] for wideband data transmission systems. Another ETSI BRAN work item for wideband data transmission systems was approved in the BRAN#104 meeting that will eventually result in a second specification [4] covering the c2 and c3 bands defined in in annex 3 of [2] or bands 75a and 75b as defined in [3] for wideband data transmission systems. But [4] is still in very early draft and not yet in a suitable state to draw meaningful limits on allowable behaviours. ETSI drafted [1] on the basis of an ETSI TR [5]. This TR has since been superseded by a second ETSI TR [6] that in turn has formed the basis for [4].

As a consequence, at least in Europe, only a sub-set of the 3GPP proposed use-cases are currently available in unlicensed spectrum above 57GHz. These uses cases are confined to low power indoor scenarios only, with Fixed Local Area Network Extension (FLANE) or Fixed Point-to-Point applications are specifically excluded in [1] while fixed outdoor is explicitly disallowed in band c1 of [2]. It should be made clear that [1] defines a lower power limit than is recommended in band c1 of [2]. It should be further noted that short range devices (SRDs) are not covered by either [1] or [4].  Non-specific short range devices (SRDs) are covered by a separate annex 1 in [2] by the n1 and n2 bands which in turn is covered by yet another ETSI harmonised standard [7] which specifies considerably lower power maximum than that available in [1] or [4].

At least in Europe and all accompanying CEPT regions, any 3GPP devices put to market in the above 57GHz unlicensed spectrum can currently only be low power indoor wideband data transmission systems or very low power SRDs. The lack of a harmonised standard (HS) covering higher power indoor devices or fixed outdoor wideband data transmission systems and SRDs, will make bringing 3GPP complaint devices to market that are not of the aforementioned, on a state-by-state basis, extremely burdensome.

It is also worth highlighting, mainly for coexistence reasons and also interference mitigation reasons, the lower half of the band 57GHz – 64GHz has also been recommended in [2] for use by Radiodetermination Applications, also known as mm-wave radar. This too is covered by an ETSI HS [8]. The FCC also has published regulations for 57 – 71 GHz in [9]. For comparison we summarize all the currently available limits for these standards in Table 1.

Since 3GPP seeks a global solution, for now at least, while regulations for fixed and outdoor scenarios are finalised, 3GPP should concentrate on what is already covered by existing regulations.

Table 1: Frequency and power limits

|  | FCC | EN 305 550 | EN 302 567 | EN 303 722 | ERC/REC/70-03 |
| --- | --- | --- | --- | --- | --- |
| Frequency (GHz) | 57 – 71 | 57 – 64 61 – 61.5 | 57 – 66 | 57 – 71 | 57 – 71 |
| Spectral density (dBm/MHz) (EIRP) | ¥ | 13 | 13 | 23 | 23 |
| Antenna gain GA > 30dBi * | n/a | n/a | n/a | 38 | 38 |
| Maximum power (EIRP) (dBm) | 40 avg + 43 peak | 20 | 40 | 40 | 40 |
| Antenna gain GA < 13dBi | n/a | n/a | n/a | 27 + GA | 40 |
| Antenna gain GA > 30dBi * | n/a | n/a | n/a | 55 | 55 |
| Antenna gain GA > 51dBi # | 82 avg 85 peak | n/a | n/a | n/a | n/a |
| Antenna gain GA < 51dBi # | 82 - 2(51 - GA) | n/a | n/a | n/a | n/a |
| * Fixed outdoor installations only, + Indoor, # Outdoor, ¥ Max 500mW for emission-BW larger than 100MHz, else 500mW × emission-BW / 100MHz |  |  |  |  |  |

The highlighted columns are the only currently published regulations that pertains to wideband data transmission systems in ITU Region 1 and ITU Region 2.

# Prioritization of use cases from the wide range of selection listed in TR 38.807

Owing to the limitations of mm-wave capabilities for mobility along with the lack of global regulatory allowances for outdoor applications we would propose to concentrate on indoor use cases first.

Observation: In a global context, currently only indoor low power use cases are universally covered worldwide for the unlicensed spectrum above 57GHz.

The obvious use case being high data rate eMBB, the foundation of NR and subsequently NR-U. Mobile offloading and private networks would be other obvious priorities along with any factory automation uses, which again would include eMBB, mobile offload and private networks. Indoor short range devices that implement point-to-point type links that could be covered by a 3GPP side-link should be considered, such as AR/VR devices, wireless connectivity such as HDMI, data-center inter-rack connectivity, etc.

Proposal: Prioritise simulation effort to use cases, scenarios, channel and traffic modelling that are relevant to indoor use cases only.

# Simulation assumptions for the prioritized use case(s)

## Layout

Given the lack of ETSI standards covering outdoor devices where Fixed Local Area Network Extension (FLANE) or Fixed Point-to-Point applications are specifically excluded in [1] while fixed outdoor is explicitly disallowed in band c1 of [2]. We would thus suggest confining our discussion to only those pertaining to indoor scenarios where the layout would be covered by one of either [1] or [8] ETSI harmonised standards. If we look to previous 3GPP studies in [10], [11], [12] or [13] and the various proposed scenarios, this would summarise to Table 2.

Table 2: Mapping of 3GPP proposed use cases to 3GPP proposed scenarios.

| Deployment scenarios | Use cases |
| --- | --- |
| Indoor hotspot | High data rate eMBB, Mobile data offloading |
| Indoor office | Private Networks, Integrated access backhaul (IAB) |
| Indoor public | High data rate eMBB, Integrated access backhaul (IAB) |
| Factory Hall | Vertical industry factory application, Factory automation/Industrial IoT(IIoT), Private Networks, smart grid automation (note: smart grid automation use case is not limited to indoors only) |
| Indoor D2D | Short-range high-data rate D2D communications, Data center inter-rack connectivity, Critical medical applications, Augmented reality/virtual reality headsets and other high-end wearables, Wireless Display Transfer |

For the same reasons as in previous sections, the layouts considered in should, for now at least, be confined to those relevant to indoor scenarios. The layouts detailed in [10], [11], [12] and [13] for indoor are listed as:

- Urban micro - street canyon

- Urban macro

- Indoor - office

- Urban micro - open square

- Indoor - shopping mall

It could be debated that urban macro and urban micro are not indoor, however there are assertions in [11] and [12] that these might extend to indoor too. Appropriate corresponding channel model parameterization for 57 to 72 GHz should be utilized in these scenario modelling. Maybe somewhat obviously, 3GPP should consider low to no mobility only.

Link budget analysis for both LOS and NLOS cases show that both the FCC and ETSI rules described in Table 1 allow for meaningful throughput with some ease in these indoor environments.

## Traffic model

Most traffic models in 3GPP studies originated from LTE [14], including two FTP models: FTP Model 1 and FTP Model 2. In LAA study, a third FTP model, FTP Model 3, was introduced in [15]. It is based on the FTP Model 2 in [14] except that packets for the same UE arrive according to a Poisson process with arrival rate 𝜆 and the transmission time of a packet is counted from the time instance it arrives in the queue. FTP Model 3 and FTP Model 1 are the two models used in the LAA co-existence study.

Sub-7 GHz NR-U study reused the traffic models from LAA, without change. All the simulation results compiled in [15] are based on these models.

The arrival rate of these traffic models varies, but the file size is fixed at 0.5 MB. They were chosen to emulate web browsing application, which is probably not what 3GPP is targeting for 57 to 72 GHz. We would suggest to update the fixed file size to support applications intended for these 57 to 72 GHz use cases.

Observation: Whatever the traffic model used for greater than 57GHz, it will likely require a larger file size than is currently specified in any previous 3GPP reports covering traffic modelling.

The Release-17 approved but postponed study item on XR may provide a valuable starting point to determining what any new file size should be for these applications expecting an average data rate of 100 Mbps and a peak data rate of 300 Mbps.

## Baseline channel access mechanism

The only regulation that specifies any rules for channel access can be found in [5]. The current version defers to an idle period of 8us followed by a random number of 5us observation slots with a maximum of 127 observation slots. However, in a recent ETSI BRAN meeting it was agreed to rewrite this to the same idle period of 8us followed by a random number of 5us observation slots instead with a minimum of 3 observation slots. A timing diagram of this is shown in Figure 1. Going forward there will not be a maximum number of observation slots.

Figure 1: EN 302 567 channel access restrictions

It has been shown in ETSI BRAN submissions towards [1] and [4] that even though coexistence is an important aspect of unlicensed spectrum and not to be ignored, the highly directional nature of > 57GHz transmissions implies that the probability of encountering competing device interference in the exact same direction at the same time at a power level that is sufficient to create reception issues is quite low. This coupled by the fact that massive bandwidths are available resulting in massive instantaneous data-rates implies on-air time is likely to be greatly reduced for the same traffic load compared to sub 7GHz channel access. As a result, the expectation is that average COT sizes will likely be of much less duration than in sub 7GHz.

Observation: Average COT durations will likely be less than 1ms in duration.

## Co-existence with other system

Both the IEEE and 3GPP are developing standards that will access 57GHz to 72GHz unlicensed spectrum, as will SRDs possibly utilizing proprietary link definitions. It is also true that IEE802.11ad is further along in this realm. It is thus important that, like the 5GHz and 6GHz unlicensed spectrum allocations, coexistence be considered an important aspect of any new specification work.

It is important for 3GPP to maintain its standing as a good neighbour at this new frequency allocation, even while the possibilities for neighbour interference are reduced due to highly directional antenna arrays.

# Conclusion

In this contribution, we make the flowing observation:

Observation: In a global context, currently only indoor low power use cases are universally covered worldwide for the unlicensed spectrum above 57GHz.

Observation: Whatever the traffic model used for greater than 57GHz, it will likely require a larger file size than is currently specified in any previous 3GPP reports covering traffic modelling.

Observation: Average COT durations will likely be less than 1ms in duration.

Further to this observation, we make the following proposal:

Proposal: Prioritise simulation effort to use cases, scenarios, channel and traffic modelling that are relevant to indoor use cases only.

# Reference

- ETSI EN 302 567 V2.1.1 (2017-05), “Multiple-Gigabit/s radio equipment operating in the 60 GHz band “, currently in draft as Draft ETSI EN 302 567 V2.1.18 (2020-03)

- European Research Council (ERC) Recommendation 70-03

- EU Commission Decision 2006/771/EC

- Draft ETSI EN 303 722 V0.0.3 (2019-12) , “Wideband Data Transmission System (WDTS) for Fixed Network Radio Equipment operating in the 57 - 71 GHz band”

- ETSI TR 102 555 V1.1.1 (2007-02), “Technical characteristics of multiple gigabit wireless systems in the 60 GHz range”

- ETSI TR 103 583 V1.1.1 (2019-08) , “Multiple Gigabit Wireless Systems (MGWS) in radio spectrum between 57 GHz and 71 GHz”

- ETSI EN 305 550 V2.1.0 (2017-10), “Short Range Devices (SRD); Radio equipment to be used in the 40 GHz to 246 GHz frequency range”

- EN 302 729 V2.1.0 (2016-04), “Short Range Devices (SRD); Level Probing Radar (LPR) equipment operating in the frequency ranges 6 GHz to 8,5 GHz, 24,05 GHz to 26,5 GHz, 57 GHz to 64 GHz, 75 GHz to 85 GHz”, currently in draft as Draft ETSI EN 302 729-1 and Draft ETSI EN 302 729-2

- FCC 47CFR 15.255: Code of Federal Regulations Title 47 part 15 - radio frequency devices (RF exposure requirements: 47CFR 1.1310, 2.1091, 2.1093).

- EC Report 288

- 3GPP TR 38.807 V16.0.0 (2019-12), “Study on requirements for NR beyond 52.6 GHz”

- 3GPP TR 38.900 V15.0.0 (2018-06), “Study on channel model for frequency spectrum above 6 GHz”

- 3GPP TR 38.913 V15.0.0 (2018-06), “Study on Scenarios and Requirements for Next Generation Access Technologies”

- 3GPP TR 38.805 V14.0.0 (2017-03), “Study on New Radio access technology; 60 GHz unlicensed spectrum”

- 3GPP TR 36.814 V9.2.0 (2017-03), “Further advancements for E-UTRA physical layer aspects”

- 3GPP TR 36.889 V13.0.0 (2015-06), “Study on Licensed-Assisted Access to Unlicensed Spectrum”