3GPP TSG RAN WG1 Meeting #101-e  		R1-2003689

e-Meeting, May 25th – June 5th, 2020

Agenda Item: 8.3.3

Source: MediaTek Inc.

Title: Discussion on coverage recovery for NR RedCap UEs

Document for: Discussion and Decision

# Introduction

In RAN plenary meeting #86, a new study item was approved to support reduced capability NR devices in Rel-17 [1]. One of the objectives of the SI is to study functionality to mitigate or limit the performance degradation of the complexity reduction features for Reduced Capability (RedCap) NR UEs.

Study functionality that will enable the performance degradation of such complexity reduction to be mitigated or limited, including [RAN1]: - Coverage recovery to compensate for potential coverage reduction due to the device complexity reduction.

In this contribution, we will discuss and evaluate the impact of UE complexity reduction features on the coverage of NR channels, and present the possible mechanisms to compensate for the coverage loss.

# Discussion UE complexity reduction features

The following features for complexity reduction are considered in the RedCap SI:

- Reduced number of UE RX/TX antennas - UE Bandwidth reduction Note: Rel-15 SSB bandwidth should be reused and L1 changes minimized - Half-Duplex-FDD - Relaxed UE processing time - Relaxed UE processing capability

NR UL Channels:

Among the complexity reduction features, reducing the number of UE RX/TX antennas is expected to impact the coverage of the NR physical channels. In NR Rel-15, the UE is not mandated to support more than 1 Tx. Thus, there is no expected coverage gap between RedCap UEs and baseline/full-capable NR UEs. Hence, no enchantments for NR UL channels are needed within the scope of this SI/WI.

- No changes are required to the minimum number (1) of mandatory Tx antenna in the standard.

- The coverage of NR UL channels is not impacted by the features for complexity reduction for RedCap NR UEs.

- Coverage compensation mechanisms for NR UL channels are not considered in RedCap NR SI.

NR DL Channels:

On the other hand, as discussed in our companion Tdoc [2], the #Rx for RedCap UEs need to be reduced as follows:

-	Above 2496 MHz: from 4 Rx to 2 Rx antennae

-	Below 2496 MHz: from 2 Rx to 1 Rx antenna

Hence, there is expected impact to NR DL channels. For DL data channel (PDSCH), NR offers several mechanisms to enhance the transmission reliability; such as PDSCH repetition and HARQ retransmission. Most importantly, the HARQ retransmission is the most efficient method to recover the performance loss that results from reducing the #Rx antennae. Hence, coverage compensation for NR PDSCH shouldn’t be of high priority in RedCap SI. However, small enhancements such as increasing the maximum number of PDSCH repetitions shouldn’t be excluded.

- Existing NR schemes, such as PDSCH repetition and HARQ retransmission, offers high flexibility for the network to recover any coverage loss for PDSCH due to reduced #Rx antennae.

- Coverage compensation mechanisms for PDSCH are considered of low priority for RedCap NR SI.

For NR PDSCH, the flexibility of DL control allocation is very limited compared to the DL data allocation due to the following reasons;

- There is no repetition/retransmission schemes for NR PDCCH

- No opportunity for soft combining, hence no SNR improvements from retransmissions

- The control channel uses selected aggregation levels (that are powers of 2) to increase user packing

- No user-specific beamforming can be applied to the DL control channel, hence no SNR improvements

- The maximum number of monitored CCEs and PDCCH candidates by a UE per slot is fixed

Thus, it is expected that the performance/coverage of PDCCH to be impacted by the reduction on #Rx antennae. Table 1 shows the performance loss for PDCCH with different aggregation levels by comparing three UE #Rx antennae configurations. The assumed target PDCCH reliability is 99%. The simulation settings and BLER curves for AL8 and AL1 are included in the Appendix. Figure 1 shows the BLER curves for PDCCH with AL6 and different number of Rx antennae for 4GHz band.

It can be noticed from the results that the PDCCH performance loss is AL dependent, where lower ALs suffer larger performance loss compared to higher ALs such as AL16. Although the system spectral efficiency is impacted by the performance loss from all ALs, the coverage loss is dominated by AL16 (the maximum AL in NR). Considering AL16 and target PDCCH reliability of 99%, there is about 2.72dB performance loss by reducing #Rx from 4 to 2, and about 6dB by reducing #Rx from 4 to 1.

Table 1: Performance loss from reducing the number of antennae.

| PDCCH BLER=1% | AL1 |  | AL8 |  | AL16 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] | Req. SNR [dB] | Loss [dB] |
| 4Rx | 2.92 | - | – 9.33 | - | – 11.67 | - |
| 2Rx | 9.48 | 6.56 | – 6.17 | 3.16 | – 8.95 | 2.72 |
| 1Rx | 17.56 | 14.64 | – 2.38 | 6.94 | – 5.71 | 5.96 |

Figure 1: PDCCH BLER for AL16 with 4, 2 and 1 Rx antennae.

- PDCCH performance is degraded by reducing the #Rx. The PDCCH performance loss is AL dependent, where lower ALs suffer larger performance loss compared to higher ALs.

- AL16 PDCCH link budget decreases by 2.72 dB for 99% target reliability in 4GHz band when the number of UE receive antennae is reduced from 4 to 2.

- AL16 PDCCH link budget decreases by about 6 dB for 99% target reliability in 4GHz band when the number of UE receive antennae is reduced from 4 to 1.

- Study techniques for improving the performance of NR PDCCH to compensate the coverage loss due to the reduction of #Rx antennae.

# PDSCH coverage enhancement

As highlighted in the previous section, there is performance loss due to the reduction of #Rx antennae. In this section, we consider the possible techniques to enhance the PDCCH performance to recover the coverage loss.

- Increasing aggregation level:

NR ALs are designed to be a power of 2 to increase the user packing given the control resources. The current maximum AL is 16 and the next possible aggregation level to be introduced is AL32. Even though increasing the AL reduces the effective code rate and delivers improved performance, it will lead to higher blockage rate. Also, due to increased frequency resources usage it would be difficult to accommodate AL32 in smaller BW that are considered for RedCap UEs [2], even with maximum CORESET size of 3 OFDM symbols. Hence, increased AL is not the best solution to recover the coverage loss for RedCap UEs.

- Increasing AL to 32 increases cannot be supported for smaller BWs that are considered for RedCap UEs.

- Increasing CORESET size:

Another approach to enhance the PDCCH reliability is to allow the CORESET size to be more than 3 symbols (e.g. up to 6 symbols). However, this could make multiplexing full-capable NR UEs and RedCap UEs in the same cell problematic or less efficient.

- Increasing CORESET size to more than 3 symbols could be one of the approaches to recover the coverage loss due to reducing #Rx antennae.

- PDCCH repetitions:

PDCCH repetitions is another technique to lower the code rate thereby improving PDCCH reliability. Even though the polar coding performance is improved over simple repetition coding, the minimum code rate used for control channel is 1/8 and to achieve code rates lower than 1/8, the buffer is cycled which makes the polar coding performance to be equivalent of repetition coding for ALs greater than AL4 for a nominal DCI payload size of ~40 bits. The advantage of PDCCH repetitions over higher AL is the ability to achieve intermediate ALs that are not the powers of 2 (for example achieving AL24 by repeating AL8 three times).

3.1) UE-transparent PDCCH repetitions:

Different options of UE-transparent PDCCH repetitions are shown in Figure 2. Option (a) provides independent control for each DL assignment of the PDSCH repetitions, and this can be adopted with minimal change in the current specification. Option (b) is very similar to the option (a) except that in option (b) the PDCCH is scheduling all the future repetitions of PDSCH. Both options (a) and (b) could suffer from performance degradation, compared to increasing the AL, if there is time-domain correlation between the channels of the repetitions, and due to the lack of soft combining of the PDCCH repetitions.

Figure 2: NR-PDCCH possible UE-transparent repetitions schemes.

3.2) UE-aware PDCCH repetitions:

The repetition scheme in Figure 3 has multiple control transmissions before the transmission of the DL data. The repetitions in this scheme can be transparent to the UE, which is quite similar to options (a) and (b) in the UE-transparent repetitions section above, and this can be implemented with the current specification with minimal changes, however it incurs performance degradation as discussed in the previous section. The other approach for this scheme is that the control repetitions are UE-aware, in which case soft combining can be exploited at the UE. However, this will require significant specification changes, and increases the UE complexity, which something not desirable for RedCap UEs.

Figure 3: NR-PDCCH possible UE-aware repetitions scheme.

In conclusion, PDCCH enhancement is required for RedCap UEs to compensate for the coverage loss due to reducing #Rx antennae. But, ALs higher than 16 (that is AL32) will be difficult to accommodate for small BWs that considered for RedCap UEs. Increasing CORESET size can be also considered as it is more suitable for the limited BWs. UE-transparent PDCCH repetitions could be another alternative that has minimum UE impact and requires minimum specifications change.

- UE-aware PDCCH repetition schemes are not considered for RedCap NR UEs.

- UE-transparent PDCCH repetitions can be considered for RedCap NR UEs.

# Conclusions

In this contribution, we discussed and evaluated the impact of UE complexity reduction features on the coverage of NR channels, and presented the possible mechanisms to compensate for the coverage loss. We have the following observations and proposals:

- No changes are required to the minimum number (1) of mandatory Tx antenna in the standard.

- The coverage of NR UL channels is not impacted by the features for complexity reduction for RedCap NR UEs.

- Existing NR schemes, such as PDSCH repetition and HARQ retransmission, offers high flexibility for the network to recover any coverage loss for PDSCH due to reduced #Rx antennae.

- PDCCH performance is degraded by reducing the #Rx. The PDCCH performance loss is AL dependent, where lower ALs suffer larger performance loss compared to higher ALs.

- AL16 PDCCH link budget decreases by 2.72 dB for 99% target reliability in 4GHz band when the number of UE receive antennae is reduced from 4 to 2.

- AL16 PDCCH link budget decreases by about 6 dB for 99% target reliability in 4GHz band when the number of UE receive antennae is reduced from 4 to 1.

- Increasing AL to 32 increases cannot be supported for smaller BWs that are considered for RedCap UEs.

- Increasing CORESET size to more than 3 symbols could be one of the approaches to recover the coverage loss due to reducing #Rx antennae

- Coverage compensation mechanisms for NR UL channels are not considered in RedCap NR SI.

- Coverage compensation mechanisms for PDSCH are considered of low priority for RedCap NR SI.

- Study techniques for improving the performance of NR PDCCH to compensate the coverage loss due to the reduction of #Rx antennae.

- UE-aware PDCCH repetition schemes are not considered for RedCap NR UEs.

- UE-transparent PDCCH repetitions can be considered for RedCap NR UEs.

# References

- RP-193238, “New SID on support of reduced capability NR devices”, Ericsson, RAN#86, December, 2019.

- R1-2003687, “On complexity reduction features for NR RedCap UEs”, MediaTek, RAN1#101-e, May 2019.

# Appendix

Link-level simulation settings for PDCCH performance evaluation are listed in Table 2.

Table 2: Link-level simulation settings for PDCCH performance evaluation.

| Parameter | Value |
| --- | --- |
| Carrier frequency for evaluation | 4 GHz |
| Channel model | TDL-C (delay spread: 300ns) as in 38.901 |
| UE speed | 3 km/h |
| BS antenna configuration | 4 Tx |
| UE antenna configuration | 4, 2 and 1 Rx antenna ports |
| System bandwidth | 40 MHz |
| Sub-carrier spacing | 30 kHz |
| Channel estimation | Practical |

Figure 4: PDCCH BLER for AL8 and AL1 with 4, 2 and 1 Rx antennae.