3GPP TSG RAN WG1 #101-e	R1- 2003718

Agenda item:		7.2.8.4

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Maintenance on PHY procedures for NR Positioning

Document for: 	Discussion and Decision

# Introduction

In RAN1#99, RAN1 has completed Rel-16 NR positioning WI, and started the maintenance discussions in RAN1#100-e. In this contribution we discuss the remaining open issue for PHY procedures for NR positioning.

# Discussion

At RAN1#99 the following agreement was reached:

Agreement:

- When the UE reports DL-PRS RSRP measurements on DL-PRS resources from one DL-PRS Resource Set, the UE may indicate in the measurement report for each TRP which DL-PRS RSRP measurements, if any, have been measured using the same Rx beam.

- Note: As previously agreed, to support Option 3 of multi-beam operation, the NW may configure DL-PRS Resources as source RS for QCL Type D for a target DL-PRS Resource. That is, Option 3 can be achieved by Option 1 with a DL-PRS as source RS for QCL Type D (Options 3/1 from previous related agreement in RAN1#97).

This agreement was discussed at length during RAN1#100b-e. The following option was proposed with a corresponding TP to TS 38.214 during the email discussion for this AI at RAN1#100b-e:

Option 3: Clarify that a "which measurement" indication is not needed; an indication that the same beam has been used is sufficient:

[…] The UE may be configured to measure and report up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate that the DL PRS RSRP measurements have been performed using the same spatial domain filter for reception. […]

This issue is arising as RAN2 in their running CR to TS 37.355 has added a new parameter nr-PRS-RxBeamIndex. This index is currently capature as shown here:

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16						TRP-ID-r16										OPTIONAL,

nr-DL-PRS-ResourceId-r16		NR-DL-PRS-ResourceId-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetId-r16		NR-DL-PRS-ResourceSetId-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16			INTEGER (FFS)									OPTIONAL,

-- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8),

nr-TimingMeasQuality-r16		NR-TimingMeasQuality-r16,

nr-DL-Aod-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16,

...

}

We would like to highlight that the original RAN1 agreement does not mention anything about an index. We would also like to highlight that the agreement in RAN1 is clear that they UE “may” report this RX beam information.The RAN1 spec is also clear that the UE may indicate this information.

Further, there is no value to the LMF knowing that a UE makes a single DL PRS-RSRP measurement using a given beam index. The only value is for the LMF to know if a group of DL PRS-RSRP measurements are performed using the same beam or not.

Proposal 1: Adpot the TP in Section 3 to clarify the reporting of information on the same Rx beam.

Proposal 2: Send an LS to RAN2 informing them of the above change and addition of higher layer parameter [TheSameRxBeam].

# Text Proposals

## TP to 38.214, Section 5.1.6.5

[…]

The UE may be configured to measure and report up to 8 DL PRS RSRP measurements on different DL PRS resources from the same cell. When the UE reports DL PRS RSRP measurements from one DL PRS resource set, the UE may indicate whichthat DL PRS RSRP measurements associated with the higher layer parameter [TheSameRxBeam] have been performed using the same spatial domain filter for reception.

[…]

# Conclusion

In this contribution we make the following proposals:

Proposal 1: Adpot the TP in Section 3 to clarify the reporting of information on the same Rx beam.

Proposal 2: Send an LS to RAN2 informing them of the above change and addition of higher layer parameter [TheSameRxBeam].

# References

[1] R1-1913687 CR : Introduction of NR positioning support, Huawei, RAN1 #99, Reno, USA, November, 2019