3GPP TSG RAN WG1 #101	R1-2003723

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	Corrections to additional SRS

Agenda item:	6.2.3.1.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss correction to additional SRS feature that has been introduced in Rel-16 as part of DL MIMO efficiency enhancement WID [1].

# Discussion on indexing of additional SRS symbols

According to TS 36.211 [2] the indexing of SRS symbols follows the index of OFDM symbol l not accounting the guard symbol. This formulation, however, may be misleading as the range of the parameter l is not defined. Moreover, in other SRS sections, parameter l is used as OFDM symbol index within the subframe. The illustration of the possible interpretations are shown in Figure 1.

(a)	                                                                                       (b)

Figure 1 Illustration of the possible OFDM symbol indexing

It should be noted that interpretation shown in Figure 1a is not correct, since it leads to “partial” SRS repetition when frequency hopping and/or antenna selection is used. More specifically, the parameter nSRS=lR which is used to derive SRS sounding band (in case of frequency hopping) and Tx antenna (in case of antenna switching) may change during SRS repetitions, if the index of the OFDM symbol l in the subframe is used as the input parameter. It is, therefore, preferable to modify the description of the parameter l  to be more consistent with Figure 1b

- change OFDM symbol number to the SRS transmission number within the subframe,

- change the notation of OFDM symbol index from l to l'

- clarify the range of l' to be starting from 0

The proposed modifications of the text are provided below, where the changes are shown in red.

Mapping to physical resources shall be done according to clause 5.5.3.2.1 with the following exceptions: - frequency hopping between OFDM symbols is supported and if a UE is configured by higher layer parameter additionalSRS-GuardSymbolFH, a guard symbol is added between every frequency hop; - nSRS=l'R where l' is the index of the OFDM symbol number carrying additional SRS transmission number within the subframe, where l'=0 corresponds to the first OFDM symbol of SRS transmission not counting guard symbol(s) , and R∈1,2,3,4,6,7,8,9,12,13 is the repetition factor given by the higher-layer parameter additionalSRS-RepNum;

# Conclusion

In this contribution we provide correction to the description of the additional SRS. It is proposed to adopt the proposed changes to Rel-16 specifications [2].

# References

[1] RP-182901, WID Revision: DL MIMO efficiency enhancements for LTE, Sorrento, Italia, December 10 – 13, 2018, Huawei, HiSilicon.

[2] TS 36.211, Physical channels and modulation, v.16.1.0.