3GPP TSG RAN WG1 Meeting #101_e                                                               R1-2003726

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.1.2

Source: 	Intel Corporation

Title:	DL Signals and Channels for NR-unlicensed

Document for: 	Discussion/Decision

# Introduction

There are some remaining issues on NR-U downlink signals and channels to be clarified in the specs based on the previous agreements.

# Intra-cell guard bands for downlink channels

It is allowed for gNB to transmit PDSCH on parts of single active BWP where CCA is successful at gNB. For the case that gNB transmits PDSCH on whole BWP only when CCA is successful for whole LBT BWs inside the BWP, same argument with the UL wideband operation can be also used so it is assumed that PRBs in intra-cell guard bands are always used if overlapped with the resource allocation.

However, for the case that gNB transmits PDSCH on parts of single active BWP where CCA is successful at gNB, additional consideration has to be taken into account for the intra-cell guard bands. If CCA is not successful for some of the LBT BW, the guard band between successful LBT BW and unsuccessful LBT BW should not be used for PDSCH transmission since the unsuccessful LBT BW will make non-negligible interference to the guard band. Since the LBT outcome is not predictable to either gNB or UE beforehand, it is desirable that UE assumes nothing is transmitted on the intra-cell guard bands.

For supporting the above two different alternative DL wideband operations, we need to have a mechanism to protect the intra-cell guard bands at least for the 2nd case. The simplest mechanism to protect the guard bands is to reuse the RateMatchPattern by using DCI format 1-1. This scheme was originally designed for avoiding the transmission of PDSCH inside the resource reserved for other purposes. Therefore, gNB can utilize this function to avoid the transmission of PDSCH by enabling rate matching around the resources reserved for intra-cell guard bands, which does not have any additional spec impacts. However, RateMatchPattern is not defined for DCI format 1-0. For the fall-back operation, we have to define a way to protect the guard bands by avoiding the mapping of PDSCH into the PRBs inside the intra-cell guard bands.

Proposal 1

- For DCI 1-1, use RateMatchPattern to avoid the PDSCH mapping to PRBs in intra-cell guard bands

- For DCI 1-0, enhance the spec 38.214 to void the PDSCH mapping to PRBs in intra-cell guard bands as given in the text proposal for 38.214

Text proposal for 38.214 ### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. A UE shall assume that the PRBs inside the intra-cell guard bands defined in Clause 7, if configured, are not available for PDSCH scheduled by PDCCH with DCI format 1_0.

# Conclusion

It is proposed to agree on the following proposal and the corresponding TP in section 2.

Proposal 1:

- For DCI 1-1, use RateMatchPattern to avoid the PDSCH mapping to PRBs in intra-cell guard bands

- For DCI 1-0, enhance the spec 38.214 to void the PDSCH mapping to PRBs in intra-cell guard bands as given in the text proposal for 38.214

# References

[1]	3GPP TS 38.211 V16.0.0

[2]	3GPP TS 38.213 V16.0.0