3GPP TSG RAN WG1 Meeting #101-e                                                  	        R1- 2003731

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	  7.2.2.2.4

Source:              Intel Corporation

Title:                   Enhancements to configured grants for NR-unlicensed

Document for:    Discussion/Decision

1 Introduction

In 3GPP TSG RAN Meeting #82, a new WID on NR-based access to Unlicensed Spectrum (RP-182878) [1] was approved. As part of the objectives of the work item, the following physical layer aspects related to the design of configured grant operation were suggested to be specified as per the outcome of the study item (TR38.889) [2]:

Physical layer procedure(s) including [RAN1, RAN2]: - Configured Grant operation: NR Type-1 and Type-2 configured grant mechanisms are the baseline for NR-U operation with modifications in line with agreements during the study phase (NR-U TR section 7.2.1.3.4). (RAN1)

In the context of enhancements for support of the configured grant operation for NR in the unlicensed spectrum, and the agreements made along this WI [3-9], a few issues will be discussed related to the following topics:

- UE’s Handling of Repetitions in Case of LBT Failure

- UE-ID

# 2 		Grant-free UL Transmission via ‘configured grant’ Mechanism

## 2.1 UE’s Handling of Repetitions in Case of LBT Failure

In previous RAN1 #99 meeting [9], the following agreement was made in regard to the case when a CG UE is configured with repK>1:

Agreement: When the UE is configured with repK > 1, repetition of a TB is mapped within a configuration in the case when an UE is configured with multiple active configurations - The UE repeats the TB in the earliest consecutive transmission occasion candidates within the same configuration instead of consecutive slots - The UE may drop repetition transmissions that fall into a subsequent configured period. - The UE terminates the repetitions if an explicit feedback indicating ACK in the DFI is received for the HARQ process.

However, RAN1 has not yet agreed on how the UE should handle the configured repetitions in the case when given a CG configuration with multiple consecutive transmission occasion candidates, one or more of them cannot be used due to LBT failure.

As an example, let’s consider the case when 8 consecutive PUSCHs are configured by a CG configuration and repK=4. By using Rel.15 mapping, the repetitions should be mapped as illustrated in Fig. 1. However, if the LBT fails for the first two PUSCH occasions, it is unclear on whether the UE will flash-out from its buffer the PUSCH repetition configured to be transmitted on the PUSCH occasion for which the LBT has failed (option 1), or it will postpone that transmission to the first available PUSCH occasion, and eventually transmit the last two PUSCH repetitions in later resources within the same CG configuration (option 2).

Fig. 1 – Illustration on two possible options on UE’s handling of repetitions in case of LBT failure.

While the merit of option 1 is to have a exemplified implementation, where there is no need for the UE to keep memory of PUSCH repetitions which were not possible to be transmitted within the same contiguous CG resources over which the burst was initiated, by postponing the transmission of the first repetition to the first available PUSCH occasion, it is given to the UE the flexibility to perform them at a later occasion or even on the same contiguous CG resources if those were overprovisioned and some are available within the same COT.

Proposal 1: A UE configured with repK>1 postpones the transmission of the first repetition to the first available PUSCH occasion for which LBT has succeeded. It is up to the UE on whether to transmit the last PUSCHs that do not fit within the consecutive CG resources over which the transmission burst was initiated in later resources within the same CG configuration.

Given the proposal above, the following text should be captured in TS 38.214 Sec 6.1.2.3.1:

---------------------------------------------- TP1: TS 38.214 section 6.1.2.3.1--------------------------------------------- *** Unchanged text is omitted *** ##### 6.1.2.3.1 Transport Block repetition for uplink transmissions with a configured grant *** Unchanged text is omitted *** For both Type 1 and Type 2 PUSCH transmissions with a configured grant, when K > 1, the UE shall repeat the TB across the K consecutive slots applying the same symbol allocation in each slot, except if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, in which case the UE repeats the TB in the repK earliest consecutive transmission occasion candidates within the same configuration. For operation with shared spectrum channel access, if the UE is provided with higher layer parameters cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16 and repK>1, the UE shall postpone the transmission burst to the earliest transmission occasion for which the related channel procedure described in 37.213 is successful. A Type 1 or Type 2 PUSCH transmission with a configured grant in a slot is omitted according to the conditions in Clause 11.1 of [6, TS38.213]. *** Unchanged text is omitted ***

## 2.3 UE-ID

During previous RAN1 meeting, it was agreed that the CG-UCI contains information regarding the HARQ process ID chosen by the CG UE, the RV used, the NDI, and the COT information for the gNB to utilize the unused resources in the CG UE’s COT:

Agreement: CG-UCI should at least include the following information: - HARQ ID - NDI - RV - COT sharing information, FFS details - FFS: other information including UE ID

If Rel.15 behaviour is used, and the UE-ID information is encoded within the DMRS, rather than being explicitly provided in the CG-UCI, then the reliability of decoding this information is greatly harmed once the number of UEs which are supported is larger than the maximum number of orthogonal DMRS sequences that can be supported. Given that this is very limitative, RAN1 should consider also more likely deployment of more than 8/12 users. In this case, it should be given the choice to the network based on the specific deployment to configure a CG-UE on whether to provide the UE-ID information based on Rel.15 behaviour, or include the UE-ID explicitly within the CG-UCI.

Proposal 2: The UE-ID is explicitly indicated within the CG-UCI upon RRC configuration.

Given the proposal above, the following text should be captured in TS 38.212 Sec 6.3.2.1.3:

---------------------------------------------- TP1: TS 38.212 section 6.3.2.1.3--------------------------------------------- *** Unchanged text is omitted *** ##### 6.3.2.1.3 CG-UCI For CG-UCI bits transmitted on a CG PUSCH, the CG-UCI bit sequence a0, a1, a2, a3, …,aA-1 is determined as follows: - set ai=oiCG-UCI for i=0,1, …, OCG-UCI-1 and A=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part. Table 6.3.2.1.3-1: Mapping order of CG-UCI fields Field; Bitwidth. HARQ process number; 4. Redundancy version; 2. New data indicator; 1. Channel Occupancy Time (COT) sharing information; log2C if both higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 and higher layer parameter cg-COT-SharingList-r16 are configured, where C is the number of combinations configured in cg-COT-SharingList-r16; 1 if higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is not configured and higher layer parameter cg-COT-SharingOffset-r16 is configured; 0 otherwise;. UE-ID; 16, if higher layer parameter cg-UEID-presence-r16 is enabled 0, otherwise.. *** Unchanged text is omitted ***

# 3 Conclusions

In this contribution, we discussed several remaining issues to support configured grant mechanism operating on unlicensed spectrum, and we derived the following proposals:

Proposal 1: A UE configured with repK>1 postpones the transmission of the first repetition to the first available PUSCH occasion for which LBT has succeeded. It is up to the UE on whether to transmit the last PUSCHs that do not fit within the consecutive CG resources over which the transmission burst was initiated in later resources within the same CG configuration.

Proposal 2: The UE-ID is explicitly indicated within the CG-UCI upon RRC configuration.

# References

[1] RP-182878, “New WID on NR-based Access to Unlicensed Spectrum”, Qualcomm Inc, 3GPP TSG RAN Meeting #82, Dec. 2018.

[2] 3GPP TR 38.889, “Study on NR-based Access to Unlicensed Spectrum (Rel-16),” V 16.0.0, Nov. 2018.

[3] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #AH 1901, January 2019.

[4] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96, February 2019.

[5] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #96b, April 2019.

[6] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #97, May 2019.

[7] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #98, August 2019.

[8] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #98bis, October 2019.

[9] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting #99, November 2019.