3GPP TSG RAN WG1 Meeting #101-E						R1-2003739

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	Corrections on PUSCH enhancements for URLLC

Agenda item:	7.2.5.3

Document for:	Discussion and Decision

# Introduction

In this contribution, we analyze remaining issues of enhanced PUSCH repetitions and provide solutions to them.

# Issues

## A-CSI and SP-CSI on PUSCH repetition type B

In the last meeting the issue was discussed in detail however no agreement was reached. The following options were identified:

For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded. Down-select among the following: - Option A: there is no scheduling restriction. - Option B: UE is not expected to be indicated with numberofrepetitions >1. - Option C: The first nominal repetition is not expected to be segmented into multiple actual repetitions. Also, we need to choose between Proposal 2, Proposal 7 and Proposal 7a: - Alt. 1. For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition. - Alt. 2. For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the last actual repetition. - Alt. 3. For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition that fulfills the CSI multiplexing timeline.

For the case of no UL-SCH on PUSCH, it seems the least restrictive approach can be agreeable, since it covers operation offered by other options. This approach in our view is Option A, i.e. no scheduling restriction.

Proposal 1

- For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded

For the case of multiplexing with UL-SCH, the intention is to follow R.15 approach in perspective of transmitting A-CSI only on one repetition. However, there are different views which repetition should be picked: first, last, or some other. While the first repetition is assumed to be a default assumption, it can incur scheduling or CSI report delays due to substantially different timeline of PUSCH preparation and CSI report preparation/multiplexing. In that sense, the last repetition can avoid undesirable delays to the start of the scheduled PUSCH, although it may incur some additional delay to the CSI report itself.

From that perspective, the Alt.2 with only last repetition seems downgrading potential CSI report delay comparing to R.15 and should not be picked as is. Alt. 3 here may look reasonable, but it needs further work to handle the cases when the timeline is not met even in the last repetition. One alternative is to pre-configure between different behaviors using RRC, like {first, last}, however this option may not be preferred due to late changes to RRC parameters. Thus, our preference is Alt.1 which looks most compatible with Rel.15 and still works in all cases.

Proposal 2

- For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition

## UCI piggybacking and PUSCH repetitions Type B

As for potential modifications to calculation of the UCI piggybacking resources, the following options were considered in last meeting:

When UCI is multiplexed on PUSCH with repetition Type B, the number of REs for UCI is calculated using the Rel-15 formula based on the nominal repetition length, with the additional limit of no more than the resources available in the actual repetition. - Option 1: The calculation is based on the nominal repetition, i.e., using the same number of REs as in TBS determination - Option 1a: with the additional limit of no more than the resources available in the actual repetition - Option 1b: UE does not expect that the number of REs required by UCI is more than the number of available REs in the actual repetition on which the UCI is multiplexed. - Option 2: The calculation is based on the actual repetition. - Option 3: The first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition.

Since the idea of the formula is to introduce variable spectrum efficiency of UCI depending on MCS of PUSCH by the first part with beta and bound it by a maximum number of REs per repetitions by the second part with alpha, in Option 3 it is reasonable to have the beta part calculated assuming regular/nominal repetition before the segmentation in order to achieve the target SE, while the alpha part to take into account the real available number of REs in this actual repetition, since the UCI is mapped only to one actual repetition. Alternatively, Option 1a also works smoothly with the explicit introduction of a third term.

Proposal 3

- When UCI is multiplexed on PUSCH with repetition Type B, the number of REs for UCI is calculated using the Rel-15 formula based on the nominal repetition length, with the additional limit of no more than the resources available in the actual repetition

  - Option 1a: The calculation is based on the nominal repetition, i.e., using the same number of REs as in TBS determination with the additional limit of no more than the resources available in the actual repetition

  - Option 3: The first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition.

# Conclusions

In this contribution, we discussed remaining issues of enhanced PUSCH repetitions:

Proposal 1

- For A-CSI/SP-CSI on PUSCH with repetition Type B without UL-SCH, A-CSI/SP-CSI is transmitted on the first actual repetition, and other actual repetitions are discarded

Proposal 2

- For A-CSI on PUSCH with repetition Type B with UL-SCH, A-CSI is transmitted on the first actual repetition

Proposal 3

- When UCI is multiplexed on PUSCH with repetition Type B, the number of REs for UCI is calculated using the Rel-15 formula based on the nominal repetition length, with the additional limit of no more than the resources available in the actual repetition

  - Option 1a: The calculation is based on the nominal repetition, i.e., using the same number of REs as in TBS determination with the additional limit of no more than the resources available in the actual repetition

  - Option 3: The first part of the equation is based on the nominal repetition, and the second part of the equation is based on the actual repetition.