3GPP TSG RAN WG1 Meeting #101-E						R1-2003758

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.11.8

Source: 	Intel Corporation

Title: 	On UE Features for NR Positioning

Document for:	Discussion and Decision

- Introduction

In this contribution, we propose revisions to the latest baseline version of the Rel-16 UE feature list for NR positioning. The outcome of the e-mail discussion captured in [1] is used as a baseline version for preparation of this contribution and provided in Annex A for reader convenience.

- Proposed Corrections

  - New Feature Groups

At the previous RAN1 WG meeting (RAN1#100bis-E), the following agreements were made:

Agreement: Introduce a new UE capability for the number of SRS resources for positioning on a symbol for intra-band CA - FFS: Capability for simultaneous SRS transmission across bands for inter-band CA - Continue discussion on capability for intra-band/inter-band CA, including potential TP to 38.214 to reflect the new capability. Agreement: A new UE capability is introduced for the number of simultaneous transmissions of SRS resources for positioning for inter-band CA, where the SRS resources are on different CCs.

The latest baseline UE feature list for NR positioning does not reflect this agreement and therefore we propose the following changes:

  - Introduce the following additional feature groups:

    - Simultaneous transmission of SRS for positioning for intra-band CA

      - Component: Number of SRS for positioning resources for simultaneous transmission on a symbol for intra-band CA case (Values: {1, 2, …, [X]})

    - Simultaneous transmission of SRS for positioning for inter-band CA

      - Component: Number of SRS for positioning resources for simultaneous transmission on a symbol for inter-band CA case (Values: {1,2, …, [X]})

  - Confirmation of Feature Groups

The following FGs (please refer to Table 1) are still under discussion and are subject to confirmation by RAN1 WG. We argue in favor of supporting FGs listed in Table 1 below.

Table 1: Feature groups under RAN1 confirmation

| FGs | FG name | Comments |
| --- | --- | --- |
| [13-7] | [Support of SSB from neighbor cell as QCL source of a DL PRS] | QCL for SSB from neighbour cell for DL PRS is ane functionality that require support of beam-management at UE side for DL PRS reception |
| [13-7a] | [Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS] | Same comment as for SSB above |
| [13-10f] | [Spatial relation maintenance] | FG is needed to indicate max number of spatial relations supported by UE for SRS for positioning |
| [13-11] | [UE Rx-Tx Measurement Report for Multi-RTT] | FG is needed to indicate: - Support of RSRP measurement jointly UE Rx-Tx time difference measurments - Max number of UE Rx-Tx time difference measurments for a single DL PRS resource set |
| [13-12] | [NR E-CID DL SSB RRM measurements with LPP support for NR Positioning] | UE need to support LPP signalling for E-CID based on SSB. LMF should be aware about E-CID support based on SSB. |
| [13-12a] | [NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning] | UE need to support LPP signalling for E-CID based on CSI-RS. LMF should be aware about E-CID support for CSI-RS for RRM. |

  - Confirm support of the NR positioning feature groups listed in Table 1,

    - 13-7 Support of SSB from neighbor cell as QCL source of a DL PRS

    - 13-7a Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS

    - 13-10f Spatial relation maintenance

    - 13-11 UE Rx-Tx Measurement Report for Multi-RTT

    - 13-12 NR E-CID DL SSB RRM measurements with LPP support for NR Positioning

    - 13-12a NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning

  - Type of Signalling and FG Pre-requisites

In this section, we provide our views for type of signaling for each FG as well as FG pre-requisites. Our proposal on these aspects are provided in Table 2.

Table 2: Pre-requisites and type of signaling for UE feature list for NR positionig

| FG | FG Name | Pre-requisite | Type of signaling |
| --- | --- | --- | --- |
| 13-1 | Common DL PRS Processing Capability | NA | Per band |
| 13-2 | DL PRS Resources for DL AoD | 13-1 | Per UE |
| 13-3 | DL PRS Resources for DL-TDOA | 13-1 | Per UE |
| 13-4 | DL PRS Resources for Multi-RTT | 13-1 | Per UE |
| 13-5 | DL PRS Measurement Report for DL-AoD | 13-2 | Per UE |
| 13-5a | Inter-frequency measurement for DL-AoD | 13-2 | Per UE |
| 13-6 | DL PRS RSTD/[RSRP] Measurement Report for DL-TDOA | 13-3 | Per UE |
| 13-6a | Inter-frequency measurement for DL-TDOA | 13-3 | Per UE |
| 13-7 | Support of SSB from neighbor cell as QCL source of a DL PRS | 13-1 | Per band |
| 13-7a | Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS | 13-1 | Per band |
| 13-8 | SRS Resources for Positioning | NA | Per FS |
| 13-8a | Support of Aperiodic SRS Resources for positioning | 13-8 | Per FS |
| 13-8b | Support of Semi-persistent SRS Resources for positioning | 13-8 | Per FS |
| 13-9 | OLPC for SRS for positioning based on PRS from the serving cell | 13-1, 13-8 | Per band |
| 13-9a | OLPC for SRS for positioning based on SSB from neighbouring cells | 13-8 and 13-9d | Per band |
| 13-9b | OLPC for SRS for positioning based on PRS from the neighbouring cells | 13-9 | Per band |
| 13-9c | OLPC for SRS for positioning based on CSI-RS from serving cell | 13-8 | Per band |
| 13-9d | OLPC for SRS for positioning based on SSB from serving cell | 13-8 | Per band |
| 13-9e | PathLoss estimate maintenance | One of {13-9, 13-9a,b,c,d} | Per band |
| 13-10 | Spatial relation for SRS for positioning based on SSB from the serving cell | 13-8 | Per band |
| 13-10a | Spatial relation for SRS for positioning based on CSI-RS from the serving cell | 13-10 | Per band |
| 13-10b | Spatial relation for SRS for positioning based on PRS from the serving cell | One of {13-2, 13-3, 13-4} and 13-8 | Per band |
| 13-10c | Spatial relation for SRS for positioning based on SRS | 13-8 | Per band |
| 13-10d | Spatial relation for SRS for positioning based on SSB from the neighbouring cell | 13-10 | Per band |
| 13-10e | Spatial relation for SRS for positioning based on PRS from the neighbouring cell | 13-10b | Per band |
| 13-10f | Spatial relation maintenance | One of {13-10, 13-10a, b, d, e} | Per band |
| 13-11a | Inter-frequency measurement for Multi-RTT | 13-4 and 13-8 | Per UE |
| 13-11 | UE Rx-Tx Measurement Report for Multi-RTT | 13-4 and 13-8 | Per UE |
| 13-12 | NR E-CID DL SSB RRM measurements with LPP support for NR Positioning | FG 1-1 | Per UE |
| 13-12a | NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning | FG 1-4 | Per UE |
| 13-13 | Simultaneous DL-AoD and DL-TDoA processing | 13-2 and 13-3 | Per band |
| 13-14 | Simultaneous DL-AoD and Multi-RTT processing | 13-2, 13-4, 13-8 | Per band |
| 13-15 | Simultaneous transmission of SRS for positioning for intra-band CA | 13-8 | Per band |
| 13-16 | Simultaneous transmission of SRS for positioning for inter-band CA | 13-8 | [Per band combination] |

  - Adopt pre-requisites and type of signalling for NR positiong FGs as provided in Table 2

  - Comments on Feature Group Components

In this section, we discuss components and their values for FG where some of the issues still need to be addressed.

### 13-1 Commom DL PRS processing capabilities

Value of X

Regarding the value X - maximum ratio of measurement gap length (MGL) to measurement gap repetition period (MGRP) we propose to support a set of values {10%, 20%, 30%} so that UE can pick the one and report together with N and T settings for maximum supported BW. The proposed set of values covers MG patterns supported by RAN4 WG.

  - Define set of X values {10%, 20%, 30%}, so that UE reports one of them together with other DL PRS processing capabilities

Case w/o MG Configured

Regarding, DL PRS processing capabilities for the case of no MG configured we are not sure that there is a need to define those. The potential issues we see is that UE can be scheduled with data transmissions in DL and UL and in case if some processing resources are shared it may be difficult to predict and analyze processing time since it may dependns on intensity of traffic, allocations sizes as well as beam management/switching procedures, etc. In addition, the actual size of the active BWP is anyway not known to LMF. In general, it is possible to make some assumptions on processing BW, reuse of hardware resources for data and DL PRS processing and even fix a value of traffic intensity, assume that data allocation size is equal to the BW of active BWP. However even if all these details are to be discussed it may still be difficult to derive reasonable numbers. It should be also point out that decision on data scheduling is under gNB control and thus LMF will have no idea on what is happening over the air. Therefore, our preference is to drop definition of such capability unless all raised above issues are addressed.

  - Do not introduce DL PRS processing capabilities for the case when no MG configured

Component Values

We suggest confirming values for all components under FG 13-1.

  - Confirm values for all components under FG 13-1

### 13-2,3,4 DL PRS resources for DL-AoD, DL-TDoA, Multi-RTT

Regarding DL PRS resources we prefer to keep component on max number of positioning frequency layers UE supports within FG and further discuss whether it is common across DL-AoD, DL-TDoA and multi-RTT.

We do not see much motivation to split candidate values of components on the set of values for FR1 and FR2 and assume that UE can report proper values from the set for each frequency range with per UE signaling and thus we think current values can be confirmed for all components.

  - For FGs 13-2/3/4 (DL PRS resources for DL-AoD, DL-TDoA and multi-RTT)

    - Keep component 6 (max number of positioning frequency layers UE supports) within each FG and further discuss whether it is common across DL-AoD, DL-TDoA and multi-RTT or not

    - Do not split candidate values among components

    - Confirm set of values for each component

### 13-6 DL PRS RSTD/[RSRP] measurement report for DL-TDOA

Support split into two components: RSRP support and RSTD measurement per DL PRS Resource Set components and corresponding sets of values.

  - For FG 13-6 (DL PRS RSTD/[RSRP] measurement report for DL-TDOA), support FG split into two components:

    - RSRP support

    - Number of RSTD measurement per DL PRS Resource Set

### 13-8/8a/8b SRS resources for positioning

It seems component #5 in FG 13-8 is redundant (a duplication of component #4) and therefore it can be removed. In addition, we propose to confirm all values for all components in FGs 13-8/8a/8b.

  - For FG 13-8, remove component#5 which is a duplication of component #4:

    - RSRP support

    - RSTD measurement per DL PRS Resource Set

### 13-9d OLPC for SRS for positioning based on SSB from serving cell

In our view this FG can be either a basic FG for UEs supporting SRS for positionng i.e. supporting FG13-8. We are also open to have FG 13-9d as a pre-requisite to other FGs covering OLPC or merge it as a component for the FG 13-8.

  - For FG 13-9d, the RAN1 to select one of the following options

    - Option 1. Define 13-9d as a basic FG for UEs supporting SRS for positioning (i.e. 13-8)

    - Option 2. Make it a pre-requisite for all FGs 13-9x

    - Option 3. Merge it as a component of the FG 13-8

### 13-9e PathLoss estimate maintenance

Regarding the FG 13-9e, we propose to change it name to “Pathloss monitoring for SRS for positioning” and are open to keep both components considering UL CA scenario and potential pathloss sharing between UL CCs of serving cell as well as to avoid configuration of all pathlosses per serving cell.

  - For FG 13-9e,

    - change name to “Pathloss monitoring for SRS for positioning”

    - keep both components

### 13-10f Spatial relation maintenance

For spatial relation maintenance, we think component #1 only is sufficient and we assume that number of maintaned spatial relations is defined across total number of SSB and DL PRS.

  - For FG 13-10f,

    - Keep only component #1

    - Clarify that max number of spatial relations is defined in total i.e. across SSBs and DL PRSs

### 13-11 UE Rx-Tx Measurement Report for Multi-RTT

Support split into two components: RSRP support and UE Rx-Tx time difference measurement per DL PRS Resource Set and corresponding sets of values.

  - For FG 13-11 (UE Rx-Tx Measurement Report for Multi-RTT) support FG split into two components:

    - RSRP support

    - UE Rx-Tx measurement per DL PRS Resource Set

- Conclusions

In this contribution, we have proposed changes to the latest version of the UE feature list for NR positioning. In summary, we have following proposals:

  - Introduce the following additional feature groups:

    - Simultaneous transmission of SRS for positioning for intra-band CA

      - Component: Number of SRS for positioning resources for simultaneous transmission on a symbol for intra-band CA case (Values: {1, 2, …, [X]})

    - Simultaneous transmission of SRS for positioning for inter-band CA

      - Component: Number of SRS for positioning resources for simultaneous transmission on a symbol for inter-band CA case (Values: {1,2, …, [X]})

  - Confirm support of the NR positioning feature groups listed in Table 1,

    - 13-7 Support of SSB from neighbor cell as QCL source of a DL PRS

    - 13-7a Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS

    - 13-10f Spatial relation maintenance

    - 13-11 UE Rx-Tx Measurement Report for Multi-RTT

    - 13-12 NR E-CID DL SSB RRM measurements with LPP support for NR Positioning

    - 13-12a NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning

  - Adopt pre-requisites and type of signalling for NR positiong FGs as provided in Table 2

  - Define set of X values {10%, 20%, 30%}, so that UE reports one of them together with other DL PRS processing capabilities

  - Do not introduce DL PRS processing capabilities for the case when no MG configured

  - Confirm values for all components under FG 13-1

  - For FGs 13-2/3/4 (DL PRS resources for DL-AoD, DL-TDoA and multi-RTT)

    - Keep component 6 (max number of positioning frequency layers UE supports) within each FG and further discuss whether it is common across DL-AoD, DL-TDoA and multi-RTT or not

    - Do not split candidate values among components

    - Confirm set of values for each component

  - For FG 13-6 (DL PRS RSTD/[RSRP] measurement report for DL-TDOA), support FG split into two components:

    - RSRP support

    - Number of RSTD measurement per DL PRS Resource Set

  - For FG 13-8, remove component#5 which is a duplication of component #4:

    - RSRP support

    - RSTD measurement per DL PRS Resource Set

  - For FG 13-9d, the RAN1 to select one of the following options

    - Option 1. Define 13-9d as a basic FG for UEs supporting SRS for positioning (i.e. 13-8)

    - Option 2. Make it a pre-requisite for all FGs 13-9x

    - Option 3. Merge it as a component of the FG 13-8

  - For FG 13-9e,

    - change name to “Pathloss monitoring for SRS for positioning”

    - keep both components

  - For FG 13-10f,

    - Keep only component #1

    - Clarify that max number of spatial relations is defined in total i.e. across SSBs and DL PRSs

  - For FG 13-11 (UE Rx-Tx Measurement Report for Multi-RTT) support FG split into two components:

    - RSRP support

    - UE Rx-Tx measurement per DL PRS Resource Set

- References

- R1-2003201, “Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR positioning”, Moderator (NTT DOCOMO, INC.)

# Annex A – UE Feature List for NR Positioning (Latest Baseline Version – before RAN1#101-E)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13. NR Positioning | 13-1 | Common DL PRS Processing Capability | - Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} - DL PRS buffering capability: Type 1 or Type 2 - Type 1 – sub-slot/symbol level buffering - Type 2 – slot level buffering - Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. - T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - N: {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50} ms Notes: - UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE - UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value - UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) - UE DL PRS processing capability is agnostic to DL PRS comb factor configuration - The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS - Max number of DL PRS resources that UE can process in a slot under it - FR1 bands: {1, 2, 4, [6], 8, 12, 16, [24], 32, [48], 64} for each SCS: 15kHz, 30kHz, 60kHz - FR2 bands: {1, 2, 4, [6], 8, 12, 16, [24], 32, [48], 64} for each SCS: 60kHz, 120kHz Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than X% (FFS: X). FFS case w/o measurement gap configured |  | Yes | N/A |  | Per band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-2 | DL PRS Resources for DL AoD | - Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} - Max number of DL PRS Resources per DL PRS Resource Set Values = {[1], 2, 4, 8, 16, 32, 64} - Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets. Values = {64, 128, 192, 256, 512, 1024, 2048} - Max number of TRPs across all positioning frequency layers per UE. Values = {[3], 6, 12, [16], 24, 32, 64, 128, 256} - Max number of DL PRS Resources per positioning frequency layer. Values = {32, 64, 128, 256, 512, 1024} - [Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | 13-1 | No | N/A |  | [Per UE] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. FFS: split of candidate values for FR1/FR2/mixed FR1-FR2 for each component | Optional with capability signaling |
| 13. NR Positioning | 13-3 | DL PRS Resources for DL-TDOA | - Max number of DL PRS Resource Sets per TRP per frequency layer. Values = {1, 2} - Max number of DL PRS Resources per DL PRS Resource Set. Values = {1, 2, 4, 8, 16, 32, 64} - Max number of DL PRS Resources across all frequency layers, TRPs and DL PRS Resource Sets. Values = {64, 128, 192, 256, 512, 1024, 2048} - Max number of TRPs across all positioning frequency layers per UE. Values = {[3,] 6, 12, [16], 24, 32, 64, 128, 256} - Max number of DL PRS Resources per positioning frequency layer. Values = {32, 64, 128, 256, 512, 1024} - [Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | 13-1 | No | N/A |  | [Per UE] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. FFS: split of candidate values for FR1/FR2/mixed FR1-FR2 for each component | Optional with capability signaling |
| 13. NR Positioning | 13-4 | DL PRS Resources for Multi-RTT | - Max number of DL PRS Resource Sets per TRP per frequency layer. Values = {1, 2} - Max number of DL PRS Resources per DL PRS Resource Set. Values = {1, 2, 4, 8, 16, 32, 64} - Max number of DL PRS Resources across all frequency layers, TRPs and DL PRS Resource Sets. Values = {64, 128, 192, 256, 512, 1024, 2048} - Max number of TRPs across all positioning frequency layers per UE. Values = {[3], [6], [12], [16], 24, 32, 64, 128, 256} - Max number of DL PRS Resources per positioning frequency layer. Values = {32, 64, 128, 256, 512, 1024} - [Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | 13-1 | Yes | N/A |  | [Per UE] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. FFS: split of candidate values for FR1/FR2/mixed FR1-FR2 for each component | Optional with capability signaling |
| 13. NR Positioning | 13-5 | DL PRS Measurement Report for DL-AoD | - Max number of DL PRS RSRP measurements on different PRS resources from the same TRP supported by the UE Values = {1, 2, 3, 4, 5, 6, 7, 8} | 13-2, | No | N/A |  | [Per UE] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-5a | Inter-frequency measurement for DL-AoD | - Support of inter-frequency measurement for DL-AoD | 13-2 | No | N/A |  | [Per band] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signalling {supported, notSupported} |
| 13. NR Positioning | 13-6 | DL PRS RSTD/[RSRP] Measurement Report for DL-TDOA | - [DL RSTD measurements per pair of TRPs. Values = {1, 2, 3, 4}] - [Support RSRP measurements. Values = {0, 1}] | 13-3 | No | N/A |  | [Per UE] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-6a | Inter-frequency measurement for DL-TDOA | - Support of inter-frequency measurement for DL-TDOA | 13-3 | No | N/A |  | [Per band] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signalling {supported, notSupported} |
| 13. NR Positioning | [13-7] | [Support of SSB from neighbor cell as QCL source of a DL PRS] | - [Support of SSB from neighbor cell as QCL source of a DL PRS] - [Support of reuse SSB measurement from RRM for receiving PRS] Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | 13-1 | No | N/A |  | [Per band] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-7a] | [Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS] | - [Support of DL PRS from serving/neighbor cell as QCL source of a DL PRS] Note: Refers to Type-D support for FR2 | 13-1 | No | N/A |  | [Per band] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-8 | SRS Resources for Positioning | - Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. - Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} - [Max number of P/SP/AP SRS Resources including the SRS resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14}] - [Max number of periodic SRS Resources for positioning supported by UE across all SRS Resource Sets per BWP. Values = {1, 2, 4, 8, 16, 32, 64}] - [Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64}] - [Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] |  | Yes | N/A |  | [Per FS] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-8a | Support of Aperiodic SRS Resources for positioning | - Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} - [Max number of aperiodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] | 13-8 | Yes | N/A |  | [Per FS] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-8b | Support of Semi-persistent SRS Resources for positioning | - Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} - [Max number of semi-persistent SRS Resources for positioning supported by UE per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}] | 13-8 | Yes | N/A |  | [Per FS] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9 | OLPC for SRS for positioning based on PRS from the serving cell | - OLPC for SRS for positioning based on PRS from the serving cell | [13-1], [One of {13-2, 13-3, 13-4}], and 13-8 | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9a | OLPC for SRS for positioning based on SSB from neighbouring cells | - OLPC for SRS for positioning based on SSB from neighbouring cells | 13-8 and [13-9d] | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9b | OLPC for SRS for positioning based on PRS from the neighbouring cells | - OLPC for SRS for positioning based on PRS from the neighbouring cells | 13-8 and 13-9 | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9c | OLPC for SRS for positioning based on CSI-RS from serving cell | - OLPC for SRS for positioning based on CSI-RS from serving cell | 13-8 | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9d | OLPC for SRS for positioning based on SSB from serving cell | - [OLPC for SRS for positioning based on SSB from serving cell] | 13-8 | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-9e | [PathLoss estimate maintenance] | - [Max number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {1,4,8,16}] - [Max number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {1,4,8,16}] | One of {13-9, 13-9a,b,c,[d]} | No | N/A |  | [Per band] | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10 | Spatial relation for SRS for positioning based on SSB from the serving cell | - Spatial relation for SRS for positioning based on SSB from the serving cell | 13-8 | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10a | Spatial relation for SRS for positioning based on CSI-RS from the serving cell | - Spatial relation for SRS for positioning based on CSI-RS from the serving cell | 13-10 | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10b | Spatial relation for SRS for positioning based on PRS from the serving cell | - Spatial relation for SRS for positioning based on PRS from the serving cell | One of {13-2, 13-3, 13-4} and13-8 | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10c | Spatial relation for SRS for positioning based on SRS | - Spatial relation for SRS for positioning based on SRS | 13-8, | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10d | Spatial relation for SRS for positioning based on SSB from the neighbouring cell | - Spatial relation for SRS for positioning based on SSB from the neighbouring cell | 13-10 | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-10e | Spatial relation for SRS for positioning based on PRS from the neighbouring cell | - Spatial relation for SRS for positioning based on PRS from the neighbouring cell | 13-10b | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-10f] | [Spatial relation maintenance] | - [Component 1: Max Number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,2,4,8,16}] - [Component 2: Max Number of maintained spatial relations for all the SRS resource sets for positioning per serving cell in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. Values = {0,1,2,4,8,16}] | One of {13-10, 13-10a, b, d, e} | No | N/A |  | [Per band] | N/A | N/A (FR2 only) | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-11a | Inter-frequency measurement for Multi-RTT | - Inter-frequency measurement for Multi-RTT | 13-4 and 13-8 | Yes | N/A |  | [Per UE] | N/A | [Yes] | N/A | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-11] | [UE Rx-Tx Measurement Report for Multi-RTT] | - Max number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set. [Note: The DL PRS resource/resource sets can be in different positioning frequency layers] - [Support RSRP measurements. Values = {0, 1}] | 13-4 and 13-8 | No | N/A |  | [Per UE] | [N/A] | [Yes] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-12] | [NR E-CID DL SSB RRM measurements with LPP support for NR Positioning] | - [NR E-CID DL SSB RRM measurements with LPP support for NR Positioning] |  | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | [13-12a] | [N R E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning] | - [NR E-CID DL CSI-RS RRM measurements with LPP support for NR Positioning] | 13-12 | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-13 | Simultaneous DL-AoD and DL-TDoA processing | - Support of simultaneous processing for DL AoD and DL TDoA measurements If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and DL TDoA measurements | 13-2 and 13-3 | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 13. NR Positioning | 13-14 | Simultaneous DL-AoD and Multi-RTT processing | - Support of simultaneous processing for DL AoD and Multi-RTT measurements If it is not indicated, a UE is not expected to perform simultaneously the processing for deriving DL AoD and M-RTT measurements | 13-2, 13-4 and 13-8 | No | N/A |  | [Per band] | [N/A] | [N/A] | [N/A] | Need for location server to know if the feature is supported. | Optional with capability signaling |