3GPP TSG RAN WG1 #101	R1-2003761

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	UE features for NR TEI

Agenda item:	7.2.11.12

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss corrections to support of aperiodic CSI-RS transmission with slot offsets of 224 and 336 has been introduced in Rel-16 as part of TEI work [1].

# SRS Tx switching

In Rel-15 UE capability for SRS Tx switching supports signaling of the xTyR as well as indication of the DL and UL bands which are impacted by the corresponding switching. In Rel-16 support of the additional xTyR configurations has been agreed as part of TEI work. The agreed xTyR signaling is a sequence of the all possible xTyR’s including the highest xTyR which should be already indicated using Rel-15 capability signaling for the backward compatibility purpose.

Considering that Rel-16 indication includes highest xTyR already indicated in Rel-15 (highlighted in yellow), reporting of the affected DL and UL bands can be reused from Rel-15 capability. On the other hand, the downgraded xTyR configuration (highlighted in green) in most of the cases (except for t1r1-t1r2-) doesn’t involve any physical switching of the antennas and can be performed without any interruptions to DL and UL bands.

- {t1r1, t1r2}

- {t1r1, t1r2, t1r4}

- {t1r1, t1r2, t2r2, t2r4}

- {t1r1, t2r2}

- {t1r1, t2r2, t4r4}

- {t1r1, t1r2, t2r2, t1r4, t2r4}

It is therefore questionable whether additional component should be  provided. Considering the discussion above the following proposal can be made:

Proposal:

- Affected DL and UL bands for downgraded xTyR configurations in Rel-16 are not applicable except for for t1r2 in {t1r1, t1r2} entry

  - Affected DL and UL bands for t1r2 are inherited from Rel-15 capability

| 14-4 | SRS Tx switch with allowing downgrading configuration | 1) Support SRS Tx port switch NOTE: NOTE: Rel-15 capability for the affected DL and UL bands are applicable for Rel-16 xTyR capability , except {t1r1, t1r2}. | 2-55 | BC | Optional with capability signalling Component 1: Candidate value set: { o{t1r1 } o{t1r1, t1r2 } o{t1r1, t1r2, t2r2 } o{t1r1} o{t1r1, t1r2, t2r2} } NOTE: Rel-15 capability for the affected DL and UL bands are not applicable for downgraded Rel-16 xTyR capability, except {t1r1, t1r2}. |
| --- | --- | --- | --- | --- | --- |

| srs-TxSwitch Defines whether UE supports SRS for DL CSI acquisition as defined in clause 6.2.1.2 of TS 38.214 [12]. The capability signalling comprises of the following parameters: - supportedSRS-TxPortSwitch indicates SRS Tx port switching pattern supported by the UE, which is mandatory with capability signaling. The indicated UE antenna switching capability of ′xTyR′ corresponds to a UE, capable of SRS transmission on ′x′ antenna ports over total of ′y′ antennas, where ′y′ corresponds to all or subset of UE receive antennas, where 2T4R is two pairs of antennas. supportedSRS-TxPortSwitch-r16, which is optional to report, indicates downgrading configuration of SRS Tx port switching pattern. If the UE indicates the support of downgrading configuration of SRS Tx port switching pattern using supportedSRS-TxPortSwitch-r16, the UE shall report the values for this as below, based on what is reported in supportedSRS-TxPortSwitch. supportedSRS-TxPortSwitch; supportedSRS-TxPortSwitch-r16. t1r2; t1r1. t1r4; t1r1-t1r2. t2r4; t1r1-t1r2-t2r2. t2r2; t1r1. t4r4; t1r1-t2r2. t1r4-t2r4; t1r1-t1r2-t2r2. - txSwitchImpactToRx indicates the entry number of the first-listed band with UL in the band combination that affects this DL, which is mandatory with capability signaling; - txSwitchWithAnotherBand indicates the entry number of the first-listed band with UL in the band combination that switches together with this UL, which is mandatory with capability signaling. For txSwitchImpactToRx and txSwitchWithAnotherBand, value 1 means first entry, value 2 means second entry and so on. All DL and UL that switch together indicate the same entry number. txSwitchImpactToRx and txSwitchWithAnotherBand are not applicable for downgrading SRS Tx port switching configuration indicated by supportedSRS-TxPortSwitch-r16 capability, except for t1r2 in {t1r1, t1r2} entry. The UE is restricted not to include fallback band combinations for the purpose of indicating different SRS antenna switching capabilities. | BC | FD | No | No |
| --- | --- | --- | --- | --- |

# Aperiodic CSI-RS triggering with beam switching timing of 224 and 336

In RAN1#98 it was agreed to define UE behaviour in Rel-16 for the UE indicating beam switching time of 224 and 336 for aperiodic CSI-RS.

Agreements: - 48 is used as the beam switching threshold for UEs reporting 224 or 336 - When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI.

Based on the agreement TP was agreed to TS 38.214 [2] and LS was sent to RAN2 [3]. The text in CR and LS to RAN2, however, assumes the Rel-15 UE capability will be used to indicate the values of 224 and 336.

It should be noted that in the UE is not aware which functionality is supported by the gNB. Due to such uncertainty UE is unlikely to report 224 or 336 values using Rel-15 capability to ensure backward compatibility for the “old” gNB potentially not supporting UE behaviour for beam switching timing of 224 and 336. Then, the agreed enhancement for aperiodic CSI-RS based on Rel-15 capability indication becomes useless.

Observation:

- Rel-15 capability signalling is not suitable for indication of the beam switching timing of 224 and 336.

In order to solve the problem, it is necessarily to introduce Rel-16 capability for (e.g., beamSwitchTiming-r16) indicating new values of {224, 336} while keep supporting Rel-15 capability for the backward compatibility purpose without any changes. New UE behaviour in TS 38.214 defining threshold of 48 symbols for aperiodic CSI-RS can be enabled depending whether UE includes Rel-16 capability or not.

It should be also noted that Rel-16 enhancement with beam switching timing of {224, 336} is supported based on UE capability and without explicit RRC configuration from gNB. Such approach was not recommended by RAN2 in the LS [4]. As the result ambiguity may occur on the actually assumed threshold for aperiodic CSI-RS, if UE in Rel-15 indicates beamSwitchTiming value other than 48 and also include new beamSwitchTiming-r16 in Rel-16 implying threshold of 48 according to TS 38.214.

To avoid ambiguity on the actually assumed threshold for aperiodic CSI-RS without explicit RRC signalling, UE including Rel-16 capability of {224, 336} should be required to include the value of 48 using Rel-15 beamSwitchTiming.

Summarizing discussion above the following proposals can be made:

Proposal:

- Introduce Rel-16 capability indicating beam switching timing of 224 and 336 while keep Rel-15 capability and UE behaviour unchanged.

- Mandate UE reporting both Rel-15 and Rel-16 capability of beam switching timing for the same band to report beam switching timing of 48 for Rel-15 to avoid ambiguity on the assumed QCL threshold.

- Send LS to RAN2 to update previous RAN1 agreement on the enhancement

| [14-7] | New capability for beamSwitchTiming values of 224 and 336 | 1. 48 is used as the beam switching threshold for UEs reporting 224 or 336 2. When using sym224 and sym336, beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. 1. Indicates the minimum number of required OFDM symbols {224, 336} between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’. | [2-28] | FFS: [per UE or per band] Per band | FFS: whether this FG is necessary or not FFS: relationship with beamSwitchTiming for FG2-28 Component 1: candidate values {224, 336} NOTE: UE indicating beamSwitchTiming-16 and beamSwitchTiming for the same band shall set beamSwitchTiming to 48 Agreements: 48 is used as the beam switching threshold for UEs reporting 224 or 336 When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI. |
| --- | --- | --- | --- | --- | --- |

The TPs capturing the above proposals to TS 38.306 are provided below:

| beamSwitchTiming-16 beamSwitchTiming-16 of value (sym224 or sym336) indicates the minimum number of required OFDM symbols between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’. UE indicating beamSwitchTiming-16 and beamSwitchTiming for the same band shall set beamSwitchTiming to 48. | Band | No | No | FR2 only |
| --- | --- | --- | --- | --- |

# References

[1] Chairman notes, 3GPP TSG RAN WG1 #98, Prague, CZ, August 26th – 30th, 2019.

[2] R1-1909857, Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336, vivo, Prague, CZ, August 26th – 30th, 2019.

[3] R1-1909845, LS on NR Rel-16 TEI, 3GPP TSG RAN WG1 #98, Prague, CZ, August 26th – 30th, 2019.

[4] R1-2001513, Guidelines for UE capability definitions, RAN2, Ericsson, Intel.