3GPP TSG RAN WG1 #101	R1-2003763

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	Maintenance of aperiodic CSI-RS triggering with beam switching timing of 224 and 336

Agenda item:	7.2.12

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss corrections to support of aperiodic CSI-RS transmission with slot offsets of 224 and 336 has been introduced in Rel-16 as part of TEI work [1].

# Discussion

In RAN1#98 it was agreed to define UE behaviour in Rel-16 for the UE indicating beam switching time of 224 and 336 for aperiodic CSI-RS.

Agreements: - 48 is used as the beam switching threshold for UEs reporting 224 or 336 - When using the higher values of the feature (sym224 and sym336), beamSwitchTiming indicates the minimum number of OFDM symbols between the DCI triggering of aperiodic CSI-RS and aperiodic CSI-RS transmission in a CSI-RS resource configured with repetition ‘ON’ to apply TCI indication in CSI-RS triggering DCI.

Based on the agreement TP was agreed to TS 38.214 [2] and LS was sent to RAN2 [3]. The text in CR and LS to RAN2, however, assumes the Rel-15 UE capability will be used to indicate the values of 224 and 336.

It should be noted that in the UE is not aware which functionality is supported by the gNB. Due to such uncertainty UE is unlikely to report 224 or 336 values using Rel-15 capability to ensure backward compatibility for the “old” gNB potentially not supporting UE behaviour for beam switching timing of 224 and 336. Then, the agreed enhancement for aperiodic CSI-RS based on Rel-15 capability indication becomes useless.

Observation:

- Rel-15 capability signalling is not suitable for indication of the beam switching timing of 224 and 336.

In order to solve the problem, it is necessarily to introduce Rel-16 capability for (e.g., beamSwitchTiming-r16) indicating new values of {224, 336} while keep supporting Rel-15 capability for the backward compatibility purpose without any changes. New UE behaviour in TS 38.214 defining threshold of 48 symbols for aperiodic CSI-RS can be enabled depending whether UE includes Rel-16 capability or not.

It should be also noted that Rel-16 enhancement with beam switching timing of {224, 336} is supported based on UE capability and without explicit RRC configuration from gNB. Such approach was not recommended by RAN2 in the LS [4]. As the result ambiguity may occur on the actually assumed threshold for aperiodic CSI-RS, if UE in Rel-15 indicates beamSwitchTiming value other than 48 and also include new beamSwitchTiming-r16 in Rel-16 implying threshold of 48 according to TS 38.214.

To avoid ambiguity on the actually assumed threshold for aperiodic CSI-RS without explicit RRC signalling, UE including Rel-16 capability of {224, 336} should be required to include the value of 48 using Rel-15 beamSwitchTiming.

The TPs capturing the above proposals to TS 38.214 and TS 38.306 are provided below:

5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology … If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336}. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming-r16 is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48 when the reported value of beamSwitchTiming-r16 is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI.

| beamSwitchTiming-16 beamSwitchTiming-16 of value (sym224 or sym336) indicates the minimum number of required OFDM symbols between the DCI triggering aperiodic CSI-RS and the corresponding aperiodic CSI-RS transmission in a CSI-RS resource set configured with repetition ‘ON’. UE indicating beamSwitchTiming-16 and beamSwitchTiming for the same band shall set beamSwitchTiming to 48. | Band | No | No | FR2 only |
| --- | --- | --- | --- | --- |

# References

[1] Chairman notes, 3GPP TSG RAN WG1 #98, Prague, CZ, August 26th – 30th, 2019.

[2] R1-1909857, Aperiodic CSI-RS Triggering for UE reporting beamSwitchTiming values of 224 and 336, vivo, Prague, CZ, August 26th – 30th, 2019.

[3] R1-1909845, LS on NR Rel-16 TEI, 3GPP TSG RAN WG1 #98, Prague, CZ, August 26th – 30th, 2019.

[4] R1-2001513, Guidelines for UE capability definitions, RAN2, Ericsson, Intel.