3GPP TSG RAN WG1 Meeting #101-E							R1-2003767

e-Meeting, May 25th – June 5th, 2020

Source:	Intel Corporation

Title:	I-IoT scenarios for NR positioning evaluations

Agenda item:	8.2.1

Document for:	Discussion and Decision

Introduction

The RAN WG approved study item on NR Positioning Enhancements [1]. The study item includes the following objective:

Define additional scenarios (e.g. (I)IoT) based on TR 38.901 to evaluate the performance for the use cases (e.g. (I)IoT).

In this contribution, we express our views on additional scenarios for I-IoT NR Positioning evaluations. Our views on other topics are provided in companion contributions [6]-[7].

# IIoT Use Cases & Positioning Requirements

The SID provides the following performance targets for high precision positioning in the justification section of the SI [1]:

(a) For general commercial use cases (e.g., TS 22.261): - sub-meter level position accuracy (< 1 m) (b) For IIoT Use Cases (e.g., 22.804): - position accuracy < 0.2 m The target latency requirement is < 100 ms; for some IIoT use cases, latency in the order of 10 ms is desired.”

The above performance targets are even more challenging to achieve comparing to the performance targets agreed in Rel.16. It should be mentioned that above requirements are formulated for enhanced positioning service areas and assuming the use of RAT-independent solutions as well [TS 22.261]. In [TR 22.804], the following I-IoT use cases and corresponding performance requirements are provided:

Mobile control panels with safety functions

  - An indoor positioning service within factory danger zones with horizontal positioning accuracy better than 1 m, 99.9% availability, heading < 0.52 rad and latency for positioning estimation of UE < 1 s.

  - An indoor positioning service for the rest of the factory hall with horizontal positioning accuracy better than 5 m, 90% availability and latency for positioning estimation of UE < 5 s.

Augmented reality

  - An indoor positioning service with horizontal positioning accuracy better than 1 m, 99% availability, heading < 10 degrees and latency for positioning estimation < 15 ms for a moving UE with speed up to 10 km/h.

Process automation - plant asset management

  - An indoor positioning service of assets in production environment with horizontal positioning accuracy better than 1 m, 90% availability, latency of less than 2 s for positioning estimation for a moving UE at the speed up to 30 km/h.

Inbound logistics for manufacturing

  - An indoor positioning service for driving trajectories (if supported by further sensors like camera, GNSS, IMU) of autonomous driving systems with horizontal positioning accuracy better than 30 cm, 99.9% availability, latency of 10 ms for positioning estimation of a moving UE at the speed up to 30 km/h.

  - An indoor positioning service for storage of goods with horizontal positioning accuracy better than 20 cm, 99% availability, latency of less than 1 s for positioning estimation of a moving UE at the speed up to 30 km/h.

Flexible, modular assembly area

  - An indoor positioning service of autonomous vehicles (only for monitoring proposes) with horizontal positioning accuracy better than 50 cm, 99% availability, latency of 1 s for positioning estimation of a moving UE at a speed up to 30 km/h.

  - An indoor positioning service for tracking of tools at the work-place location with horizontal positioning accuracy better than 1 m (relative positioning), 99% availability and latency of 1 s for positioning estimation of a moving UE at a speed up to 30 km/h.

Based on analysis of the above use cases it clear that almost all of them are for indoor scenarios. In addition, the performance targets as provided in study item description document can be considered as a target for evaluation of NR positioning enhancements.

  - Performance targets provided in study item description document are confirmed as design targets for evaluation of NR positioning enhancements

# I-IoT Scenarios for Positioning Evaluation

The Indoor Factory deployment scenarios defined for URLLC studies are primary candidates for additional NR Positioning evaluations. The TR 38.901[2], defines the following five Indoor Factory (InF) scenarios:

InF-SL - Indoor Factory w/ sparse clutter and low base station height (Tx, Rx are below the average height of the clutter)

InF-DL - Indoor Factory w/ dense clutter and low base station height (Tx, Rx are below the average height of the clutter)

InF-SH - Indoor Factory w/ sparse clutter and high base station height (Tx or Rx elevated above the clutter)

InF-DH - Indoor Factory w/ dense clutter and high base station height (Tx or Rx elevated above the clutter)

InF-HH - Indoor Factory w/ high Tx and high Rx (both elevated above the clutter)

Among the above scenarios, we suggest selecting the three representative scenarios for evaluation of NR positioning performance: InF-SL, InF-SH, InF-DH.

  - Prioritize three representative I-IoT scenarios for NR Positioning evaluations

  - Use the following three I-IoT representative scenarios for NR positioning evaluations in Rel-17:

    - InF-SL

    - InF-SH

    - InF-DH

# Evaluation Assumptions for I-IOT Scenarios

Evaluation Parameters Common Across In-Factory Scenarios

In Rel-16, the system parameters common to all scenarios for NR positioning evaluation were defined in Table 6.1.1-1 in TR 38.855 [3] (see Table 1), which includes the carrier frequency, the bandwidth of reference signals, subcarrier spacing, gNB/UE noise figures, UE max. TX power, UE antenna configuration, UE radiation pattern and network synchronization, etc. These common parameters may be re-used for I-IOT scenarios with slight modifications as highlighted by red color in Table 1 below.

Table 1: Common scenario parameters applicable for all scenarios (copy of the Table 6.1.1-1 in TR 38.855)

|  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- |
| Carrier frequency, GHz | 2GHz, 4GHz – Note 1 | 30 GHz – Note 1 |
| Bandwidth, MHz | 5MHz, 50MHz for 2GHz 100MHz for 4GHz | 100MHz, 400MHz |
| Subcarrier spacing, kHz | 15kHz for 5MHz and 50MHz 30kHz for 100MHz | 120kHz |
| gNB model parameters |  |  |
| gNB noise figure, dB | 5dB | 7dB |
| UE model parameters |  |  |
| UE max. TX power, dBm | 23dBm – Note 1 | 23dBm – Note 1 EIRP should not exceed 43 dBm. |
| UE noise figure, dB | 9dB – Note 1 | 13dB – Note 1 |
| UE antenna configuration | Panel model 1 – Note 1 Mg = 1, Ng = 1, P = 2, dH = 0.5λ, (M, N, P, Mg, Ng) = (1, 2, 2, 1, 1) | Multi-panel Configuration 1 and Panel Configuration a – Note 1 - Multi-panel Configuration 1: (Mg, Ng) = (1, 2); Θmg,ng=90°; Ω0,1=Ω0,0+180°; (dg,H, dg,V)=(0,0) - Panel Configuration a: - Each antenna array has shape dH=dV=0.5λ - Config a: (M, N, P) = (2, 4, 2), - the polarization angles are 0° and 90° - The antenna elements of the same polarization of the same panel is virtualized into one TXRU - Optional: Provided by company |
| UE antenna radiation pattern | Omni, 0dBi | Antenna model according to Table 6.1.1-2 |
| PHY/link level abstraction | Explicit simulation of all links, individual parameters estimation is applied. Companies to provide description of applied algorithms for estimation of signal location parameters. |  |
| Network synchronization | The network synchronization error, per UE dropping, is defined as a truncated Gaussian distribution of (T1 ns) rms values between an eNB and a timing reference source which is assumed to have perfect timing, subject to a largest timing difference of T2 ns, where T2 = 2*T1 – That is, the range of timing errors is [-T2, T2] – T1: 0ns (perfectly synchronized), 50ns |  |
| Note 1: According to 3GPP TR 38.802 Note 2: According to 3GPP TR 38.901 |  |  |

The details of UE antenna model for evaluations in FR2 band are described in Table 2 below and follow evaluation assumptions in 3GPP TR 38.855.

Table 2: UE antenna radiation pattern (FR2) - (copy of the Table 6.1.1-2 in TR 38.855)

| Parameter | Values |
| --- | --- |
| Antenna element radiation pattern in θ'' dim (dB) |  |
| Antenna element radiation pattern in φ'' dim (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5dBi |
|  | Note: are in local coordinate system. |

The details of gNB antenna configurations for positioning evaluations in FR1 and FR2 bands are described in Table 3 below and follow evaluation assumptions in 3GPP TR 38.855 defined for Scenario 1. Indoor office scenario parameters. In addition, the system level evaluation assumptions on gNB max TX power are provided.

Table 3: gNB antenna configuration and TX power (copy of Table 6.1.1-3 in TR 38.855)

|  |  | FR1 Specific Values | FR2 Specific Values |
| --- | --- | --- | --- |
| Layout |  | Indoor floor: (12BSs per 120m x 50m), TRP number per floor:12, Inter-gNB distance = 20m - Note 1 |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |
| Channel model |  | Indoor open office – Note 2 |  |
| Peneteration loss |  | 0dB |  |
| Number of floors |  | 1 |  |
| UE drop procedure |  | 100% indoor, uniformly distributed over the horizontal area |  |
| UE mobility |  | 3km/h |  |
| UE antenna height |  | 1.5m |  |
| Min gNB-UE distance (2D), m |  | 0m |  |
| gNB antenna height |  | 3m |  |
| 3-sector | Antenna element vertical radiation pattern (dB) |  |  |  |
|  | Antenna element horizontal radiation pattern (dB) |  |  |  |
|  | Combining method for 3D antenna element pattern (dB) |  |  |  |
|  | Maximum directional gain of an antenna element GE,max | 8dBi |  |  |
|  | Electric tilting | 110 degree |  |  |
| Note 1: According to 3GPP TR 38.802 (Table A.2.1-7: Indoor BS antenna radiation pattern for above 6GHz) – reproduced above for 3-sector case |  |  |  |  |

  - Reuse common system parameters as provided in Table 1 for NR Positioning evaluations in Rel-17 with the following minor changes relative to the 3GPP TR 38.855

    - FR1: Keep only 4GHz carrier, 100MHz BW and 30kHz SCS

    - FR2: Keep only 400MHz BW and 120kHz SCS

  - Reuse UE antenna model from the 3GPP TR 38.855 as defined in Table 1 for FR1 and Table 2 for FR2

  - Reuse gNB antenna model corresponding to indoor office deployment scenario in the 3GPP TR 38.855 and as defined by Table 3 in this document

Parameters for Indoor Factory Scenarios

The indoor factory (InF) scenarios mainly focus on factory halls of varying sizes and with varying levels of density of "clutter". Details of the InF scenarios are summarized in TR 38.901 in Table 7.2-4 (Evaluation parameters for InF) and Table 7.8-7: (Simulation assumptions for large scale calibration for the indoor factory scenario). For convenience, these tables are reproduced here in Table 4 and Table 5 respectively.

Table 4: Evaluation parameters for InF (copy of Table 7.2-4 Evaluation parameters for InF)

|  |  | InF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameters |  | InF-SL (sparse clutter, low BS) | InF-DL (dense clutter, low BS) | InF-SH (sparse clutter, high BS) | InF-DH (dense clutter, high BS) | InF-HH (high Tx, high Rx) |
| Layout | Room size | Rectangular: 20-160000 m2 |  |  |  |  |
|  | Ceiling height | 5-25 m | 5-15 m | 5-25 m | 5-15 m | 5-25 m |
|  | Effective clutter height hc | < Ceiling height, 0-10 m |  |  |  |  |
|  | External wall and ceiling type | Concrete or metal walls and ceiling with metal-coated windows |  |  |  |  |
| Clutter type |  | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Big machineries composed of regular metallic surfaces. For example: several mixed production areas with open spaces and storage/commissioning areas | Small to medium metallic machinery and objects with irregular structure. For example: assembly and production lines surrounded by mixed small-sized machineries. | Any |
| Typical clutter size, dclutter |  | 10 m | 2 m | 10 m | 2 m | Any |
| Clutter density r (percentage of surface area occupied by clutter) |  | Low clutter density (<40%) | High clutter density (≥40%) | Low clutter density (<40%) | High clutter density (≥40%) | Any |
| BS antenna height hBS |  | Clutter-embedded, i.e. the BS antenna height is below the average clutter height |  | Above clutter |  | Above clutter |
| UT location | LOS/NLOS | LOS and NLOS |  |  |  | 100% LOS |
|  | Height hUT | Clutter-embedded |  |  |  | Above clutter |

Table 5: Simulation assumptions for indoor factory scenario (copy of Table 7.8-7 from the 3GPP TR 38.901)

| Parameter | Values |
| --- | --- |
| Scenario | InF-SL, InF-DL, InF-SH, InF-DH |
| Hall size | InF-SL: 120x60 m InF-DL: 300x150 m InF-SH: 300x150 m InF-DH: 120x60 m |
| Room height | 10 m |
| Sectorization | None |
| BS antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| UT antenna configurations | 1 element (vertically polarized), Isotropic antenna gain pattern |
| Handover margin (for calibration) | 0dB |
| BS deployment | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m BS height = 1.5 m for InF-SL and InF-DL BS-height = 8 m for InF-SH and InF-DH |
| UT distribution | uniform dropping for indoor with minimum 2D distance of 1 m UT height = 1.5 m |
| UT attachment | Based on pathloss |
| UT noise figure | 9 dB |
| Carrier frequency | 3.5 GHz, 28 GHz |
| Bandwidth | 100 MHz |
| Clutter density: r | Low clutter density: 20% High clutter density: 60% |
| Clutter height: hc | Low clutter density: 2 m High clutter density: 6 m |
| Clutter size: dclutter | Low clutter density: 10 m High clutter density: 2 m |

  - Reuse evaluation parameters for indoor factory evaluations as defined in Table 4 and Table 5 with proposed modifications marked in red colour

## Indoor Factory Channel Models

Channel models for Indoor Factory scenarios are described in the 3GPP TR 38.901. We propose to reuse InF channel models for NR Positioning evaluations including modeling of NLOS time offset in propagation delay for NLOS models.

  - Reuse InF channel models defined in the 3GPP TR 38.901 including modelling of NLOS offset in propagation delay for NLOS models

Conclusions

In this contribution we have provided our views on the I-IOT use cases and scenarios for NR positioning evaluations as a part of the Rel-17 SI on NR Positioning Enhancements. In summary, we have following list of proposals:

  - Performance targets provided in study item description document are confirmed as design targets for evaluation of NR positioning enhancements

  - Prioritize three representative I-IoT scenarios for NR Positioning evaluations

  - Use the following three I-IoT representative scenarios for NR positioning evaluations in Rel-17:

    - InF-SL

    - InF-SH

    - InF-DH

  - Reuse common system parameters as provided in Table 1 for NR Positioning evaluations in Rel-17 with the following minor changes relative to the 3GPP TR 38.855

    - FR1: Keep only 4GHz carrier, 100MHz BW and 30kHz SCS

    - FR2: Keep only 400MHz BW and 120kHz SCS

  - Reuse UE antenna model from the 3GPP TR 38.855 as defined in Table 1 for FR1 and Table 2 for FR2

  - Reuse gNB antenna model corresponding to indoor office deployment scenario in the 3GPP TR 38.855 and as defined by Table 3 in this document

  - Reuse evaluation parameters for indoor factory evaluations as defined in Table 4 and Table 5 with proposed modifications marked in red colour

  - Reuse InF channel models defined in the 3GPP TR 38.901 including modelling of NLOS offset in propagation delay for NLOS models

References

- RP-193237, “New SID on NR Positioning Enhancements”, Qualcomm Incorporated, Sitges, Spain, December 9th – 12th, 2019.

- 3GPP TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”,V16.0.0 (2019-10)

- 3GPP TR 38.855, “Study on NR positioning support”,  V16.0.0 (2019-03)

- 3GPP TR 22.804, “Study on Communication for Automation in Vertical Domains”, V16.2.0 (2018-12)

- R1-1913674, “Updated consolidated parameter list for Rel-16 NR”

- R1-2003768, “Initial analysis of NR positioning performance in I-IoT scenarios”, Intel Corporation, May 2020

- R1-2003769, “Potential NR positioning enhancements”, Intel Corporation, May 2020