3GPP TSG RAN WG1 #101                                                                                      R1-2003773

e-Meeting, May 25th – June 5th, 2020

Source: 	Intel Corporation

Title:	Discussion on baseline coverage performance for FR1

Agenda item:	8.4.1.1

Document for:	Discussion and Decision

# Introduction

In NR coverage enhancement SID, the following objectives have been identified [1]:

- The target scenarios and services include

  - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1

  - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2.

  - TDD and FDD for FR1.

  - VoIP and eMBB service for FR1.

  - eMBB service as first priority and VoIP as second priority for FR2.

  - LPWA services and scenarios are not included.

- Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation

  - UL channels (including PUSCH and PUCCH) are prioritized for FR1.

  - Both DL and UL channels for FR2.

- Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services

  - The target channels include at least PUSCH/PUCCH

  - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions)

  - Study the additional enhanced solutions for FR2 if any

  - Evaluate the performance of the potential solutions based on link level simulation.

In the contribution, we discuss baseline coverage performance for FR1, with primary focus on MCL analysis and link level simulation results for various DL/UL physical channels for FR1. Our view on baseline coverage performance for FR2 is described in our companion contributions [2].

# MCL analysis for FR1

Based on the link level simulation results as presented in the Appendix, Table 1 and Table 2 illustrate MCL analysis for rural and urban deployment scenario in FR1, respectively. Note that in the link level simulations, it is assumed

- 700MHz carrier frequency with 15kHz subcarrier spacing for rural scenario;

- 4GHz carrier frequency with 30kHz subcarrier spacing for urban scenario.

Further, when calculating the MCL for PDSCH and PUSCH, MCS and the number of PRBs/symbols are selected in order to meet the target throughput for rural and urban scenarios, respectively, as identified in the NR coverage enhancement SID [1]:

- Urban scenario: DL 10Mbps, UL 1Mbps

- Rural scenario: DL 1Mbps, UL 100kbps

Table 1. MCL analysis for rural deployment scenario in FR1 (700MHz carrier frequency)

Table 2. MCL analysis for urban deployment scenario in FR1 (4GHz carrier frequency)

From the tables, it can be observed that for FR1,

- For rural deployment scenario, considering target data rate of 1Mbps and 100kbps for DL and UL, respectively,

  - The gap between maximum MCL and minimum MCL can be up to 15.8dB.

  - The gap between PDSCH and PUSCH in term of MCL can be 7.5dB.

  - PUSCH and PUCCH format 3 are the coverage bottleneck.

- For urban deployment scenario, considering target data rate of 10Mbps and 1Mbps for DL and UL, respectively,

  - The gap between maximum MCL and minimum MCL can be up to 19.4dB

  - The gap between PDSCH and PUSCH in term of MCL can be 7.2dB

  - PUSCH is the coverage bottleneck.

Note that in the MCL analysis, it is assumed same PSD is uniformly distributed across system bandwidth for the transmission of DL physical channels. In case when PSD boosting is applied for the transmission of DL physical channels, additional coverage extension for DL transmission can be achieved.

Observation 1

- For rural deployment scenario, considering target data rate of 1Mbps and 100kbps for DL and UL, respectively,

  - The gap between maximum MCL and minimum MCL can be up to 15.8dB.

  - The gap between PDSCH and PUSCH in term of MCL can be 7.5dB.

  - PUSCH and PUCCH format 3 are the coverage bottleneck.

- For urban deployment scenario, considering target data rate of 10Mbps and 1Mbps for DL and UL, respectively,

  - The gap between maximum MCL and minimum MCL can be up to 19.4dB

  - The gap between PDSCH and PUSCH in term of MCL can be 7.2dB

  - PUSCH is the coverage bottleneck.

To investigate potential solutions for different physical channels, RAN1 needs to first identify overall coverage enhancement target for FR1, which may also depend on specific deployment scenario, e.g., urban and rural scenarios. Further, based on the overall coverage enhancement target, corresponding coverage enhancement target for each physical channel, especially for the physical channel with coverage bottleneck, can be derived accordingly.

As indicated in the NR coverage enhancement SID [1], urban scenario with outdoor gNB serving indoor UEs, and rural scenario including extreme long distance rural scenario, e.g. ISD = 30km are considered for coverage enhancement. Taking into account the deployment scenarios, it is expected a relatively large overall coverage enhancement target can be considered.

Table 3 illustrates one example of coverage enhancement target for various physical channels for urban and rural scenarios, respectively. In the table, it is assumed 10dB overall coverage enhancement target. From the table, it can be observed that PUSCH and PUCCH format 3 are the bottleneck for coverage performance.

Table 3. Coverage enhancement target for FR1

| Physical Channels | PDSCH | PDCCH | PBCH | PRACH | PUSCH | PUCCH(PF1) | PUCCH(PF3) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Coverage enhancement target for urban (dB) | 2.8 | 0 | 0 | 0 | 10.0 | 0 | 2.6 |
| Coverage enhancement target for rural (dB) | 2.5 | 2.6 | 0 | 4.5 | 10.0 | 0 | 7.5 |

Based on the above discussions, coverage performance for PUSCH and PUCCH format 3 in FR1 needs to be improved. Further, RAN1 should conduct study on potential techniques for coverage enhancement of these two physical channels in FR1. Note that potential solutions for coverage enhancement of uplink transmission in FR1 and FR2 are discussed in our companion contribution [4].

Proposal 1

- RAN1 to specify overall coverage enhancement target for FR1, which may be common or different for urban and rural scenarios.

Proposal 2

- RAN1 to study potential techniques for coverage enhancement of PUSCH and PUCCH in FR1.

# Conclusions

In this contribution, we discussed the baseline coverage performance for FR1, with primary focus on MCL analysis and link level simulation results for various DL/UL physical channels for FR1. Further, we summarize the observation and proposals as follows:

Observation 1

- For rural deployment scenario, considering target data rate of 1Mbps and 100kbps for DL and UL, respectively,

  - The gap between maximum MCL and minimum MCL can be up to 15.8dB.

  - The gap between PDSCH and PUSCH in term of MCL can be 7.5dB.

  - PUSCH and PUCCH format 3 are the coverage bottleneck.

- For urban deployment scenario, considering target data rate of 10Mbps and 1Mbps for DL and UL, respectively,

  - The gap between maximum MCL and minimum MCL can be up to 19.4dB

  - The gap between PDSCH and PUSCH in term of MCL can be 7.2dB

  - PUSCH is the coverage bottleneck.

Proposal 1

- RAN1 to specify overall coverage enhancement target for FR1, which may be common or different for urban and rural scenarios.

Proposal 2

- RAN1 to study potential techniques for coverage enhancement of PUSCH and PUCCH in FR1.

# References

- RP-193240, “New SID on NR coverage enhancement”, China Telecom, Sitges, Spain, December 2019

- R1-2003774, “Discussion on baseline coverage performance for FR2”, Intel Corporation, e-Meeting, May 2020

- R1-2003776, “Discussion on simulation assumption for NR coverage enhancement”, Intel Corporation, e-Meeting, May 2020

- R1-2003775, “Discussion on potential techniques for NR coverage enhancement”, Intel Corporation, e-Meeting, May 2020

# Appendix

## PDSCH baseline performance

Figure 1 illustrates PDSCH baseline performance for FR1. The simulation assumptions are outlined in our companion contribution [3]. In the simulations, it is assumed that MCS and the number of PRBs are selected in order to meet the target throughput for rural and urban scenarios, i.e., 1Mbps and 10Mbps, respectively as described in [1].

Figure 1. PDSCH baseline performance for FR1

Based on the link level simulation results, Table 4 summarizes the required SNRs for PDSCH in urban and rural scenarios for FR1.

Table 4. Required SNRs for PDSCH in FR1

|  | Urban (FR1) | Rural (FR1) |
| --- | --- | --- |
| Required SNR (dB) | 3.30 | -0.67 |

## PDCCH baseline performance

Figure 2 illustrates PDCCH baseline performance for FR1. The simulation assumptions are outlined in our companion contribution [3]. In the simulations, it is assumed CORESET size with 2 symbols in time, aggregation level of 4, cyclic precoding and interleaved mapping for PDCCH transmission.

Figure 2. PDCCH baseline performance for FR1

Based on the link level simulation results, Table 5 summarizes the required SNRs for PDCCH in urban and rural scenarios for FR1.

Table 5. Required SNRs for PDCCH in FR1

|  | Urban (FR1) | Rural (FR1) |
| --- | --- | --- |
| Required SNR (dB) | -1.55 | -0.55 |

## PBCH baseline performance

Figure 4 illustrates PBCH baseline performance for FR1. The simulation assumptions are outlined in our companion contribution [3]. 4 accumulations are assumed for the PBCH decoding.

Figure 3. PBCH baseline performance for FR1

Based on the link level simulation results, Table 6 summarizes the required SNR for PBCH in urban scenario for FR1.

Table 6. Required SNRs for PBCH in FR1

|  | Urban (FR1) |
| --- | --- |
| Required SNR (dB) | -8.9 |

## PUSCH baseline performance

Figure 4 illustrates PUSCH baseline performance for FR1. The simulation assumptions are outlined in our companion contribution [3]. In the simulations, it is assumed that MCS and the number of PRBs are selected in order to meet the target throughput for rural and urban scenarios, i.e., 100kbps and 1Mbps, respectively as described in [1]. Further, intra-slot frequency hopping is employed for PUSCH transmission.

Figure 4. PUSCH baseline performance for FR1

Based on the link level simulation results, Table 7 summarizes the required SNRs for PUSCH in urban and rural scenarios for FR1.

Table 7. Required SNRs for PUSCH in FR1

|  | Urban (FR1) | Rural (FR1) |
| --- | --- | --- |
| Required SNR (dB) | 0.79 | 2 |

## PUCCH baseline performance

Figure 5 illustrates PUCCH baseline performance for FR1. The simulation assumptions are outlined in our companion contribution [3]. In the simulations, it is assumed that 1 bit and 50 bit UCI payload are carried by PUCCH format 1 and format 3, respectively. Further, intra-slot frequency hopping is employed for PUCCH transmission.

Figure 5. PUCCH baseline performance for FR1

Based on the link level simulation results and assuming 1% miss-detection probability target for PUCCH format 1 and 1% BLER target for PUCCH format 3, Table 8 summarizes the required SNRs for PUCCH in urban and rural scenarios for FR1.

Table 8. Required SNRs for PUCCH in FR1

|  | Urban (FR1) | Rural (FR1) |
| --- | --- | --- |
| PF1: Required SNR (dB) for 1% miss-detection | -8 | -6.8 |
| PF3: Required SNR (dB) for 1% BLER | 1.2 | 2.5 |

## PRACH baseline performance

Figure 6 illustrates PRACH baseline performance for FR1. The simulation assumptions are outlined in our companion contribution [3]. In the simulations, it is assumed PRACH format 0 and 0.1% false alarm target.

Figure 6. PRACH baseline performance for FR1

Based on the link level simulation results and assuming 1% miss-detection probability target for PRACH, Table 9 summarizes the required SNRs for PRACH in urban and rural scenarios for FR1.

Table 9. Required SNRs for PRACH in FR

|  | Urban (FR1) | Rural (FR1) |
| --- | --- | --- |
| Required SNR (dB) for 1% miss-detection | -8.3 | -8.3 |