3GPP TSG RAN WG1 #101		R1-2003787

e-Meeting, May 25th – June 5th, 2020

Agenda item:	6.2.5.1

Source: 	Qualcomm Incorporated

Title: 	UE features for eMTC

Document for:	Discussion and Decision

# Background

In this document we present our comments to the current eMTC feature list (from R1-2003196). The proposed changes are provided with change marks with respect to the current version.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. LTE_eMTC5 | 1-1 | Group WUS without group resource alternation | 1. Group WUS without group resource alternation | Rel-15 MWUS | Yes | N/A | The UE will be paged with Rel-15 MWUS or without MWUS instead of Group WUS. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-2 | Group WUS with group resource alternation | 1. Group WUS with group resource alternation | 1-1 | Yes | N/A | If UE does not support group resource alternation and the eNB enables group resource alternation, UE falls back to Rel-15 MWUS when Rel-15 MWUS is configured or no MWUS when Rel-15 MWUS is not configured. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-3 | PUR for full-PRB in CEmodeA | 1. PUR for full-PRB in CEmodeA | CEmodeA | Yes | N/A | UL data transmission will use EDT or connected mode instead of PUR in CEmodeA. | Per UE | Yes | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 1-3a | Combination of PUR for full-PRB in CEmodeA with max UL TBS 2984 bits | 1. Combination of PUR for full-PRB in CEmodeA with max UL TBS 2984 bits | 1-3, and ce-PUSCH-NB-MaxTBS | Yes | N/A | PUR transmission will not use the larger UL TBS. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-4 | PUR for full-PRB in CEmodeB | 1. PUR for full-PRB in CEmodeB 2. Potential UE-specific cyclic shift for DMRS | 1-3, and CEmodeB | Yes | N/A | UL data transmission will use EDT or connected mode instead of PUR in CEmodeB. | Per UE | Yes | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 1-5 | PUR for sub-PRB in CEmodeA | 1. PUR for sub-PRB in CEmodeA | 1-3, and UL sub-PRB | Yes | N/A | PUR will not use sub-PRB allocation in CEmodeA. | Per UE | Yes | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 1-6 | PUR for sub-PRB in CEmodeB | 1. PUR for sub-PRB in CEmodeB | 1-4, and UL sub-PRB | Yes | N/A | PUR will not use sub-PRB allocation in CEmodeB. | Per UE | Yes | N/A | RAN2 has agreed that PUR with UP and CP solutions have separate indications, but this is not captured in this RAN1 UE feature list. | Optional with capability signalling |
|  | 1-7 | PUR serving cell RSRP TA validation | 1. Serving cell RSRP for TA validation for PUR | 1-3 | Yes | N/A | PUR will not use serving cell RSRP for TA validation. | Per UE | Yes | N/A | TA validation mechanisms based on ‘Serving cell changes’ and ‘TA timer for idle mode’ (and ‘TA always valid’) are mandatory for PUR UEs. | Optional with capability signalling |
|  | 1-8 | PUR frequency hopping | 1. Frequency hopping for PUR | 1-3 | Yes | N/A | PUR will not use frequency hopping. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-9 | PUR L1 ACK | 1. L1 ACK for PUR | 1-3 | Yes | N/A | PUR will not use L1 ACK. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-10 | Multi-TB unicast for DL in CEmodeA | 1. Multi-TB unicast scheduling for DL in CEmodeA | CEmodeA | Yes | N/A | Each DCI will schedule a single TB instead of multiple TBs in DL in CEmodeA. | Per UE | Yes | N/A | Following legacy capabilities are reused to support combinations. o Rel-14 feature for 2984 bits max UL TBS in 1.4 MHz in CE mode A o Rel-14 feature for new numbers of repetitions for PUSCH in CE mode A o Rel-14 feature for modulation restrictions for PDSCH/PUSCH in CE mode A o Rel-15 features for flexible starting PRB for PDSCH/PUSCH in CE mode A/B | Optional with capability signalling |
|  | 1-11 | Multi-TB unicast for DL in CEmodeB | 1. Multi-TB unicast scheduling for DL in CEmodeB | CEmodeB | Yes | N/A | Each DCI will schedule a single TB instead of multiple TBs in DL in CEmodeB. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-12 | Multi-TB unicast for UL in CEmodeA | 1. Multi-TB unicast scheduling for UL in CEmodeA | CEmodeA | Yes | N/A | Each DCI will schedule a single TB instead of multiple TBs in UL in CEmodeA. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-13 | Multi-TB unicast for UL in CEmodeB | 1. Multi-TB unicast scheduling for UL in CEmodeB | CEmodeB | Yes | N/A | Each DCI will schedule a single TB instead of multiple TBs in UL in CEmodeB. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-14 | Multi-TB unicast TB interleaving | 1. TB interleaving for multi-TB unicast scheduling | 1-10 or 1-11 or 1-12 or 1-13 | Yes | N/A | Multi-TB unicast will not use TB interleaving. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-15 | Multi-TB unicast HARQ bundling | 1. DL HARQ bundling for multi-TB unicast scheduling | 1-10 | Yes | N/A | Multi-TB unicast will not use HARQ bundling. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-16 | Multi-TB unicast UL sub-PRB | 1. UL sub-PRB allocation for multi-TB unicast scheduling | 1-12 or 1-13, and UL sub-PRB | Yes | N/A | Multi-TB unicast will not use UL sub-PRB. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-17 | Multi-TB unicast UL early termination | 1. UL early termination for multi-TB unicast scheduling | 1-12, or 1-13 | Yes | N/A | Multi-TB unicast will not use UL early termination. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-18 | Multi-TB unicast DL 64QAM for CE mode A | 1. DL 64QAM for multi-TB unicast scheduling for CE mode A | 1-10, and DL 64QAM | Yes | N/A | Multi-TB unicast will not use DL 64QAM. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-19 | Multi-TB unicast frequency hopping | 1. Frequency hopping for multi-TB unicast scheduling | 1-10 or 1-11 or 1-12 or 1-13 | Yes | N/A | Multi-TB unicast will not use frequency hopping. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-21 | Multi-TB SC-MTCH in CEmodeA | 1. Multi-TB SC-MTCH scheduling in CEmodeA 2. Potential scheduling gaps for multi-TB SC-MTCH scheduling in CEmodeA | CEmodeA, and SC-PTM | Up to RAN2 | N/A | UE will not be able to receive SC-PTM transmissions using multi-TB scheduling in CEmodeA. | Per UE | Yes | N/A | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |
|  | 1-22 | Multi-TB SC-MTCH in CEmodeB | 1. Multi-TB SC-MTCH scheduling in CEmodeB 2. Potential scheduling gaps for multi-TB SC-MTCH scheduling in CEmodeB | CEmodeB, and SC-PTM | Up to RAN2 | N/A | UE will not be able to receive SC-PTM transmissions using multi-TB scheduling in CEmodeB. | Per UE | Yes | N/A | The basic multicast (SC-PTM) functionality was introduced for LTE-M/NB-IoT in Rel-14 without capability signaling. | Up to RAN2 |
|  | 1-23 | Subframe level resource reservation for DL in CEmodeA | 1. Subframe-level time-domain resource reservation in DL in CEmodeA 2. RBG-level frequency-domain resource reservation in DL in CEmodeA | CEmodeA | Yes | N/A | Whole DL subframe(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-23a | Slot/symbol level resource reservation for DL in CEmodeA | 1. Slot/symbol-level time-domain resource reservation in DL in CEmodeA 2. RBG-level frequency-domain resource reservation in DL in CEmodeA | 1-23 | Yes | N/A | Whole DL PRB pair(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-24 | Subframe level resource reservation for DL in CEmodeB | 1. Subframe-level time-domain resource reservation in DL in CEmodeB 2. RBG-level frequency-domain resource reservation in DL in CEmodeB | CEmodeB | Yes | N/A | Whole DL subframe(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-24a | Slot/symbol level resource reservation for DL in CEmodeB | 1. Slot/symbol-level time-domain resource reservation in DL in CEmodeB 2. RBG-level frequency-domain resource reservation in DL in CEmodeB | 1-24 | Yes | N/A | Whole DL PRB pair(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-25 | Subframe level resource reservation for UL in CEmodeA | 1. Subframe-level time-domain resource reservation in UL in CEmodeA | CEmodeA | Yes | N/A | Whole UL subframe(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-25a | Slot/symbol-level resource reservation for UL in CEmodeA | 1. Slot/symbol-level time-domain resource reservation in UL in CEmodeA | 1-25 | Yes | N/A | Whole UL PRB pair(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-26 | Subframe level resource reservation for UL in CEmodeB | 1. Subframe-level time-domain resource reservation in UL in CEmodeB | CEmodeB | Yes | N/A | Whole UL subframe(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-26a | Slot/symbol-level resource reservation for UL in CEmodeB | 1. Slot/symbol-level time-domain resource reservation in UL in CEmodeB | 1-26 | Yes | N/A | Whole UL PRB pair(s) may need to be configured as invalid in order to avoid NR collision. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-27 | Subcarrier puncturing for DL in CEmodeA | 1. Subcarrier puncturing for DL in CEmodeA | CEmodeA | Yes | N/A | The UE will suffer a slight DL performance degradation if eNB punctures anyway. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-28 | Subcarrier puncturing for DL in CEmodeB | 1. Subcarrier puncturing for DL in CEmodeB | CEmodeB | Yes | N/A | The UE will suffer a slight DL performance degradation if eNB punctures anyway. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-29 | DL quality report in Msg3 in Idle | 1. Using 2 bits in Msg3 in Idle 2. Using 4 bits in Msg3 in Idle | CEmodeA | Up to RAN2 | N/A | The eNB will have to rely on other information, e.g. CSI reports if available. | Per UE | Yes | N/A | It is up to RAN2 whether to have separate capabilities for CE mode A and B. | Up to RAN2 |
|  | 1-30 | DL quality report in Connected | 1. DL quality report using 4 bits in Connected | CEmodeA | Up to RAN2 | N/A | The eNB will have to rely on other information, e.g. CSI reports if available. | Per UE | Yes | N/A | It is up to RAN2 whether to have separate capabilities for CE mode A and B. | Up to RAN2 |
|  | 1-31 | MPDCCH performance improvement with precoder cycling in CEmodeA | 1. MPDCCH performance improvement with precoder cycling in CEmodeA | CEmodeA | Yes | N/A | MPDCCH demodulation will rely on DMRS only (not CRS) in CEmodeA. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-32 | MPDCCH performance improvement with precoder cycling in CEmodeB | 1. MPDCCH performance improvement with precoder cycling in CEmodeB | CEmodeB | Yes | N/A | MPDCCH demodulation will rely on DMRS only (not CRS) in CEmodeB. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-33 | MPDCCH performance improvement with CSI-based mapping for CE mode A | 1. MPDCCH performance improvement with CSI-based mapping for CE mode A | 1-31 | Yes | N/A | MPDCCH performance improvement does not use CSI-based mapping. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-34 | MPDCCH performance improvement with reciprocity-based candidates in TDD in CEmodeA | 1. MPDCCH performance improvement with reciprocity-based candidates in TDD for CE mode A | 1-31 | Yes | N/A | MPDCCH performance improvement does not use reciprocity-based candidates in TDD. | Per UE | TDD only | N/A |  | Optional with capability signalling |
|  | 1-35 | CSI-RS-based feedback for non-BL UE | 1. CSI-RS-based feedback for non-BL UE in CEmodeA | tm9-CE-ModeA-r13 | Yes | N/A | CSI feedback will be based on CRS. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-35a | Codebook subset restriction for CSI-RS-based feedback for non-BL UE in CEmodeA | 1. Codebook subset restriction for CSI-RS-based feedback for non-BL UE in CEmodeA | 1-35 | Yes | N/A | CSI feedback will be based CSI-RS without codebook subset restriction (or on CRS). | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-36 | ETWS/CMAS indication in connected mode for non-BL UE in CEmodeA | 1. ETWS/CMAS indication in connected mode for non-BL UE in CEmodeA | CEmodeA | Yes | N/A | UE will need to be released to idle mode before it can receive the ETWS/CMAS indication. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-37 | ETWS/CMAS indication in connected mode for non-BL UE in CEmodeB | 1. ETWS/CMAS indication in connected mode for non-BL UE in CEmodeB | CEmodeB | Yes | N/A | UE will need to be released to idle mode before it can receive the ETWS/CMAS indication. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-38 | LTE control region use for MPDCCH in CEmodeA | 1. LTE control region use for MPDCCH in CEmodeA | CEmodeA | Yes | N/A | MPDCCH reception will rely only on symbols transmitted in the LTE data region. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-39 | LTE control region use for MPDCCH in CEmodeB | 1. LTE control region use for MPDCCH in CEmodeB | CEmodeB | Yes | N/A | MPDCCH reception will rely only on symbols transmitted in the LTE data region. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-40 | LTE control region use for PDSCH in CEmodeA | 1. LTE control region use for PDSCH in CEmodeA | CEmodeA | Yes | N/A | PDSCH reception will rely only on symbols transmitted in the LTE data region. | Per UE | Yes | N/A |  | Optional with capability signalling |
|  | 1-41 | LTE control region use for PDSCH in CEmodeB | 1. LTE control region use for PDSCH in CEmodeB | CEmodeB | Yes | N/A | PDSCH reception will rely only on symbols transmitted in the LTE data region. | Per UE | Yes | N/A |  | Optional with capability signalling |