3GPP TSG RAN WG1 #101	        		             R1-2003891

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.10.1

Source: 	Samsung

Title: 				    Remaining Issues on UL Power Control for NR-DC

Document for:	Discussion and Decision

# Introduction

This contribution considers the open issues for UL power control in NR-DC and primarily the cases of periodic transmissions on the MCG and the impact of UE group-common DCI formats on the MCG for dynamic power sharing.

# Remaining Issues for Dynamic Power Sharing

Dynamic power sharing relies on the MCG ensuring that, at a time T, scheduling on the MCG is delayed by Toffset so that, for any transmission scheduled at the time T on the SCG, the UE can use all available power on the SCG without having to reduce the power due to a new transmission on the MCG before time T+Toffset. It should be noted that an underlying assumption for this condition is that the UE does not require support of short latency services (e.g. some URLLC services) on the MCG.

For Type 2 CG-PUSCH transmissions on the MCG, the “scheduled by DCI formats” in the current specifications can be viewed as also including activation or release of UL grant Type 2 CG-PUSCH. The same applies for PUCCH transmissions with HARQ-ACK information in response to detections of DCI formats (HARQ-ACK information in response to SPS PDSCH reception can be predetermined without a timeline). For CG-PUSCH transmissions or for P/SP-PUCCH or for P/SP-SRS, the UE can determine the transmission power without any time constraint (TPC commands by DCI format 2_2 are not considered at this point).

Observation 1: Type 2 CG-PUSCH activation and PUCCH with HARQ-ACK information in response to detection of DCI formats should be included in “transmissions scheduled by a DCI format”.

Observation 2: A UE can account for a power of CG-PUSCH transmissions and of PUCCH transmissions without HARQ-ACK information in response to detections of DCI formats on the MCG without considering a timeline.

For GC-DCI formats, the UE-specific scheduling restrictions on the MCG need to extend to a group of UEs. DCI format 2_0 switches flexible symbols to DL or UL. The former case (F to DL) does not matter – it either has no impact on UL transmissions or cancels configured UL transmissions on the MCG. It can be up to the UE implementation to use for transmissions on the SCG any additional power from canceled transmissions on the MCG. The latter case (F to UL) enables configured UL transmissions. Further, if the gNB does not transmit the PDCCH with DCI format 2_0 or if the UE fails to detect it, the UE cancels configured transmissions that include flexible symbols. As DCI format 2_0 is a GC-DCI format (and in practice, a cell-specific DCI format), it is not reasonable to require that a UE does not expect to have configured transmissions within Toffset from the time the UE detects DCI format 2_0. Instead, the UE should account for the power of configured transmissions regardless of whether or not they include flexible symbols as described in Observation 2.

Observation 3: DCI format 2_0 does not have an impact on DPS for NR-DC.

For DCI format 2_2, the network needs to ensure that no UE from the group of UEs has a CG-PUSCH or a P/SP PUCCH transmission within Toffset from the time the PDCCH with DCI format 2_2 is transmitted if the TPC command for that UE is non-zero. This can be expressed as “a UE does not expect to detect a DCI format 2_2 providing a non-zero TPC command for a transmission that is within Toffset from the time of the PDCCH reception with the DCI format 2_2”. Another approach is to not have any such restriction and leave it up to the MCG whether there is a chance that the UE needs to reduce a transmission power on the SCG (will happen if the TPC command is positive and would result to PTotalNR-DC being exceeded. This can be acceptable as (a) a TPC command is less likely to result to PTotalNR-DC being exceeded compared to a scheduled PUSCH/PUCCH transmission, (b) such an event is under the control of the MCG (e.g. MCG can choose to transmit a zero TPC command instead of a positive one and take a small increase in the BLER when appropriate),  and (c) does not place larger restrictions on the network to manage DCI format 2_2 transmissions to a group of UEs.

Observation 4: DPS for NR-DC can operate properly without any conditions on when a UE can expect to detect a DCI format 2_2 with TPC commands.

For DCI format 2_3, the same arguments as for DCI format 2_2 apply in case DCI format 2_3 provides only TPC commands for SRS transmissions. If DCI format 2_3 also triggers an SRS transmission, the MCG can set the value of slotOffset to be larger than Toffset. In either case, although a “UE does not expect …” statement can apply for triggered SRS transmissions, this can be also left to gNB implementation.

Observation 5: DPS for NR-DC can operate properly without any conditions on when a UE can expect to detect a DCI format 2_3 with TPC commands for SRS transmissions. SRS activation can be included in “transmissions scheduled by a DCI format”.

DCI format 2_4 can only cancel transmissions and it is therefore of no concern as it may only lead to an overestimation of the power on the MCG. Further, it’s use is expected to be infrequent in practice is associated with the existence of low latency services on the MCG in which case the basic assumption for DPS may not hold.

Observation 6: DCI format 2_4 does not need to be considered for DPS in NR-DC.

PRACH transmission on the MCG, including by a PDCCH order, cannot be precluded at any time (subject to the configuration of PRACH resources) and may lead to power scaling on the SCG. Nevertheless, such transmissions are also expected to be infrequent in practice and will therefore not have a material impact on using DPS for NR-DC.

Observation 7: DPS for NR-DC can operate properly without any conditions while allowing PRACH transmission at any time on the MCG.

As it was discussed in past meetings, and as it is evident from the above, DPS is useful if Toffset is minimized. In RAN1#100bis-e, it was identified that Tproc,CSI should also be removed (in addition to Tproc,CSImux) when the UE indicates the enhanced (second) capability for Toffset. Since the value of Tproc,CSI is practically same as the value of Tproc,CSImux, its presence in Toffset when the UE indicates the second capability is considered to be an oversight; otherwise, there is practically no reason from having the second capability for Toffset.

Observation 8: If Tproc,CSI is included in Toffset for the second capability, there is practically no reason for the existence of the second capability and associated benefits for DPS do not exist.

Finally, the LS form RAN2 in [2] indicates that “MN and SN are not required to comprehend each other’s UE configuration for MR-DC”. There is no impact from the LS in [2] to the RAN1 specifications as when the MN and SN both “comprehend” the UE configurations, DPS can be either according to first or second capability; otherwise, DPS can be according to the first capability.

Observation 9: There is no impact from the LS in [2] on the existing RAN1 specifications for DPS in NR-DC.

Considering the above observations, the following update to the specifications is proposed.

Proposal: Update the specifications for DPS in NR-DC in Clause 7.6.2 of TS 38.213 v.16.1.0 as follows.

## *** Unchanged text is omitted *** If a UE - is provided dynamic for nrdc-PCmode-FR1-r16 or for nrdc-PCmode-FR2-r16, and - indicates a capability to determine a total transmission power on the SCG at a first symbol of a transmission occasion on the SCG by determining transmissions on the MCG that - are scheduled by DCI formats in PDCCH receptions with a last symbol that is earlier by more than Toffset from the first symbol of the transmission occasion on the SCG, or are configured by higher layers, and - overlap with the transmission occasion on the SCG the UE determines a maximum transmission power on the SCG at the beginning of the transmission occasion on the SCG as - minPSCG,PTotalNR-DC- PMCGactual, if the UE determines transmissions on the MCG with a PMCGactual total power - PTotalNR-DC, if the UE does not determine any transmissions on the MCG where - Toffset=max⁡{Tproc,MCGmax,Tproc,SCGmax}, - Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI, Tproc,releasemux, Tproc,2mux, and Tproc,CSImux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a first value for the capability, and - Tproc,MCGmax and Tproc,SCGmax is the maximum of Tproc,2, Tproc,CSI Tproc,releasemux, Tproc,2mux based on the configurations on the MCG and the SCG, respectively, when the UE indicates a second value for the capability. The UE does not expect to have PUSCH, PUCCH, or SRS transmissions on the MCG that - are scheduled/triggered by DCI formats in PDCCH receptions with a last symbol that is earlier by less than or equal to Toffset from the first symbol of the transmission occasion on the SCG, and - overlap with the transmission occasion on the SCG ## *** Unchanged text is omitted ***

# References:

- R1-2003183, “Corrections on NR-DC and on Cross-carrier Scheduling with Different Numerologies”

- R1-2003262, “LS reply on uplink power control for NR-NR Dual-Connectivity”