3GPP TSG RAN WG1 #101-e	R1-2003923

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.12

Source:	NEC

Title:	On ambiguous TBS due to ambiguity of Ninfo

Document for:	Discussion and Decision

Introduction

It has been identified in RAN1#99 that there would be different UE implementation of TBS derivation in the market due to ambiguity of Ninfo, which may lead to different TBS understanding between gNB and UE. RAN1 concluded this issue is postponed to Rel-16.

In this contribution, we provide our view on this issue.

Discussion

There is an ambiguity of Ninfo, whether it should be interpreted as an integer or floating point in derivation of TBS. This may result in different TBS understanding between gNB and UE for some parameter combinations where floating point value NRE∙R∙Qm∙υ results larger than 3824 and less than 3825.

In Rel-15, to address this, gNB may, for example, avoid ambiguous TBS which may be different due to different implementation of Ninfo.

If ambiguity of Ninfo could be fixed to either integer or floating point in Rel-16, gNB may distinguish Rel-15 UE and Rel-16 or later UE, use any TBS for Rel-16 or later UE but keep avoiding ambiguous TBS for Rel-15 UE. Looking at contributions in previous meetings, there are different view how to fix type of Ninfo.

“Do nothing” in Rel-16 would also be one way as the issue happens only when floating point value NRE∙R∙Qm∙υ results larger than 3824 and less than 3825. Or, to clarify this, it could be captured by the specification or chairman’s notes as a conclusion that TBS ambiguous due to Ninfo is not supported in Rel-16, without clarification of type of Ninfo, e.g.

- Any TBS derived from NRE∙R∙Qm∙υ (floating point value) larger than 3824 and less than 3825 is not supported in this release of the specification

Summary

In this contribution, we discuss about ambiguity of Ninfo in derivation of TBS. In Annex, a TP for the case to clarify the specification that ambiguous TBS is not supported in Rel-16.

Reference

[1] RAN1#99 Chairman’s notes.

Annex. TP for TS38.214

#### 5.1.3.2	Transport block size determination

<Unchanged texts are omitted>

The NDI and HARQ process ID, as signalled on PDCCH, and the TBS, as determined above, shall be reported to higher layers.

Any TBS resulting from floating point value NRE∙R∙Qm∙υ more than 3824 and less than 3825 are not supported in this version of the specification.