3GPP TSG RAN WG1 #101	                         	R1-2004014

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.2.2

Source: 	LG Electronics

Title: 	Remaining issues of initial access and mobility for NR-U

Document for:	Discussion and decision

- Introduction

In this contribution, we discuss the following remaining issues of initial access and mobility for NR-U, to be handled in RAN1#101-e.

- Value ranges of measDuration-r16 for RSSI measurement

- PDSCH rate-matching around SS/PBCH block candidate indexes

- Support of no gap between PRACH and Msg. A PUSCH for 2-step RACH procedure

- SFN validation for RAR

- Value ranges of measDuration-r16 for RSSI measurement

In RAN1#100bis-e meeting [1], the following intermediate agreement was made for RSSI measurement.

Agreement: - 1 symbol duration is included for each numerology (approx. 71 us, 35 us, 17 us respectively). - This is a natural consequence of previous agreement: The L1 averaging duration of RSSI measurements (within a configured measurement duration) is limited to 1 OFDM symbol of a configured reference subcarrier spacing. - Retain LTE LAA values {sym1, sym14, sym28, sym42, sym70} at least for 15 kHz numerology - Measurement duration can be no more than 5 ms for each numerology (requires sym70, sym140, sym280/sym240 respectively)

During the follow-up discussion through [100b-e-NR-unlic-NRU-InitAccessProc-03], two alternatives were discussed, as summarized in [2]:

Proposal: For the value ranges for measDuration-r16, - Alt 1: {sym1, sym14or28or56or48, sym28or56or112or96, sym42or84or168or144, sym70or140 or280or240} - “sym14or28or56or48” refers to 14 symbols for 15 kHz SCS, 28 symbols for 30 kHz SCS, 56 symbols for 60 kHz SCS with NCP, and 48 symbols for 60 kHz SCS with ECP, respectively, and so on - Inform RAN2 of this decision (can be within updated RRC parameter spread sheet that we send to RAN2, not necessarily a separate LS) - Alt 2: {sym1, sym14or12, sym28or24, sym42or36, sym70or60} - “sym14or12” refers to 14 symbols for NCP and 12 symbols for ECP, respectively, and so on - Inform RAN2 of this decision (can be within updated RRC parameter spread sheet that we send to RAN2, not necessarily a separate LS) - If measured bandwidth of RSSI overlaps with the active DL BWP, UE performs RSSI measurement with the SCS of the active DL bandwidth part during the measurement duration derived from combination of measDuration-r16 and rmtc-ref-SCS-CP. The UE expects an integer number of symbol(s) to perform RSSI measurement with respect to the SCS of the active DL BWP.

Considering the effectiveness of higher layer signalling and the diversity of configured measurement duration, Alt 2 is preferred.

Proposal #1: The value ranges for measDuration-r16 are {sym1, sym14or12, sym28or24, sym42or36, sym70or60}.

- “sym14or12” refers to 14 symbols for NCP and 12 symbols for ECP, respectively, and so on

- If measured bandwidth of RSSI overlaps with the active DL BWP, UE performs RSSI measurement with the SCS of the active DL bandwidth part during the measurement duration derived from combination of measDuration-r16 and ref-SCS-CP-r16. The UE expects an integer number of symbol(s) to perform RSSI measurement with respect to the SCS of the active DL BWP.

- PDSCH rate-matching around SS/PBCH block candidate indexes

Similar to Rel-15 NR, ssb-PositionsInBurst and Q for NR-U can be used to provide rate-matching pattern within a DRS transmission window. The simplest way would be for UE to perform rate-matching for all of SS/PBCH block candidate position indices QCLed with actually transmitted SS/PBCH block indices that are provided by ssb-PositionsInBurst in RMSI. For the example in Figure 1 where Q is equal to 4 and ssb-PositionsInBurst is provided with ‘10100000’, only SS/PBCH block indices 0 and 2 can be transmitted, UE will perform rate-matching around time/freq. resource of SS/PBCH block candidate position indices 0/2/4/6/8/10/12/14/16/18. Therefore, the following proposal and text proposal for TS 38.214 section 5.1.4 can be made.

Figure 1. Example of SS/PBCH block candidate positions when Q is set to 4 and ssb-PositionsInBurst is provided with ‘10100000’.

Proposal #2: UE performs rate-matching for all of SS/PBCH block candidate position indices (within configured DRS transmission window) QCLed with actually transmitted SS/PBCH block indices that are provided by ssb-PositionsInBurst in RMSI. Adopt the following text proposal in TS 38.214 section 5.1.4.

<Text proposal>

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of candidate SS/PBCH block indexes according to SS/PBCH block indexes provided by ssb-PositionsInBurst as described in [Section 4.1, TS 38.213].

- Support of no gap between PRACH and Msg. A PUSCH for 2-step RACH procedure

No gap between PRACH and Msg. A PUSCH is being proposed as the UE feature for the NR-U UE. The proposal seems to be beneficial because the UE is not required to do additional LBT procedure for Msg. A PUSCH transmission. On the other hand, according to PRACH configuration and PUSCH allocation for the licensed band, the minimum timing gap between RO and PO exists (i.e., 2 symbols for 15/30 kHz SCS, and 4 symbols for 60 kHz SCS). Given that, in order to support Msg. A PUSCH transmission without additional LBT procedure in 2-step RACH, a method to fill the minimum timing gap with certain signal can be considered. One possible way to fill the timing gap is supporting the CP extension for Msg. A PUSCH transmission, which was already introduced in PUSCH/PUCCH transmission in NR-U.

In order to support the CP extension for 2-step RACH procedure in NR-U, following consideration points can be discussed. First of all, it is necessary to determine the (maximum) size of timing gap between RO and PO to allow the CP extension. For an example, the CP extension can be allowed when the timing gap between RO and PO is not larger than 1 OFDM symbol for each SCS, which is based on the basic UE feature for CP extension in NR-U. For another example, the CP extension can be allowed if the timing gap between RO and PO is not larger than 2 OFDM symbol for 15/30 kHz SCS or 4 OFDM symbol for 60 kHz SCS, which is based on the current restriction of RO/PO configuration. Alternatively, it can be considered that the gNB provide the maximum size of timing gap between RO and PO to allow the CP extension via SIB.

Secondly, it is necessary to decide the criteria to allow the CP extension (even if the size of the above timing gap is satisfied). For an example, once the timing gap between RO and PO is less than or equal to the maximum allowable timing gap defined as in the above, the CP extension is allowed for UE in 2-step RACH procedure without any constraint. For another example, even if the timing gap between RO and PO is less than or equal to the maximum allowable timing gap, the CP extension is allowed for UE in 2-step RACH procedure only if the gNB indicates the availability of CP extension for 2-step RACH via SIB.

Proposal #3: In order to support no gap between PRACH and Msg. A PUSCH for the 2-step RACH, the CP extension can be used with consideration of following points:

- Size of timing gap between RO and PO to allow the CP extension

- Criteria to finally allow the CP extension for the allowable gap size

- SFN validation for RAR

In last RAN1 meeting, it was discussed how to reflect the SFN validation procedure for RAR reception via 2 LSB bits of SFN in DCI scheduling RAR in RAN1 specifications [3]. In our view, UE should not be mandated to always use the SFN LSB bits in the DCI for RAR reception especially for the case of contention free random access or for the case where RAR window size is configured as equal to or smaller than 10ms. This is because for contention free random access, if UE receives RAR message assigned by RA-RNTI or MSGB-RNTI, UE anyhow can identify the target UE by RAPID included in the RAR message. Also, for the case where RAR window size is configured as equal to or smaller than 10ms, SFN validation is not necessary since the RAR window can be uniquely mapped to the RO where the UE has transmitted PRACH preamble. Therefore, it should be up to UE implementation whether the UE should use the SFN LSB bits in the DCI for the DCI validation or not.

As the outcome of the discussion the text proposal for section 8.2 and 8.2A includes condition of “if included and applicable” for the usage of SFN LSB bits in the DCI. We believe that text is reasonable since that condition gives the UE implementation choice of when to apply the SFN validity check by its own discretion. However, if some companies think that description is problematic due to some ambiguity we may consider more precise text as an alternative.

In addition, the latest text proposal has a logical error in the paragraph quoted below, where the highlighted text can be interpreted as once the UE detects DCI with invalid SFN LSB bits within the RAR window, the UE should go to the next step of RAR reception failure, which is not the intention. Therefore this part should be also corrected reflecting the text regarding SFN validation part appropriately.

- Section 8.2 – <text omitted> If the UE does not detect the DCI format with CRC scrambled by the corresponding RA-RNTI, or for operation with shared spectrum channel access, if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI but the two LSB bits of the SFN in the DCI does not match the SFN of PRACH occasion used to transmit the random access preamble, or if the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

Therefore, we suggest two alternative TPs in the annex of this contribution, where the alternative 1 keeps the latest text in the FL summary in [3] but revising the text related to the RAR failure case. Alternative 2 include more precise text regarding SFN validation and also revising the text related to the RAR failure case.

Proposal #4: Regarding SFN validation for RAR, choose between TP alternative 1 and TP alternative 2 in the annex of this contribution.

- Conclusion

In this contribution, we provided our views on the remaining issues of initial access and mobility for NR-U and proposals are as follows.

Proposal #1: The value ranges for measDuration-r16 are {sym1, sym14or12, sym28or24, sym42or36, sym70or60}.

- “sym14or12” refers to 14 symbols for NCP and 12 symbols for ECP, respectively, and so on

- If measured bandwidth of RSSI overlaps with the active DL BWP, UE performs RSSI measurement with the SCS of the active DL bandwidth part during the measurement duration derived from combination of measDuration-r16 and ref-SCS-CP-r16. The UE expects an integer number of symbol(s) to perform RSSI measurement with respect to the SCS of the active DL BWP.

Proposal #2: UE performs rate-matching for all of SS/PBCH block candidate position indices (within configured DRS transmission window) QCLed with actually transmitted SS/PBCH block indices that are provided by ssb-PositionsInBurst in RMSI. Adopt the following text proposal in TS 38.214 section 5.1.4.

<Text proposal>

### 5.1.4 PDSCH resource mapping When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 0, the UE shall assume that no SS/PBCH block is transmitted in REs used by the UE for a reception of the PDSCH. When receiving the PDSCH scheduled with SI-RNTI and the system information indicator in DCI is set to 1, RA-RNTI, MsgB-RNTI, P-RNTI or TC-RNTI, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst, and if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE expects a configuration provided by ssb-PositionsInBurst in ServingCellConfigCommon to be same as a configuration provided by ssb-PositionsInBurst in SIB1. When receiving PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCHs with SPS, the REs corresponding to the configured or dynamically indicated resources in Clauses 5.1.4.1, 5.1.4.2 are not available for PDSCH. Furthermore, the UE assumes SS/PBCH block transmission according to ssb-PositionsInBurst if the PDSCH resource allocation overlaps with PRBs containing SS/PBCH block transmission resources, the UE shall assume that the PRBs containing SS/PBCH block transmission resources are not available for PDSCH in the OFDM symbols where SS/PBCH block is transmitted. A UE is not expected to handle the case where PDSCH DM-RS REs are overlapping, even partially, with any RE(s) not available for PDSCH. For operation with shared spectrum channel access, SS/PBCH block transmission according to ssb-PositionsInBurst represents all of candidate SS/PBCH block indexes according to SS/PBCH block indexes provided by ssb-PositionsInBurst as described in [Section 4.1, TS 38.213].

Proposal #3: In order to support no gap between PRACH and Msg. A PUSCH for the 2-step RACH, the CP extension can be used with consideration of following points:

- Size of timing gap between RO and PO to allow the CP extension

- Criteria to finally allow the CP extension for the allowable gap size

Proposal #4: Regarding SFN validation for RAR, choose between TP alternative 1 and TP alternative 2 in the annex of this contribution.

- Reference

RAN1#100bis-e chairman’s note

R1-2002851, “Outcome of email thread [100b-e-NR-unlic-NRU-InitAccessProc-03],” Charter Communications, RAN1#100bis-e, Apr. 2020

R1-2002996, Summary of email discussion on LSB of SFN validation, Moderator (Qualcomm)

Annex

================ TP alternative 1 for 38.213 8.2 and 8.2A================

8.2	Random access response - Type-1 random access procedure

-------Unchanged part omitted---------------------

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI, and the LSBs of SFN field in the DCI format 1_0, if included and applicable, match the LSBs of the SFN in which the PRACH is transmitted, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window but the LSBs of SFN field in the DCI format 1_0, if included and applicable, do not match the LSBs of the SFN in which the PRACH is transmitted, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming  corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For , the UE assumes  [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines  assuming SCS configuration .

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and the LSBs of SFN field in the DCI format 1_0, if included and applicable, match the LSBs of the SFN in which the PRACH is transmitted and the UE receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for a secondary cell, the UE may assume the DM-RS antenna port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set for receiving the PDCCH that includes the DCI format 1_0.

A RAR UL grant schedules a PUSCH transmission from the UE. The contents of the RAR UL grant, starting with the MSB and ending with the LSB, are given in Table 8.2-1.

If the value of the frequency hopping flag is 0, the UE transmits the PUSCH without frequency hopping; otherwise, the UE transmits the PUSCH with frequency hopping.

The UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [6, TS 38.214].

The TPC command value  is used for setting the power of the PUSCH transmission, as described in Clause 7.1.1, and is interpreted according to Table 8.2-2.

The CSI request field is reserved.

The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213].

-------Unchanged part omitted---------------------

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding RA-RNTI where the LSBs of SFN field in the DCI format 1_0, if included and applicable, match the LSBs of the SFN in which the PRACH is transmitted, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

8.2A	Random access response - Type-2 random access procedure

-------Unchanged part omitted---------------------

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI, and the LSBs of SFN field in the DCI format 1_0, if included and applicable, match the LSBs of the SFN in which the PRACH is transmitted, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆, where n is a slot of the PDSCH reception and ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214]

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window but the LSBs of SFN field in the DCI format 1_0, if included and applicable, do not match the LSBs of the SFN in which the PRACH is transmitted, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding MsgB-RNTI where the LSBs of SFN field in the DCI format 1_0, if included and applicable, match the LSBs of the SFN in which the PRACH is transmitted, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

===========================================================

================ TP alternative 2 for 38.213 8.2 and 8.2A================

8.2	Random access response - Type-1 random access procedure

-------Unchanged part omitted---------------------

If the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI, and the LSBs of SFN field in the DCI format 1_0, if included, match the LSBs of the SFN in which the PRACH is transmitted, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers parse the transport block for a random access preamble identity (RAPID) associated with the PRACH transmission. If the higher layers identify the RAPID in RAR message(s) of the transport block, the higher layers indicate an uplink grant to the physical layer. This is referred to as random access response (RAR) UL grant in the physical layer.

If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window but the LSBs of SFN field in the DCI format 1_0, if included, do not match the LSBs of the SFN in which the PRACH is transmitted, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit a PRACH. If requested by higher layers, the UE is expected to transmit a PRACH no later than  msec after the last symbol of the window, or the last symbol of the PDSCH reception, where  is a time duration of  symbols corresponding to a PDSCH processing time for UE processing capability 1 assuming  corresponds to the smallest SCS configuration among the SCS configurations for the PDCCH carrying the DCI format 1_0, the corresponding PDSCH when additional PDSCH DM-RS is configured, and the corresponding PRACH. For , the UE assumes  [6, TS 38.214]. For a PRACH transmission using 1.25 kHz or 5 kHz SCS, the UE determines  assuming SCS configuration .

If the UE detects a DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI and the LSBs of SFN field in the DCI format 1_0, if included, match the LSBs of the SFN in which the PRACH is transmitted and the UE receives a transport block in a corresponding PDSCH, the UE may assume same DM-RS antenna port quasi co-location properties, as described in [6, TS 38.214], as for a SS/PBCH block or a CSI-RS resource the UE used for PRACH association, as described in Clause 8.1, regardless of whether or not the UE is provided TCI-State for the CORESET where the UE receives the PDCCH with the DCI format 1_0.

If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for the SpCell [11, TS 38.321], the UE may assume that the PDCCH that includes the DCI format 1_0 and the PDCCH order have same DM-RS antenna port quasi co-location properties. If the UE attempts to detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI in response to a PRACH transmission initiated by a PDCCH order that triggers a contention-free random access procedure for a secondary cell, the UE may assume the DM-RS antenna port quasi co-location properties of the CORESET associated with the Type1-PDCCH CSS set for receiving the PDCCH that includes the DCI format 1_0.

A RAR UL grant schedules a PUSCH transmission from the UE. The contents of the RAR UL grant, starting with the MSB and ending with the LSB, are given in Table 8.2-1.

If the value of the frequency hopping flag is 0, the UE transmits the PUSCH without frequency hopping; otherwise, the UE transmits the PUSCH with frequency hopping.

The UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [6, TS 38.214].

The TPC command value  is used for setting the power of the PUSCH transmission, as described in Clause 7.1.1, and is interpreted according to Table 8.2-2.

The CSI request field is reserved.

The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213].

-------Unchanged part omitted---------------------

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding RA-RNTI where the LSBs of SFN field in the DCI format 1_0, if included, match the LSBs of the SFN in which the PRACH is transmitted, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

In the procedures above, UE may ignore the LSBs of SFN field when contention free random access procedure is applied or RAR window size is equal to or smaller than 10ms.

8.2A	Random access response - Type-2 random access procedure

-------Unchanged part omitted---------------------

If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI, and the LSBs of SFN field in the DCI format 1_0, if included, match the LSBs of the SFN in which the PRACH is transmitted, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer

-	an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in Clause 8.2 when the UE detects a RAR UL grant, or

-	transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where

-	a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon

-	a slot for the PUCCH transmission is indicated by a PDSCH-to-HARQ_feedback timing indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆, where n is a slot of the PDSCH reception and ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214]

-	the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214]

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR

-	the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission

If the UE detects the DCI format 1_0 with CRC scrambled by a C-RNTI and a transport block in a corresponding PDSCH within the window, the UE transmits a PUCCH with HARQ-ACK information having ACK value if the UE correctly detects the transport block or NACK value if the UE incorrectly detects the transport block and the time alignment timer is running [11, TS 38.321].

The UE does not expect to be indicated to transmit the PUCCH with the HARQ-ACK information at a time that is prior to a time when the UE applies a TA command that is provided by the transport block. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding MsgB-RNTI within the window but the LSBs of SFN field in the DCI format 1_0, if included, do not match the LSBs of the SFN in which the PRACH is transmitted, or if the UE does not correctly receive the transport block in the corresponding PDSCH within the window, or if the higher layers do not identify the RAPID associated with the PRACH transmission from the UE, the higher layers can indicate to the physical layer to transmit only PRACH according to Type-1 random access procedure or to transmit both PRACH and PUSCH according to Type-2 random access procedure [11, TS 38.321]. If requested by higher layers, the UE is expected to transmit a PRACH no later than NT,1+0.75 msec after the last symbol of the window, or the last symbol of the PDSCH reception, where NT,1 is a time duration of N1 symbols corresponding to a PDSCH processing time for UE processing capability 1 when additional PDSCH DM-RS is configured. For μ=0, the UE assumes N1,0=14 [6, TS 38.214].

Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message.

If the UE does not detect the DCI format with CRC scrambled by the corresponding MsgB-RNTI where the LSBs of SFN field in the DCI format 1_0, if included, match the LSBs of the SFN in which the PRACH is transmitted, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321].

In the procedures above, UE may ignore the LSBs of SFN field when contention free random access procedure is applied or RAR window size is equal to or smaller than 10ms

===========================================================