3GPP TSG RAN WG1 #101                                R1-2004038

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.1.1

Source: 	LG Electronics

Title: 	Consideration on required physical layer changes to support NR above

52.6 GHz

Document for:	Discussion and decision

- Introduction

A new SI to study on supporting NR with existing DL/UL waveform in high frequency band from 52.6 GHz to 71 GHz was approved in RAN#86 with following objectives [1].

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4]. - Identify potential critical problems to physical signal/channels, if any [RAN1]. - Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1]. - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we discuss and provide initial views on the required changes to support NR in high frequency range from 52.6 GHz to 71 GHz (simply denoted as “FR-X” hereafter), in terms of applicable numerology (including SCS and (maximum) carrier BW) considering the FR-X band characteristics and potential impacts to FR2 physical layer design.

- Required changes to support NR in FR-X from 52.6GHz to 71GHz

- Supported numerology for FR2 and consideration points for FR-X

Regarding the supported numerology for current FR2 NR, first of all, the SCS values of 120 kHz or 240 kHz are supported for SSB transmission, and the SCS values of 60 kHz or 120 kHz are supported for other channel/signal transmission such as CORESET (PDCCH) or PDSCH/PUSCH. Accordingly, (maximum) NR carrier BWs are determined based on the SCS values used for CORESET/PDSCH/PUSCH and the FFT size of 4096, and specifically 200 MHz and 400 MHz are defined as the maximum carrier BW with the SCS values of 60 kHz and 120 kHz, respectively.

On the support of NR in FR-X from 52.6 GHz to 71 GHz, there are some aspects to be considered further compared to current FR2 NR. Firstly, in FR-X band, the effects of phase noise (common phase error (CPE) and, especially inter-carrier interference (ICI)) and Doppler shift might be larger compared to FR2 band. These phase noise and Doppler shift would affect the BLER/SE performance and/or EVM requirement, especially for the case with high MCS, and thus may need to be taken into account for the determination of the supported SCS values and/or PTRS design. In addition, under FR-X band, the channel delay spread and target cell coverage might be different (e.g. smaller) compared to FR2 band. These aspects would be related to the target deployments/scenarios for NR in FR-X, and may need to be taken into account for the determination of the CP length and OFDM symbol duration which might affect the beam switching time. Furthermore, for supporting NR in higher/wider FR-X band, it may need to consider frequency utilization efficiency, UE implementation, and unlicensed band operation (with multi-RAT coexistence). These aspects may need to be taken into account for the determination of the supported maximum carrier BW, with consideration of FFT size, ADC complexity, and channelization in unlicensed band.

- Observations on previous NR study on high frequency band in Rel-15

According to the previous NR study proceeded during the initial phase of Rel-15, there were already some observations on certain high frequency band (e.g. 60 GHz) which belongs to the FR-X [2 – 6]. Firstly, in case with the SCS values of 60 kHz or 120 kHz for the high frequency band of 60 GHz, the performance in terms of BLER and T-put would be degraded for high MCS case, even in case with (ideal) CPE compensation based on the PTRS transmission due to the effects of ICI. On the other hand, the performance loss is not significant or negligible for low MCS even in case without CPE compensation (no PTRS transmission).

Besides, in case with the SCS value of 480 kHz for the frequency band of 60 GHz, the performance gain would be significant for high MCS case with CPE compensation, compared to the case with SCS values of 60 kHz or 120 kHz. In other words, the performance degradation caused by the ICI could be much mitigated if higher SCS value is used for the data (e.g. PDSCH/PUSCH) transmission.

Moreover, in Rel-15 NR, the PTRS structure/mapping was designed by focusing on the CPE compensation for the SCS values of up to 120 kHz and for the FR2 band of up to 52.6 GHz, with the underline assumption that the CPE is dominant while the ICI is negligible. More specifically, the Rel-15 PTRS design could provide adaptive PTRS density in time/frequency domain according to MCS level/index and/or the transmission BW (with consideration of T-put performance). Furthermore, it could also provide flexible PTRS density in time/frequency domain by setting the threshold values of MCS/BW for determination of multiple different PTRS densities (with consideration of UE channel/situation).

- Potential numerology and further considerations to support NR in FR-X

As stated in the SID [1], FR-X NR design is targeting to operate in higher frequency band than the current FR2 where the ICI term in phase noise might not be negligible. Thus, based on the above observations, it might be expected that performance degradation for the case with high MCS would probably be significant if the SCS values and PTRS structure defined in Rel-15 are used even for FR-X band as it is. Given that, in order to mitigate potential degradation caused by the ICI effect, one of the following two approaches would need to be adopted for FR-X NR design.

- Approach 1

  - Introduce new larger SCS values (e.g. 240 kHz and/or 480 kHz) than that for the current FR2 (e.g. 120 kHz), and reuse the existing Rel-15 PTRS structure

- Approach 2

  - Introduce new additional PTRS structure/mapping which could make compensation of the ICI term possible, and reuse the existing Rel-15 SCS values in FR2

Between these two approaches, Approach 1 is preferred for FR-X NR design with consideration of: 1) frequency utilization efficiency with wider carrier BW as well as multi-RAT coexistence/channelization in unlicensed band, 2) robustness to the ICI effect (as well as large Doppler shift) by using higher SCS value, 3) impacts to UE implementation which would be involved with new time domain processing (e.g. filtering) and new PTRS mapping/estimation in case of Approach 2. Consequently, the followings are proposed as potential candidate numerologies to support NR in FR-X band in the context of Approach 1. To be specific, an existing 240 kHz SCS could be reused for SSB transmission applying very low MCS robust to phase noise effects, and a new 240/480 kHz SCS would need to be introduced for other channel/signal (e.g. PDSCH/PUSCH) possibly applying high MCS sensitive to phase noise effects. Correspondingly, 800 MHz and/or 1.6 GHz could be considered as the maximum carrier BW for the 240/480 kHz SCSs by employing the current FFT size of 4096 (or 2048 for ADC complexity reduction).

Proposal #1: Consider the followings as candidate numerologies to support NR in FR-X band by taking frequency utilization efficiency, unlicensed band operation, the ICI mitigation, and the UE implementation into account.

- SCS for SSB transmission

  - Reuse the existing value of 240 kHz (and/or 120 kHz)

- SCS for other channels/signals

  - Introduce new value as 240 kHz and 480 kHz (and/or 120 kHz)

- Maximum carrier BW

  - 800 MHz for the SCS of 240 kHz

  - 1.6 GHz (and/or 800 MHz) for the SCS of 480 kHz

Considering to adopt larger SCS values of 240/480 kHz for supporting FR-X NR, correspondingly, the OFDM symbol duration would be reduced and also the CP length would be shortened. For this reason, it may be required to study further on potential impact (and relevant handling) due to the shortening of symbol duration and CP length in terms of whether/how to handle cell coverage and/or beam switching time (e.g. by employing the extended CP and/or grouping multiple OFDM symbols as a unit).

Proposal #2: Study further on potential impacts (and relevant handling) due to the shortening of OFDM symbol duration and CP length by adopting larger SCS value.

- Whether/how to handle impact to cell coverage and/or beam switching time (e.g. by employing the extended CP and/or grouping multiple OFDM symbols as a unit)

Furthermore, considering potential phase noise effects in FR-X band to certain control channel (e.g. PUCCH) which was designed without PTRS in Rel-15, further study may also be necessary (e.g. whether/how to handle phase noise effects on such channel).

- Potential impacts to FR2 PHY design to support NR in FR-X band

- SSB transmission

As described in Section 2, the numerology (e.g., 240 kHz SCS) defined for the SSB transmitted in FR2 can be reused for the SSB transmission in FR-X band as well. Furthermore, support of up to 64 SSB (candidate) indexes for FR2 can be also applied for NR in FR-X band, especially for licensed spectrum. On the other hand, for unlicensed band operation, it could be beneficial to provide more opportunities for SSB transmission to cope with LBT failure. Similar to the mechanism already introduced for NR-U, within a SSB transmission window, multiple candidate SSB indexes corresponding to a SSB index can be defined with wrap-around manner. For instance, as shown in Figure 1, in addition to 64 SSB candidate indexes (from 0 to 63) positioned in the first 2.5 msec within 5 msec window as in Rel-15 NR FR2 design, additional 64 SSB candidate indexes (from 64 to 127) can be considered in the second 2.5 msec within the window. In this case, the total number of candidate SSB indexes within 5 msec window is increased to 128, which may lead to the necessity of discussing how to express candidate SSB indexes up to 128 (e.g., by using MIB or PBCH DM-RS). Furthermore, QCL relationship among candidate SSB indexes may need to be defined, for example, by using mod(candidate SSB index or PBCH DM-RS index, Q) where Q value can be signalled, for example, as one of {8, 16, 32, 64}.

Figure 1. Example of SSB transmission for NR in FR-X unlicensed band

Proposal #3: Consider the enhancements for the SSB transmission to provide more opportunities in FR-X unlicensed band.

- Multiplexing between SSB and CORESET#0

If new numerologies such as the SCS values of 240 kHz and 480 kHz are defined for DL signal/channels other than SSB to support NR in FR-X band, it should be discussed how to multiplex SSB and corresponding CORESET#0. In Rel-15 NR FR2 design, three patterns for the multiplexing between SSB and CORESET#0 are supported as follows:

- Pattern 1: TDM between SSB and CORESET#0

- Pattern 2: FDM between SSB and CORESET#0 having different SCS from SSB

- Pattern 3: FDM between SSB and CORESET#0 having the same SCS as for SSB

Therefore, for SSB with 240 kHz SCS, Pattern 1 and/or Pattern 3 can be supported for CORESET#0 with 240 kHz SCS, and Pattern 1 and/or Pattern 2 can be supported for CORESET#0 with 480 kHz SCS. Especially for Pattern 1, CORESET#0 can be located close to the corresponding SSB such that CORESET#0(s) and corresponding SSB(s) can be contiguously transmitted for unlicensed band operation.

Proposal #4: Study further how to multiplex SSB and corresponding CORESET#0 in case of using new numerologies such as 240/480 kHz SCSs for the DL signal/channels other than SSB.

- Conclusions

In this contribution, the required changes to support NR in high frequency range (FR-X) from 52.6 GHz to 71 GHz were discussed, and the followings are proposed.

Proposal #1: Consider the followings as candidate numerologies to support NR in FR-X band by taking frequency utilization efficiency, unlicensed band operation, the ICI mitigation, and the UE implementation into account.

- SCS for SSB transmission

  - Reuse the existing value of 240 kHz (and/or 120 kHz)

- SCS for other channels/signals

  - Introduce new value as 240 kHz and 480 kHz (and/or 120 kHz)

- Maximum carrier BW

  - 800 MHz for the SCS of 240 kHz

  - 1.6 GHz (and/or 800 MHz) for the SCS of 480 kHz

Proposal #2: Study further on potential impacts (and relevant handling) due to the shortening of OFDM symbol duration and CP length by adopting larger SCS value.

- Whether/how to handle impact to cell coverage and/or beam switching time (e.g. by employing the extended CP and/or grouping multiple OFDM symbols as a unit)

Proposal #3: Consider the enhancements for the SSB transmission to provide more opportunities in FR-X unlicensed band.

Proposal #4: Study further how to multiplex SSB and corresponding CORESET#0 in case of using new numerologies such as 240/480 kHz SCSs for the DL signal/channels other than SSB.

- References

RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71 GHz”, RAN#86, Intel Corporation

RAN1 Chairman’s Notes, RAN1#87

RAN1 Chairman’s Notes, RAN1#AH1701

R1-1700608, “Discussion and evaluation on NR-PSS/SSS structure”, NTT DOCOMO

RAN1 Chairman’s Notes, RAN1#88

RAN1 Chairman’s Notes, RAN1#88bis