3GPP TSG RAN WG1 #101                                  R1-2004039

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	8.1.2

Source: 	LG Electronics

Title: 	Considerations on channel access mechanism to support NR above 52.6 GHz

Document for:	Discussion and decision

- Introduction

A new SI to study on supporting NR in high frequency band from 52.6 GHz to 71 GHz was approved in RAN#86 with following objectives [1].

- Study of required changes to NR using existing DL/UL NR waveform to support operation between 52.6 GHz and 71 GHz - Study of applicable numerology including subcarrier spacing, channel BW (including maximum BW), and their impact to FR2 physical layer design to support system functionality considering practical RF impairments [RAN1, RAN4]. - Identify potential critical problems to physical signal/channels, if any [RAN1]. - Study of channel access mechanism, considering potential interference to/from other nodes, assuming beam based operation, in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6 GHz and 71 GHz [RAN1]. - Note: It is clarified that potential interference impact, if identified, may require interference mitigation solutions as part of channel access mechanism.

In this contribution, we discuss and provide initial views on the channel access mechanism to support NR unlicensed band operation in high frequency range from 52.6 GHz to 71 GHz (simply denoted as “FR-X” hereafter), in terms of ED threshold adaptation mechanism and the directional LBT/transmission considering the regulations of the FR-X band.

- ED threshold adaptation mechanism considering regulatory requirement and numerology for NR above 52.6 GHz

According to 38.807 [2], most countries declare frequency band from 57 GHz to 66 GHz as unlicensed spectrum and have the power requirements of max EIRP about 500mW (40dBm), and some countries (EU, Korea, Singapore) have the maximum power spectral density (PSD) requirement of 13dBm/MHz. For spectrum access requirements, Europe regulation requires an adequate spectrum sharing mechanism such as LBT, and Japan requires carrier sensing functionality when the transmission power is more than 10dBm. Therefore, if there is an incumbent system operating in the 60 GHz band, an appropriate spectrum sharing mechanism might be required for fair coexistence.

In order to support NR in the frequency range from 52.6 GHz to 71 GHz, the numerology would need to be changed at least considering phase noise effects, and CC (component carrier) bandwidth would be determined accordingly. For example, if 240 kHz or 480 kHz is used as the candidate subcarrier spacing (SCS) of the data channel for FR-X as discussed in our companion paper [3], the CC bandwidth of NR above 52.6GHz can be up to 800 MHz or 1600 MHz.

However, assuming that the NR complies with the regulatory requirement, the output power or PSD may need to be adjusted depending on the operating CC BW. For example, if the CC bandwidth is 500 MHz and the transmission is performed with the maximum PSD (=13dBm/MHz), the total output power can already reach the maximum output power (=40dBm) allowed by regulation. Therefore, in order to transmit the CC bandwidth greater than 500MHz, it might be necessary to adjust the PSD to be lower than 13dBm/MHz. For example, in order to transmit a CC bandwidth of up to 800 MHz corresponding to the SCS value of 240 kHz, it may be required to transmit with the PSD of 11 dBm/MHz which is lower than the maximum PSD allowed by regulation.

Therefore, to support NR in FR-X band, multiple parameters including the numerology, channel bandwidth of NR and the power requirements such as maximum output power and power spectral density, should be considered in determining the channel access mechanism.

Proposal #1: In order to support NR operation in unlicensed band from 52.6GHz to 71GHz, multiple parameters including the numerology, channel bandwidth of NR and the power requirements such as maximum output power and power spectral density, should be considered in determining the channel access mechanism.

If a channel access mechanism is introduced with consideration of coexistence with an incumbent system (e.g., WiGig) operating in the frequency range from 52.6 GHz to 71 GHz, the LBT parameter setting (e.g., energy detection threshold adaptation procedure) may need to be enhanced considering the channel bandwidth of NR and the power requirements such as maximum output power and maximum PSD.

To explain a method of the ED threshold setting according to the channel bandwidth of NR, the output power and the PSD, the reference LBT parameter set can be defined as {reference (LBT) BW, reference (max output) power, reference ED (threshold)}. The value of each parameter in the reference LBT parameter set can be determined based on the regulatory requirements and/or the (nominal) system bandwidth, the maximum output power (allowed for that BW size), and the ED threshold defined by other RATs. For example, when considering the WiGig system as a reference, the reference LBT parameter set can be expressed as {BWiGig = 2.16GHz, PWiGig = 40dBm, TWiGig = -48dBm}.

If the unit LBT BW of NR is considered to be the same as its CC BW (=BNR) and if BNR = BWiGig, the maximum output power of NR (=PNR) and ED threshold of NR (=TNR) can be set to the same values as in the reference LBT parameter sets (used for WiGig). In other words, the LBT parameter set of NR can be expressed as {BNR = 2.16GHz, PNR = 40dBm, TNR = -48dBm}, and in this case, it would be obvious that NR can fairly coexist with WiGig.

Even in case when BNR <BWiGig, the maximum output power and ED threshold of NR could be set to the same values as in the reference LBT parameter sets used for WiGig as long as it does not violate the regulatory requirement. However, since NR performs LBT with a smaller bandwidth than WiGig, PNR and TNR might need to be adjusted with consideration of coexistence. For example, in case when BNR < BWiGig, PNR can be set to be lower than PWiGig, and TNR can be adapted to be higher than TWiGig. As a result, the NR can perform the channel access procedure using the LBT parameter set by {BNR (<BWiGig), PNR (<PWiGig), TNR (> TWiGig)}.

Proposal #2: It is necessary to enhance the method of determining ED threshold with consideration of the maximum output power and the unit LBT bandwidth applied in NR and the fair coexistence with the incumbent system (e.g., WiGig) operating in frequency range from 52.6GHz to 71 GHz.

- Directional channel access procedure for beam based operation of NR above 52.6 GHz

Since the cell coverage can be reduced in the high frequency band, the beam-based transmission using a multi-antenna beamforming technique may be useful. Therefore, the directional CCA can be utilized for channel access mechanism to enhance the performance of beam-based transmission in above 52.6GHz band.

However, it is noted that whether to introduce the directional CCA can be determined based on performance analysis between the beam-based transmission using directional CCA and beam-based transmission using the (legacy) omni-directional CCA. If beam-based transmission using directional CCA shows a performance gain, the following issues related to directional CCA operation can be discussed.

■ Transmitter performs channel sensing before transmission

  - CCA threshold

    - CCA threshold for directional transmission may need to be different from the case of omni-directional transmission since the coverage and interference from a directional transmission will be different from an omni-directional transmission

  - In case of directional CCA, relationship between transmission direction and CCA direction

    - gNB or UE’s real implementation may not be able to well match the reception direction and the transmission direction. Moreover, applying same spatial direction as transmission direction for CCA may not be effective since interference to the receiver will be estimated in the reverse direction from the transmission direction as depicted in figure 1. In that sense, we may consider receiver performs channel sensing before reception as well.

Figure 1. Illustration of interference caused by directional transmission

■ Transmission with multiple candidate directions

For some broadcast signals/channels (e.g. SSB) in NR, multi-beam sweeping transmission is necessary. Depending on the further discussion on the LBT operation for directional transmission, directional CCA at transmitter side may be necessary. In that case, it should be further studied how to perform omni-directional or directional CCA for multiple-beam sweeping transmission.

In addition, if either transmitter or receiver side channel sensing is introduced per transmission direction or reception direction, it should be also discussed how to define CWS management, e.g., per-direction management or across-direction management.

Proposal #3: If directional CCA procedure with beam based transmission is identified as beneficial, the followings for directional CCA procedure can be considered:

- CCA threshold setting

- Relationship between transmission direction and CCA direction

- Directional LBT for broadcast/unicast transmission

- CWS management

- Conclusions

In this contribution, the channel access mechanism to support NR in high frequency range from 52.6 GHz to 71 GHz was discussed, and the followings are proposed.

Proposal #1: In order to support NR operation in unlicensed band from 52.6GHz to 71GHz, multiple parameters including the numerology, channel bandwidth of NR and the power requirements such as maximum output power and power spectral density, should be considered in determining the channel access mechanism.

Proposal #2: It is necessary to enhance the method of determining ED threshold with consideration of the maximum output power and the unit LBT bandwidth applied in NR and the fair coexistence with the incumbent system (e.g., WiGig) operating in frequency range from 52.6GHz to 71 GHz.

Proposal #3: If directional CCA procedure with beam based transmission is identified as beneficial, the followings for directional CCA procedure can be considered:

- CCA threshold setting

- Relationship between transmission direction and CCA direction

- Directional LBT for broadcast/unicast transmission

- CWS management

- References

- RP-193259, “New SID: Study on supporting NR from 52.6GHz to 71 GHz”, RAN#86, Intel Corporation.

- 3GPP TR 38.807: “Study on requirements for NR beyond 52.6 GHz”.

- R1-2004038, “Consideration on required physical layer changes to support NR above 52.6 GHz”, LG Electronics.