3GPP TSG RAN WG1 #101	R1-2004041

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.2.1.3

Source: 	Fujitsu

Title:	Remaining issues on UL signals and channels for NR-U

Document for:	Discussion/Decision

- Introduction

In this contribution, we would like to discuss some remaining issues on resource allocation for PUSCH.

Some relevant agreements are listed in Appendix A.

- Discussion

2.1. Resource allocation for PUSCH scheduled by DCI 0_0 in CSS and RAR UL grant

Regarding resource allocation for PUSCH scheduled by DCI 0_0 in CSS and RAR UL grant, some discussion and progress were made in the last meeting. However, there are still issues remained to be further discussed.

During the discussion in the last meeting, some companies proposed to adopt same principle of resource allocation for PUSCH scheduled by DCI 0_0 addressed to TC-RNTI and RAR UL grant, since they are facing the same issue that gNB may not know the active UL BWP of the UE if consider CBRA. And we share the same view. Therefore, in the following, we will separately discuss two cases, i.e.

- Case 1: PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI

- Case 2: PUSCH scheduled by DCI 0_0 addressed to TC-RNTI or RAR UL grant

Case 1

For case 1, the following agreement was made.

Agreement: - For PUSCH scheduled by DCI 0_0 received in a CSS when UL resource allocation Type 2 is configured, PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which DCI 0_0 is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP. - FFS1: PUSCH allocation within the active UL BWP corresponding to an UL carrier without intra-cell guard bands - FFS2: Whether or not the first bullet is modified to “…the active DL BWP in which the first REG of the received DCI 0_0 is located,” in order to facilitate a CORESET not confined to a single RB set.

Regarding the FFS1, as per agreements in the last meeting, RB sets are configured even for an UL carrier without intra-cell guard bands. Therefore, no modification specifically for this scenario is needed.

Regarding the FFS2, we think the first bullet should be modified to facilitate a CORESET not confined to a single RB set. But to figure out how to modify it, one more scenario, where the DL carrier is configured without intra-cell guard bands, should be also considered. In this scenario, the rule is infeasible since it’s relying on RB set of the DL while there is no definition of RB set for a DL carrier without intra-cell guard bands. Therefore, taking both of the scenarios into account, we propose to decouple the RB set in UL and RB set in DL. For example, the first bullet could be modified as follows.

- For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI when UL resource allocation Type 2 is configured, PUSCH is allocated to the first RB set of the active UL BWP that overlaps with the first REG of the received DCI 0_0. If there is no overlap, PUSCH is allocated to RB Set 0 of the active UL BWP.

Observation 1: For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI, the rule of resource allocation is infeasible in scenarios where a CORESET is not confined to a single RB set and where the DL carrier is configured without intra-cell guard bands.

Proposal 1: For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI, the rule of resource allocation should be modified.

- For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI when UL resource allocation Type 2 is configured, PUSCH is allocated to the first RB set of the active UL BWP that overlaps with the first REG of the received DCI 0_0. If there is no overlap, PUSCH is allocated to RB Set 0 of the active UL BWP.

  - Adopt TP1 for Clause 6.1.2.2.3 of TS 38.214 in Appendix B.

Case 2

For case 2, the following agreement was made.

Agreement: To resolve the FFS in TP#1, support one of the following two alternatives for the PUSCH allocation rule. - When UL resource allocation Type 2 is configured, the UE assumes that PUSCH is allocated as follows: - Alt-1: PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which the DCI 0_1 that schedules the PDSCH containing the RAR UL grant is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP. - Alt-2: PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to RB Set 0 of the active UL BWP. - FFS: Rule for PUSCH allocation for an UL carrier without intra-cell guard bands.

Firstly, Alt-1 is following the similar rule as that for case 1. Based on the discussion in case 1, Alt-1 could be updated accordingly as follows.

- Alt-1: PUSCH is allocated to the first RB set of the active UL BWP that overlaps the first REG of the received DCI 0_0 addressed to TC-RNTI or of the received DCI 1_0 that schedules the PDSCH containing the RAR UL grant is received. If there is no overlap, PUSCH is allocated to RB Set 0 of the active UL BWP.

Secondly, as mentioned above, the big issue in case 2 is that the gNB may not know the exact active UL BWP of the UE, since it cannot identify the UE in CBRA. Then it may be problematic for gNB receiving the Msg3 PUSCH from the UE as well as scheduling from system perspective. Considering that, in CBRA, a rule that can guarantee common understanding on the resources of Msg3 PUSCH between the gNB and the UE is preferred.

Based on the above, we will further discuss the rule of Msg3 PUSCH allocation considering different scenarios.

The first scenario is where the active UL BWP of the UE is fully overlapping with the initial UL BWP. As noted in the FFS, the rule for an UL carrier without intra-cell guard bands needs to be discussed. Meanwhile, according to the following agreement in the lasting meeting, whether a BWP can be configured to be partially overlapping with a RB set is FFS.

Agreement: For an UL carrier without intra-cell guard bands when the parameter useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated: - The UL carrier can be configured with NRB-set,UL≥1non-overlapping RB set(s) - For each RB set except for RB set 0, the starting CRB index is given by startCRB-r16 - For RB set 0, the starting CRB index is given by Ngrid,ULstart,μ - The UE expects nrofCRBs-r16 set to 0 for all GBs between two adjacent RB sets within the UL carrier. - The UE expects N RBs contained in each interlace of each RB set, wherein 10 <= N <= 11. - For 30 kHz SCS, the number of RBs within any RB set is between 50 and 55, and for 15 kHz SCS, the number of RBs within any RB set is between 100 and 110 - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP (from RAN1#99 agreement) - Note: It’s up to gNB’s configuration to fulfill RAN4 requirement with e.g., on maximum transmission bandwidth configuration, spectral emission mask, and so on. - Note: In order to reuse existing PUCCH/PUSCH resource allocation mechanisms, this proposal applies to all supported carrier bandwidths except 10 MHz - FFS: Whether BWP can be configured to be partially overlapping with a RB set

If it is supported, an example is given in Figure 1. It is assuming that UE1’s active UL BWP is the initial UL BWP, UE2’s active UL BWP is fully overlapping with the initial UL BWP, and the initial UL BWP is partially overlapping with RB set 0. As illustrated in the figure, based on Alt-1, the determined resources for Msg3 PUSCH would be different for UE1 and UE2, while based on alt-2, it could be same. Therefore, at least if the initial UL BWP is partially overlapping with a RB set, using initial UL BWP is a better choice when the active UL BWP overlaps with the initial UL BWP.

Figure 1. Example of determined resources at UE side for Msg3 PUSCH in the first scenario

Proposal 2: At least if initial UL BWP can be partially overlapping with a RB set, PUSCH should be allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP.

The second scenario is where the active UL BWP of a UE is not fully overlapping with the initial UL BWP. That is, for the UE, it cannot use the initial UL BWP to transmit Msg3 PUSCH. It’s necessary to have another method.

In Rel-15, in this scenario, UE would use the lower part of resources of the active UL BWP. However, for the same reason mentioned above, it cannot guarantee common understanding on resources of Msg3 PUSCH between the gNB and the UE. To have the common understanding, there would be e.g. restriction on BWP configuration. In Alt-2, using RB Set 0 of active UL BWP in the scenario would be in the similar situation as in Rel-15, e.g. as shown in (1) of Figure 2.

However, since we are considering resource allocation in the units of RB set, there is another alternative which could fix the issue in a better way. That is, the PUSCH is allocated to the RB set where the PRACH is transmitted on the active BWP, as shown in (2) of Figure 2.

Figure 2. Example of determined resources at UE side for Msg3 PUSCH in the second scenario

Based on the discussions above, Alt-2 is preferred with some modification as in Proposal 3.

Proposal 3: For PUSCH scheduled by DCI 0_0 scrambled by TC-RNTI or RAR UL grant, PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to the RB set where PRACH is transmitted in the active BWP.

- Adopt TP2 for Clause 6.1.2.2.3 of TS 38.214 in Appendix B.

Apart from discussions above, the impact of SCS of active UL BWP on resource allocation may also need to be considered. From UE’s perspective, the X bits in FDRA field and the allocated resources are determined based on the SCS of the active UL BWP. However, in CBRA, gNB may also not know the SCS of the active UL BWP of the UE. Then the understanding on the allocated resources would be not matched between the gNB and UE. To avoid that mismatch, one option is that the SCS of PRACH is always same as the SCS of the UL BWP where it is configured.

Proposal 4: For PUSCH scheduled by DCI 0_0 scrambled by TC-RNTI or RAR UL grant, to avoid the mismatch of understanding on allocated resources between gNB and UE, the SCS of PRACH should be always same as the SCS of the UL BWP where it is configured.

- Adopt TP3 for Clause 8.3 of TS 38.213 in Appendix B.

2.2. Transmission in intra-cell guard band

According to an LS [5] with respect to UE feature from RAN4, if the UE is scheduled a contiguous allocation that is wider than a 20MHz subband, whether the UE can transmit in the intra-cell guard band depends on UE capability. Specifically, if UE cannot transmit in the guard bands, it could only transmit in the subbands.

However, according to clause 6.1.2.2.3 of TS 38.214, RBs in intra-cell guard bands between indicated RB sets, if any, would be allocated to the UE for PUSCH transmission, regardless of the UE capability. Therefore, in order to clarify resource allocation for PUSCH considering UE capability as per the RAN4’s LS, we provide a TP as follows.

Proposal 5: In order to clarify resource allocation for PUSCH considering UE capability of transmission in intra-cell guard bands, adopt TP4 for Clause 6.1.2.2.3 of TS 38.214 in Appendix B.

- Conclusion

According to the discussions above, we have the following observation and proposals.

Observation 1: For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI, the rule of resource allocation is infeasible in scenarios where a CORESET is not confined to a single RB set and where the DL carrier is configured without intra-cell guard bands.

Proposal 1: For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI, the rule of resource allocation should be modified.

- For PUSCH scheduled by DCI 0_0 in a CSS addressed to C-RNTI, MCS-C-RNTI or CS-RNTI when UL resource allocation Type 2 is configured, PUSCH is allocated to the first RB set of the active UL BWP that overlaps with the first REG of the received DCI 0_0. If there is no overlap, PUSCH is allocated to RB Set 0 of the active UL BWP.

  - Adopt TP1 for Clause 6.1.2.2.3 of TS 38.214 in Appendix B.

Proposal 2: At least if initial UL BWP can be partially overlapping with a RB set, PUSCH should be allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP.

Proposal 3: For PUSCH scheduled by DCI 0_0 scrambled by TC-RNTI or RAR UL grant, PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to the RB set where PRACH is transmitted in the active BWP.

- Adopt TP2 for Clause 6.1.2.2.3 of TS 38.214 in Appendix B.

Proposal 4: For PUSCH scheduled by DCI 0_0 scrambled by TC-RNTI or RAR UL grant, to avoid the mismatch of understanding on allocated resources between gNB and UE, the SCS of PRACH should be always same as the SCS of the UL BWP where it is configured.

- Adopt TP3 for Clause 8.3 of TS 38.213 in Appendix B.

Proposal 5: In order to clarify resource allocation for PUSCH considering UE capability of transmission in intra-cell guard bands, adopt TP4 for Clause 6.1.2.2.3 of TS 38.214 in Appendix B.

Corresponding to different proposals above, there are multiple TPs, TP1, TP2, and TP4, provided for Clause 6.1.2.2.3 of TS 38.214. For convenience, a compact version, TP5, is also provided in Appendix B.

References

- R1-2003276 (R4-2005192), LS on Rel-16 RAN4 UE features lists for LTE and NR, 3GPP TSG RAN WG1 #101, e-Meeting, May 25th – June 5th, 2020

- Chairman's Notes RAN1#98 final, 3GPP TSG RAN WG1 Meeting #98, Aug. 2019.

- Chairman's Notes RAN1#99 final, 3GPP TSG RAN WG1 Meeting #99, Nov. 2019.

- RAN1#100-e list of email discussion approvals final, 3GPP TSG RAN WG1 Meeting #100-e, Feb. 2020.

- Chairman's Notes RAN1#100b-e final, 3GPP TSG RAN WG1 Meeting #100b-e, Apr. 2020.

Appendix A: Relevant Agreements

| RAN1#98[2] | Agreement: - For interlaced PUSCH transmission in a BWP, X bits of the PUSCH frequency domain resource allocation field are used for indicating which combination of M interlaces is allocated to the UE. - This applies to PUSCH of the following types: - Msg3 PUSCH - PUSCH Scheduled by fallback and non-fallback DCI - Type 1 and Type 2 Configured Grant PUSCH - For 30 kHz SCS - Support X = 5 (5-bit bitmap to indicate all possible interlace combinations) - For 15 kHz SCS - Down-select between the following two alternatives: - Alt-1: Support X = 10 (10-bit bitmap to indicate all possible interlace combinations) - Alt-2: Support X = 6 bits to indicate start interlace index and number of contiguous interlace indices (RIV) and using remaining up to 9 RIV values to indicate specific pre-defined interlace combinations |
| --- | --- |
| RAN1#99[3] | Agreement: - For interlaced PUSCH transmission in a BWP, Y bits of the frequency domain resource allocation (FDRA) field indicate which RB sets (corresponding to LBT bandwidths) are allocated to the UE - This applies to PUSCH of the following types - PUSCH scheduled by at least non-fallback DCI - FFS: applicability to fallback DCI - Configured Grant PUSCH Type 2 (FDRA indicated by DCI) - Configured Grant PUSCH Type 1 (FDRA configured by RRC) - The UE determines the overall PUSCH frequency domain resource allocation by the intersection of the following: - Allocated interlaces (indicated by X bits of the FDRA field, as previously agreed) - Available PRBs derived at least from the allocated RB sets (indicated by Y bits of the FDRA field) and intra-carrier guard bands between RB sets corresponding to contiguous LBT bandwidths - Note: An RB set contains PRBs within an LBT bandwidth and does not include any inter or intra carrier guard PRBs - Note: The PRBs between adjacent RB sets comprise an intra-carrier guard - Y is determined by the number of RB sets contained in the BWP - The Y bits indicate a first RB set and a number of RB sets corresponding to contiguous LBT bandwidths - Note: The maximum possible value of Y is thus log2NN+12 where N is the number of RB sets contained in the BWP |
| RAN1#100-e[4] | Agreement: Modify the agreement from RAN1#99 as follows: - The UE transmits PUSCH scheduled by fallback DCI in CSS within the initial BWP on the interlaces indicated by the X bits of the FDRA field - Note: The FDRA field for fallback DCI in CSS does not include Y bits |
| RAN1#100b-e[5] | Agreement: - For PUSCH scheduled by DCI 0_0 received in a CSS when UL resource allocation Type 2 is configured, PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which DCI 0_0 is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP. - FFS1: PUSCH allocation within the active UL BWP corresponding to an UL carrier without intra-cell guard bands - FFS2: Whether or not the first bullet is modified to “…the active DL BWP in which the first REG of the received DCI 0_0 is located,” in order to facilitate a CORESET not confined to a single RB set. Agreement: Adopt TP#1 in R1-2002913 for 38.213 Section 8.3 and TP#2 for 38.214 Section 6.1.2.2.3 where the last bullet of TP#1 is modified as follows: - The UE assumes the RB set allocation in the active UL BWP for a PUSCH scheduled by the RAR UL grant is given by [FFS: rule for for RB set allocation] - FFS: PUSCH allocation rule within the interlaces indicated by the frequency domain resource allocation field Agreement: To resolve the FFS in TP#1, support one of the following two alternatives for the PUSCH allocation rule. - When UL resource allocation Type 2 is configured, the UE assumes that PUSCH is allocated as follows: - Alt-1: PUSCH is allocated to the RB set of the active UL BWP that intersects the RB set of the active DL BWP in which the DCI 0_1 that schedules the PDSCH containing the RAR UL grant is received. If there is no intersection, PUSCH is allocated to RB Set 0 of the active UL BWP. - Alt-2: PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to RB Set 0 of the active UL BWP. - FFS: Rule for PUSCH allocation for an UL carrier without intra-cell guard bands. Agreement: For an UL carrier without intra-cell guard bands when the parameter useInterlacePUCCH-PUCCH is configured in any of BWP-UplinkCommon and BWP-UplinkDedicated: - The UL carrier can be configured with NRB-set,UL≥1non-overlapping RB set(s) - For each RB set except for RB set 0, the starting CRB index is given by startCRB-r16 - For RB set 0, the starting CRB index is given by Ngrid,ULstart,μ - The UE expects nrofCRBs-r16 set to 0 for all GBs between two adjacent RB sets within the UL carrier. - The UE expects N RBs contained in each interlace of each RB set, wherein 10 <= N <= 11. - For 30 kHz SCS, the number of RBs within any RB set is between 50 and 55, and for 15 kHz SCS, the number of RBs within any RB set is between 100 and 110 - Note: This configuration may be used for the case where transmission only occurs in a BWP if LBT is successful in all RB sets within the BWP (from RAN1#99 agreement) - Note: It’s up to gNB’s configuration to fulfill RAN4 requirement with e.g., on maximum transmission bandwidth configuration, spectral emission mask, and so on. - Note: In order to reuse existing PUCCH/PUSCH resource allocation mechanisms, this proposal applies to all supported carrier bandwidths except 10 MHz - FFS: Whether BWP can be configured to be partially overlapping with a RB set |

Appendix B: Text Proposals

- TP1 (corresponding to Proposal 1)

------------------------------------ Text Proposal 1 for Clause 6.1.2.2.3 of TS 38.214------------------------------------

*** Unchanged text omitted ***

6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to  NRB-set,ULBWP  contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For DCI 0_0 monitored in a common search space with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. The uplink RB set is the first one that overlaps with the first REG of the received DCI 0_0. If there is no overlap, the uplink RB set is RB set 0 in the active uplink BWP.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

- TP2 (corresponding to Proposal 3)

------------------------------------- Text Proposal 2 for Clause 6.1.2.2.3 of TS 38.214-----------------------------------*** Unchanged text omitted ***

6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to  NRB-set,ULBWP  contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. For DCI 0_0 monitored in a common search space, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. The uplink RB set is the one that intersects with the downlink RB set of the active downlink BWP in which the UE detects the DCI 0_0. If there is no intersection, the uplink RB set is RB set 0 in the active uplink BWP. For DCI 0_0 with CRC scrambled by TC-RNTI, if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, the UE shall determine the allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the initial UL BWP, otherwise as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP, and the uplink RB set is the one where the corresponding PRACH is transmitted. PUSCH is allocated to the initial UL BWP if the active UL BWP fully overlaps the initial UL BWP, otherwise PUSCH is allocated to the RB set where PRACH is transmitted in the active BWP.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

- TP3 (corresponding to Proposal 4)

------------------------------------------ Text Proposal 3 for Clause 8.3 of TS 38.213-------------------------------------***Unchanged text omitted ***

If useInterlace-PUCCH-PUSCH is provided in BWP-UplinkCommon or BWP-UplinkDedicated, the frequency domain resource allocation is by uplink resource allocation type 2 [6, TS 38.214]. A UE processes the frequency domain resource assignment field as follows

- truncate the frequency domain resource assignment field to the X=6 LSBs if μ=0, or to the X=5 LSBs if μ=1.

- interpret the truncated frequency domain resource assignment field for the active UL BWP as for the X MSBs of the frequency domain resource assignment field in DCI format 0_0 with CRC scrambled by TC-RNTI [6, TS 38.214]. The UE expects same SCS of PRACH and the UL BWP where the PRACH is configured.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

- TP4 (corresponding to Proposal 5)

------------------------------------- Text Proposal 4 for Clause 6.1.2.2.3 of TS 38.214-----------------------------------*** Unchanged text omitted ***

6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to  NRB-set,ULBWP  contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, if the UE indicates the capability of transmission in intra-cell guard bands, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces, and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. Otherwise, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets. For DCI 0_0 monitored in a common search space, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. The uplink RB set is the one that intersects with the downlink RB set of the active downlink BWP in which the UE detects the DCI 0_0. If there is no intersection, the uplink RB set is RB set 0 in the active uplink BWP.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------

- TP5 (compact version based on TP1, TP2 and TP4 for Clause 6.1.2.2.3 of TS 38.214)

------------------------------------ Text Proposal 1 for Clause 6.1.2.2.3 of TS 38.214------------------------------------

*** Unchanged text omitted ***

6.1.2.2.3	Uplink resource allocation type 2

In uplink resource allocation of type 2, the resource block assignment information defined in [5, TS 38.212] indicates to a UE a set of up to M interlace indices, and for DCI 0_0 monitored in a UE-specific search space and DCI 0_1 a set of up to  NRB-set,ULBWP  contiguous RB sets, where M and interlace indexing are defined in Clause 4.4.4.6 in [4, TS 38.211]. For DCI 0_0 monitored in a UE-specific search space and DCI 0_1, if the UE indicates the capability of transmission in intra-cell guard bands, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces, and the indicated set of RB sets and intra-cell guard bands defined in Clause 7 between the indicated RB sets, if any. Otherwise, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the indicated set of RB sets. For DCI 0_0 monitored in a common search space with CRC scrambled by C-RNTI, MCS-C-RNTI or CS-RNTI, the UE shall determine the resource allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP. The uplink RB set is the first one that overlaps with the first REG of the received DCI 0_0. If there is no overlap, the uplink RB set is RB set 0 in the active uplink BWP. For DCI 0_0 with CRC scrambled by TC-RNTI, if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, the UE shall determine the allocation in frequency domain as an intersection of the resource blocks of the indicated interlaces and the initial UL BWP, otherwise as an intersection of the resource blocks of the indicated interlaces and a single uplink RB set of the active UL BWP, and the uplink RB set is the one where the corresponding PRACH is transmitted.

*** Unchanged text omitted ***

------------------------------------------------------ End Text Proposal -------------------------------------------------------