3GPP TSG RAN WG1 Meeting #101-e	R1-2004149

E-meeting, 25 May – 5 June, 2020

Agenda Item:	7.2.11.9

Source:	Huawei, HiSilicon

Title:	Rel-16 UE features for mobility enhancement

Document for:	Discussion and Decision

Introduction

In this paper, we share our views on Rel-16 UE features for mobility enhancement. The discussions here are in reference to the latest draft in R1-2003073.

Corrections to existing UE features

As discussed in the companion contribution [1], UE implementation, in addition to defining a proper timeline, needs to be considered when deciding whether defining a UE feature regarding uplink cancelation, e.g, an identical RF chain or different RF chains for transmissions to different cells. It seems flexible for UE implementation if defining such a UE capability. The expected UE behaviors are:

- When UE reports support of uplink cancelation, UE behavior complies with the TP agreed in the last meeting [2].

- When UE does not report support of uplink cancelation, it is expected that NW will not schedule uplink transmission on source that overlaps with the transmission on target cell.

In addition, as explained in our companion contribution, it was agreed that the gNB can configure the UE to perform UL power sharing if the UE indicates support of FG 21-2/2a/2b. Similarly, it was also agreed that the gNB can indicate absence of UL power sharing by not configuring UplinkPowerSharingDAPS-HO-mode. The consequence of FG 21-2 not being supported should be updated to “the UE is only able to drop the transmission to the source”.

Proposal 1: Adopt the corrections listed below:

- Remove the FFS and the consequence of not supporting FG 21-2 as “The UE is only able to drop the transmission to the source”.

- Removing the brackets for FG 21-2d, i.e., keep the UE feature of “UL transmission cancellation”.

Summary of proposals

The proposals in this paper are summarized as follows.

Proposal 1: Adopt the corrections listed below:

- Remove the FFS and the consequence of not supporting FG 21-2 as “The UE is only able to drop the transmission to the source”.

- Removing the brackets for FG 21-2d, i.e., keep the UE feature of “UL transmission cancellation”.

- References

- R1-2003506, “Remaining issues on DAPS HO,” Huawei, HiSilicon, RAN1#101-e, May 25- June 5, 2020.

- Chairman’s Notes for RAN1#100b-e, April 20-30, 2020.