3GPP TSG RAN WG1 Meeting #101-e	                              R1-2004155

E-meeting, 25 May – 5 June, 2020

Agenda Item:	8.4

Source:	Huawei, HiSilicon

Title:	Overview of coverage enhancement: scenarios, channels, services and potential solutions

Document for:	Discussion and Decision

- Introduction

In RANP#86 meeting, a new study item, i.e., study on NR coverage enhancement was approved for Release 17 [1]. The SI focuses on identifying target scenarios, services and channels for coverage enhancement and investigation of potential solutions to increase cell coverage, the detailed objectives are copied as follows.

- The target scenarios and services include - Urban (outdoor gNB serving indoor UEs) scenario, and rural scenario (including extreme long distance rural scenario) for FR1 - Indoor scenario (indoor gNB serving indoor UEs), and urban/suburban scenario (including outdoor gNB serving outdoor UEs and outdoor gNB serving indoor UEs) for FR2. - TDD and FDD for FR1. - VoIP and eMBB service for FR1. - eMBB service as first priority and VoIP as second priority for FR2. - LPWA services and scenarios are not included. - Identify baseline coverage performance for both DL and UL for the above scenarios and services based on link-level simulation - UL channels (including PUSCH and PUCCH) are prioritized for FR1. - Both DL and UL channels for FR2. - Identify the performance target for coverage enhancement, and study the potential solutions for coverage enhancements for the above scenarios and services - The target channels include at least PUSCH/PUCCH - Study enhanced solutions, e.g., time domain/frequency domain/DM-RS enhancement (including DM-RS-less transmissions) - Study the additional enhanced solutions for FR2 if any - Evaluate the performance of the potential solutions based on link level simulation.

Regarding the above mentioned objectives in SID, in this contribution, we discuss the following aspects:

- Clarify requirements of NR coverage towards different scenarios (i.e., urban/rural/rural with long distance scenario), channels (i.e., DL and UL channels) and services (i.e., eMBB and VoIP services) with different target data rates.

- Provide baseline simulation assumptions, evaluation metrics and identify the bottleneck channel of coverage based on link budget calculations.

- Analyze key factors which put limitations to coverage, for example, accuracy of channel estimation, overhead of reference signal, time domain scheduling with resource waste, maximum UE transmission power, etc. And those factors may lead to some enlightenment on the potential coverage enhancement solutions.

# Discussion

## On requirements for coverage enhancement

The following two types of services are considered in the SI:

- eMBB service which requires high data rate to enable applications such as surveillance cameras in cities and homes, live video communication and meetings, cloud VR and so on.

- VoIP service which requires a high reliability with a low rBLER and high mean opinion score to ensure good experience.

Target data rates of eMBB and VoIP services under different scenarios are specified as follows:

- Urban scenario with ISD of 300~500 meters, for example, office zones, coffee houses, industrial factories in urban cities, high data rates are required for eMBB services such as no less than 1Mbps uplink and 10Mbps downlink rates.

- Rural scenario with ISD of 1~3 kilometers, for example, farm houses, parks, etc., moderate data rates are required for eMBB services, such as no less than 100kbps uplink and 1Mbps downlink rates.

- Rural scenario with ISD of 10-100km such as beaches and forests, moderate data rates are required for eMBB services, such as above 100kbps uplink and 1Mbps downlink data rates.

- For VoIP services, the target data rate 12.2kbps should be ensured for above three scenarios, with a small packet transmission period of 20 ms.

As agreed in the email discussion [2] that ‘both DL and UL should be taken into account for coverage enhancement for FR1 and FR2’, channels need to be studied for UL and DL are described as follows:

- UL channels:

  - Coverage of PUSCH with high uplink data rate requirements should be studied while the UE ability in uplink transmission is limited.

  - Coverage of PUCCH needs to be studied considering the importance of PUCCH which carries key feedback information such as HARQ ACK/NACK indications. Coverage of PUCCH format 1 can be studied as a reference to judge whether PUCCH needs coverage enhancement.

  - Coverage of Msg3 need to be studied considering its importance for uplink connection and following PUSCH transmissions.

- DL channels:

  - Coverage of PDSCH with a very high data rate requirement for cell edge UEs should be studied, such as 10Mbps.

  - Coverage of PDCCH should be studied considering the importance of PDCCH which carries key scheduling information.

- Other channels (at FR2):

  - For FR2, signals at higher frequency band will experience a much larger pathloss than FR1. Thus, besides above mentioned DL and UL channels, the coverage of other significant channels such as SS/PBCH and PRACH can also be studied.

Usually, coverage of UL transmission is more challenging than that of DL transmission due to the limited UE ability, such as limited maximum transmission power under regulatory restrictions while BS can have higher transmission powers and so on. Thus, according to above analysis of NR coverage requirements, we have following proposals:

Proposal 1: For FR1, coverage of PUSCH and PUCCH need to be studied with a higher priority.

- For PUSCH at FR1: the target data rate for eMBB service is 1Mbps, 100kbps and 100kbps for urban (ISD=500m), rural (ISD=1732 m) and rural with long distance (ISD>10 km) scenarios, respectively.For PUCCH: coverage of PUCCH format 1 should be studied as a reference to judge whether PUCCH needs coverage enhancement.

Proposal 2: Coverage of other channels needed to be studied include:

- For FR1, coverage of Msg3, PDSCH and PDCCH can also be studied to determine which channel needs coverage enhancement;

- For FR2, the coverage of Msg3, PDSCH, PDCCH, SS/PBCH and PRACH can also be studied to determine which channel needs coverage enhancement.

## On evaluation methodology, assumptions and metrics for baseline simulation

In this section, we present the evaluation methodology and evaluation assumptions for different channels towards multiple scenarios.

### Evaluation methodology and revised parameters

Since IMT-2020 self-evaluation has already determined a full set of simulation parameters, the most convenient way is reusing the link budget methodology adopted in IMT-2020 self-evaluation for 3GPP submission [3] as much as possible. Considering this early stage for evaluation, we reuse channel mode A as a start. The detailed evaluation procedure for available path loss and corresponding maximum coverage range, i.e., parameter setting and calculation formula, are cited as in the following table.

Table 1. Link budget template

| Parameter | Values |
| --- | --- |
| Scenario |  |
| Frame structure |  |
| Carrier frequency (Hz) |  |
| BS antenna heights (m) |  |
| UT antenna heights (m) |  |
| Cell area reliability for control channel |  |
| Cell area reliability for data channel |  |
| Transmission bit rate for control channel (bit/s) |  |
| Transmission bit rate for data channel (bit/s) |  |
| Target packet error rate for the required SNR in item (19a) for control channel |  |
| Target packet error rate for the required SNR in item (19b) for data channel |  |
| Spectral efficiency (bit/s/Hz) |  |
| Pathloss model (select from LoS or NLoS) |  |
| UE speed (km/h) |  |
| Feeder loss (dB) |  |
| Transmitter |  |
| (1) Number of transmit antennas. (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (2) Maximal transmit power per antenna (dBm) |  |
| (3) Total transmit power = function of (1) and (2) (dBm) (The value shall not exceed the indicated value in § 8.4 of Report ITU-R M.2412-0) |  |
| (4) Transmitter antenna gain (dBi) |  |
| (5) Transmitter array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, CDD (cyclic delay diversity), etc.) (dB) |  |
| (6) Control channel power boosting gain (dB) |  |
| (7) Data channel power loss due to pilot/control boosting (dB) |  |
| (8) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for downlink) |  |
| (9a) Control channel EIRP = (3) + (4) + (5) + (6) – (8) dBm |  |
| (9b) Data channel EIRP = (3) + (4) + (5) – (7) – (8) dBm |  |
| Receiver |  |
| (10) Number of receive antennas (The number shall be within the indicated range in § 8.4 of Report ITU-R M.2412-0) |  |
| (11) Receiver antenna gain (dBi) |  |
| (11bis) Receiver array gain (depends on transmitter array configurations and technologies such as adaptive beam forming, etc.) (dB) |  |
| (12) Cable, connector, combiner, body losses, etc. (enumerate sources) (dB) (feeder loss must be included for and only for uplink) |  |
| (13) Receiver noise figure (dB) |  |
| (14) Thermal noise density (dBm/Hz) |  |
| (15a) Receiver interference density for control channel (dBm/Hz) |  |
| (15b) Receiver interference density for data channel (dBm/Hz) |  |
| (16a) Total noise plus interference density for control channel = 10 log (10^(((13) + (14))/10) + 10^((15a)/10)) dBm/Hz |  |
| (16b) Total noise plus interference density for data channel = 10 log (10^(((13) + (14))/10) + 10^((15b)/10)) dBm/Hz |  |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) |  |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) |  |
| (18a) Effective noise power for control channel = (16a) + 10 log((17a)) dBm |  |
| (18b) Effective noise power for data channel = (16b) + 10 log((17b)) dBm |  |
| (19a) Required SNR for the control channel (dB) |  |
| (19b) Required SNR for the data channel (dB) |  |
| (20) Receiver implementation margin (dB) |  |
| (21a) H-ARQ gain for control channel (dB) |  |
| (21b) H-ARQ gain for data channel (dB) |  |
| (22a) Receiver sensitivity for control channel = (18a) ++ (19a) + (20) – (21a) dBm |  |
| (22b) Receiver sensitivity for data channel = (18b) ++ (19b) + (20) – (21b) dBm |  |
| (23a) Hardware link budget for control channel = (9a) + (11) + (11bis) − (22a) dB |  |
| (23b) Hardware link budget for data channel = (9b) + (11) + (11bis) − (22b) dB |  |
| Calculation of available pathloss |  |
| (24) Lognormal shadow fading std deviation (dB) |  |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) |  |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) |  |
| (26) BS selection/macro-diversity gain (dB) |  |
| (27) Penetration margin (dB) |  |
| (28) Other gains (dB) (if any please specify) |  |
| (29a) Available path loss for control channel = (23a) – (25a) + (26) – (27) + (28) – (12) dB |  |
| (29b) Available path loss for data channel = (23b) – (25b) + (26) – (27) + (28) – (12) dB |  |
| Range/coverage efficiency calculation |  |
| (30a) Maximum range for control channel (based on (29a) and according to the system configuration section of the link budget) (m) |  |
| (30b) Maximum range for data channel (based on (29b) and according to the system configuration section of the link budget) (m) |  |

Proposal 3: Reuse the evaluation methodology of link budget in IMT-2020 self-evaluation for Rel-17 NR coverage enhancement.

Although above methodology of IMT-2020 self-evaluation can be reused, values of some parameter needs to be updated because some scenarios, channels and data rates considered in NR coverage enhancement are different from that of IMT-2020 self-evaluation. Hence, following parameters should be revised:

- Frequency carrier, which changes along with scenarios.

- Target data rates with appropriate number of scheduled PRBs and spectrum efficiency.

- Lognormal shadow fading std deviation (dB), which are closely related to scenarios and values are defined in 3GPP TR 38.901 document.

- Margins of shadow fading for control channel and data channel need to be recalculated under different scenarios, such as rural with long distance scenario with TDL-E channel model, which is not considered in IMT-2020 self-evaluation.

- Margins of penetration loss should be re-calculated once the frequency carrier changes.

Considering above mentioned target data rates under different channels, revised values of above parameters are listed in following Table 2.

Table 2: Revised parameters for NR coverage enhancement evaluation

| Modified Parameters in Link Budget Template.xlsx | Urban TDD | Rural NLoS TDD | Rural NLoS FDD | Rural with long distance FDD |
| --- | --- | --- | --- | --- |
| Carrier frequency (GHz) | 4 | 4 | 2 | 0.7 |
| Transmission bit rate for data channel (bit/s) | 1Mbps eMBB 12.2kbps VoIP | 100kbps eMBB; 12.2kbps VoIP | 100kbps eMBB; 12.2kbps VoIP | 100kbps eMBB; 12.2kbps VoIP |
| (17a) Occupied channel bandwidth for control channel (for meeting the requirements of the traffic type) (Hz) | 360000 (1*12*30*1000) | 360000 (1*12*30*1000) | 180000 (1*12*15*1000) | 180000 (1*12*15*1000) |
| (17b) Occupied channel bandwidth for data channel (for meeting the requirements of the traffic type) (Hz) | 10800000 eMBB (30*12*30*1000) 1440000 VoIP (4*12*30*1000) | 1440000 (4*12*30*1000) | 720000 (4*12*15*1000) | 720000 (4*12*15*1000) |
| Spectral efficiency(2) (bit/s/Hz) | 0.27 eMBB | 0.2 eMBB | 0.14 eMBB | 0.14 eMBB |
| (24) Lognormal shadow fading std deviation (dB) | 7(NLoS) | 8 (NLoS) | 8 (NLoS) | 6 (LoS) |
| (25a) Shadow fading margin for control channel (function of the cell area reliability and (24)) (dB) | 7.56 | 10.45 (NLoS O2O) 8.45 (NLoS O2I) | 10.45 (NLoS O2O) 8.45 (NLoS O2I) | 6 |
| (25b) Shadow fading margin for data channel (function of the cell area reliability and (24)) (dB) | 4.48 | 6.61 (NLoS O2O) 5.13 (NLoS O2I) | 6.61 (NLoS O2O) 5.13 (NLoS O2I) | 4.79 |
| (27) Penetration margin (dB) | 26.25 | 9 (NLoS O2O) 14.53 (NLoS O2I) | 9 (NLoS O2O) 12.5 (NLoS O2I) | 9 |

Notes for values of above revised parameters is provided as below:

On the determination of the number of PRB

Taking the eMBB service under urban scenario as an example, 1Mbps UL target data rate, 30 kHz SCS in TDD duplex mode with 7:3 DL-UL slot configuration, the time duration of a frame is 5 millisecond, hence, the minimum transmission block size requirement under 10% BLER for a half-frame could be calculated as:

1×106 bps0.9×5×10-3s≈5555 bits

For UEs at cell edge, a small index MCS is used to ensure the coverage, e.g., QPSK modulation with coding rate 251/1024, and one single symbol DMRS is configured within each slot, thus number of bits transmitted for each UL slot per PRB is

12×14-1×2×2511024≈76.5 bits

Given 7:3 DL-UL slot configuration, the number of bits per PRB in UL transmission in each half-frame is (76.5*3) bits

Hence, the minimum number of scheduled PRBs should be:

555576.5*3≈25,

Thus, 30 PRBs are scheduled to meet the above requirement. Following the similar principle, number of scheduled PRBs towards other services and scenarios are provided as well in Table 2.

On the determination of the number of DMRS symbols

Given 3 km/h UE speed under outdoor to indoor scenario, one DMRS symbol is able to satisfy the requirement for channel estimation. When UE speed reaches 120km/h under outdoor to outdoor scenario, additional DMRS symbol should be configured to combat the large Doppler frequency shifts.

On the determination of margins

According to TS 38.901, we could find the recommended value and calculation formula for the aforementioned parameters such as shadow fading margin, penetration margin, respectively.

Proposal 4: Adopt the revised values in Table 2 to the link budget template for the baseline link budget evaluation of Rel-17 coverage enhancement.

### Evaluation assumptions and metrics

#### Evaluation assumptions and metrics for PUSCH

Considering both eMBB and VoIP services for all scenarios (e.g. urban, rural and rural with long distance), link level evaluation parameters for PUSCH are listed in Table 3.

Table 3: Evaluation assumptions and metrics for PUSCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 4GHz TDD | 4GHz TDD 2GHz FDD | 700MHz FDD |
| Transmission bit rate for data channel (bit/s) | 1 Mbps for eMBB 12.2kbps for VoIP | 100 kbps for eMBB 12.2kbps for VoIP | 100kbps for eMBB 12.2kbps for VoIP |
| Frame structure | DDDSU DDDSUDDSUU DDDDDDDSUU | DDDSU DDDSUDDSUU DDDDDDDSUU | - |
| Subcarrier spacing | 30kHz for TDD | 30kHz for TDD 15kHz for FDD | 15kHz for FDD |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-E |
| Pathloss model (select from LoS or NLoS) | NLos | NLos | Los |
| Delay Spread | 300ns | 300ns | 300ns |
| UE velocity | 3 km/h for indoor | 120km/h for outdoor 3 km/h for indoor | 120 km/h for outdoor |
| Occupied channel bandwidth | eMBB: 30 PRB VoIP: 4PRB | eMBB: 4PRB VoIP: 4PRB | eMBB: 4PRB VoIP: 4PRB |
| Number of BS antennas & antenna configuration (M,N,P,Mg,Ng;Mp,Np) | 192 (12,8,2,1,1;1,1) | 64 (8,4,2,1,1;1,1) | 64 (8,4,2,1,1;1,1) |
| Number of TXRU for BS | 2 | 2 | 2 |
| Number of UE antennas | 2 | 2 | 1 |
| Number of TRXU for UE | 2 | 2 | 1 |
| Overhead assumption for required SNR simulation | DMRS: Type I, one front-loaded DMRS symbol; no multiplexing with data | DMRS: Type I, one front-loaded DMRS (3km/h); one front-loaded DMRS + one add DMRS(120km/h); no multiplexing with data | DMRS: Type I, one front-loaded DMRS + one add DMRS(120km/h); no multiplexing with data |
| Target BLER/rBLER | Target BLER =10% for eMBB service; Target rBLER=2% for VoIP service |  |  |

#### Evaluation assumptions and metrics for PUCCH

Link level evaluation parameters for PUCCH are listed in Table 4.

Table 4: Evaluation assumptions and metrics for PUCCH

| Parameters | Values |  |  |
| --- | --- | --- | --- |
| Scenario | Urban | Rural | Rural with long distance |
| Duplexing scheme and frequency | 4GHz TDD 2GHz FDD | 4GHz TDD/ 2GHz FDD | 700MHz FDD |
| Format type | Format 1 (long PUCCH with 14 OFDM symbols), 2bits | Format 1 (long PUCCH with 14 OFDM symbols), 2bits | Format 1 (long PUCCH with 14 OFDM symbols), 2bits |
| Subcarrier spacing | 30kHz for TDD | 30kHz for TDD 15kHz for FDD | 15kHz for FDD |
| Channel model for link-level simulation | TDL-C | TDL-C | TDL-E |
| Delay Spread | 300ns | 300ns | 300ns |
| UE velocity | 3 km/h for indoor | 120km/h for outdoor 3 km/h for indoor | 120 km/h for outdoor |
| Number of BS antennas & antenna configuration (M,N,P,Mg,Ng;Mp,Np) | 192 (12,8,2,1,1;1,1) | 64 (8,4,2,1,1;1,1) | 64 (8,4,2,1,1;1,1) |
| Number of TXRU for BS | 2 | 2 | 2 |
| Number of UE antennas | 2 | 2 | 1 |
| Number of TRXU for UE | 2 | 2 | 1 |
| Target BLER | BLER=1% |  |  |

#### Evaluation assumptions and metrics for others channels

As mentioned above, coverage of other channels such as PDCCH, PDSCH and Msg3, etc., should also be studied. Related key evaluation parameter settings are listed in Table 5.

Table 5: Evaluation assumptions and metrics for other channels

| Other channels | Msg3 | PDCCH | PDSCH |
| --- | --- | --- | --- |
| Key parameters | MCS: 0 (2,120/1024) Scheduled PRB: 2 Payload size: 56 bit Channel model: TDL-C Moving speed: - 3km/h & 120km/h DMRS overhead: - 1 front-loaded DMRS - 1 front loaded DMRS + 1 additional DMRS | Aggregation level: 8 DCI payload size: 60 bit CRC length: 24 bit CORRESET PRB: 48 CORRESET symbols: 2 Channel model: TDL-C Moving speed: - 3km/h & 120km/h DMRS overhead: - 1 front-loaded DMRS - 1 front loaded DMRS + 1 additional DMRS | MCS: 2 (2,251/1024) Scheduled PRB: - Calculated Channel model: TDL-C Moving speed: - 3km/h & 120km/h DMRS overhead: - 1 front-loaded DMRS - 1 front loaded DMRS + 1 additional DMRS Target data rate: - 10Mbps for urban - 1Mbps for rural |
| Target BLER | 10% | 1% | 10% |

Additional notes on above evaluation assumptions and metrics:

- MCS selection:

Low MCS levels are selected to ensure the coverage, especially for uplink transmission, such as MCS 0 for Msg3 and MCS 2 for PDSCH.

- Determination of number of scheduled PRB:

Following the determination method of number of PRB in section 2.2 and considering target data rates of 10 Mbps for urban scenario and 1Mbps for other scenarios with MCS 2, the number of scheduled PRBs for PDSCH are listed as follows:

| Parameters | Urban TDD | Rural(O2I) FDD | Rural(O2O) FDD | Rural(O2I) TDD | Rural(O2O) TDD | Rural long distance FDD |
| --- | --- | --- | --- | --- | --- | --- |
| Num of PRB | 154 | 30 | 32 | 16 | 16 | 32 |
| TBS | 5640 | 1128 | 1128 | 608 | 552 | 1128 |

- Moving speed and DMRS overhead:

A moving speed of 3 km/h is assumed for NLoS O2I scenario with indoor UEs while a moving speed of 120km/h is assumed for NLoS O2O scenario with outdoor UE in a car.

Under the low moving speed of being 3km/h, 1 front-loaded single-symbol DMRS is enough to estimate the fading channel; under the high moving speed of being 120km/h, 1 front-loaded and 1 additional single-symbol DMRSs are used to ensure the channel estimation accuracy with large Doppler frequency shifts.

- DCI payload size of PDCCH:

A small payload size of PDCCH being 60 bits is used as a reference since usually a smaller payload size of PDCCH can achieve a better coverage.

Note: Above key evaluation parameters for other channels will be used under all mentioned scenarios (e.g. urban, rural and rural with long distance scenarios) for both TDD and FDD frame structures.

## On factors which limit the coverage

According to above evaluation assumptions and metrics, our discussions on NR coverage enhancement are presented in [3, 4]. Observations show that coverage of some channels are limited, especially for the uplink transmission (i.e., PUSCH). Following, we analyze the key factors which limit coverage before we could come out with potential solutions.

### Factor 1: Accuracy of channel estimation

In NR, DMRS is configured within each slot, which consists of front-loaded DMRS and additional DMRS, maximum 4 symbols can be configured as DMRS according to the channel coherence time. Usually, more DMRS should be configured for fading channels with short channel coherence time and large Doppler frequency shifts to ensure an accurate channel estimation.

### Factor 2: Overhead of reference signal

As mentioned above, necessary reference signals such as DMRS are configured for each slot to ensure accurate channel estimation and good system performance. However, REs occupied by reference signals would not be used to transmit data, which would degrade the spectrum efficiency. Thus, a proper reference signal overhead should be considered.

Figure 1: DMRS configuration in each slot: dmrs-type1, maxLength=single,

dmrs-additionalPosition=Pos1

### Factor 3: Inaccurate scheduling

According to uplink channel measurements on DMRS or SRS, BS determines the appropriate scheduling parameters for either DL or UL transmission. Once a data packet fails, HARQ retransmission can be performed. Usually, there will be gaps between practical scheduling and best scheduling in theory due to measurement inaccuracy and finite MCS levels (finite coding rate values), etc., which would cause resource waste. As shown in Figure 2, for example, to decode current TBS, there is a minimum number of coded bits need to be transmitted (obtained at RX) and 4 HARQ transmissions are scheduled until the TBS is correctly decoded. Due to the inaccurate scheduling, extra wasted resources (in blue color) are transmitted and the coverage  would be degraded.

Figure 2: HARQ retransmission to decode TBS

### Factor 4: Maximum UE transmission power

UE power class: To enable proper scheduling, UE will cooperate with BS to report UE capability, including UE power class which defines the maximum output power for any uplink transmission within the bandwidth of NR carrier. Typically, there are 3 UE power classes to match the UE hardware ability, namely power class 1 with 31 dBm (used in TDD and CPE), power class 2 with 26 dBm and power class 3 with 23 dBm. Most UEs maintain power class 3 with very limited uplink transmission power because of RF hardware with low cost, however, this results in limited maximum uplink coverage.

Besides, SAR (Specific Absorption Ratio) limitation should also be considered to avoid large signal radiation harm to human body, such as the average transmit power within a long period should be no more than 23dBm.

### Factor 5: TDD UL-DL configuration

Compared to FDD frame structure, a small UL transmission portion in TDD mode is another key limitation for uplink coverage, such as 4:1, 8:2, 7:3 of DL:UL TDD configurations.

### Factor 6: UL transmission frequency band

Signals at higher frequency bands will experience larger pathloss and penetration loss, thus received signals with very low powers would degrade the channel estimation and demodulation performance.

### Other factors

Other factors which put limitations on cell coverage can also be studied, such as maximum power reduction, network interference, limited UE antennas, etc.

# Conclusion

Proposal 1: For FR1, coverage of PUSCH and PUCCH need to be studied with a higher priority.

- For PUSCH at FR1: the target data rate for eMBB service is 1Mbps, 100kbps and 100kbps for urban (ISD=500m), rural (ISD=1732 m) and rural with long distance (ISD>10 km) scenarios, respectively.

- For PUCCH: coverage of PUCCH format 1 should be studied as a reference to judge whether PUCCH needs coverage enhancement.

Proposal 2: Coverage of other channels needed to be studied include:

- For FR1, coverage of Msg3, PDSCH and PDCCH can also be studied to determine which channel needs coverage enhancement;

- For FR2, the coverage of Msg3, PDSCH, PDCCH, SS/PBCH and PRACH can also be studied to determine which channel needs coverage enhancement.

Proposal 3: Reuse the evaluation methodology of link budget in IMT-2020 self-evaluation for Rel-17 NR coverage enhancement.

Proposal 4: Adopt the revised value in Table 2 to the link budget template in table 1 for the baseline link budget evaluation of Rel-17 coverage enhancement.

# References

- RP-193240, “New SID on NR coverage enhancement”, RAN#86, Sitges, Spain, December 9 – 12, 2019

- RP-191886, “Summary of email discussion on NR coverage enhancement”, China Telecom, Sep, 2019

- R1-2003298, “Baseline coverage performance for FR1”, RAN1#101-e, E-meeting, 25 May – 5 June, 2020

- R1-2003299, “Discussion on baseline coverage evaluation for FR2”, RAN1#101-e, E-meeting, 25 May – 5 June, 2020