3GPP TSG RAN WG1 Meeting #101-e	R1-2004164

E-meeting, May 25 – June 5, 2020

Agenda Item:	6.2.2.1

Source:	Huawei, HiSilicon

Title:	Corrections on UE-group wake-up signal

Document for:	Discussion and Decision

# Introduction

In NB-IoT Rel-16, the agreements related to UE-group wake-up signal are summarized in [1]. This paper presents text proposals for TS 36.211.

# Discussion

Agreement If the group WUS resource is configured to be shared by Rel-15 WUS and Rel-16 WUS, the common WUS sequence for all the group WUS UEs in the same WUS resource can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence. Agreement For common WUS, g = 126 unless in a shared resource and common WUS is configured to be legacy WUS in which case g = 0.

As shown in the agreements above, the common WUS sequence can be configured to be the Rel-15 WUS sequence or a Rel-16 WUS sequence.

The corresponding RRC signaling in the running CR of TS 36.331 [2] is as follows:

… (Copied from the running CR of TS 36.331 [2]) … GWUS-Config-NB information element … GWUS-Config-NB-r16 ::= SEQUENCE { … commonSequence-r16 ENUMERATED {g0, g126} OPTIONAL, -- Need OR … GWUS-Config-NB field descriptions. commonSequence Presence of the field indicates common WUS sequence is configured. Value g0 indicates common WUS sequence for the shared WUS resource is g=0, value g126 indicates common WUS sequence for the shared WUS resource is g=126, see TS 36.211[21].. …. …

The related description in the latest TS 36.211 is as follows:

… (Copied from TS 36.211) … #### 10.2.6B.1 Sequence generation … The common NWUS sequence shall be determined by g=126 unless the resource is shared with non-group NWUS and common NWUS is configured to be non-group NWUS in which case g=0. …

As shown above, the terminology is not aligned, i.e., TS 36.211 uses “non-group NWUS”, and TS 36.331 uses “g0”.

And it seems the field description in TS 36.331, i.e., “value g126 indicates common WUS sequence for the shared WUS resource is g=126” is duplicated with TS 36.211. According to current TS 36.211, if the configured value is not “g0”, the common WUS sequence will be g=126.

To solve the issues above, and to avoid referencing HL parameter name directly, the following proposal and TP are given.

Proposal 1: Agree on the TP in Section 2 and capture its contents in TS 36.211.

------------------------------------- Start of Text Proposal for TS 36.211------------------------------- ------------------------------------------- Unchanged parts omitted -------------------------------------- #### 10.2.6B.1 Sequence generation … where M is the actual duration of NWUS as defined in [4]. For a UE not configured with group NWUS, g=0. For a UE configured with group NWUS, g=14NgroupWUS+1 for 0≤NgroupWUS≤7, where NgroupWUS is determined by the UE group to which the UE is associated as determined by higher layers [10]. In resource that is not shared with non-group NWUS, the common NWUS sequence shall be determined by g=126. In resource that is shared with non-group NWUS, the common NWUS sequence is determined by higher layers [9].g=0 -------------------------------------------- Unchanged parts omitted ------------------------------------- ---------------------------------------------- End of Text Proposal ----------------------------------------

# Conclusion

The contribution discusses the corrections on UE-group wake-up signal. The proposal based on the analysis in the contribution is as following:

Proposal 1: Agree on the TP in Section 2 and capture its contents in TS 36.211.

# References

- R1-1913595, “RAN1 agreements for Rel-16 Additional Enhancements for NB-IoT”, Futurewei, RAN1#99, Reno, USA, 18th – 22nd November 2019.

- R2-2004040, “Miscellaneous corrections to 36.331 for Rel-16 NB-IoT”, Huawei, HiSilicon, RAN2#109bis-e, Online, 20th - 30th April 2020