3GPP TSG RAN WG1 #101-e	                         			 R1-2004307

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.2.7.2

Source:	InterDigital, Inc.

Title:	Remaining issues of cross-slot scheduling for UE power saving

Document for:	Discussion and Decision

# Introduction

Several remaining details regarding cross-slot scheduling were discussed in RAN1 #100b-e [1]. One of the agreements was the following:

Agreements:

- The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with MsgB-RNTI.

- When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with SP-CSI-RNTI with K2 smaller than the applicable minimum scheduling offset restriction K2min.

- For an active BWP with scheduling offset restriction(s) configured and when UE detects an invalid TDRA entry violating current applied K0min/K2min from DCI format 1_0/0_0, one of the following is decided in RAN1 #101-e meeting:

  - Alt 1: Additional RAN1 specification is defined for handling such error case

    - Solution to be converged from companies’ proposals to RAN1 #101-e

  - Alt 2: No additional RAN1 specification is defined for handling such error case

- Aperiodic CSI-RS triggering offset value range is extended from {0, 1, 2, 3, 4, 16, 24} to {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24}

- (Working assumption) The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI or MCS-C-RNTI in the search space set provided by recoverySearchSpac Id when monitoring PDCCH as described in Section 6 [38.213].

As can be seen from the agreement, handling an invalid TDRA entry violating current applied K0min/K2min from DCI format 1_0/0_0 was left to be decided in this meeting. In this contribution, we present our view on this remaining issue.

# Discussion

## Misdetection identification and correction

According to the above agreement, if the time domain resource allocation field in the scheduling DCI points to an entry of the TDRA table with K0 < K0min and K2 < K2min, two alternatives may be considered:

- Alt 1: Additional RAN1 specification is defined for handling such error case

- Alt 2: No additional RAN1 specification is defined for handling such error case

In Alt 1, the misalignment between the gNB and the UE on the current value of the minimum scheduling offset can be resolved by the UE by falling back to a predetermined default value. This default value could be the minimum k0/k2 values contained in the TDRA table or the configured minimum scheduling offset with the lowest index.

If not corrected, the misalignment could result in significant data loss. In some cases, the error can be corrected in the future slots based on the HARQ feedback from the UE; however, this would result in throughput loss until the error is corrected. In some other cases, correction may not be possible at all or it may take longer for the correction. Such a case may occur, for example, if DCI without the indication field (i.e., format 0_0, 1_0) is used in the future slots. Therefore, it is desirable to specify the UE behavior so that it can correct the misalignment as soon as it is detected.

We do not think that the UE behavior can be left to UE implementation. According to 38.213, “If a UE detects a DCI format with inconsistent information, the UE discards all the information in the DCI format”, and therefore not transmit HARQ feedback. So, even if the error may eventually be corrected, this would come at the expense of a possibly significant loss in throughput.

In addition to the above, in another scenario, the UE may also receive in the scheduling DCI an aperiodic CSI-RS triggering offset that is smaller than the applicable K0min, indicating to the UE that a misdetection has occurred.

Based on this discussion, we propose that this error case be corrected with a simple UE behavior:

Proposal 1: When the UE is scheduled with a DCI to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than the applicable minimum scheduling offset restriction K0min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

Proposal 2: When the UE is scheduled with a DCI to transmit a PUSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K2 smaller than the applicable minimum scheduling offset restriction K2min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

Proposal 3: When the UE is triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

# Conclusion

In this contribution, we have discussed the misdetection of minimum scheduling offset indication and provided the following proposals to handle this issue. The corresponding text proposals are also provided in the following.

Proposal 1: When the UE is scheduled with a DCI to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than the applicable minimum scheduling offset restriction K0min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

Proposal 2: When the UE is scheduled with a DCI to transmit a PUSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K2 smaller than the applicable minimum scheduling offset restriction K2min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

Proposal 3: When the UE is triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

Text Proposals (38.214):

#### 5.1.2.1	Resource allocation in time domain

…

When the UE configured with [minimumSchedulingOffset] in an active DL BWP it applies a minimum scheduling offset restriction indicated by the ['Minimum applicable scheduling offset indicator'] field in DCI format 0_1 or 1_1. When the UE configured with [minimumSchedulingOffset] in active DL BWP and it has not received ['Minimum applicable scheduling offset indicator'] field in DCI format 0_1 or 1_1, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than the applicable minimum scheduling offset restriction K0min. When the UE is scheduled with a DCI to receive a PDSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K0 smaller than the applicable minimum scheduling offset restriction K0min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'. The minimum scheduling offset restriction is not applied when PDSCH transmission is scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI in common search space associated with CORESET0 and default PDSCH time domain resource allocation is used or when PDSCH transmission is scheduled with SI-RNTI or RA-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

#### 6.1.2.1	Resource allocation in time domain

…

When the UE configured with [minimumSchedulingOffset] in active UL BWP it applies a minimum scheduling offset restriction indicated by the ['Minimum applicable scheduling offset indicator'] field in DCI format 0_1 or 1_1. When the UE configured with [minimumSchedulingOffset] in active UL BWP and it has not received ['Minimum applicable scheduling offset indicator'] field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K2 smaller than the applicable minimum scheduling offset restriction K2min in slot n. When the UE is scheduled with a DCI to transmit a PUSCH scheduled with C-RNTI, CS-RNTI or MCS-C-RNTI with K2 smaller than the applicable minimum scheduling offset restriction K2min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'. The minimum scheduling restriction is not applied when PUSCH transmission is scheduled by RAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1.

#### 5.2.1.5	Triggering/activation of CSI Reports and CSI-RS

…

The UE does not expect that aperiodic CSI-RS is transmitted before the OFDM symbol(s) carrying its triggering DCI. When the minimum scheduling offset restriction is applied, UE is not expected to be triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min. When the UE is triggered by CSI triggering state indicated by the CSI request field in DCI in which CSI-RS triggering offset is smaller than the currently applicable minimum scheduling offset restriction K0min, UE shall apply a minimum scheduling offset restriction indicated based on ['Minimum applicable scheduling offset indicator'] value '0'.

# References

[1] Chairman’s Notes, 3GPP WG1 #100b-e