3GPP TSG RAN WG1 #101			R1-2004403

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (NTT DOCOMO, INC.)

Title:	Summary on UE features for NR-U

Agenda Item:	7.2.11.2

Document for: 	Discussion and Decision

- Introduction

This contribution summarizes the discussions and proposals in AI 7.2.11.2 regarding UE features for NR-U.

Based on the discussions summarized in Section 2, following is the suggested list of email discussions/approvals for AI 7.2.11.2.

FL proposal of list of email discussion/approval:

[101-e-NR-UEFeatures-NRU-01] Email discussion/approval on feature group structure for NR-U (25th – 29th May)

- Discuss and decide whether FG10-2a (SSB-based RRM [for semi-static channel access mode]) is kept or merged to FG10-2 (SSB-based RRM [for dynamic channel access mode])

- Discuss and decide whether FG10-2d (SSB-based RLM [for semi-static channel access mode]) is kept or merged to FG10-2c (SSB-based RLM [for dynamic channel access mode])

- Discuss and decide whether FG10-19 (Number of LBT bandwidth) is kept or removed

- Discuss and decide whether FG10-19a ([Support DL reception in a carrier with intra-cell guard-bands]) is kept or removed

- Discuss and decide whether FG10-19b ([Support UL transmission with subset of RB sets passing LBT]) is kept or removed

- Discuss and decide whether FG10-21b (Support UL to DL COT sharing) is kept or removed

- Discuss and decide whether FG10-31 ([Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0]) is kept or removed

- Discuss and decide whether any additional FG(s) related to SSB-based BFD and CBD with Q is added or not

- Discuss and decide whether any additional FG(s) related to intra-cell guard band length smaller than the default intra-cell guard band length defined in RAN4 is added or not

- Discuss and decide capability signaling design for FG(s) decided to be kept/added in this email discussion (if any)

[101-e-NR-UEFeatures-NRU-02] Email discussion/approval on capability signaling design for existing FGs for NR-U (25th May – 2nd June)

- Discuss and decide capability signaling design (including components, candidate values, reporting type, xDD/FRx differentiations) for existing FGs

- Discuss and decide any other necessary update for the UE features list for NR-U based on identified issues/proposals in R1-2004403

[101-e-NR-UEFeatures-NRU-03] Email discussion/approval on basic FGs for each NR-U operation/scenario (29th May – 4th June)

- Discuss and down-select from Alt.1 or Alt.2 based on the working assumption

- Discuss and decide basic FGs for each NR-U operation/scenario based on finalized FG structure for NR-U

Companies are encouraged to check above FL proposals and to provide feedback if any in below.

| Company | Comment |
| --- | --- |
| Nokia, NSB | In general the scope is fine, but we are surprised to see FG10-19 being discussed again. At least it should be clear that the discussion it is not about keeping or removing it, it is about reverting a previous agreement to introduce it, and that has a much higher bar to clear. This should not be prioritized over other topics where agreements are not present yet, unless there is a serious issue identified with the existing agreement, but it does not seem to be the case. |
| Moderator (NTT DOCOMO) | Thank you very much for feedback. For FG10-19, as replied to LG’s comment on reflector, I will remove the bullet unless the proponent provides clear justification to revert the agreement. Or at least it will be clarified in the bullet that it is already agreed to be removed. |

1.1	Updated FL proposal

Based on the preparation phase email discussion, the moderator recommends following email discussion/approval for UE features for NR-U in AI 7.2.11.2.

[101-e-NR-UEFeatures-NRU-01] Email discussion/approval on feature group structure for NR-U (25th – 29th May)

- Discuss and decide whether FG10-2a (SSB-based RRM [for semi-static channel access mode]) is kept or merged to FG10-2 (SSB-based RRM [for dynamic channel access mode])

- Discuss and decide whether FG10-2d (SSB-based RLM [for semi-static channel access mode]) is kept or merged to FG10-2c (SSB-based RLM [for dynamic channel access mode])

- Discuss and decide whether FG10-19a ([Support DL reception in a carrier with intra-cell guard-bands]) is kept or removed

- Discuss and decide whether FG10-19b ([Support UL transmission with subset of RB sets passing LBT]) is kept or removed

- Discuss and decide whether FG10-21b (Support UL to DL COT sharing) is kept or removed

- Discuss and decide whether FG10-31 ([Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0]) is kept or removed

- Discuss and decide whether any additional FG(s) related to SSB-based BFD and CBD with Q is added or not based on proposals identified in R1-2004403

- Discuss and decide whether any additional FG(s) related to intra-cell guard band length smaller than the default intra-cell guard band length defined in RAN4 is added or not based on proposals identified in R1-2004403

- Discuss and decide capability signaling design for FG(s) decided to be kept/added in this email discussion (if any)

[101-e-NR-UEFeatures-NRU-02] Email discussion/approval on capability signaling design for existing FGs for NR-U (25th May – 2nd June)

- Discuss and decide capability signaling design (including components, candidate values, reporting type, xDD/FRx differentiations) for existing FGs

- Discuss and decide any other necessary update for the UE features list for NR-U based on identified issues/proposals in R1-2004403

[101-e-NR-UEFeatures-NRU-03] Email discussion/approval on basic FGs for each NR-U operation/scenario (29th May – 4th June)

- Discuss and down-select from Alt.1 or Alt.2 based on the working assumption

- Discuss and decide basic FGs for each NR-U operation/scenario based on finalized FG structure for NR-U

- Discussion on UE features for NR-U

2.1	FG10-1/1a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type ( 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-1 | UL channel access for dynamic channel access mode | 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission |  | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-1a | UL channel access for semi-static channel access mode | 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- Component of FG10-1

  - Include contention window size adjustment under the first component (i.e., Type 1 channel access) or as a separate component: [6]

- Component of FG10-1a

  - Add “FFS: CP extension up to 1 symbol for PUSCH/PUCCH transmission” as one of its components: [4]

- Prerequisite feature groups for FG10-1a

  - No prerequisite feature groups: [12]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [4] | For FG10-1a, since Type 2C is one of its components, CP extension should be, too. Proposal 3: In FG10-1a, add “FFS: CP extension up to 1 symbol for PUSCH/PUCCH transmission” as one of its components. |
| --- | --- |
| [6] | For FG 10-1, it is generic FG for dynamic channel access including various types of channel access mode. For Type 1 channel access, contention window size adjustment procedure should be required. However, it is not clearly stated in FG 10-1. During e-mail discussion, it was proposed to include contention window size adjustment procedure as a component in FG 10-1. Although Type 1 channel access in the component includes contention window size adjustment procedure, in our view, explicit description to avoid miss-leading of FG 10-1 is desirable. Therefore, it is proposed to include contention window size adjustment under the first component (i.e., Type 1 channel access) or as a separate component in FG 10-1. Proposal 1: Include contention window size adjustment under the first component (i.e., Type 1 channel access) or as a separate component in FG 10-1 |
| [12] | 10. NR-unlicensed; 10-1; UL channel access for dynamic channel access mode; 1. Type 1 channel access 2. Type 2A channel access 3. Type 2B channel access 4. Type 2C channel access 5. 20MHz LBT bandwidth 6. CP extension up to 1 symbol for PUSCH/PUCCH transmission; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-1a; UL channel access for semi-static channel access mode; 1. Type 2C channel access 2. Single sensing slot of 9us channel access 3. 20MHz LBT bandwidth; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. |

2.2	FG10-2/2a/2b/2c/2d/2e/2f

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-2 | SSB-based RRM [for dynamic channel access mode] | 1. SSB-based RRM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2a | SSB-based RRM [for semi-static channel access mode] | 1. SSB-based RRM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2b | MIB reading on unlicensed cell | 1. MIB reading on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2c | SSB-based RLM [for dynamic channel access mode] | 1. SSB-based RLM with Q [for dynamic channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2d | SSB-based RLM [for semi-static channel access mode] | 1. SSB-based RLM with Q [for semi-static channel access mode] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Q indicates the value of RAN1 parameter NSSBQCL | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2e | SIB1 reception on unlicensed cell | 1. SIB1 reception on unlicensed cell | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-2f | Support monitoring of extended RAR window | 1. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- Necessity of FG10-2a

  - FG is removed and merged to FG10-2: [4], [5], [11]

  - FG is kept: [7], [9], [12], [13]

- Necessity of FG10-2d

  - FG is removed and merged to FG10-2c: [4], [5]

  - FG is kept: [7], [9], [11], [12], [13]

- Component of FG10-2a

  - Modify the component of FG 10-2a from “SSB-based RRM with Q [for semi-static channel access mode]” to “SSB-based RRM with Q for semi-static channel access mode, when SMTC window is no longer than the fixed frame period”: [12]

- Component of FG10-2b

  - Modify the component of FG 10-2b from “MIB reading on unlicensed cell” to “MIB reading on unlicensed cell for PCell and PSCell”: [10]

- Component of FG10-2d

  - Modify the component of FG 10-2a from “SSB-based RLM with Q [for semi-static channel access mode]” to “SSB-based RLM with Q for semi-static channel access mode, when DRS window is no longer than the fixed frame period”: [12]

- Component of FG10-2e

  - Modify the component of FG 10-2e from “SIB1 reception on unlicensed cell” to “SIB1 reception on unlicensed cell for PCell”: [10]

- Component of FG10-2f

  - Remove the brackets of [40ms]: [9], [12], [13]

- Others

  - Discuss how to handle SSB-based BFD and CBD capabilities support by considering two alternatives below: [10]

    - Alt.1: Adding SSB-based BFD/CBD with Q as additional components in FG 10-2c/2d.

      - FG-10-2c: SSB-based RLM/BFD/CBD with Q [for dynamic channel access mode]

      - FG-10-2d: SSB-based RLM/BFD/CBD with Q [for semi-static channel access mode]

    - Alt.2: Create four new FGs

      - FG 10-2g: SSB-based BFD with Q [for dynamic channel access mode]

      - FG 10-2h: SSB-based BFD with Q [for semi-static channel access mode]

      - FG 10-2I: SSB-based CBD with Q [for dynamic channel access mode]

      - FG 10-2J: SSB-based CBD with Q [for semi-static channel access mode]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [4] | For downlink features, there is no need to distinguish FBE from LBE. Hence, FGs 10-2a and 10-2d are not necessary. Proposal 4: Remove FG10-2a and FG10-2d. |
| --- | --- |
| [5] | It does not seem necessary to have separate FGs defined for dynamic and semi-static channel access mode for RLM and RRM. It seems like 2/2a can be merged and 2c/2d can be merged. It can be further discussed if there is a need to introduce a range of values for various configurable parameters to allow a UE to report its capability. Proposal 2 FG 2a/2b can be merged. FG 2c/2d can be merged. FG 10-2f “Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0” in our view, it is not critical if NR-U capable UEs do not support the extended RAR window. Collisions do not happen frequently, and if there is a collision, a UE can retry to access the channel again. It is true that the gNB does not know the UE’s capability if the RACH procedure is initiated by the IDLE/INACTIVE UE. However, if support of extended RAR is a separate FG with its own an capability bit, this can be used to collect statistics on UE capabilities, and the gNB may decide based on the penetration and use case whether to configure the extended RAR window or not. If considered useful, this can be implemented in the initial phase. Otherwise, UEs may also be upgraded with this capability if enhancements are considered needed. Proposal 3 FG 10-2f for support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0 does not need to be part of basic operation. |
| [7] | In RAN1#100-e meeting, the following conclusion was made. Conclusion: For semi-static channel access, SSBs that (partially) fall in the idle region of a fixed frame period should be considered as invalid. No PDSCH rate matching and no RLM/RRM measurement will be done for those candidate SSB positions.. The above conclusion implies that, for FBE case, if location of a candidate SS/PBCH block is (partially) overlapped with idle region of a fixed frame period, UE shall not perform RRM/RLM/BFD/CBD operation for the SS/PBCH block. Therefore, based on this observation, we can remove brackets of FG 10-2/2a/2c/2d. Proposal #5: Keep FG 10-2/2a/2c/2d separate feature groups as is, and remove brackets for FG 10-2/2a/2c/2d. |
| [9] | 10-2; SSB-based RRM [for dynamic channel access mode]; 1. SSB-based RRM with Q [for dynamic channel access mode]. 10-2a; SSB-based RRM [for semi-static channel access mode]; 1. SSB-based RRM with Q [for semi-static channel access mode]. Proposal: keep the two FGs separate. There is no reason that a FBE UE should have to support measurements for LBE, and vice-versa. 10-2c; SSB-based RLM [for dynamic channel access mode]; 1. SSB-based RLM with Q [for dynamic channel access mode]. 10-2d; SSB-based RLM [for semi-static channel access mode]; 1. SSB-based RLM with Q [for semi-static channel access mode]. Proposal: keep the two FGs separate. There is no reason that a FBE UE should have to support measurements for LBE, and vice-versa. 10-2f; Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0; 1. Support of RAR extension from 10ms to [40ms] by decoding of the 2-bit SFN indication in DCI 1_0. Proposal: remove the brackets. |
| [10] | On FG 10-2b and 10-2e We would like to add “PCell and PSCell” to the FG10-2b to scope the intended use case of this FG. For PSCell, reading MIB is still needed to acquire the SFN timing of the SCG (which maybe different from the MCG). This modification is important to provide desirable flexibility so that UE that only supports NR-U deployment scenario A together with FG 10-23 (reportCGI for neighbour) does not necessarily support this feature. In other words, this modification gets rid of the overlapping part (i.e. MIB/SIB1 decoding) between FG 10-2b (PCell/PSCell) and FG 10-23 (neighbour cells). 10-2b; MIB reading on unlicensed cell; 1. MIB reading on unlicensed cell for PCell and PSCell; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. For SIB-1 reading i.e. FG 10-2e, we propose to update the component to include “for PCell”. It should be noted that, according to TS 38.331 section 5.2.1, SIB-1 information for SCell, including PSCell, is provided by dedicated RRC signaling in SCell addition procedure or SCG addition procedure and therefore no need to read SIB1 for any SCell. 10-2e; SIB1 reception on unlicensed cell; 1. SIB1 reception on unlicensed cell for PCell; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. Proposal 2: - Modify the component of FG 10-2b to be “MIB reading on unlicensed cell for PCell and PSCell” - Modify the component of FG 10-2e to be “SIB1 reception on unlicensed cell for PCell” On FG 10-2c/10-2d 10-2c; SSB-based RLM [for dynamic channel access mode]; 1. SSB-based RLM with Q [for dynamic channel access mode]; Q indicates the value of RAN1 parameter NSSBQCL; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10-2d; SSB-based RLM [for semi-static channel access mode]; 1. SSB-based RLM with Q [for semi-static channel access mode]; Q indicates the value of RAN1 parameter NSSBQCL; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. As shown in Table above, the current FG 10-2c/10-2d only includes SSB-based RLM functionality. RAN1 needs to discuss how to handle other SSB-based functionalities, e.g. BFD (Beam Failure Detection) and CBD (Candidate Beam Detection) for NR-U operation, i.e. either adding them into 10-2c and 10-2d as components (Alt.1) or creating new FGs e.g. FG 10-2g/2h/2I/2J (Alt.2) Proposal 3: - RAN1 to discuss how to handle SSB-based BFD and CBD capabilities support by considering two alternatives below: - Alt.1: Adding SSB-based BFD/CBD with Q as additional components in FG 10-2c/2d. - FG-10-2c: SSB-based RLM/BFD/CBD with Q [for dynamic channel access mode] - FG-10-2d: SSB-based RLM/BFD/CBD with Q [for semi-static channel access mode] - Alt.2: Create four new FGs - FG 10-2g: SSB-based BFD with Q [for dynamic channel access mode] - FG 10-2h: SSB-based BFD with Q [for semi-static channel access mode] - FG 10-2I: SSB-based CBD with Q [for dynamic channel access mode] - FG 10-2J: SSB-based CBD with Q [for semi-static channel access mode] |
| [11] | - FG 10-2: SSB-based RRM [for dynamic channel access mode] - As commented to 10-2a, we don’t see the necessity to separate FGs for LBE and FBE. 10-2 and 10-2a should be combined into a single FG. - FG 10-2a: SSB-based RRM [for semi-static channel access mode] - Considering RRM for measurement cell, FFP of the measurement cell may not be always the same FFP of the serving cell. In that case, UE may perform several RRM for the measurement cell and has to support SSB-based RRM with an arbitrary FFP. In that sense, we don’t see the necessity to separate FGs for LBE and FBE. 10-2 and 10-2a should be combined into a single FG. As there was a discussion in the last RAN1 meeting that measurement cells would have the same FFP as the serving cells in some cases since they are deployed by the same operator, we would like to know whether this assumption can be the baseline to define the FG. - FG 10-2c: SSB-based RLM with Q [for dynamic channel access mode] - As commented to 10-2d, we are OK to separate SSB-based RLM for LBE and FBE. - FG 10-2d: SSB-based RLM with Q [for semi-static channel access mode] - We think the motivation to separate the capability of SSB-based RLM for LBE and that for FBE comes from reducing the number of RLM within DRS half slot for FFP of 5/10ms (e.g. UE only perform RLM for candidate SSB index from #0 to #7 within DRS half slot for FFP of 5ms). In that sense, we are OK to separate SSB-based RLM for LBE and FBE. |
| [12] | - We do prefer to keep 10-2 and 10-2a separate, and keep 10-2c and 10-2d separate, i.e., remove square brackets, consider the SSB transmission patterns are different between LBE and FBE systems at least during the idle period. - As discussed in the previous round of email discussion, for 10-2a and 10-2c, we would like to only keep SMTC window or DRS window length shorter or equal to the fixed frame period as the component, consider the UE behavior can be different if the SSB burst can be spreaded across multiple FFPs. We can introduce separate capability for SMTC window or DRS window longer than FFP if necessary. 10. NR-unlicensed; 10-2; SSB-based RRM for dynamic channel access mode; 1. SSB-based RRM with Q for dynamic channel access mode; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Q indicates the value of RAN1 parameter NSSBQCL; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2a; SSB-based RRM for semi-static channel access mode; 1. SSB-based RRM with Q for semi-static channel access mode, when SMTC window is no longer than the fixed frame period; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Q indicates the value of RAN1 parameter NSSBQCL; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2b; MIB reading on unlicensed cell; 1. MIB reading on unlicensed cell; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2c; SSB-based RLM for dynamic channel access mode; 1. SSB-based RLM with Q for dynamic channel access mode; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Q indicates the value of RAN1 parameter NSSBQCL; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2d; SSB-based RLM for semi-static channel access mode; 1. SSB-based RLM with Q for semi-static channel access mode, when DRS window is no longer than the fixed frame period; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Q indicates the value of RAN1 parameter NSSBQCL; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2e; SIB1 reception on unlicensed cell; 1. SIB1 reception on unlicensed cell; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10. NR-unlicensed; 10-2f; Support monitoring of extended RAR window; 1. Support of RAR extension from 10ms to 40ms by decoding of the 2-bit SFN indication in DCI 1_0; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. |
| [13] | - 10-2/2a/2c/2d: it is OK to keep the differentiation depending on dynamic/static access modes. - 10-2f: RAN2 has agreed already that 40ms is mandatory. |

2.3	FG10-7

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-7 | UL channel access for 10 MHz SCell | - 10 MHz LBT bandwidth | one of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- No remaining issues or proposals are identified for FG10-7

2.4	FG10-10

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-10 | RSSI and channel occupancy measurement and reporting | - RSSI measurement - Channel occupancy reporting | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- Reporting type of FG10-10

  - Per UE: [5]

  - Per band: [9], [10], [11], [12]

- Whether FG10-10 can be extended to licensed band

  - Not support: [4], [6], [9], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| --- | --- |
| [5] | It is not clear why RSSI channel occupancy measurement and reporting needs to be per band instead of being per UE as it was in the case of LTE-LAA. It is simpler to make this per UE, especially considering there are at least two bands already (5 and 6 GHz) and there may potentially also be other unlicensed bands that may open up in the future. Proposal 6 FG 10-10 should be per UE |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: - The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | - We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-10; RSSI and channel occupancy measurement and reporting; - RSSI measurement Channel occupancy reporting; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. |

2.5	FG10-11

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-11 | SRS starting position at any OFDM symbol in a slot | - Support transmitting SRS starting in all symbols (0,…,13) of a slot | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- Reporting type of FG10-11

  - Per UE: [2], [5], [9]

  - Per band: [10], [11], [12]

- Whether FG10-11 can be extended to licensed band

  - Support: [2], [3], [5], [6], [9], [14]

  - Not support: [4], [6], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - For SRS starting position at any OFDM symbol in a slot (10-11), it offers more flexibility on the placement of SRS which will also benefit the licensed band. Proposal 1: SS group switching related features (10-9, 10-9b, 10-9c, 10-9d) and SRS starting position at any OFDM symbol in a slot (10-11) could be extended to licensed use and adopt “Per UE” type. |
| --- | --- |
| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [5] | The ability to configure an SRS resource to start at any OFDM symbol in a slot is generally useful, regardless of the band. For example, it can enable 1T4R antenna switching in the same slot (currently 2-slots are needed), it can be useful for positioning, and also URLLC. Hence this FG should be per UE. Proposal 7 FG 10-11 should be per UE |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: - The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-11; SRS starting position at any OFDM symbol in a slot; Support transmitting SRS starting in all symbols (0,…,13) of a slot; TBD; Yes; N/A; Per UE; N/A; N/A; N/A; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. |
| [14] | Observation 1: Support of SRS starting position at any symbol in a slot will alleviate the shortage of SRS capacity and is beneficial for licensed bands as well. Observation 2: Support of SRS starting position at any symbol in a slot for licensed bands will lead to NO additional standardization efforts. Based on the above discussions and observations, we have the following proposal Proposal 1: Support SRS resource starting at any symbol in a slot for licensed bands in Rel-16, i.e., UE feature 10-11 is also applicable to licensed bands. |

2.6	FG10-20/20a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-20 | Support search space set configuration with freqMonitorLocation-r16 | 1. Support search space set configuration with freqMonitorLocations-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |
| 10. NR-unlicensed | 10-20a | Support coreset configuration with rb-Offset | 1. Support coreset configuration with rb-Offset | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- Component of FG10-20

  - Modify the component of FG10-20 from “Support search space set configuration with freqMonitorLocations-r16” to “Maximum number of frequency domain locations for a search space set configuration with freqMonitorLocations-r16”: [4]

- Whether FG10-20/20a can be extended to licensed band

  - Support: [3]

  - Not support: [4], [6]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| --- | --- |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. For 10-20, the number of PDCCH search space sets/CORESETs/candidates that UE can monitor is highly related to UE’s complexity. The more cost a UE is willing to pay, the more candidates/search spaces sets/CORESETs the UE is able to process. Therefore, 10-20 should not be just a “whether or not” indication. Instead, it should be a “how many” indication. Proposal 10: For FG10-20, change the component to “Maximum number of frequency domain locations for a search space set configuration with freqMonitorLocations-r16” |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |

2.7	FG10-23

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-23 | CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality | 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A | Support reading RMSI from an unlicensed cell [with an off-sync raster SSB] for ANR | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- FG name of FG10-23

  - Change FG name from “CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality” to “CGI reading on unlicensed cell for ANR functionality”: [4], [7], [11]

  - Remove the brackets of [based on off-sync raster SSB]: [9]

- Prerequisite feature groups for FG10-23

  - Add FG 4-5: [10]

- Note of FG10-23

  - Remove “[based on off-sync raster SSB]”: [4], [7], [11]

  - Remove the brackets of [based on off-sync raster SSB]: [9]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [4] | With the introduction of discovery burst transmission windows, the number of SSB positions that UE has to monitor for measurements and PBCH reading is increased dramatically especially for small Q values. In addition, PCI collision is no new issue in NR-U. LTE-LAA has the same issue. However, it is resolved by eNB without mandating UE to read and report CGI. Hence, we think the CGI reading of an unlicensed cell for ANR functionality should be an optional feature. Proposal 11: CGI reading of an unlicensed cell for ANR functionality should be UE capability regardless whether its SSB is on or off sync rater. Proposal 12: For FG10-23, change FG name from “CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality” to “CGI reading on unlicensed cell for ANR functionality”. |
| --- | --- |
| [7] | In our opinion, it would be preferable that CGI reading for ANR purpose is an optional feature regardless of whether the corresponding SSB is located in sync raster or not. Therefore, “[based on off-sync raster SSB]” can be removed for FG 10-23. Proposal #6: Remove “[based on off-sync raster SSB]” for FG 10-23. |
| [9] | 10-23; CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality; 1. Support acquisition of relevant information from a neighbouring NR unlicensed cell in an unlicensed carrier by reading the RMSI of the neighbouring unlicensed cell and reporting the acquired information to the network. Proposal: remove the brackets. |
| [10] | Proposal 4: - Adding the FG 4-5 as prerequisite feature groups for FG 10-23. - Modify the “cat4” in FG 10-21a to “Type 1” to align with TS 37.213 specification. |
| [11] | - FG 10-23: CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality - Since CGI reading for both on and off sync raster are defined for unlicensed band, the capability including both cases may be sufficient, i.e., the sentence in the bracket in the columns of Feature group and Note should be removed. |

2.8	FG10-25

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-25 | Enable configured UL transmissions when DCI 2_0 is configured but not detected | 1. Support configuration of enableConfiguredUL-r16 and enable transmission of higher-layer configured UL *SRS, PUCCH, CG-PUSCH etc) when DCI 2_0 is configured but not detected | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- Components of FG10-25

  - Change “DCI 2_0” to “SFI”: [2]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - On 10-25 (Enable configured UL transmission out of COT), to be more accurate, we suggest to change “DCI 2_0” to “SFI” since DCI 2_0 is not SFI only in NRU Rel-16. Index; Feature group; Components; Prerequisite feature groups. 10-25; Enable configured UL transmission out of COT; 1. Support configuration of enableConfiguredUL-r16 and enable Cat 4 LBT based transmission of RRC configured UL *SRS, PUCCH, CG-PUSCH etc) out of COT when DCI 2_0 is configured but not detected; 10-1. Proposal 5: Change “DCI 2_0” to “SFI” in the component of 10-25. |
| --- | --- |

2.9	FG10-27

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-27 | Wideband PRACH | - Enhanced PRACH design for NR-U by adopting a single long ZC sequence, with ZC sequence = 1151 for 15kHz and ZC sequence = 571 for 30kHz | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- No remaining issues or proposals are identified for FG10-27

2.10	FG10-29

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-29 | Support available RB set indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read availableRB-Sets-r16 | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- No remaining issues or proposals are identified for FG10-29

2.11	FG10-30

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-30 | Support channel occupancy duration indicator field in DCI 2_0 | - Support monitoring DCI 2_0 to read COT duration | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signaling This FG may be a part of basic operation for a particular scenario |

- No remaining issues or proposals are identified for FG10-30

2.12	FG10-8

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-8 | Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | - Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision | 5-6a | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Note length 9/10 with DMRS shift due to CRS collision are already covered by 14-3 | Optional with capability signalling |

- FG name of FG10-8

  - Remove the brackets of [9, 10,]: [9], [11], [13]

- Components of FG10-8

  - Remove the brackets of [9, 10,]: [9], [11], [13]

- Reporting type of FG10-8

  - Per UE: [5], [9]

  - Per band: [10], [11], [12]

- Whether FG10-8 can be extended to licensed band

  - Support: [3], [5], [6], [8], [9]

  - Not support: [4], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| --- | --- |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [5] | Since the PDSCH mapping lengths are generally useful, regardless of the band, in our view this feature should be per UE. Proposal 4 FG 10-8 should be per UE. |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant - 10-28 Configured grant enhanced resource configuration 10-8; Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; - Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision. Proposal: remove the brackets. |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. - FG 10-8: Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision - We are OK to remove the bracket on PDSCH length [9, 10] for supporting them without DMRS shift due to CRS collision. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-8; Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; 5-6a; Yes; N/A; Per band; N/A; N/A; N/A; Note length 9/10 with DMRS shift due to CRS collision are already covered by 14-3; Optional with capability signalling. |
| [13] | - 10-8: We are OK to remove brackets from [9,10]. |

2.13	FG10-9/9b/9c/9d

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-9 | Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring | 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support switching the search space set group with PDCCH decoding in group 1 4. Support a timer to switch back to original search space set group 5. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9b | Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring | 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
| 10. NR-unlicensed | 10-9c | Joint search space group switching across multiple cells | - Configured with a group of cells and switch search space set group jointly over these cells | one of {10-9, 10-9b} | Yes | N/A |  | FFS: Per UE or per band or per BC | N/A | N/A | N/A | Without this capability, the UE will switch search space set groups for different cells independently | Optional with capability signalling |
| 10. NR-unlicensed | 10-9d | Support Search space set group switching capability 2 | - Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS | one of {10-9, 10-9b} | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Without this capability, the UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2 | Optional with capability signalling |

- FG name of FG10-9

  - Change from “Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring” to “Search space set group switching with DCI 2_0 monitoring”: [4]

- Reporting type of FG10-9/9b/9d

  - Per UE: [2], [5]

  - Per band: [4], [6], [9], [10], [11], [12]

- Reporting type of FG10-9c

  - Per UE: [2], [5]

  - Per band: [4], [6], [10], [11]

  - Per BC: [9], [12]

- Whether FG10-9/9b/9c/9d can be extended to licensed band

  - Support: [2], [3], [5]

  - Not support: [4], [6], [9], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - For search space set (SS) group switching related features (10-9, 10-9b, 10-9c, 10-9d), it is beneficial for power saving purpose in licensed band, i.e. one SS with sparse PDCCH monitoring in power saving mode and switch to another SS with frequent PDCCH monitoring when traffic arrives. Proposal 1: SS group switching related features (10-9, 10-9b, 10-9c, 10-9d) and SRS starting position at any OFDM symbol in a slot (10-11) could be extended to licensed use and adopt “Per UE” type. |
| --- | --- |
| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. For FG10-9, we suggest to change from “Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring” to “Search space set group switching without DCI 2_0 monitoring.” Proposal 5: For FG10-9, change from “Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring” to “Search space set group switching without DCI 2_0 monitoring.” |
| [5] | In our view, this feature is useful for UE power saving, regardless of the operating band. Hence this feature should be per UE. Please note that we previously commented that 10-9c should be FFS Per UE or Per Band to be consistent with 10-9,-9b,-9d. Proposal 5 FGs 10-9/9b/9c/9d should be per UE. |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For 10-9c, consider this is CA related, may need to consider this is “per band” or “per BC” - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-9; Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring; 1. Two groups of search space sets 2. Monitor DCI 2_0 with a search space set switching field 3. Support switching the search space set group with PDCCH decoding in group 1 4. Support a timer to switch back to original search space set group 5. Monitor DCI 2_0 for channel occupancy time and use the end of channel occupancy time to switch back to the original search space set group; TBD; Yes; N/A; per band; N/A; N/A; N/A; Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups.; Optional with capability signalling. 10. NR-unlicensed; 10-9b; Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring; 1. Two groups of search space sets 2. Support switching the search space set group with PDCCH decoding in group 1 3. Support a timer to switch back to original search space set group; TBD; Yes; N/A; per band; N/A; N/A; N/A; Being configured with two groups of search spaces, and switch between them. Some search space sets can be configured in both groups.; Optional with capability signalling. 10. NR-unlicensed; 10-9c; Joint search space group switching across multiple cells; Configured with a group of cells and switch search space set group jointly over these cells; one of {10-9, 10-9b}; Yes; N/A; per BC; N/A; N/A; N/A; Without this capability, the UE will switch search space set groups for different cells independently; Optional with capability signalling. 10. NR-unlicensed; 10-9d; Support Search space set group switching capability 2; Search space set group switching Capability-2: P=10/12/22 symbols for µ = 0/1/2 SCS; one of {10-9, 10-9b}; Yes; N/A; per band; N/A; N/A; N/A; Without this capability, the UE supports search space set group switching capability-1: P=25/25/25 symbols for µ=0/1/2; Optional with capability signalling. |

2.14	FG10-14

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | - Support configuration of a value for dl-DataToUL-ACK indicating an inapplicable time to report HARQ ACK | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | If non-numerical K1 value is supported | Optional with capability signalling |

- Reporting type of FG10-14

  - Per UE: [9]

  - Per band: [10], [11], [12]

  - Per unlicensed band: [2]

- Whether FG10-14 can be extended to licensed band

  - Support: [3], [9]

  - Not support: [2], [4], [6], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - For other UE features, we do not see the need of extension to licensed band since it is introduced due to LBT requirement on unlicensed band which doesn’t exist in licensed band and no benefit is Proposal 2: For UE features that are not agreed to be extended to licensed use, update “per band” to “per unlicensed band”. |
| --- | --- |
| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-14; Non-numerical PDSCH to HARQ-ACK timing; Support configuration of a value for dl-DataToUL-ACK indicating an inapplicable time to report HARQ ACK; TBD; Yes; N/A; per band; N/A; N/A; N/A; If non-numerical K1 value is supported; Optional with capability signalling. |

2.15	FG10-15

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-15 | Enhanced dynamic HARQ codebook | 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 (configuration of nfi-TotalDAI-Included) 2. Support of bit field in DCI 0_1 for other group total DAI if configured. (configuration of ul-TotalDAI-Included) 3. Support the retransmission of HARQ ACK (pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16) | TBD | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |

- Reporting type of FG10-15

  - Per UE: [9]

  - Per band: [10], [11], [12]

  - Per unlicensed band: [2]

- Whether FG10-15 can be extended to licensed band

  - Support: [3], [9]

  - Not support: [2], [4], [6], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - For other UE features, we do not see the need of extension to licensed band since it is introduced due to LBT requirement on unlicensed band which doesn’t exist in licensed band and no benefit is Proposal 2: For UE features that are not agreed to be extended to licensed use, update “per band” to “per unlicensed band”. |
| --- | --- |
| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-15; Enhanced dynamic HARQ codebook; 1. Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 (configuration of nfi-TotalDAI-Included) 2. Support of bit field in DCI 0_1 for other group total DAI if configured. (configuration of ul-TotalDAI-Included) 3. Support the retransmission of HARQ ACK (pdsch-HARQ-ACK-Codebook = enhancedDynamic-r16); TBD; Yes; N/A; per band; N/A; N/A; N/A; Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups; Optional with capability signalling. |

2.16	FG10-16

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-16 | One-shot HARQ ACK feedback | - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH - Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |

- Reporting type of FG10-16

  - Per UE: [5]

  - Per band: [10], [11], [12]

  - Per unlicensed band: [2]

- Whether FG10-16 can be extended to licensed band

  - Support: [3]

  - Not support: [2], [4], [6], [9], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - For other UE features, we do not see the need of extension to licensed band since it is introduced due to LBT requirement on unlicensed band which doesn’t exist in licensed band and no benefit is Proposal 2: For UE features that are not agreed to be extended to licensed use, update “per band” to “per unlicensed band”. |
| --- | --- |
| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [5] | Proposal 8 FG 10-16 should be per UE |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-16; One-shot HARQ ACK feedback; - Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_1 scheduling a PDSCH Support feedback of type 3 HARQ-ACK codebook , triggered by a DCI 1_1 without scheduling a PDSCH using a reserved FDRA value; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group.; Optional with capability signalling. |

2.17	FG10-17

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-17 | Multi-PUSCH UL grant | 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1 | TBD | Yes | N/A |  | FFS: Per band or Per UE | N/A | N/A | N/A |  | Optional with capability signalling |

- Reporting type of FG10-17

  - Per UE: [5], [9]

  - Per band: [10], [11], [12]

  - Per unlicensed band: [2]

- Whether FG10-17 can be extended to licensed band

  - Support: [3], [5], [9]

  - Not support: [2], [4], [6], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - For other UE features, we do not see the need of extension to licensed band since it is introduced due to LBT requirement on unlicensed band which doesn’t exist in licensed band and no benefit is Proposal 2: For UE features that are not agreed to be extended to licensed use, update “per band” to “per unlicensed band”. |
| --- | --- |
| [3] | - TypeB PDSCH length & SRS starting position: including FG 10-8 and FG 10-11 - CORESET/SS: including FG 10-9/9b/9c, 10-20/20a. - HARQ enhancements: including FG 10-14 ~ 10-17. In general, the above enhancements on PDSCH, SRS, CORESET/SS, and HARQ could be beneficial to licensed spectrum in terms of enhanced flexibility and reliability. On the other hand, it may introduce implementation complexity for NR UEs. Probably they can be considered as optional features to be applied to NR licensed spectrum. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [5] | Multi-PUSCH UL grants should be per UE instead of per band. Firstly, the functionality will be very useful in any band where PDCCH capacity can be constrained. Secondly, it is functionality that once implemented is fundamentally not related to the band of operation. Proposal 9 FG 10-17 should be per UE |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-17; Multi-PUSCH UL grant; 1. Support of scheduling up to 8 PUSCH with a single DCI 0_1; TBD; Yes; N/A; Per band; N/A; N/A; N/A; Optional with capability signalling. |

2.18	FG[10-19a/19b]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | [10-19a] | [Support DL reception in a carrier with intra-cell guard-bands] | 1. [When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted | Optional with capability signalling |
| 10. NR-unlicensed | [10-19b] | [Support UL transmission with subset of RB sets passing LBT] | 1. [When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located] | TBD | Yes | N/A |  | Per band | N/A | N/A |  | Without this capability, UE will transmit UL when all RB sets in the UL BWP pass LBT | Optional with capability signalling |

- Necessity of FG10-19 (Number of LBT bandwidth)

  - FG is kept: [5]

- Necessity of FG10-19a

  - FG is removed: [13]

  - FG is kept: [4], [5], [11]

- Necessity of FG10-19b

  - FG is kept: [5], [11], [13]

- Potential new FG

  - introduce a new FG to signal the UE capability 2 for intra-cell guard band: [8]

    - Support intra-cell guard band length smaller than the default intra-cell guard band length defined in RAN4

- FG name of FG 10-19a

  - Modify to “support DL reception in a carrier with intra-cell guard-bands including additional CSI-RS reception”: [9]

- Components of FG10-19a

  - Modify from “[When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets]” to “When DL carrier/BWP has multiple RB sets, support the reception of a DL signal/channel even when gNB may have failed LBT in RB sets that the DL signal/channel does not occupy.”: [4]

  - Add “When DL BWP has multiple RB sets, support PDSCH reception on a subset of RB sets”: [5]

- Components of FG10-19b

  - Modify to “When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only a subset of the RB sets”: [5]

- Note of FG10-19a

  - Modify to “If UE does not indicate this capability, UE will be configured with zero intra-cell guard-band. Furthermore, UE will assume all RB sets in a DL BWP are all transmitted or none of them are transmitted.”: [4]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | - On 10-19a (Support DL reception with subset of RB sets), it seems only related with CSI-RS from component description but not aligned with the feature name or description part. It should be clarified what’s the intention for this UE feature. Index; Feature group; Components; Prerequisite feature groups. 10-19a; Support DL reception with subset of RB sets; 1. When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets; 10-1 or 10-2. Proposal 4: Clarify the intention of 10-19a. |
| --- | --- |
| [4] | For FG10-19a, we think this is important to reduce UE complexity for wideband operation and should be kept as a FG. When the serving gNB can transmit DL transmissions on a subset of RB sets in a wideband BWP even when it does not pass all RB sets in a BWP, it means some of the RB sets may be occupied by other non-serving gNB/eNB/Wifi devices. If the interference on these unavailable RB sets is very strong (say it is caused by a nearby Wifi device), it is very difficult for UE to set a proper AGC gain. If UE does not indicate this capability, it means it does not have to worry some of the RB sets in its active DL BWP are occupied by devices other than its serving cell. Note: in our understanding, this FG actually regulates gNB so that gNB has to pass LBT in all RB sets of a DL BWP in order to transmit to UEs that do not support this capability. Suggested modification to FG10-19a: - Component: Change from “[When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets]” to “When DL carrier/BWP has multiple RB sets, support the reception of a DL signal/channel even when gNB may have failed LBT in RB sets that the DL signal/channel does not occupy.” - Note: If UE does not indicate this capability, UE will be configured with zero intra-cell guard-band. Furthermore, UE will assume all RB sets in a DL BWP are all transmitted or none of them are transmitted. Proposal 7: For FG10-19a, change the column of component from “[When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets]” to “When DL carrier/BWP has multiple RB sets, support the reception of a DL signal/channel even when gNB may have failed LBT in RB sets that the DL signal/channel does not occupy.” Proposal 8: For FG10-19a, add the following to the column of Note: - If UE does not indicate this capability, UE will be configured with zero intra-cell guard-band. Furthermore, UE will assume all RB sets in a DL BWP are all transmitted or none of them are transmitted. Proposal 9: Support FG10-19a. |
| [5] | FG 10-19 Regarding the FFS, this feature group is needed, as it is important for the network to know the UE capability to enable proper UL scheduling. Proposal 10: Keep FG 10-19; FFS can be deleted FG 10-19a It seems that this capability is more general and applies not only to CSI-RS but also PDSCH. We suggest to add a second component to this feature group. Proposal 11: Keep FG. Add a component to FG 10-19a for reception of PDSCH over a subset of RB sets as follows [10-19a]; [Support DL reception in a carrier with intra-cell guard-bands]; 1. [When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets] 2. When DL BWP has multiple RB sets, support PDSCH reception on a subset of RB sets; Without this capability, UE will assume all RB sets in the DL BWP are all transmitted or none of them are transmitted. FG 10-19b We support keeping this is a FG, but the component description needs rewording to “When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only a subset of the RB sets.” Proposal 12: Keep FG. Change component description to “When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only a subset of the RB sets.” |
| [8] | Intra-cell guard band In wideband operation scenario, if UE is not configured with intra-cell guard band by higher layer parameters, the UE should determine intra-cell guard band and corresponding RB set according to the default intra-cell GB pattern from [2]. The default pattern, the number of usable RBs per sub-channel and the number of intra-carrier guard PRBs for different SCS is copied here in Table 1 and Table 2 according to [2]: Table 1 the default pattern SCS; 20MHz Channels; 40MHz Channels; 60MHz Channels; 80MHz Channels. 15KHz; 106; 105-6-105; Max. 216; N/A; N/A. 30KHz; 51; 50-6-50; Max. 106; 50-6-50-6-50; Max. 162; 50-6-50-5-50-6-50; Max. 217. Alt. 1 60KHz; 24; [23-5-23]; Max. 51; [23-5-23-5-23]; Max. 79; [23-5-23-5-23-5-23]; Max. 107. Alt. 2 60KHz; [25]; [24-3-24]; Max. 51; [24-3-25-3-24]; Max. 79; [24-4-24-3-24-4-24]; Max. 107. Table 2 the number of usable RBs per sub-channel and intra-carrier guard PRBs 15kHz; 30kHz; 60kHz. Usable RBs; Guard RBs; Usable RBs; Guard RBs; Usable RBs; Guard RBs. 104, 105; 6, 7; 49, 50, 51; 5, 6, 7; [22, 23, 24]; [3, 4, 5]. On the other hand, RRC signallings which are used to configure intra-cell guard band for UE are specified in 38.331 as follows [3]: ServingCellConfigCommon ::= SEQUENCE { <omit> intraCellGuardBandUL-r16 IntraCellGuardBand-r16 OPTIONAL, -- Need S intraCellGuardBandDL-r16 IntraCellGuardBand-r16 OPTIONAL -- Need S ]] } IntraCellGuardBand-r16 ::= SEQUENCE (SIZE (1..ffsValue)) OF GuardBand-r16 -- FFS upper size 4, assuming 100Mhz cell GuardBand-r16 ::= SEQUENCE { startCRB-r16 INTEGER (0..ffsValue), --FFS upper range 275 nrofCRBs-r16 INTEGER (0..ffsValue) }. ServingCellConfigCommon field descriptions. intraCellGuardBandDL, intraCellGuardBandUL List of guard bands in a BWP. For each entry in the list, startCRB indicates the starting RB of the guard band and nrofCRBs indicates the length of the guard band in RBs. For intraCellGuardBandUL, when nrofCRBs is 0, zero-size or no guard band is used. If not configured, the guard bands are defined according the TS 38.101-X.. The default intra-cell GB pattern defined in RAN4 was intended to target a minimum requirement for a UE to support a BWP containing multiple RB sets via intra-cell guard-bands configuration. Moreover, the default GB length was studied to control the out-of-intra-band power leakage which is further tightly related to the RF implementation. In this regard, a UE, who declares to support multiple RB sets in a BWP, should be allowed to implement the RF filtering which satisfies the RAN4 minimum requirement. It is to say that for a moderate UE, who satisfies the minimum requirement, the configured GB length should not be smaller than the RAN4 default GB length. For this reason, we propose to define a UE capability, e.g. intra-cell GB capability 2, of supporting all the configurable GB lengths. Proposal 2: introduce a new FG10-19c to signal the UE capability 2 for intra-cell guard band. FG10-19c; UE capability 2 for intra-cell guard band; Supporting intra-cell guard band length smaller than the default intra-cell guard band length defined in RAN4.. |
| [9] | [10-19a]; [Support DL reception in a carrier with intra-cell guard-bands]; 1. [When DL BWP has multiple RB sets, support using the available RB set bitmap in DCI 2_0 to validate the periodic CSI-RS transmission if the CSI-RS is over multiple RB-sets]. [10-19b]; [Support UL transmission with subset of RB sets passing LBT]; 1. [When UL BWP has multiple RB sets, support transmission of UL signal or channels when LBT passes for only the RB sets the UL signals or channels are located]. Proposal: 10-19a title could be changed to “support DL reception in a carrier with intra-cell guard-bands including additional CSI-RS reception” |
| [11] | - [FG 10-19a: Support DL reception in a carrier with intra-cell guard-bands] - We think this FG can be kept and the bracket in the Components can be removed. - [FG 10-19b: Support UL transmission with subset of RB sets passing LBT] - We think this FG can be kept and the bracket in the Components can be removed. |
| [13] | - 10-19a: This FG is not needed in our view, and in any case this should not be based on guard-bands being allowed or not, but whether UE is capable of inband blocking or not - 10-19b: This FG is OK in principle, but it should be clear how many RB sets are supported. |

2.19	FG10-26/26a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-26 | CSI-RS based RLM for NR-U | [CSI-RS based RLM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-26a | CSI-RS based RRM for NR-U | [CSI-RS based RRM for NR-U] | TBD | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- No remaining issues or proposals are identified for FG10-26/26a

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [11] | - FG 10-26: CSI-RS based RLM for NR-U - As this FG is related to the discussion on the validation of periodic CSI-RS, which wasn’t agreed in the last RAN1 meeting, it would be better to discuss the definition of this FG after the agreement on the validation of periodic CSI-RS is made in NR-U agenda. - FG 10-26a: CSI-RS based RRM for NR-U - As this FG is related to the discussion on the validation of periodic CSI-RS, which wasn’t agreed in the last RAN1 meeting, it would be better to discuss the definition of this FG after the agreement on the validation of periodic CSI-RS is made in NR-U agenda. |
| --- | --- |

2.20	FG[10-31]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | [10-31] | [Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0] | [· Perform CSI measurements for reporting and tracking using CSI-RS resources that are not within a COT duration indicated by DCI 2_0 · Note: This includes the cases when DCI 2_0 is not configured and when DCI 2_0 is configured but COT duration is not provided by either CO duration field or SFI.] | TBD | Yes | N/A |  | [Per band] | N/A | N/A |  |  | Optional with capability signaling |

- Necessity of FG10-31

  - FG is removed: [5]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [5] | We don’t believe that such a feature group is needed or useful. It is well understood that in unlicensed spectrum, quality cannot be ensured and is always dependent on the number of other devices trying to access the carrier. It is also well understood that CSI-RS measurements for reporting and tracking are highly implementation dependent and rely on the UE admitting measurements into its averaging processes and tracking loops when the UE is able to ascertain reliably that the measurement is legitimate. Furthermore, with regard to CSI-RS measurements for reporting, it is possible for the system to operate well without any changes to UE implementation since the gNB can set measurement restrictions for CSI-RS measurements to ensure that any particular measurement report is associated with a single instance of the CSI-RS. The gNB can then use only the measurements that correspond to CSI-RS occasions where the gNB transmitted so that there is no risk of noisy measurements being used. Hence, no special capability or functionality at the UE is required for measurement reporting. When it comes to CSI-RS for tracking, once again the system can operate by configuring the periodic TRS appropriately and by relying on aperiodic TRS. However, the ability to operate when some information via DCI 2_0 is not available should be mandatory for operation in shared spectrum and there is no necessity to define a capability for this. Furthermore, even if a capability were to be defined, it is not good to define this capability in terms of the CO-duration since slots within the CO-duration are not guaranteed to carry the periodic CSI-RS transmissions. The gNB needs to maintain flexibility to be able to schedule any slot as an uplink slot and if no SFI is configured to the UE, then other UEs will assume CSI-RS is present when it is not and pollute their tracking loops. Considering all of this, it is best to just leave it to RAN4 to define a reasonable test case for the UE to be able to measure CSI-RS when DCI 2_0 is not configured or no pertinent information is received in DCI 2_0. Proposal 13 FG 10-31 should be removed |
| --- | --- |
| [11] | - [FG 10-31: Support of CSI-RS measurements for CSI reporting and tracking without COT duration from DCI 2_0] - As this FG is related to the discussion on the validation of periodic/semi-persistent CSI-RS, which wasn’t agreed in the last RAN1 meeting, it would be better to discuss the definition of this FG after the agreement on the validation of periodic CSI-RS is made in NR-U agenda. |
| [13] | - 10-31: This is still open in NR-U maintenance, wait for conclusion. |

2.21	FG10-3/3a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-3 | PRB interlace mapping for PUSCH | 1. PRB interlace frequency domain resource allocation for PUSCH | TBD One of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUSCH | Optional with capability signalling |
| 10. NR-unlicensed | 10-3a | PRB interlace mapping for PUCCH | - PRB interlace frequency domain resource allocation for PUCCH format 0 and format 1 - PRB interlace frequency domain resource allocation for PUCCH format 2 - PRB interlace frequency domain resource allocation for PUCCH format 3 | TBD One of {10-1, 10-1a} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support of PRB interlace PUCCH format 0/1 | Optional with capability signalling |

- No remaining issues or proposals are identified for FG10-3/3a

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | - Interlaced mapping including FG 10-3/3a. The interlaced mapping is proposed to simplify the signalling of the FDRA of interlace in order to satisfy the OCB requirement from ESTI. There is no such OCB requirement for NR licensed spectrum, so we do not think there is a need to extend the application range for this feature. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| --- | --- |

2.22	FG10-12

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-12 | OCC for PRB interlace mapping for PF2 and PF3 | 1. OCC2 2. OCC4 | 10-3a | Yes | N/A |  | Per band | N/A | N/A | N/A | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |

- No remaining issues or proposals are identified for FG10-12

2.23	FG10-13a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-13a | Extended CP range of more than one symbol for CG-PUSCH | - UE supports generating a CP extension of length longer than 1 symbol for Configured Grant PUSCH transmission | TBD One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | How long a UE can generate the CP extension beyond 1 symbol for CG-PUSCH | Optional with capability signalling |

- No remaining issues or proposals are identified for FG10-13a

2.24	FG10-18

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-18 | Configured grant with retransmission in CG resources | 1. Support retransmission in CG resources 2. Support configured grant retransmission timer 3. Support DFI monitoring 4. Support CG-UCI in CG-PUSCH | TBD One of {5-19, 5-20} | Yes | N/A |  | Per band | N/A | N/A | N/A | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |

- No remaining issues or proposals are identified for FG10-18

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | - CG enhancement: FG 10-18, 10-24, and 10-28. The enhancement on the configured grant, for example the CG-UCI and retransmission on CG resources have been discussed in Rel-15, but not agreed at that moment. And then in Rel-16 URLLC WI, the configured grant has been enhanced with different approaches. If the enhancements in NR-U are applied to licensed spectrum, there will be two ways to do configured grant which are not compatible. It is not clear on the configurations and UE behavior for the operation of configured grant. So we think the CG enhancement shall not be applied to licensed spectrum at least in Rel-16. We can further discuss how to optimize the URLLC for NR-U in Rel-17. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. - Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| --- | --- |

2.25	FG10-21a

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-21a | Support using ED threshold given by gNB for UL to DL COT sharing | 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information | TBD 10-1 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Potential new FGs

  - Keep FG10-21b “Support UL to DL COT sharing” separately to allow UE COT sharing without using ULtoDL-CO-SharingED-Threshold-r16: [8], [11]

- Components of FG10-21a

  - Modify the “cat4” in FG 10-21a to “Type 1” to align with TS 37.213 specification: [10]

- Components of FG10-21b

  - 1. Support cat 4 LBT for scheduled UL to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16: [8]

  - 2. Support cat 4 LBT for CG-PUSCH to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16: [8], [11]

  - 3. Indicate in CG-UCI the COT sharing information: [8], [11]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [8] | UL to DL COT sharing FG10-21a, FG10-21b should be kept as is, in last RAN1 meeting there was an agreement that if a UE is provided with ULtoDL-CO-SharingED-Threshold-r16 the UE can select if the UE uses the configured ED threshold or not. If the UE chooses to not use this threshold, the UE shall not share the COT with the gNB. Agreement: For at least PUSCH transmissions with configured grants, a UE is allowed to choose between the ED threshold given by ul-toDL-CO-SharingED-Threshold-r16 and the default one. Whether a spec change is required needs further discussion. Discuss and decide the possible TPs in the next meeting.. In this case, FG10-21b is necessary, because if the UE only supports the COT sharing without ULtoDL-CO-SharingED-Threshold-r16, it must be informed to the gNB, otherwise, the gNB might configure ULtoDL-CO-SharingED-Threshold-r16, but the UE does not implement this threshold so that the UE cannot share the COT. Keeping FG10-21b, it allows the UE to continue sharing its COT without using ULtoDL-CO-SharingED-Threshold-r16. 10-21a; Support using ED threshold for UL to DL COT sharing; 1. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for scheduled UL to share COT with gNB for DL 2. Use ULtoDL-CO-SharingED-Threshold-r16 for cat 4 LBT for CG-PUSCH to share COT with gNB for DL 3. Indicate in CG-UCI the COT sharing information as configured in cg-COT-Sharing-r16. 10-21b; Support UL to DL COT sharing; 1. Support cat 4 LBT for scheduled UL to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16 2. Support cat 4 LBT for CG-PUSCH to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16 3. Indicate in CG-UCI the COT sharing information. Proposal 1: keeping FG10-21a and FG10-21b separately to allow UE COT sharing without using ULtoDL-CO-SharingED-Threshold-r16. |
| --- | --- |
| [10] | Proposal 4: - Adding the FG 4-5 as prerequisite feature groups for FG 10-23. - Modify the “cat4” in FG 10-21a to “Type 1” to align with TS 37.213 specification. |
| [11] | - FG 10-21a: Support using ED threshold given by gNB for UL to DL COT sharing - In addition to this FG, 10-21b which was discussed in the last RAN1 meeting can be added as follows: 10-21b; Support UL to DL COT sharing; 1. Support cat 4 LBT for CG-PUSCH to share COT with gNB for DL without ULtoDL-CO-SharingED-Threshold-r16 2. Indicate in CG-UCI the COT sharing information. |

2.26	FG10-24

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-24 | CG-UCI multiplexing with HARQ ACK | 1. Support multiplexing CG-UCI with HARQ ACK | 10-18 | Yes | N/A |  | Per band | N/A | N/A | N/A |  | Optional with capability signalling |

- No remaining issues or proposals are identified for FG10-24

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | - CG enhancement: FG 10-18, 10-24, and 10-28. The enhancement on the configured grant, for example the CG-UCI and retransmission on CG resources have been discussed in Rel-15, but not agreed at that moment. And then in Rel-16 URLLC WI, the configured grant has been enhanced with different approaches. If the enhancements in NR-U are applied to licensed spectrum, there will be two ways to do configured grant which are not compatible. It is not clear on the configurations and UE behavior for the operation of configured grant. So we think the CG enhancement shall not be applied to licensed spectrum at least in Rel-16. We can further discuss how to optimize the URLLC for NR-U in Rel-17. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| --- | --- |

2.27	FG10-28

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10. NR-unlicensed | 10-28 | Configured grant with Rel-16 enhanced resource configuration | 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16, | TBD One of {5-19, 5-20} | Yes | N/A |  | FFS: Per UE or per band | N/A | N/A | N/A |  | Optional with capability signalling |

- Reporting type of FG10-28

  - Per UE: [9]

  - Per band: [10], [11], [12]

- Whether FG10-28 can be extended to licensed band

  - Support: [3], [5], [9]

  - Not support: [3], [4], [6], [10]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [3] | - CG enhancement: FG 10-18, 10-24, and 10-28. The enhancement on the configured grant, for example the CG-UCI and retransmission on CG resources have been discussed in Rel-15, but not agreed at that moment. And then in Rel-16 URLLC WI, the configured grant has been enhanced with different approaches. If the enhancements in NR-U are applied to licensed spectrum, there will be two ways to do configured grant which are not compatible. It is not clear on the configurations and UE behavior for the operation of configured grant. So we think the CG enhancement shall not be applied to licensed spectrum at least in Rel-16. We can further discuss how to optimize the URLLC for NR-U in Rel-17. Proposal 2: - The interlace structure and enhancement on configured grant shall not be applied to NR licensed spectrum. Enhancements on TypeB PDSCH length, SRS starting position, HARQ and CORESET/SS can be considered to be applied to NR licensed spectrum as optional features. |
| --- | --- |
| [4] | During the previous email discussion, a question was raised about the extension of NR-U features to licensed operation. In principle, we do not think NR-U features should be applied for licensed operation unless the use cases and benefits are well justified. The reason is that NR-U features are introduced to mitigate the impact of LBT and/or to meet regional regulations such as OCB or PSD. In contrast, these regulations are not required for licensed spectrum access. We hence do not see the need to apply NR-U features to licensed operation. Proposal 1: NR-U features can only be extended to licensed operation when uses cases and benefits are well justified. |
| [6] | NR-U functions have been introduced to handle inherit problem of unlicensed band such as LBT failure and regulation. Hence, in our view, except FG-8 and FG-11 which are general function for licensed band, applicability of NR-U feature groups should be restricted to unlicensed band. If some of NR-U feature groups are identified to be beneficial for licensed band operation, we will be able to make an agreement for each. Proposal 3: UE features for NR-U should be used only for unlicensed band. |
| [9] | Functionality; FGs; Need for licensed band operation. Type B PDSCH length; 10-8 Type B PDSCH length {3, 5, 6, 8, [9, 10,] 11, 12, 13} without DMRS shift due to CRS collision; Per UE The additional scheduling flexibility could be exploited by the network.. Search space set group switching; 10-9 Search space set group switching with explicit DCI 2_0 bit field trigger or with implicit PDCCH decoding with DCI 2_0 monitoring 10-9b Search space set group switching with implicit PDCCH decoding without DCI 2_0 monitoring 10-9c Joint search space group switching across multiple cells 10-9d Support Search space set group switching capability 2; 10-9/9b/9d: per band 10-9c: per BC It is unclear what benefit could be obtained for operation on a licensed carrier since the monitoring periodicity of PDCCH search spaces would generally not need to change frequently nor depend on implicit rules.. RSSI and channel occupancy measurement and reporting; 10-10 RSSI and channel occupancy measurement and reporting; Per band Unclear what additional information those measurements could bring in case of licensed band operation.. SRS starting position at any OFDM symbol in a slot; 10-11 SRS starting position at any OFDM symbol in a slot; Per UE It is well-known that SRS capacity is always an issue even in licensed bands.. HARQ enhancements; 10-14 Non-numerical PDSCH to HARQ-ACK timing 10-15 Enhanced dynamic HARQ codebook 10-16 One-shot HARQ ACK feedback; 10-14 & 10-15: per UE 10-16: per band It is unclear why many instances of HARQ feedback from a UE would fail simultaneously in licensed operation, requiring that all the HARQ processes are reported by a UE in one-shot. Multi-PUSCH UL grant; 10-17 Multi-PUSCH UL grant; Per UE This feature is beneficial for reducing control overhead on licensed bands. To avoid additional complexity, we suggest no further optimization for this feature in Rel-16, so it should be limited to time-consecutive PUSCHs even on licensed bands.. Configured grant enhanced resource configuration; 10-28 Configured grant with Rel-16 enhanced resource configuration; Per UE. Proposal 2: The following FGs could be extended to licensed bands, i.e. reported “per UE”: - 10-8 Type B PDSCH length - 10-11 SRS starting position at any OFDM symbol in a slot - 10-14 and 10-15 HARQ enhancements - 10-17 Multi-PUSCH UL grant 10-28 Configured grant enhanced resource configuration |
| [10] | In general, our view is that the features developed under NR-U WI should be limited to unlicensed operation by default and exceptions should be discussed case by case with strong justifications. This is important to avoid unnecessary complications/impacts on the operation of licensed band operation. To be more specific, in our view the features listed with “FFS: Per band or Per UE” should be put “per band” to provide the flexibility in implementation and IOT testing Proposal 1: The features listed with “FFS: Per band or Per UE” should be put “per band” |
| [11] | We think the type of all FGs in NR-U should be “per band” and whether a FG can be applied to licensed band as well can be discussed later. |
| [12] | - For all the features listed as “FFS:Per band or Per UE”, to allow the most flexibility in implementation and IOT testing, we would like to make them “per band” 10. NR-unlicensed; 10-28; Configured grant with Rel-16 enhanced resource configuration; 1. Support configuration of resources with cg-nrofSlots-r16 and cg-nrofPUSCH-InSlot-r16,; TBD One of {5-19, 5-20}; Yes; N/A; per band; N/A; N/A; N/A; Optional with capability signalling. |

2.28	Potential new FGs

- No remaining issues or proposals are identified for potential new FGs

Above proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [11] | - [FG 10-22: No gap 2-step RACH msgA transmission] - As this FG was discussed but wasn’t agreed in the last RAN1 meeting, it would be better to discuss the definition of this FG after any agreement/conclusion on MsgA PRACH-PUSCH gap for NR-U is made in NR-U agenda. |
| --- | --- |

  - Basic FGs

Following agreements and working assumption were made at RAN1#100bis-e meeting.

Agreements (RAN1#100bis-e): - Define new basic FGs with components that have tightly related functionality to replace current basic FGs - In “mandatory/optional” column for the possible basic FGs, it should be clarified that the FG may be a part of basic operation for a particular scenario - If the FG is decided as a basic FG, the note will be updated to clarify that the FG is “optional with capability signaling and is required to be supported for the scenario” - Note: each basic FG will have capability bit - [Working assumption] Take either one of following alternatives Alt.1: - Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification - Note: the table does not have impact on capability signaling - Note: the grouping of FGs in the table does not have impact on “prerequisite FGs” column in features list Alt.2: - Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list

- Confirm the working assumption on the association between the basic FGs and NR-U deployment scenarios

  - Alt.1 (Define a table to capture the basic FGs required for a certain NR-U deployment scenario in specification): [3]

  - Alt.2 (Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list): [2], [7]

- Define basic FG(s) for a certain NR-U deployment scenario (whether to keep the text “This FG may be a part of basic operation for a particular scenario”)

  - 10-1 (UL channel access for dynamic channel access mode): [3], [4], [5], [6], [7], [13]

  - 10-1a (UL channel access for semi-static channel access mode): [3], [4], [5], [6], [7], [13]

  - 10-2 (SSB-based RRM [for dynamic channel access mode]): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2a (SSB-based RRM [for semi-static channel access mode]): [5], [7], [13]

    - Objected by: [6]

  - 10-2b (MIB reading on unlicensed cell): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2c (SSB-based RLM with Q [for dynamic channel access mode]): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2d (SSB-based RLM with Q [for semi-static channel access mode]): [5], [7], [13]

    - Objected by: [6]

  - 10-2e (SIB1 reception on unlicensed cell): [3], [4], [5], [7], [13]

    - Objected by: [6]

  - 10-2f (Support monitoring of extended RAR window): [4], [13]

    - Objected by: [5], [6], [7]

  - 10-3 (PRB interlace mapping for PUSCH): [4], [7]

  - 10-3a (PRB interlace mapping for PUCCH): [4], [7]

  - 10-7 (UL channel access for 10 MHz SCell): [4]

    - Objected by: [5], [6], [7]

  - 10-10 (RSSI and channel occupancy measurement and reporting):

    - Objected by: [4], [5], [6], [7]

  - 10-11 (SRS starting position at any OFDM symbol in a slot):

    - Objected by: [5], [6], [7]

  - 10-14 (Non-numerical PDSCH to HARQ-ACK timing): [4]

  - 10-15 (Enhanced dynamic HARQ codebook): [4]

  - 10-19a ([Support DL reception in a carrier with intra-cell guard-bands]): [13]

  - 10-19b ([Support UL transmission with subset of RB sets passing LBT]): [13]

  - 10-20 (Support search space set configuration with freqMonitorLocation-r16): [13]

    - Objected by: [5], [6], [7]

  - 10-20a (Support coreset configuration with rb-Offset):

    - Objected by: [5], [6], [7]

  - 10-23 (CGI reading on unlicensed cell [based on off-sync raster SSB] for ANR functionality):

    - Objected by: [5], [6], [7]

  - 10-25 (Enable configured UL transmissions when DCI 2_0 is configured but not detected):

    - Objected by: [5], [6], [7]

  - 10-27 (Wideband PRACH): [4]

    - Objected by: [5], [6], [7]

  - 10-29 (Support available RB set indicator field in DCI 2_0): [4], [13]

    - Objected by: [5], [6], [7]

  - 10-30 (Support channel occupancy duration indicator field in DCI 2_0): [4]

    - Objected by: [5], [6], [7]

Above remaining issues and proposals are identified based on following feedbacks provided in contributions for the RAN1#101-e meeting.

| [2] | For the capture of association between the FGs to a certain NR-U scenario, we think it is enough to clarify that the FG is a part of basic operation for a particular scenario and the association between the FGs and a certain NR-U deployment scenario is established. Note that, one FG could be clarified as a part of basic operation for multiple scenarios. For example, 10-1 is for NRU standalone scenario with dynamic channel access and CA DL+UL scenario with dynamic channel access. Proposal 3: Clarifying that the FG is a part of basic operation for a particular scenario in “mandatory/optional” column of UE feature list is enough to establish the association between the FGs to a certain NR-U scenario. |
| --- | --- |
| [3] | In our view, Alt.1 is clear as the table is defined with the intention that the basic FGs would be defined with tightly related functionality, and the basic FGs would try to avoid overlapping functionality as much as possible. It would be easier to build the relationship between the basic FGs and other optional FGs in terms of prerequisite condition. Proposal 1: - The following table is defined in specification to capture the basic FGs required for a certain NR-U deployment scenario. Basic FGs; Deployment scenarios. A-1 (SCell); A-2 (SCell); A-2 (SCell); C (Standalone), D (Standalone + SUL); C (Standalone), D (Standalone + SUL); B (EN-DC), E (NR-NR DC); B (EN-DC), E (NR-NR DC). DL only; DL+UL LBE; DL+UL FBE; DL+UL LBE; DL+UL FBE; DL+UL LBE; DL+UL FBE. 10-1: UL channel access for LBE; X; X; X. 10-1a: UL channel access for FBE; X; X; X. 10-2: SSB based RRM; X; X; X; X; X; X; X. 10-2b: MIB reading on unlicensed cell; X; X; X; X. 10-2c: SSB-based RLM; X; X; X; X. 10-2e: SIB1 reception; X; X. |
| [4] | MIB reading should basic FG for SA/DC deployment scenarios. RACH related FGs should be basic FGs for SA/DC deployment scenarios. RAR windows are extended to 40ms to cope with the transmission timing uncertainty caused by LBT. We think it should be supported by UE that plans to support NR-U SA and DC deployments. On the other hand, for UE that only supports NR-U CA operation, then it is not necessary, because RAR is transmitted in SpCell (PCell or PSCell). PRB interlaced mapping for PUSCH should be basic FGs for CA with UL and SA/DC deployment scenarios. PRB interlaced mapping for PUCCH should be basic FGs for SA/DC deployment scenarios. In our opinion, monitoring DCI 2_0 to acquire COT structure in both time (CO duration) and frequency (availableRB-Sets-r16) dimensions is beneficial to UE in terms of AGC adjustment, COT detection, skipping of PDCCH monitoring and CSI-RS reception, etc. Therefore, we suggest to merge FG10-29 and 10-30 as a basic FG for all NR-U deployment scenarios. In our opinion, FG10-11 (“SRS starting position at any OFDM symbol in a slot”) is an enhancement and should not be regarded as a basic feature group. FG10-14 (“Non-numerical PDSCH to HARQ-ACK timing”) is beneficial to unlicensed operation when channel occupancy time is limited by regulation. Specifically, gNB can tell UE to postpone its HARQ-ACK feedback to a PDSCH that is scheduled in the end of a COT. We hence think it should be a basic feature group for UE that supports NR-U SA and DC deployments. On the other hand, for a UE that only supports NR-U CA operation, then the support of this feature group should be optional. We think FG10-15 (“enhanced dynamic HARQ codebook”) should be regarded as a basic FG for UE that supports NR-U SA and DC operation. Similar to the motivation for introducing DRS windows to NR-U, enhanced HARQ procedures that have designed for NR-U can provide multiple transmission opportunities for HARQ-ACK feedbacks. As “dynamic HARQ-ACK codebook” is chosen to be the default codebook for Rel-15, we believe FG10-15 (“enhanced dynamic HARQ codebook”) rather than FG10-16 and 10-16a (one-shot codebook) should be the basic feature group for SA and DC deployment scenarios. Table 1: Proposed basic feature groups for NR-U deployment scenarios Scenario A with DL-only; Scenario A with both DL and UL in LBE; Scenario A with both DL and UL in FBE; Scenario B in LBE; Scenario B in FBE; Scenario C; Scenario D; Scenario E in LBE; Scenario E in FBE. 10-1 (LBT LBE); X; X; X; X. 10-1a (LBT FBE); X; X; X; X. 10-2 (SSB RRM); X; X; X; X; X; X; X; X; X. 10-2b (MIB); X; X; X; X; X; X. 10-2c (SSB RLM); X; X; X; X; X; X. 10-2e (SIB1); X; X; X; X; X; X. 10-2f (ext. RAR); X; X; X; X. 10-3 (interlaced PUSCH); X; X; X; X; X; X; X. 10-3a (interlaced PUCCH); X; X; X; X; X. 10-7 (10M UL); X. 10-14 (non-numerical HARQ); X; X; X; X; X. 10-15; X; X; X; X; X. 10-27 (wide PRACH); X; X; X; X; X. 10-29 (DCI 2_0); X; X; X; X; X; X; X; X; X. 10-30 (DCI 2_0); X; X; X; X; X; X; X; X; X. Proposal 2: Adopt the proposed basic feature groups in Table 1 for NR-U deployment scenarios. For FG10-10, remove “This FG may be a part of basic operation for a particular scenario” from the column of “Mandatory/Optional.” According to RAN2’s agreements, FG10-10 should be optional. Proposal 6: For FG10-10, remove “This FG may be a part of basic operation for a particular scenario” from the column of “Mandatory/Optional.” |
| [5] | In our view, the following FGs make sense to be part of a basic operation for a particular scenario as listed in the last column of the spreadsheet: - FG 10-1, -1a, -2, -2a, -2b, -2c, -2d, -2e However, the following features should be listed only as “Optional with capability signaling,” and not be part of basic operation for a particular scenario - FG 10-2f, -7, -10, -11, -20, -20a, -23, -25, -27, -29, -30 |
| [6] | In the lists, there are many FGs with the statement, ‘This FG may be a part of basic operation for a particular scenario’ in Mandatory/Optional column. Examples are listed as below. 10-10; RSSI and channel occupancy measurement and reporting; 1. RSSI measurement 2. Channel occupancy reporting; FFS: Per band or Per UE; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10-11; SRS starting position at any OFDM symbol in a slot; 1. Support transmitting SRS starting in all symbols (0,…,13) of a slot; FFS: Per band or Per UE; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. 10-20; Support search space set configuration with freqMonitorLocation-r16; 1. Support search space set configuration with freqMonitorLocations-r16; Per band; Optional with capability signaling This FG may be a part of basic operation for a particular scenario. …; …; …; …; …. However, in our view, all of those FGs are not essential features to operate NR-U in a particular scenario. For example, FG 10-10 and FG 10-11, it is quite sure that those feature groups should not necessary regardless of deployment scenarios. Therefore, it is proposed to remove (or put FFS) the statements in Mandatory/Optional column or put FFS for the further discussion. Proposal 2: Remove (or put FFS) ‘This FG may be a part of basic operation for a particular scenario’ for FGs other than FG 10-1 and FG 10-1a. |
| [7] | In RAN1#100bis-e meeting, it was agreed to define BFGs with working assumption for two alternatives on how to capture the relationship between BFG and the corresponding deployment scenario. Between two alternatives, Alt 2 is preferred since it seems sufficient to capture the association relationship between BFG and the corresponding deployment scenario in mandatory/optional column in UE features list, and capturing and managing the association in separate specification as in Alt 1 could be redundant. Proposal #1: Confirm the working assumption by adopting Alt 2 (Capture an association between the basic FGs required to be supported and a certain NR-U deployment scenario in the UE features list). Considering the deployment scenarios identified in NR-U WID [2], in our view, the following feature groups can be considered for BFGs applicable to a specific deployment scenario, with the principle that the minimum set of BFGs should be defined for each deployment scenario. - FG 10-1 (UL channel access for dynamic channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-1a (UL channel access for semi-static channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-2 (SSB-based RRM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2a (SSB-based RRM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2b (MIB reading on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D - FG 10-2c (SSB-based RLM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2d (SSB-based RLM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2e (SIB1 reception on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D - FG 10-3 (PRB interlace mapping for PUSCH): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-3a (PRB interlace mapping for PUCCH): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E For all of feature groups other than above BFGs, we propose to remove “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column. Proposal #2: For FG 10-1/1a/2/2a/2b/2c/2d/2e, update “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column as follows, where each scenario is given by RP-191575: - FG 10-1 (UL channel access for dynamic channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-1a (UL channel access for semi-static channel access mode): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-2 (SSB-based RRM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2a (SSB-based RRM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios A, B, C, D, and E - FG 10-2b (MIB reading on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D - FG 10-2c (SSB-based RLM [for dynamic channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2d (SSB-based RLM [for semi-static channel access mode]): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E - FG 10-2e (SIB1 reception on unlicensed cell): This FG is optional with capability signaling and is required to be supported for the scenarios C and D Proposal #3: Remove “This FG may be a part of basic operation for a particular scenario” in mandatory/optional column for all FGs except for FG 10-1/1a/2/2a/2b/2c/2d/2e. Proposal #4: For FG 10-3/3a, add the following statement in mandatory/optional column, where each scenario is given by RP-191575: - FG 10-3 (PRB interlace mapping for PUSCH): This FG is optional with capability signaling and is required to be supported for the scenarios A (except for DL-only), B, C, D, and E - FG 10-3a (PRB interlace mapping for PUCCH): This FG is optional with capability signaling and is required to be supported for the scenarios B, C, D, and E |
| [13] | Table 1 and 2 below provides our proposed mapping for the basic FGs to the NR-U deployment scenarios for dynamic and semi-static channel access modes, respectively. In the tables, ‘O’ indicates the FG can be optional for the corresponding scenario, while ‘M’ indicates the FG should be mandatory for the corresponding scenario. Table 1: Proposed mapping for the basic FGs to the NR-U deployment scenarios assuming dynamic channel access mode Index; FG; Scen. A (DL-only); Scen. A (UL+DL); Scen. B; Scen. C; Scen. D; Scen. E; Comment. 10-1; UL channel access for dynamic channel access mode; N/A; M; M; M; N/A; M; Implies support of transmission of relevant UL channels/signals as well. 10-2; SSB-based RRM [for dynamic channel access mode]; M; M; M; M; M; M. 10-2b; MIB reading on unlicensed cell; N/A; N/A; M; M; M; M. 10-2c; SSB-based RLM [for dynamic channel access mode]; N/A; N/A; M; M; M; M. 10-2e; SIB1 reception on unlicensed cell; N/A; N/A; M; M; M; M. 10-2f; Support monitoring of extended RAR window; N/A; N/A; M; M; M; M. 10-30; Support channel occupancy duration indicator field in DCI 2_0; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; Depends on whether SFI is mandatory or optional in DCI 2_0. Table 2: Proposed mapping for the basic FGs to the NR-U deployment scenarios assuming semi-static channel access mode Index; FG; Scen. A; Scen. A (UL+DL); Scen. B; Scen. C; Scen. D; Scen. E; Comment. 10-1a; UL channel access for semi-static channel access mode; M (DL+UL); M; M; M; N/A; M; Not applicable to Scenario A with DL-only operation.. 10-2a; SSB-based RRM [for semi-static channel access mode]; M; M; M; M; M; M. 10-2b; MIB reading on unlicensed cell; N/A; N/A; M; M; M; M. 10-2d; SSB-based RLM [for semi-static channel access mode]; N/A; N/A; M; M; M; M. 10-2e; SIB1 reception on unlicensed cell; N/A; N/A; M; M; M; M. 10-2f; Support monitoring of extended RAR window; N/A; N/A; M; M; M; M. 10-30; Support channel occupancy duration indicator field in DCI 2_0; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; [O/M]; This depends on whether “end of FFP is treated as end of COT in the specification”. Other aspects can be considered when mapping basic feature groups to scenarios. One such aspect is wideband operation modes 2/3, where such a mapping is proposed in Table 3 below. Given the simpler mapping in this case, it might be straightforward to handle the relationships by mean of pre-requisites in the FG table as well, depending on the final decisions regarding the FGs themselves and applicability to deployment scenarios. Table 3: Basic FGs for wideband operation mode 2/3 Index; FG; Comment. 10-20; Support search space set configuration with freqMonitorLocation-r16; Applies for scenarios A, B, C, D, E. 10-29; Support available RB set indicator field in DCI 2_0; Applies for scenarios A, B, C, D, E. [10-19a]; [Support DL reception in a carrier with intra-cell guard-bands]; If FG is confirmed. [10-19b]; [Support UL transmission with subset of RB sets passing LBT]; If FG is confirmed. |

# Reference

[1]	R1-2003198	Summary on email discussion [100b-e-NR-UEFeatures-Remaining] NR-unlicensed	Moderator (NTT DOCOMO, INC.)

[2]	R1-2003416	Discussion on UE features for NRU	vivo

[3]	R1-2003460	Discussion on the remaining issues of the UE features for NR-U	ZTE, Sanechips

[4]	R1-2003694	Views on Rel-16 UE features for NR-U	MediaTek Inc.

[5]	R1-2003848	UE features for NR-U	Ericsson

[6]	R1-2003894	UE features for NR-U	Samsung

[7]	R1-2004019	Discussion on UE features for NR-U	LG Electronics

[8]	R1-2004091	Discussion on UE feature for NRU	OPPO

[9]	R1-2004152	Rel-16 UE features for NR-U	Huawei, HiSilicon

[10]	R1-2004241	Discussions on NR-U UE features	Apple

[11]	R1-2004402	UE features for NR-U	NTT DOCOMO, INC

[12]	R1-2004477	Discussion on NR-U UE features	Qualcomm Incorporated

[13]	R1-2004560	On UE features NR Unlicensed	Nokia, Nokia Shanghai Bell

[14]	R1-2004062	Discussion on the support of SRS transmission in all symbols of a slot	OPPO