3GPP TSG-RAN WG1 Meeting #101-e	R1-2004429

e-Meeting, May 25th – June 5th, 2020

Agenda Item:	7.1.2

Source:	Ericsson

Title:	Clarification on sp-CSI-RS capability

Document for:	Discussion, Decision

# 1	Issue

In TS 38.306, the capability for sp-CSI-RS is captured as follows:

| sp-CSI-RS Indicates whether the UE supports semi-persistent CSI-RS. | UE | Yes | No | Yes |
| --- | --- | --- | --- | --- |

However, it is not clear if this capability refers to

- only SP NZP CSI-RS,

- only SP ZP CSI-RS, or

- both SP ZP CSI-RS and SP NZP CSI-RS

Hence, some clarification is needed to the text in TS 38.306.

Since SP ZP CSI-RS was introduced primarily for the purpose of rate matching PDSCH around SP NZP CSI-RS, it is natural associate both SP ZP CSI-RS and SP NZP CSI-RS with the sp-CSI-RS capability.

Hence, the following clarification needs to be added to 38.306:

#### 4.2.7.10        Phy-Parameters

| sp-CSI-RS Indicates whether the UE supports semi-persistent NZP CSI-RS and semi-persistent ZP CSI-RS. | UE | Yes | No | Yes |
| --- | --- | --- | --- | --- |

Hence, it is proposed to send an LS to RAN2 that the above clarification needs to be added to 38.306.

Send an LS to RAN2 that asks for a clarification to be added to TS 38.306 that the capability sp-CSI-RS indicates whether the UE supports semi-persistent NZP CSI-RS and semi-persistent ZP CSI-RS.