3GPP TSG RAN WG1 Meeting #101-e	R1-2004610

E-meeting, 25 May – 5 June, 2020

Agenda Item:	8.4.3

Source:	Huawei, HiSilicon

Title:	System-level simulation assumptions and evaluation metrics for coverage enhancement

Document for:	Discussion and Decision

Introduction

In RAN#86 meeting, the study item (SI) of NR coverage enhancement was approved [1]. The objective of this SI is to identity the limited channels and study potential coverage enhancement solutions for specific scenarios for both FR1 and FR2. The link-level simulation is specified to determine the basic coverage performance for a variety of downlink and uplink channels and evaluate the performance gain for potential solutions.  In the link-level simulation, the users’ geometry distribution and network’s layout cannot be characterized, which would have impacts on antenna gain, interference strength, etc.. As a compensated evaluation methodology, the system-level simulation is useful for performance evaluation from a network’s aspect.

In this contribution, considerations on the system-level simulation for FR1 are provided for NR coverage enhancement.

Considerations on system-level simulation

In this section, the evaluation scenarios, evaluation metrics, and the detailed evaluation parameters will be discussed. These evaluation parameters can be considered as the baseline simulation assumption to evaluation the performance of the baseline and the gain for the potential solutions. In [1], the performance evaluation for uplink PUSCH and PUCCH channels are prioritized for FR1. To evaluate the achieved rate for eMBB and VoIP services, at least PUSCH should be evaluated by the system-level simulation.

Focused scenarios and configurations

For the system-level simulation, the focused evaluation scenarios and configurations are included in Table 1.  In IMT-2020 self-evaluation [2], the inter-site distance for rural scenario can be 1732 m and 6000 m. In addition, the extreme long distance (e.g. 30 km) is also discussed for NR coverage enhancement.

Table 1 Focused evaluation scenarios and configurations for FR1

| Scenario | Urban | Rural |
| --- | --- | --- |
| Service type | eMBB service, VoIP service | eMBB service, VoIP service |
| Physical channel for evaluation | PUSCH | PUSCH |

Focused solutions

As discussed in [4], the potential solutions are proposed to improve the uplink coverage, including cross-slot joint channel estimation, DMRS-less transmission, finer RV transmission, instant higher power transmission, etc.. In addition, other solutions are also mentioned during the email discussion, e.g. sub-band precoding and joint receiving for multiple TRxPs. For cross-slot joint channel estimation and DMRS-less transmission, the link-level simulation is enough to identify the performance gain. For other solutions, the system-level simulation can be considered as a supplementary evaluation methodology to determine performance gain. In the system-level simulation, the channel model including large scale parameters and small scale parameters, geometry location for UE and BS, scheduling, and interference are modelled. For higher power transmission, the scheduled duration and traffic load have impacts on the opportunity for using instant higher power.  The variation of interference cannot be evaluated in the link-level simulation when instant higher power is used. The channel model in the link-level is less accurate than that in the system-level simulation, which would impact the performance for sub-band precoding. In addition, the performance gain for multiple TRxP receiving should be evaluated in the system-level simulation.

Observation 1: System-level simulation could be used for performance evaluation for the specific solutions, e.g. multiple TRxP receiving.

Evaluation metrics

The target rate for different scenarios is defined in [1], which can ensure the good user experience for eMBB and VoIP services. In IMT-2020 self-evaluation [3], user experienced data rate is defined as one of metrics related to the minimum technical performance of IMT-2020 candidate radio interface technologies. For coverage enhancement, use experienced data rate can be reused to identify the performance gain for potential solutions.

In [3], user experienced data rate is the 5% point of the cumulative distribution function (CDF) of the user throughput. User throughput (during active time) is defined as the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time. In case of one frequency band and one layer of transmission reception points (TRxP), the user experienced data rate could be derived from the 5th percentile user spectral efficiency through equation (1). Let W denote the channel bandwidth and SEuser denote the 5th percentile user spectral efficiency. Then the user experienced data rate, Ruser is given by:

Ruser = W × SEuser 	(1)

The 5th percentile user spectral efficiency defined in [3] is the 5% point of the CDF of the normalized user throughput.

Proposal 1: If the system-level evaluation is needed to identify the performance gain for potential solutions, take the user experienced data rate as the evaluation metric.

Detailed evaluation parameters

For Urban and Rural evaluation scenarios, the evaluation parameters defined for IMT-2020 self evaluation [2] can be considered as references for coverage evaluation. It is noted that Urban scenario and Rural scenario with extreme long distance are not defined in [2], but most of evaluation parameters for Dense Urban – eMBB test environment and Rural configuration A – eMBB test environment can be applied for these two scenarios, respectively. The detailed parameters are discussed in the following sections.

Evaluation parameters for Urban scenario

The detailed evaluated parameters for PUSCH in Urban scenario are illustrated in Table 2. Other evaluation parameters align with the Dense Urban – eMBB test environment [2].

Table 2 Evaluation parameters for PUSCH in Urban scenario

| Parameter |  | Values |
| --- | --- | --- |
| Scenario |  | Urban |
| Carrier frequency |  | 4 GHz |
| Waveform |  | CP-OFDM/DFT-S-OFDM |
| Inter-site distance |  | 500 m |
| Channel model |  | UMa_B [2] |
| Duplexing mode |  | TDD |
| Subcarrier spacing |  | 30 kHz |
| TDD frame structure |  | DDDSU / DDDDDDDSUU / DDDSUDSUUD |
| Symbols number per slot |  | 14 |
| Number of antenna elements per TRxP |  | Cross-polarized antenna, (M,N,P,Mg,Ng) = (12,8,2,1,1) |
| Number of TXRU per TRxP |  | 64Rx (vertical 4-to-12) |
| Number of antenna elements per UE |  | 2Tx with 0°,90° polarization |
| Maximal transmit power for UE |  | 26 dBm |
| Mechanic tilt |  | 90 degree in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 105 degree in LCS |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF |
| MIMO mode |  | SU-MIMO with rank adaptation |
| BS receiver type |  | MMSE-IRC |
| UE precoder scheme |  | Codebook / Non-codebook based |
| Sub-band granularity |  | Wideband / Subband |
| UL CSI derivation |  | Non-precoded SRS / Preceded SRS based |
| Power control |  | α=0.6， P0=-60 dBm (Companies are encouraged to report this value ) |
| Power backoff model |  | Follow TS 38.101 in Section 6.2.2; |
| Overhead | PUCCH | 2 RBs and 14 OFDM symbols |
|  | DMRS | Type I, 1 FL DMRS symbol, no multiplexing with data |
|  | SRS | 2 symbols per 5 slots, |
|  | PTRS | N/A |
| Channel estimation |  | Non-ideal |
| Traffic model |  | Full buffer |

Evaluation parameters for Rural scenario

The following evaluated parameters can be considered for PUSCH simulation in Rural scenario. Other evaluation parameters align with Rural configuration A – eMBB test environment [2].

Table 3 Evaluation parameters for PUSCH in Rural scenario

| Parameter |  | Values |  |
| --- | --- | --- | --- |
| Scenario |  | Rural configuration 1 | Rural configuration 2 (extreme long distance) |
| Carrier frequency |  | 2 GHz for FDD; 4 GHz for TDD | 700 MHz |
| Waveform |  | CP-OFDM/DFT-S-OFDM | CP-OFDM/DFT-S-OFDM |
| ISD distance |  | 1732 m | 30 km |
| Channel model |  | RMa_B [2] | RMa_B [2] |
| Duplexing mode |  | FDD / TDD | FDD |
| Subcarrier spacing |  | 15 kHz for FDD; 30 kHz for TDD | 15 kHz for FDD |
| TDD frame structure |  | DDDSU / DDDDDDDSUU / DDDSUDSUUD | N/A |
| Symbols number per slot |  | 14 | 14 |
| Number of antenna elements per TRxP |  | Cross-polarized antenna, (M,N,P,Mg,Ng) = (8,4,2,1,1). | Cross-polarized antenna, (M,N,P,Mg,Ng) = (8,4,2,1,1) |
| Number of TXRU per TRxP |  | 8Rx (vertical 1-to-8) | 8Rx (vertical 1-to-8) |
| Number of antenna elements per UE |  | 1Tx | 1Tx |
| Maximal transmit power for UE |  | 23 dBm | 23 dBm |
| Mechanic tilt |  | 90 degree in GCS (pointing to the horizontal direction) | 90 degree in GCS (pointing to the horizontal direction) |
| Electronic tilt |  | 105 degree in LCS | 100 degree in GCS (pointing to the horizontal direction) |
| UT attachment |  | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 | Based on RSRP (Eq. (8.1-1) in TR36.873) from port 0 |
| Scheduling |  | SU-PF | SU-PF |
| MIMO mode |  | SU-SIMO with rank adaptation | SU-SIMO |
| BS receiver type |  | MMSE-IRC | MMSE-IRC |
| Sub-band granularity |  | Wideband / Subband | Wideband / Subband |
| UL CSI derivation |  | Non-precoded SRS | Non-precoded SRS |
| Power control |  | α=0.8， P0=-76 dBm; (Companies are encouraged to report this value ) | α=0.8， P0=-76 dBm; (Companies are encouraged to report this value ) |
| Power backoff model |  | Follow TS 38.101 in Section 6.2.2; | Follow TS 38.101 in Section 6.2.2; |
| Overhead | PUCCH | 2 RBs and 14 OFDM symbols | 2 RBs and 14 OFDM symbols |
|  | DMRS | Type I, 1 FL DMRS symbol + 1 additional DMRS symbol, no multiplexing with data | Type I, 1 FL DMRS symbol + 1 additional DMRS symbol, no multiplexing with data |
|  | SRS | 2 symbols per 5 slots | 2 symbols per 5 slots |
|  | PTRS | N/A | N/A |
| Channel estimation |  | Non-ideal | Non-ideal |
| Traffic model |  | Full buffer | Full buffer |

Proposal 2: If the system-level evaluation is needed to identify the performance gain for potential solutions, use the evaluated parameters as listed in Table 2 and Table 3 in Section 2.4.

Conclusions

In this contribution, the evaluation metric and evaluation parameters for system-level simulation are discussed. These evaluation parameters can be considered as the baseline simulation assumption to evaluation the performance gain for the potential solutions. The observations and proposals are listed as following.

Observation 1: System-level simulation could be used for performance evaluation for the specific solutions, e.g. multiple TRxP receiving.

Proposal 1: If the system-level evaluation is needed to identify the performance gain for potential solutions, take the user experienced data rate as the evaluation metric.

Proposal 2: If the system-level evaluation is needed to identify the performance gain for potential solutions, use the evaluated parameters as listed in Table 2 and Table 3 in Section 2.4.

- References

RP-193240, “New SID on NR coverage enhancement”, RAN#86, Sitges, Spain, December, 2019.

Report ITU-R M.2412-0, “Guidelines for evaluation of radio interface technologies for IMT-2020”, October, 2017.

Report ITU-R M.2410-0, “Minimum requirements related to technical performance for IMT-2020 radio interface(s)”, October, 2017.

R1-2004155, “Overview of coverage enhancement: scenarios, channels, services and potential solutions”, RAN1#101-e, Huawei, HiSilicon, May 2020.