3GPP TSG RAN WG1 Meeting #101-e				 R1-2004903

E-meeting, May 25 – June 5, 2020

Title:	[DRAFT] Reply LS on transmit power of CSI-RS across different occasions

Response to:	R1-2003273 (R4-2005377)

Release:	Rel-16

Work Item:	NR_unlic-core,

Source:	RAN WG1

To:	RAN WG4

Cc:

Contact Person:

Name:	Jiayin Zhang

Tel. Number:

E-mail Address:	zhangjiayin@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

RAN1 received the RAN4 LS on the transmit power of CSI-RS across different occasions and is requested to confirm whether a UE can assume that CSI-RS (for L1-RSRP, RLM, BFD and CBD) is transmitted with the same transmit power across different occasions during the measurement period, and whether this also applies to SSB-based RRM measurements.

RAN1 confirms that UE can assume same transmit power of NZP CSI-RS or SSB (for L1-RSRP, RLM, BFD CBD and RRM) across different occasions and can average the measurement during the measurement period.

2. Actions:

To RAN4.

ACTION: 	RAN1 kindly requests RAN4 to take the above information into account when considering the design of L1-RSRP, RLM, BFD, CBD and SSB-based RRM measurements for NR-U

3. Date of Next TSG-RAN1 Meetings:

TSG RAN WG1 Meeting #102-e	Aug 2020				e-Meeting

TSG RAN WG1 Meeting #102-bis	12 – 16 Oct 2020			China