3GPP TSG RAN WG1 #101	DRAFT R1-2004638

e-Meeting, May 25th – June 5th, 2020

Agenda item:	7.2.10.4

Source:	Moderator (Nokia)

Title:	FL summary on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH

Document for:	Discussion and Decision

1	Introduction

This contribution is a pre-meeting summary of the 5 documents submitted for the AI7.2.10.4 on aperiodic CSI-RS triggering with different numerology between CSI-RS and triggering PDCCH.

< Companies are invited to provide comments on the FL proposals to this draft in section 3 >

2	Summary of issues addressed in the Tdocs

| Issue # | Description | Source |
| --- | --- | --- |
| 1 | For cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption. | Vivo #1 |
| 2 | Proposal 2 (vivo): Capture the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier. Proposal 2 (Qualcomm): In case of same numerology A-CSI RS triggering, when the offset between A-CSI RS and triggering DCI is less than beamSwitchTiming, capture the default QCL agreement in specification. - If no CORESET configured on the carrier for receiving the A-CSI RS, UE receives the A-CSI RS by applying the QCL parameters of the activated PDSCH TCI state with lowest ID. | Vivo #2 QCom #2 |
| 3 | Support to define the FG 18-5a for default QCL assumption for cross-carrier A-CSI-RS triggering. | QCom #3 |
| 4 | RRC parameter name alignment - CORESET ID controlResourceSetID - minimumSchedulingOffset minimumSchedulingOffsetK0-r16 | Oppo |
| 5 | Correct the indentation level for three bullets for default QCL assumption determination. | Huawei, QCom #1 |
| 6 | Introduction of a missing RRC parameter in three places - aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16 | Ericsson |

3	Discussion on the scope of the RAN1#101

Issue #1: [Vivo #1]

- For cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption. [vivo]

FL proposal: Discuss the proposal in RAN1#101

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #1 |
| --- | --- |
| MTK | We are fine to discuss the proposal. For cross-carrier ACSI-RS triggering, according to GTW session of DC/CA UE features in RAN1 #100-bis-e, companies agreed that UE capabilities 18-6a “Default QCL assumption for cross-carrier A-CSI-RS triggering” apply to both same/different numerologies. Therefore, [enableDefaultBeamForCCS] should also apply to both same/different numerologies. |
| vivo | Support |

Issue #2: [vivo #2, Qualcomm #2]

Proposal 2 [vivo]: Capture the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier.

Proposal 2 [Qualcomm]: In case of same numerology A-CSI RS triggering, when the offset between A-CSI RS and triggering DCI is less than beamSwitchTiming, capture the default QCL agreement in specification.

- If no CORESET configured on the carrier for receiving the A-CSI RS, UE receives the A-CSI RS by applying the QCL parameters of the activated PDSCH TCI state with lowest ID.

FL proposal: Discuss the two proposals in RAN1#101

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #2 |
| --- | --- |
| MTK | We are fine to discuss the proposal. For cross-carrier ACSI-RS triggering, according to GTW session of DC/CA UE features in RAN1 #100-bis-e, companies agreed that UE capabilities 18-6a “Default QCL assumption for cross-carrier A-CSI-RS triggering” apply to both same/different numerologies. Therefore, [enableDefaultBeamForCCS] should also apply to both same/different numerologies. |
| vivo | Support |

Issue #3: [Qualcomm #3]

- Support to define the FG 18-5a for default QCL assumption for cross-carrier A-CSI-RS triggering.

FL proposal: Discuss the proposal in RAN1#101

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #3 |
| --- | --- |
| MTK | This proposal seems already agreed before. To our understanding, according to GTW session of DC/CA UE features in RAN1 #100-bis-e, companies agreed that UE capabilities 18-6a “Default QCL assumption for cross-carrier A-CSI-RS triggering” apply to both same/different numerologies. |
| vivo | Better to discuss in UE feature session. |

Issue #4: [Oppo]

- RRC parameter name alignment

  - CORESET ID  controlResourceSetID

  - minimumSchedulingOffset  minimumSchedulingOffsetK0-r16

FL proposal: Discuss the proposal in RAN1#101

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #4 |
| --- | --- |
| MTK | We are fine to discuss the proposal. |
| vivo | We are fine to discuss the proposal. |

Issue #5: [Huawei, Qualcomm #1]

- Correct the indentation level for three bullets for default QCL assumption determination.

FL proposal: Discuss the proposal in RAN1#101

Comments on the FL proposal

| Company | Comment on FL proposal wrt. issue #5 |
| --- | --- |
| MTK | We are fine to discuss the proposal. |
| vivo | We are fine with the proposal. |

Issue #6: [Ericsson]

- Introduction of a missing RRC parameter in three places

  - aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16

FL proposal: Discuss the proposal in RAN1#101

Discussion on views wrt. issue 6:

| Company | Comment on FL proposal wrt. issue #5 |
| --- | --- |
| MTK | We are fine to discuss the proposal. |
| vivo | We are fine to discuss the proposal. |

4	Conclusion on the scope of the RAN1#101

To be completed

References

- R1-2003412 Remaining issues on aperiodic CSI-RS triggering, vivo

- R1-2004058 Text proposals for aperiodic CSI-RS triggering with different numerologies, OPPO

- R1-2004150 Remaining issues on the A-CSI RS triggering with different numerology, Huawei, HiSilicon

- R1-2004204 Maintenance for cross-carrier CSI-RS triggering, Ericsson

- R1-2004475 Remaining issues for aperiodic CSI-RS triggering with different numerology, Qualcomm Inc.

Annex – Proposals of the submitted Tdocs

R1-2003412 Remaining issues on aperiodic CSI-RS triggering, vivo

Proposal 1:

- For cross-carrier A-CSI-RS triggering with different SCS, RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption.

TS 38.214

5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies < Unchanged parts are omitted > Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else if the UE is configured with [enableDefaultBeamForCCS], when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. < Unchanged parts are omitted >

Proposal 2:

- Capture the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS when the scheduling offset is smaller than the threshold and no CORESET is configured on A-CSI-RS carrier.

- RRC parameter [enableDefaultBeamForCCS] is used to enable the default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS.

TS 38.214

5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology < Unchanged parts are omitted > - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else if the UE is configured with [enableDefaultBeamForCCS], - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. < Unchanged parts are omitted >

R1-2004058 Text proposals for aperiodic CSI-RS triggering with different numerologies, OPPO

Text Proposal for 38.214:

##### 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions: Beam switch timing: (omitted part) - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. (omitted part) Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset, including the case that the UE is not configured with minimumSchedulingOffsetK0-r16 for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states.. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where (omitted part)

R1-2004150 Remaining issues on the A-CSI RS triggering with different numerology, Huawei, HiSilicon

Proposal 1: Adopt the following TP in the TS 38.214 [correcting the indentation of the else-if branch]

5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies When the triggering PDCCH and the triggered aperiodic CSI-RS are of different numerologies, the behavior defined in 5.2.1.5.1 for the case where the numerologies are the same applies with the following exceptions: Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d∙2μCSIRS/2μPDCCH in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+ d∙2μCSIRS/2μPDCCH in CSI-RS symbols when the reported value of beamSwitchTiming is one of the values of {224, 336}, the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. <Unchanged parts omitted>

R1-2004204 Maintenance for cross-carrier CSI-RS triggering, Ericsson

Proposal: Adopt following TP (changes in blue [highlight on top of the April-agreed CR]) for 38.214.

5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology <omitted text> When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 16, 24} slots. If the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states , the CSI-RS triggering offset is fixed to zero. The aperiodic triggering offset of the CSI-IM follows offset of the associated NZP CSI-RS for channel measurement. <omitted text> 5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies <omitted text> Aperiodic CSI-RS timing: - When the aperiodic CSI-RS is used with aperiodic CSI reporting, the CSI-RS triggering offset X is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16, including the case that the UE is not configured with [minimumSchedulingOffset] for any DL or UL BWP and all the associated trigger states do not have the higher layer parameter qcl-Type set to 'QCL-TypeD' in the corresponding TCI states.. The CSI-RS triggering offset has the values of {0, 1,…,31} slots when the µPDCCH < µCSIRS and {0, 1, 2, 3, 4, 16, 24} when the µPDCCH > µCSIRS.. The aperiodic CSI-RS is transmitted in a slot , if UE is configured with ca-SlotOffset for at least one of the triggered and triggering cell, and Ks = , otherwise, and where - n is the slot containing the triggering DCI, X is the CSI-RS triggering offset in the numerology of CSI-RS according to the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffsetExt-r16, - μCSIRS and μPDCCH are the subcarrier spacing configurations for CSI-RS and PDCCH, respectively, <omitted text>

R1-2004475 Remaining issues for aperiodic CSI-RS triggering with different numerology, Qualcomm Inc.

Proposal 1: Adopt the proposed text proposal 1 for 5.2.1.5.1a in TS 38.214 to correct the indent level for three bullets for default QCL assumption determination.

5.2.1.5.1a Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have different numerologies Beam switch timing: - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info is smaller than the UE reported threshold beamSwitchTiming + d in CSI-RS symbols, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48+ d when the reported value of beamSwitchTiming is one of the values of {224, 336} and where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero - if one of the associated trigger states has the higher layer parameter qcl-Type set to 'QCL-TypeD', - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming + d in CSI-RS symbols when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48+ d when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else, - if at least one CORESET is configured for the BWP in which the aperiodic CSI-RS is to be received, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET-ID in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored. - else, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received. - If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources is equal to or greater than the UE reported threshold beamSwitchTiming + d in CSI-RS symbols, when the reported value is one of the values of {14,28,48}, or is equal to or greater than 48+d when the reported value of beamSwitchTiming is one of the values of {224, 336} , the UE is expected to apply the QCL assumptions in the indicated TCI states for the aperiodic CSI-RS resources in the CSI triggering state indicated by the CSI trigger field in DCI. ----------------------------------------------------------- End of text proposal --------------------------------------------------

Proposal 2: In case of same numerology A-CSI RS triggering, when the offset between A-CSI RS and triggering DCI is less than beamSwitchTiming, capture the default QCL agreement in specification.

- If no CORESET configured on the carrier for receiving the A-CSI RS, UE receives the A-CSI RS by applying the QCL parameters of the activated PDSCH TCI state with lowest ID.

Proposal 3: Support to define the FG 18-5a for default QCL assumption for cross-carrier A-CSI-RS triggering.

- Adopt the proposed text proposal 2 in 5.2.1.5.1 in TS 38.214

Based on the two proposals 2 and 3 above, the corresponding text proposal is provided as follows

5.2.1.5.1 Aperiodic CSI Reporting/Aperiodic CSI-RS when the triggering PDCCH and the CSI-RS have the same numerology >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted <<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< If the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without the higher layer parameter repetition is smaller than the UE reported threshold beamSwitchTiming, as defined in [13, TS 38.306], when the reported value is one of the values of {14, 28, 48}, or is smaller than 48 when the reported value of beamSwitchTiming is one of the values of {224, 336}. - if there is any other DL signal with an indicated TCI state in the same symbols as the CSI-RS, the UE applies the QCL assumption of the other DL signal also when receiving the aperiodic CSI-RS. The other DL signal refers to PDSCH scheduled with offset larger than or equal to the threshold timeDurationForQCL, as defined in [13, TS 38.306], aperiodic CSI-RS scheduled with offset larger than or equal to the UE reported threshold beamSwitchTiming when the reported value is one of the values {14,28,48}, aperiodic CSI-RS scheduled with offset larger than or equal to 48 when the reported value of beamSwitchTiming is one of the values {224, 336}, periodic CSI-RS, semi-persistent CSI-RS; - else if the active BWP of the serving cell for receiving the aperiodic CSI-RS has configured ControlResourceSet, when receiving the aperiodic CSI-RS, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored; - else if the UE is configured with [enableDefaultBeamForCCS] and when receiving the aperiodic CSI-RS, the UE applies the QCL assumption of the lowest-ID activated TCI state applicable to the PDSCH within the active BWP of the cell in which the CSI-RS is to be received.