3GPP TSG RAN WG1 #101	R1-200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	moderator (vivo)

Title:	Feature lead summary on [101-e-NR-unlic-NRU-CG-01] week2

Agenda Item:	7.2.2.2.4

Document for:	Discussion and Decision

Introduction

Following is agreed in RAN1#101e

Agreement:

- Value range of the RRC parameter cg-COT-SharingList-r16 is 1709

- The value range of the RRC parameters cg-StartingFullBW-InsideCOT-r16, cg-StartingFullBW-OutsideCOT-r16 is 7

  - cg-StartingFullBW-InsideCOT-r16  SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)

  - cg-StartingFullBW-OutsideCOT-r16  SEQUENCE (SIZE (1..7)) OF INTEGER (0..6)

- The value range of the RRC parameter cg-COT-SharingOffset-r16 has been agreed in RAN1#100b-e, it is confirmed that the step size is 14 symbols.

Agreement:

The maximum configurable value for cg-nrofPUSCH-InSlot-r16 can be set as 7

Agreement:

For a given shared COT, UE should provide consistent COT sharing information in multiple consecutive PUSCHs in the same UE-initiated COT.

Discuss TP(s) needed for above agreements and TPs for HARQ-ACK for CBG based PUSCH (Issue 8) until 6/3.

Further discuss TP for 3rd agreement above, and the two alternatives under issue#8.

Remaining issues

TP for the agreement under issue6

TP for 38.212

-------------------------------------start---------------------------------------------

6.3.2.1.3	CG-UCI

For CG-UCI bits transmitted on a CG PUSCH, the CG-UCI bit sequence a0, a1, a2, a3, …,aA-1  is determined as follows:

-	set ai=oiCG-UCI  for i=0,1, …, OCG-UCI-1 and A=OCG-UCI, where the CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1, mapped in the order from upper part to lower part.

Table 6.3.2.1.3-1: Mapping order of CG-UCI fields

| Field | Bitwidth |
| --- | --- |
| HARQ process number | 4 |
| Redundancy version | 2 |
| New data indicator | 1 |
| Channel Occupancy Time (COT) sharing information | log2C if both higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 and higher layer parameter cg-COT-SharingList-r16 are configured, where C is the number of combinations configured in cg-COT-SharingList-r16; 1 if higher layer parameter ULtoDL-CO-SharingED-Threshold-r16 is not configured and higher layer parameter cg-COT-SharingOffset-r16 is configured; 0 otherwise; For a given shared COT, UE should provide consistent COT sharing information in multiple consecutive PUSCHs in the same UE-initiated COT. |

-------------------------------------end---------------------------------------------

| Company | Comments |
| --- | --- |
| Intel | We are Ok with the proposed TP, with the following minor edits: For a given shared COT, a UE should provide consistent COT sharing information in multiple consecutive PUSCHs occurring within in the same UE’s initiated COT. |

Issue 8: HARQ-ACK for CBG based PUSCH

Proposal:

- Agree to introduce the first correction in TP#1 as below

================= Start of TP#1 for TS 38.213 ====================

10.5 HARQ-ACK information for PUSCH transmissions

< Unchanged Texts Omitted >

For a PUSCH transmission scheduled by a DCI format, if a UE is provided PUSCH-CodeBlockGroupTransmission for a serving cell, a value of HARQ-ACK information for a transport block of a corresponding HARQ process number is ACK if at least one of CBGs for the PUSCH is ACK; otherwise, a value of HARQ-ACK information is NACK

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK.

< Unchanged Texts Omitted >

========================== End of TP#1 for TS 38.213 =========================

- Down select between 2 alternatives below:

  - Alt1: second correction in TP#1

================= Start of TP#1 for TS 38.213 ====================

For an initial PUSCH transmission configured by ConfiguredGrantConfig, if a UE is provided PUSCH-CodeBlockGroupTransmission for a serving cell, a value of HARQ-ACK information for a transport block of a corresponding HARQ process number is ACK if the transport block all of CBGs for the PUSCH are is ACK; otherwise, a value of HARQ-ACK information is NACK.

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK.

< Unchanged Texts Omitted >

======================== End of TP#1 for TS 38.213 =========================

  - Alt2: TP#2

=================== Start of TP for TS 38.213 =======================

10.5 HARQ-ACK information for PUSCH transmissions

< Unchanged Texts Omitted >

For a PUSCH transmission scheduled by a DCI format, if a UE is provided PUSCH-CodeBlockGroupTransmission for a serving cell, a value of HARQ-ACK information for a transport block of a corresponding HARQ process number is ACK if the transport block for all of CBGs for the PUSCH  is ACK; otherwise, a value of HARQ-ACK information is NACK.

For a PUSCH transmission scheduled by a DCI format, HARQ-ACK information for a transport block of a corresponding HARQ process number is valid if a first symbol of the PDCCH reception is after a last symbol of the PUSCH transmission or, if the PUSCH transmission is over multiple slots,

-	after a last symbol of the PUSCH transmission in a first slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is ACK.

-	after a last symbol of the PUSCH transmission in a last slot from the multiple slots by a number of symbols provided by cg-minDFIDelay-r16, if a value of the HARQ-ACK information is NACK.

< Unchanged Texts Omitted >

================ End of TP for TS 38.213 ==========================

| Company | Comments |
| --- | --- |
| Intel | Our understanding is that for the TPs listed above, we should break the discussion as follows: - HARQ interpretation for CGB based retransmissions for DG PUSCH received over a DFI: for this topic we should down-select between the first correction in TP#1 and Alt2. Among them we prefer Alt2. - HARQ interpretation for an initial CBG-based CG PUSCH transmission: in this case we are OK with Alt1. |

- Agree to introduce TP#3

=================== Start of TP for TS 37.213 =======================

*** Unchanged text is omitted ***

4.2.2.2 	Contention window adjustment procedures for UL transmissions scheduled/configured by gNB

If a UE transmits transmissions using Type 1 channel access procedures that are associated with channel access priority class p on a channel, the UE maintains the contention window value CWp and adjusts CWp for those transmissions before step 1 of the procedure described in subclause 4.2.1.1, using the following steps:

1)	For every priority class p∈1,2,3,4, set CWp=CWmin,p;

2)	If HARQ-ACK feedback is available after the last update of CWp,  go to step 3. Otherwise, if the UE transmission after procedure described in subclause 4.2.1.1 does not include a retransmission or is transmitted within a duration Tw from the end of the reference duration corresponding to the earliest UL transmission burst after the last update of CWp transmitted after the procedures described in subclause 4.1.1, go to step 5; otherwise go to step 4.

3)	The HARQ-ACK feedback(s) corresponding to PUSCH(s) in the reference duration for the latest UL transmission burst for which HARQ-ACK feedback is available is used as follows:

a.	If at least one HARQ-ACK feedback is 'ACK' for PUSCH(s) with transport block (TB) based feedback or at least 10% of HARQ-ACK feedbacks is 'ACK' for PUSCH(s) with code block group (CBG) based feedback go to step 1; otherwise go to step 4.

4)	Increase CWp for every priority class p∈1,2,3,4 to the next higher allowed value;

5)	For every priority class p∈1,2,3,4, maintain CWp as it is; go to step 2.

*** Unchanged text is omitted ***

---------------------------------------------------

# References

[1] R1-2004796, “Feature lead summary on [101-e-NR-unlic-NRU-CG-01]”, vivo, RAN1#101e