3GPP TSG RAN WG1 Meeting #101-E						     R1- 200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (Intel Corporation)

Title:	TPs based on outcome of [101-e-NR-5G_V2X_NRSL-Mode-2-01]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

The following agreements to be captured in RAN1 specifications were made in [101-e-NR-5G_V2X_NRSL-Mode-2-01].

Agreements: - Tproc,0 is {1, 1, 2, [2 or 4]} physical slots for {15, 30, 60, 120} kHz sub-carrier spacing respectively Agreements: - Confirm that sensing window size parameter T0 is (pre)-configured between two values: 1100 ms and 100 ms Working assumption - Tproc,0 is 4 physical slots for 120 kHz sub-carrier spacing Agreements: - T3 = Tproc,1, where the UE is only required to include sensing information from time earlier than ‘m – T3 – Tproc,0’ for pre-emption and re-evaluation check at time ‘m – T3’ Working assumption: - Tproc,1 is 2 ms converted to physical slots+ 1 slot, i.e. {3, 5, 9, 17} for {15, 30, 60, 120} kHz sub-carrier spacing respectively

TP to TS 38.214

### 8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission:

-	the resource pool from which the resources are to be reported;

-	L1 priority, prioTX;

-	the remaining packet delay budget;

-	the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH;

-	optionally, the resource reservation interval, Prsvp_TX, in units of ms.

The following higher layer parameters affect this procedure:

-	t2min_SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter t2min_SelectionWindow for the given value of prioTX.

-	SL-ThresRSRP_pi_pj: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 0-1 and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX.

-	RSforSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1.

-	reservationPeriodAllowed

-	t0_SensingWindow: internal parameter T0 is defined as the number of slots corresponding to t0_SensingWindow ms.

The resource reservation interval, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX'.

Notation:

t0SL,t1SL,t2SL,... denotes the set of slots which can belong to a sidelink resource pool and is defined in [TBD].

The following steps are used:

1)	A candidate single-slot resource for transmission Rx,y is defined as a set of LsubCH contiguous sub-channels with sub-channel x+j in slot tySL where j=0,...,LsubCH-1. The UE shall assume that any set of LsubCH contiguous sub-channels included in the corresponding resource pool within the time interval [n+T1,n+T2] correspond to one candidate single-slot resource, where

-	selection of T1 is up to UE implementation under 0 ≤ T1≤ Tproc,1SL , where Tproc,1SL is defined in slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP;

-	if  T2min is shorter than the remaining packet delay budget (in slots) then T2 is up to UE implementation subject to  T2min ≤ T2 ≤ remaining packet budget (in slots); otherwise T2 is set to the remaining packet delay budget (in slots).

The total number of candidate single-slot resources is denoted by Mtotal.

2)	The sensing window is defined by the range of slots [n –T0,n–Tproc,0SL) where T0 is defined above and Tproc,10SL is defined in slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP. The UE shall monitor slots which can belong to a sidelink resource pool within the sensing window except for those in which its own transmissions occur. The UE shall perform the behaviour in the following steps based on PSCCH decoded and RSRP measured in these slots.

3)	The internal parameter Th(pi) is set to the corresponding value from higher layer parameter SL-ThresRSRP_pi_pj for  pj  equal to the given value of prioTX and each priority value pi.

4)	The set SA is initialized to the set of all the candidate single-slot resources.

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter reservationPeriodAllowed and a hypothetical SCI format 0-1 received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause [TBD] in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than  ThprioRX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots,  Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is FFS.

7)	If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4.

The UE shall report set SA to higher layers.

Table 8.1.4-1 Tproc,0SL depending on sub-carrier spacing

| μSL | Tproc,0SL [slots] |
| --- | --- |
| 0 | 1 |
| 1 | 1 |
| 2 | 2 |
| 3 | 4 |

Table 8.1.4-2 Tproc,1SL depending on sub-carrier spacing

| μSL | Tproc,1SL [slots] |
| --- | --- |
| 0 | 3 |
| 1 | 5 |
| 2 | 9 |
| 3 | 17 |

| Source | Comments |
| --- | --- |
| OPPO | The unit for Tproc,1SL and Tproc,1SL can be defined in the description similar to T0 as: - Tproc,0SL is defined as number of slots in Table 8.1.4-1 where μSL is the SCS configuration of the SL BWP - Tproc,1SL is defined as number of slots in Table 8.1.4-2 where μSL is the SCS configuration of the SL BWP Agree to capture the definition for T3=Tproc,1SL as part of thread #5 TP. FL: added “slots” in both text and tables, thanks |

- References

- R1-2003310	Remaining details of Resource Allocation Mode 2	Nokia, Nokia Shanghai Bell

- R1-2003379	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2003495	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2003549	Remaining issues in Mode-2	ZTE, Sanechips

- R1-2003559	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2003563	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2003613	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2003653	Remaining Issues on Resource Allocation in NR Sidelink Mode 2	ITRI

- R1-2003671	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2003703	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2003735	Remaining details of Mode-2 NR V2X sidelink design	Intel Corporation

- R1-2003807	Remaining details on mode-2 resource allocation	Futurewei

- R1-2003874	On Mode 2 for NR Sidelink	Samsung

- R1-2003991	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2004043	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2004074	Discussion on remaining open issue for mode 2	OPPO

- R1-2004171	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2004217	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2004295	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2004310	Remaining issues on resource allocation Mode 2	NEC

- R1-2004328	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2004385	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2004452	Sidelink Resource Allocation Mode 2	Qualcomm Incorporated

- R1-2004531	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2004544	Resource allocation Mode 2 for NR SL	Ericsson