3GPP TSG RAN WG1 Meeting #101-E						     R1- 200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (Intel Corporation)

Title:	TPs based on outcome of [101-e-NR-5G_V2X_NRSL-Mode-2-03]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

The following agreements to be captured in RAN1 specifications were made in [101-e-NR-5G_V2X_NRSL-Mode-2-03].

Agreements: - X% is is (pre-)configured per pool per L1 priority from a set of {20, 35, 50} %, - RSRP threshold adaptation triggering issue is not further discussed in Rel-16 Agreements: - Confirm the following working assumption from RAN1#100bis-e with "shall”: Working assumption: Agreements: - The UE should/shall indicate min(Nselected, N) first-in-time resources when setting the values of frequency resource assignment and time resource assignment in SCI format 0_1, where - Nselected is the number of resources selected by MAC within 32 slots (including the current one) - N is the maximum number of resources that can be signalled in one SCI - To discuss and conclude “should vs. shall” in RAN1#101. - To also add the above agreements (without change marks) to the RAN2 LS, indicating that the agreement is not intended to be in conflict with the corresponding QoS requirements. If RAN2 sees any issues, please inform RAN1 accordingly.

TP to 38.213 capturing SCI signalling of Nselected resources

In FL understanding, this is a signalling aspect and needs to be captured in 38.213, section 16.4 as follows:

## 16.4 UE procedure for transmitting PSCCH A UE can be provided a number of symbols in a resource pool, by timeResourcePSCCH, starting from a second symbol that is available for SL transmissions in a slot, and a number of PRBs in the resource pool, by frequencyResourcePSCCH, for a PSCCH transmission with a SCI format 0_1. A UE that transmits a PSCCH with SCI format 1-A using sidelink resource allocation mode 2 [6, TS 38.214] - sets the values of the Frequency resource assignment field and the Time resource assignment field as described in clause 8.1.5 in [6, TS 38.214] to indicate the N resources from the set Ry selected by higher layer as described in [11, TS 38.321] with N smallest slot indices yi such that y0≤yi≤y0+31 and yi<yi+1 for any i, where: - N = min(Nselected, sl-MaxNumPerReserve), where Nselected is the number of resources in the set Ry selected by higher layer as described in [11, TS 38.321] with slot indices yi such that y0≤yi≤y0+31 and yi<yi+1 for any i, and sl-MaxNumPerReserve is provided by higher layer. - Ry is a set of resources, each corresponding to LsubCH contiguous sub-channels and a slot in a set of slots {tySL}, where LsubCH is the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot. - t0SL,t1SL,t2SL,... denotes the set of slots which can belong to a sidelink resource pool and is defined in [TBD]. - y0 is the index of the slot in which the SCI format 1-A is transmitted.

TP to 38.214 capturing (pre-)configured X%

Current section 8.1.4 needs to be updated to capture the configurability of X% threshold.

### 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 In resource allocation mode 2, the higher layer can request the UE to determine a subset of resources from which the higher layer will select resources for PSSCH/PSCCH transmission. To trigger this procedure, in slot n, the higher layer provides the following parameters for this PSSCH/PSCCH transmission: - the resource pool from which the resources are to be reported; - L1 priority, prioTX; - the remaining packet delay budget; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - optionally, the resource reservation interval, Prsvp_TX, in units of ms. The following higher layer parameters affect this procedure: - t2min_SelectionWindow: internal parameter T2min is set to the corresponding value from higher layer parameter t2min_SelectionWindow for the given value of prioTX. - SL-ThresRSRP_pi_pj: this higher layer parameter provides an RSRP threshold for each combination pi, pj, where pi is the value of the priority field in a received SCI format 0-1 and pj is the priority of the transmission of the UE selecting resources; for a given invocation of this procedure, pj = prioTX. - RSforSensing selects if the UE uses the PSSCH-RSRP or PSCCH-RSRP measurement, as defined in clause 8.4.2.1. - reservationPeriodAllowed - t0_SensingWindow: internal parameter T0 is defined as the number of slots corresponding to t0_SensingWindow ms. - sl-xPercentage: internal parameter X for a given prioTX is defined as sl-xPercentage(prioTX) converted from percentage to ratio The resource reservation interval, Prsvp_TX, if provided, is converted from units of ms to units of logical slots, resulting in Prsvp_TX'. << UNCHANGED PARTS OMITTED >> 7) If the number of candidate single-slot resources remaining in the set SA is smaller than 0.2X⋅Mtotal, then Th(pi) is increased by 3 dB for each priority value Th(pi) and the procedure continues with step 4. The UE shall report set SA to higher layers.

- References

- R1-2003310	Remaining details of Resource Allocation Mode 2	Nokia, Nokia Shanghai Bell

- R1-2003379	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2003495	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2003549	Remaining issues in Mode-2	ZTE, Sanechips

- R1-2003559	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2003563	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2003613	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2003653	Remaining Issues on Resource Allocation in NR Sidelink Mode 2	ITRI

- R1-2003671	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2003703	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2003735	Remaining details of Mode-2 NR V2X sidelink design	Intel Corporation

- R1-2003807	Remaining details on mode-2 resource allocation	Futurewei

- R1-2003874	On Mode 2 for NR Sidelink	Samsung

- R1-2003991	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2004043	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2004074	Discussion on remaining open issue for mode 2	OPPO

- R1-2004171	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2004217	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2004295	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2004310	Remaining issues on resource allocation Mode 2	NEC

- R1-2004328	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2004385	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2004452	Sidelink Resource Allocation Mode 2	Qualcomm Incorporated

- R1-2004531	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2004544	Resource allocation Mode 2 for NR SL	Ericsson