3GPP TSG RAN WG1 Meeting #101-E						     R1- 200xxxx

e-Meeting, May 25th – June 5th, 2020

Source:	Moderator (Intel Corporation)

Title:	Discussion #4 [101-e-NR-5G_V2X_NRSL-Mode-2-04]

Agenda item:	7.2.4.2.2

Document for:	Discussion and Decision

Introduction

This contribution provides discussion on critical issues for the fourth thread [101-e-NR-5G_V2X_NRSL-Mode-2-04].

[101-e-NR-5G_V2X_NRSL-Mode-2-04] Email discussion/approval with respect to periodic reservation:

5a – Conversion of periodicity to logical slots of a resource pool, handling of UL-DL configurations, Tscal

New – How to capture L1 procedure of periodicity signalling in SCI

By 6/1, with potential TPs till 6/4 – Sergey (Intel)

Discussion

## Conversion of periodicity to logical slots of a resource pool, handling of UL-DL configurations, Tscal

It may be uncertain in specification, how the period value in ms for both TX and RX is converted to logical slots, taking also into account the UL-DL configurations. Note, in PHY structure AI, currently it is a working assumption, that slots with insufficient number of UL symbols in semi-static UL-DL configuration are excluded.

There are a few proposals leaning towards reusing procedures similar to LTE [3][16][14]. However, there may be opinions that there is no need for additional conversions.

Q1: How Prsvp_TX and Prsvp_RX measured in ms are converted to logical slot periods P'rsvp_TX and P'rsvp_RX respectively? E.g. is it required to introduce P'rsvp_TX and P'rsvp_RX as a function of Pstep which is determined from a semi-static UL-DL configuration?

| Source | Comments / Motivation |
| --- | --- |
| Intel | Main principle is to do all operations in physical domain i.e. in ms or physical slots and then convert to logical indexing using the floor function to find the nearest logical slot for transmission. Specific implementation of this is a matter of TP preparation. In anyway, we assume that LTE-V2X principle will be reused here. |
| Qualcomm | LTE principle is used, details can be discussed in TP. |
| Sharp | Reuse the function of Pstep as in LTE V2X. |
| Huawei/HiSilicon | LTE-V mechanism should be reused to avoid any confusion about the conversion. The conversion of periodicity to logical slots can be formulated as Prsvp_TX'=S×PrsvpTX/P , where P denotes the periodicity of DL-UL pattern in section 11.1 of TS 38.213, provided by higher layer parameter dl-UL-TransmissionPeriodicity, S denotes the number of configured slots for sidelink in the pattern, and PrsvpTX/P is equal to the number of DL-UL pattern occurrences within the physical interval PrsvpTX. |
| OPPO | Similar to Intel, to reuse the LTE mechanism is fine, but to use the ceil function instead of floor function, in order to avoid P'rsvp_TX or P'rsvp_RX equal zero. |

Currently Tscal is FFS in 214. Assuming the main purpose of this value is to increase the interval of the TX UE selection window to check for periodic collision in case of small reservation periods, what is the value of Tscal that can be adopted in NR?

Q2: How to define Tscal in 38.214, section 8.1.4?

- Option 1: Tscal=PDB

- Option 2: Tscal=100 ms

- Other options, please specify

| Source | Short answer | Comments |
| --- | --- | --- |
| Intel | Option 2 | In order to comply with LTE V2X procedure |
| Qualcomm | Other | PDB if packet is aperiodic, 100ms otherwise. |
| Sharp | Option 2 | Share same thought with Intel. |
| Huawei/HiSilicon | Option 1 | Since the T_scal corresponds the upper bound of resource selection window, which is 100 in LTE-V2X and PDB in NR-V2X, so T_scal should be equal to PDB. |
| OPPO | Other | Tscal=T2, since it is not necessary to take into consideration of reserved resources that are outside the selection window and anyway T2 can equal to remaining PDB based on UE selection. Furthermore, if Tscal=PDB all the time, then this may cause the Q value to be large and the selecting UE will exclude excessive resources, especially when Prsvp_RX is small. |

Finally, it is worth confirming whether Cresel=10*SL_RESOURCE_RESELECTION_COUNTER which is provided by higher layers, or it needs modification comparing to LTE.

Q3: Is it agreeable to reuse LTE rule and let Cresel=10*SL_RESOURCE_RESELECTION_COUNTER?

| Source | Short answer | Comments |
| --- | --- | --- |
| Intel | Agree | In order to comply with LTE V2X procedure. Only for semi-persistent processes in Mode-2 |
| Sharp | Agree | Share same thought with Intel. |
| Huawei/HiSilicon | Agree | Reuse LTE-V rule |
| OPPO | Agree | Reuse the LTE mechanism. BTW, isn’t this a RAN2 topic? |

## How to capture L1 procedure of periodicity signalling in SCI

There are a few places in 38.212 and 38.214 referring to the procedure how a UE sets/determines a period in SCI 0_1. However, there is no such clause currently in specification. In FL understanding, it can be implemented in 38.213, section 16.4.

There are two main aspects:

- How to ensure / capture that for periodic reservation enabled case, a UE can still set the period to 0, i.e. no periodic reservation

- If such a code-point in SCI always available, in which cases the period can be set to 0

Q4: Please indicate your preference how to ensure that a period value 0 corresponding to no periodic reservation is always present in SCI:

- Option 1:  A fixed codepoint in the “Resource reservation period” field of SCI 1-A, e.g. corresponding to 0. Other configured periods are provided by RRC and are indexed starting from 1.

- Option 2: A UE does not expect to be (pre-)configured with a set sl-MultiReserveResource not containing value of 0 ms

| Source | Short answer | Comments |
| --- | --- | --- |
| Intel | Option 2 is preferred | Straight forward approach. |
| Qualcomm | Option 2 |  |
| Panasonic | Option 2 |  |
| Sharp | Option 1 | We propose to reuse the LTE V2X method which uses a code point ‘0000’ to indicate no periodical reservation. |
| Huawei/HiSilicon | Option 2 | Option 2 is simple and straightforward |
| ASUSTeK | Option 2 |  |
| NTT DOCOMO | Option 2 |  |
| OPPO | Option 2 |  |

Q5: Is it agreeable to capture two different cases for setting “Resource reservation period” to 0 ms?

- The higher layer decides not to keep the resource for the transmission in the next period

- The higher layer does not provide a period value

| Source | Short answer | Comments |
| --- | --- | --- |
| Intel | Agree | We assume that “The higher layer decides not to keep the resource for the transmission in the next period” also covers the case when in a given period UE reserves dynamic resources for the same or another TB transmission |
| Qualcomm | Agree |  |
| Panasonic | Agree |  |
| Sharp | Not support | It seems in LTE V2X, there is always ‘X’ indicated by higher layers and does not include the 2nd case. The interpretation of ‘0000’ in TS36.213 is for the 1st case. |
| Huawei/HiSilicon | Agree | We think the first bullet is also related to the discussion in Mode2-02 Q5/Q6. For example, if “reselected resources should not be periodically reserved” is chosen for Q5/Q6 in Mode2-02, we assume this belongs to the 1st bullet, i.e., “The higher layer decides not to keep the resource for the transmission in the next period”. |
| ASUSTeK | Agree |  |
| NTT DOCOMO | Agree |  |
| OPPO | Agree |  |

- References

- R1-2003310	Remaining details of Resource Allocation Mode 2	Nokia, Nokia Shanghai Bell

- R1-2003379	Remaining issues on mode 2 resource allocation mechanism	vivo

- R1-2003495	Remaining details of sidelink resource allocation mode 2	Huawei, HiSilicon

- R1-2003549	Remaining issues in Mode-2	ZTE, Sanechips

- R1-2003559	Remaining Issues on Sidelink Mode 2 Resource Allocation	Panasonic Corporation

- R1-2003563	Discussion on resource allocation for Mode 2	LG Electronics

- R1-2003613	Remaining issues on Mode 2 resource allocation in NR V2X	CATT

- R1-2003653	Remaining Issues on Resource Allocation in NR Sidelink Mode 2	ITRI

- R1-2003671	Sidelink mode-2 resource allocation	MediaTek Inc.

- R1-2003703	Remaining issues for Mode 2 resource allocation in NR V2X	ASUSTeK

- R1-2003735	Remaining details of Mode-2 NR V2X sidelink design	Intel Corporation

- R1-2003807	Remaining details on mode-2 resource allocation	Futurewei

- R1-2003874	On Mode 2 for NR Sidelink	Samsung

- R1-2003991	Remaining issues in NR sidelink mode 2 resource allocation	Spreadtrum Communications

- R1-2004043	Remaining details on mode 2 resource allocation for NR V2X	Fujitsu

- R1-2004074	Discussion on remaining open issue for mode 2	OPPO

- R1-2004171	Resource allocation for NR sidelink Mode 2	TCL Communication Ltd.

- R1-2004217	Remaining Issues of Mode 2 Resource Allocation	Apple

- R1-2004295	Remaining Issues on NR Sidelink Mode 2 Resource Allocation	InterDigital, Inc.

- R1-2004310	Remaining issues on resource allocation Mode 2	NEC

- R1-2004328	Remaining issues on resource allocation mode 2 for NR sidelink	Sharp

- R1-2004385	Remaining issues on resource allocation mechanism mode 2	NTT DOCOMO, INC.

- R1-2004452	Sidelink Resource Allocation Mode 2	Qualcomm Incorporated

- R1-2004531	Remain details on mode-2 resource allocation for NR V2X	ITL

- R1-2004544	Resource allocation Mode 2 for NR SL	Ericsson